3GPP TSG-RAN WG3 #117bis-e	R3-225926

Online, 10. – 18. Oct 2022

Agenda Item:	14.4

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on Selective Activation (CB #MobilityEnh3_Others)

Document for:	Approval

# Introduction

CB: # MobilityEnh3_Others

- Discuss on NR-DC with selective activation, and focus on high level topics.

- Feasibility of simultaneous data transmission from the UPF to the group of nodes prepared for selective activation.

- SCG selective activation is prioritized in Rel-18?

- The source MN or the source SN can initiate the procedure for SCG selective activation?

- Supporting CPC/CPA configuration including intra-SN CPC configuration for the reduction of coordination and overhead?

- Other issues related with selective activation.

- Coexistence between Rel-17 CPAC and Rel-18 CPAC.

- Capture the agreements and open issues

# For the Chairman’s Notes

After the first round of the discussion following is proposed:

RAN3 will start the work on all three scenarios for Selective Activation: MN-initiated CPA, MN-initiated CPC and SN-initiated CPC.

RAN3 will work to enable direct early data forwarding in Selective Activation.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

WA: Some indication of Selective Activation is needed in the Addition Request with “reject” criticality to enable the target node to recognise the request (and react, if it does not support the feature). Pre-configuration of supporting neighbours via OAM is not expected to be needed.

Any optimisations to the signalling are FFS (more details are needed, contribution-driven). Proposals made so far:

- activation/deactivation of groups of PSCells prepared for Selective Activation

- parallel data transmission from the UPF to all prepared PSCells

For the 2nd round RAN3 will further discuss the scenarios for Selective activation, in particular:

- if RAN3 shall consider keeping prepared PSCell when DC operation is temporarily released;

- if RAN3 shall consider preparing more PSCells than can be configured for the UE;

- Are there scenarios where only indirect data forwarding will be allowed.

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion (1st round)

## Initiating node and general operation

In [5373], [5418] and [5770] the initiative to start Selective Activation is discussed:

- [5373] and [5770] propose to prioritise or focus exclusively on MN-initiated Selective Activation operation;

- [5418] on the other hand proposes to work on solution that would enable both, the MN- and SN-initiated Selective Activation operation.

It seem therefore, there is no doubt to work on MN-initiated Selective Activation operation. The question concerns SN-initiated operation. However, as observed in [5770], in case of MN-initiated operation, it may be both, CPC and CPA. We have therefore 3 types of Selective Activation operation:

- MN-initiated CPA

- MN-initiated CPC

- SN-initiated CPC

Question 1: Please, comment if any of the operations above should be postponed (or excluded) from RAN3 work.

| Company | Operation to be postponed | Possible comment |
| --- | --- | --- |
| Nokia | None | Postponing/deprioritising will likely mean excluding it from Rel.18. However, all of the operations, including SN-initiated CPC, are likely relevant. |
| CATT | None | We may support all the scenarios. But we need to treat them in different priority |
| Huawei | 3)? | It is better to focus on 1) and 2) first, and only discuss 3) if time allows. |
| Lenovo | upon RAN2? | They are all technically possible, however, it doesn’t seem a wise decision for RAN3 to dive into detailed procedures. We can keep an eye on RAN2 discussion during this meeting. Or, we can send an LS to check RAN2’s view or to trigger the scenario discussion there. |
| Qualcomm | None | All the above types of operations seem relevant. |
| ZTE | 1) | We wonder CPA, because when after first CPA execution and subsequent selective activation, the other candidate SN will be changed to CPC, which is different from MN/SN initiated CPC, especially for the delta SN configuration. More, we think we can coordinate with RAN2’s view. |
| Intel | None from R3 point of view | But tend to agree with Lenovo. We think RAN2 needs to progress before we dive into details for this subsequent CPAC. |
| China Telecom | None | All the scenarios should be supported. |
| NEC | None | All are relevant. But agree with Lenovo that we may need to align with RAN2 if RAN2 will prioritize. In [5373] we did not propose to exclude any scenario, but we discuss who to decide to apply the selective activation, and both MN and S-SN is possible while not persuade to let T-SN to take the decision. |
| NTT DOCOMO | None | Agree with the prioritization as 1) – 3). |
| Google | None | The RAN2 progress should be monitored |
| Samsung | None | But we suggest 1) and 2) have higher priority. |
| E/// | None | May down-prioritize 1). Need to sync with RAN2’s progress. |

In [5423], [5713] and [5770], it is discussed if the solution for Selective Activation shall enable keeping the last serving PSCell as prepared cell after UE changes to another PSCell.

Question 2: Please, comment if you agree that the last serving (source) PSCell should be possible to be kept as prepared after the UE changes the serving PSCell?

| Company | Yes / No | Possible comment |
| --- | --- | --- |
| Nokia | Yes | The last serving (source) cell/node is a close neighbout after UE changes the cell, so keeping it prepared is relevant from the Selective Activation perspective. |
| CATT |  | We may study the options: keep it or release/add by MN, which option is more efficient and signalling saving if we keep it, the configuration should be updated and set condition for it. |
| Huawei | Yes | It is possible for UE to switch back to the source PSCell when executing subsequent PSCell change. |
| Lenovo | Yes, but upon RAN2 | Although we have the same understanding, it is essentially upon RAN2 conclusion since it will impact UE behaviour as well. |
| Qualcomm | Yes | Agree with Nokia’s comment. |
| ZTE | Yes | The last serving PSCell can be the new candidate PSCell, but the initiating node can finally decide it, i.e., keep it or release it. |
| Intel |  | Should this be decided by RAN2? |
| China Telecom | Yes | UE may handover back to the source PSCell when performing subsequent PSCell change, so configuring the source PSCell as a new candidate PSCell for the UE is reasonable. |
| NEC | Yes |  |
| NTT DOCOMO | Yes | There can be some cases in which keeping source PSCell as candidate target PSCell in subsequent CPC is helpful. |
| Google | Yes |  |
| Samsung | Yes | Same view as Nokia |
| E/// | Yes |  |

## Coexistence

At the last meeting, RAN3 made following agreement:

From RAN3 point of view, Rel-16/Rel-17 CPAC procedures are considered as start point for the Rel-18 work.

This hints, the Rel.16/17 procedures will be enhanced to support Selective Activation operation and thus the initiation will be action of X2/Xn signalling. However, in [5367], it is suggested initiation of Selective Activation operation may require pre-configuration of the information if the possible target SN supports it.

Question 3: Please, comment if you expect that OAM support will be necessary to initiate Selective Activation operation? Do you have any other comments concerning OAM support?

| Company | Comment |
| --- | --- |
| Nokia | No, we don’t think that initiation of the Selective Activation operation must rely on OAM. Similarly like with other features, Selective Activation will likely require adding new IEs, which, based on criticalities, will enable a node to handle a scenario where the target node does not support Rel.18 Selective Activation. |
| CATT | We don’t think the OAM involvement is needed |
| Huawei | No. An indication of selective activation over Xn/X2 signalling is sufficient to aid the target SN to decide whether to support the new feature. |
| Lenovo | Same view as Nokia. |
| Qualcomm | No, we don’t think OAM involvement is needed. |
| ZTE | In our contribution [5367], we ask a question, can nodes only supporting R17 CPAC and nodes supporting R18 CPAC coexist during the selective activation procedure? For instance, in case of SN initiated CPC, the initiating node (i.e., source SN) and MN support R18 CPAC, some candidate SN also support R18CPAC, but some candidate SN does not support R18 CPAC. Then after CPC execution, during selective activation period, can we release those candidate SN not supporting R18 CPAC and keep other candidate SN supporting R18 CPAC? |
| Intel | We think that, if RAN2 progresses and agrees to make this selective activation based on the explicit configuration from NW (i.e. the UE is explicitly configured to perform selective activation), then it may be better to go with having an explicit indicator for “selective activation” during SN ADD procedure even though there is nothing to enhance in terms of signalling compared to Rel-17 CPAC. With the criticality, non-supporting candidate SNs can be filtered out without relying on OAM. |
| China Telecom | Same view as Nokia. |
| NEC | As usual, OAM configuration is always the default way. Whether we need explicit signalling to indicate whether supporting the Selective Activation operation, may be this is not needed, as we will likely have new IE at least to indicate to keep the prepared CG configuration. |
| NTT DOCOMO | Same view as CATT. |
| Google | Share the same view as NEC. |
| Samsung | Same view as Nokia. |
| E/// | No. This is about node capability, which can be handled normally as other features. Any pre-configuration is leaving for implementation. |

Also, in [5367] it is clearly declared that Selective Activation operation can coexist with classic CPAC operations.

However, there are two variants of coexistence:

- Using different procedures (e.g. Rel.17 CPAC and Rel.18 SelAct) in the RAN for different UEs;

- Using different procedures (e.g. Rel.17 CPAC and Rel.18 SelAct) for the same UE at the same time.

Question 4: Please, comment if you see any issues with the coexistence of Selective Activation operations with other features within RAN for different UEs or at the same time for the same UE.

| Company | Comment |
| --- | --- |
| Nokia | For (1), we don’t see any issue with using any Rel.17 feature together with the Selective activation within RAN for different UEs. For (2), possible handing of Rel.17 CPAC and Rel.18 Selective Activation within the same UE at the same time should be discussed rather in RAN2. |
| CATT | For 1), 2), the UE should be informed which version is used. But it is RAN2 scope |
| Huawei | Agree with Nokia. |
| Lenovo | From [5367], it seems “coexist” means a candidate SN in a R17 CPAC can configure R18 SelAct during R17 CPAC preparation? If the understanding is correct, we think it should be discussed after CHO+CPAC discussion and “CPC/CPA configuration including intra-SN CPC configuration” discussion. |
| Qualcomm | For 1), we agree with Nokia. For 2), we agree that RAN2 involvement is needed. It is not clear why the network would provide the UE with both the configurations at the same time. |
| ZTE | Agree with CATT, it is RAN2’s issue for UE behaviour. We shall consider to allocate the new data forwarding address during selective activation period. |
| Intel | Agree with Nokia. |
| China Telecom | Agree with Nokia. |
| NEC | We understand 5367 is talking about the peer nodes that some support Rel-17 CPAC but some support Rel-17 CPAC+Rel-18 Selective Activation, whether the Selective Activation initiating node (e.g. MN) will need to run different procedures to handle differently. If some peer nodes do not support Rel-18 Selective Activation but support only Rel-17 CPAC, we believe it has no other choice but only to handle differently. And whether can use different procedure for different UEs, since this control is per UE context, it should have no issue using different procedures for different UEs. (May be we misunderstand the question.) |
| NTT DOCOMO | Agree with Nokia. |
| Google | Agree with Nokia. |
| Samsung | Agree with Nokia. |
| E/// | Agree with Nokia. |

## Signalling effort

Two papers address possible signalling effort related to the Selective Activation operation. In [5713] and in [5392] some optimisations are considered that help managing large groups of possibly prepared PSCells:

- In [5713], it is proposed to enable a mechanism that will enable activating or deactivating groups of prepared PSCells, so that only a limited part of them is active for the UE, while frequent reconfigurations of the set of prepared PSCells does not need to be updated.

- In [5392] early data forwarding towards prepared target nodes is considered and the effort of frequent reconfiguration of the data forwarding (in case serving SN changes more often than the set of prepared target SNs).

Even though the objectives are quite different, both papers consider that the group of prepared PSCells (or SNs) may be large and stable, i.e. SN nodes do not have to be added or removed from the set often. In [5392], also an alternative is considered: UE staying long in a given PSCell, while other SNs/PSCells are configured just temporarily, e.g. for the moment of cell border crossing.

Question 5: Please, comment on the Selective Activation scenario: do you expect Selective Activation to be used for UEs changing PSCells frequently among large group of prepared SNs? Also, do you expect that direct early data forwarding should be enabled in Selective Activation?

| Company | Comment |
| --- | --- |
| Nokia | Yes, this is one of relevant scenarios for Selective Activation, if not the most relevant one. To make an illustration, we imagine Selective Activation to be used in a large area, covered by many SNs, all of which remain prepared (and thus the group of possibly prepared nodes remain about-stable), which the UE moves among them. |
| CATT | Yes The direct data forwarding should be supported |
| Huawei | Selective activation intends to apply to the frequent PSCell change case scenario, but we are not sure if there will be large group of prepared SNs, there could be multiple prepared SCGs, but may not large number of prepared SNs, e.g. in Rel-17 there are at maximum 8 prepared SCGs for a UE. Early data forwarding is the main gain of selective activation. Without early data forwarding, selective activation is more or less meaningless, direct data forwarding or indirect data forwarding, could be further discussed case by case. |
| Lenovo | Partially, we agree UE may change PSCells frequently, but whether it is among large group of prepared SNs is unclear. How many SNs shall we expect and how large the area is? |
| Qualcomm | We agree that the scenario with UE changing among multiple prepared PSCells/SNs while it is moving is quite relevant. However, as Huawei mentions, there has to be a maximum number of PSCells configured for a UE, similar as Rel-16/Rel-17 CPA/CPC. Early data forwarding should be enabled, and direct data forwarding is also fine. |
| ZTE | Yes, in our view, the selective activation is use for this changing PSCell frequent scenario. The direct early data forwarding shall be used after R17 CPAC execution, if direct early data forwarding is already used. But if the indirect early data forwarding is used in the previous CPAC preparation, then direct early data forwarding cannot be used during selective activation period. |
| Intel | Agree with Nokia and Huawei. |
| China Telecom | We agree that the selective activation is use for this changing PSCell frequent scenario, but we think the maximum number of candidate SNs should be discussed. Since the larger the number of candidates, the more complex the network, we prefer to keep the prepared SNs in a reasonable range. For data forwarding used in frequent PSCell changes, we think the directed early data forwarding should be supported. |
| NEC | We understand that motivation of such selective activation is when the UE move here and there quite frequently then to keep the prepared configuration even if the UE has selected other cell, the purpose is to reduce signalling when again need to do the CPAC. So the answer should be Yes. |
| NTT DOCOMO | Yes. It seems that prepared cells activation/deactivation could avoid both a frequent candidate cell reconfiguration and a waste of resources in prepared cells, and this scenario (i.e. so many candidates configured) could also be possible. Agree with Huawei about early data forwarding. |
| Google | Yes and direct data forwarding can be supported. |
| Samsung | The direct early data forwarding should be supported. |
| E/// | Early data forwarding should be supported. Whether indirect or direct needs further discussion. |

Since [5392] assumes early data transmission, it is also worthwhile to confirm that it should be enabled for Selective Activation.

Question 6: Please, comment, if direct early data forwarding should be enabled in Selective Activation?

| Company | Yes / No | If the answer is ‘no’, please, explain |
| --- | --- | --- |
| Nokia | Yes |  |
| CATT | Yes |  |
| Huawei | Yes? | Early data forwarding should be enabled, but whether direct or indirect data forwarding could be further discussed case by case. |
| Lenovo | Yes, in principle | In principle yes, otherwise the service continuity will suffer. On the other hand, the complexity of possible solution and scenario needs careful assessment. |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Intel | In principle, Yes |  |
| China Telecom | Yes |  |
| NEC | Yes |  |
| NTT DOCOMO | Yes | Same view as Huawei. |
| Google | Yes |  |
| Samsung | Yes |  |
| E/// | Yes | Agree with Huawei. |

If the scenario in Q6 is likely, RAN3 shall consider if the signalling optimisation solution proposed in [5713] is worth further work.

Question 7: Please, comment if RAN3 shall work further on the signalling optimisation related to the activation/deactivation of groups of PSCells prepared for Selective Activation.

| Company | Yes / No | If the answer is ‘no’, please, explain |
| --- | --- | --- |
| Nokia | Yes, but… | We are fine to study the proposed solution further, but at this stage, it does not seem to realise any promised optimisations: activation/deactivation of groups of prepared cells may be as burdensome as preparing/releasing some targets. But we are fine to evaluate it further. |
| CATT |  | We may have more study whether the group concept is introduced. |
| Huawei | Yes | Agree with Nokia. Not at this point but for later. |
| Lenovo | Yes, after scenarios become clear |  |
| Qualcomm |  | At this stage we can probably focus on how the basic procedure looks like and discuss signalling optimizations later. |
| ZTE | Not sure | We appreciate the motivation from the contribution [5713]. However, because it shall introduce activation/deactivation group besides candidate group, so, I wonder if it will bring additional group change between the two groups, and it maybe ask RAN2 to have different RRC signalling for the two groups. |
| Intel |  | We prefer to wait for RAN2 to progress more before delving into solutions for optimizations. |
| China Telecom |  | Similar view with QC, we should focus on the basic procedures first and then evaluate whether optimization is needed. |
| NEC |  | Fine to study to see first any benefit. |
| NTT DOCOMO | Yes | Agree with Huawei. |
| Google |  | Agree with Intel. |
| Samsung | Yes | Activation and deactivation of the candidate PSCells/CGs is a solution to reduce the signalling overhead for pre-configured candidate cells change with the mobility of UE, especially with the use of FR2. |
| E/// | Yes with comments | RAN2 is going to figure out the scenarios first, we are fine to discuss some basic signalings, but prefer later. |

Then, similarly, if the scenario in Q6 is likely and direct early data forwarding possible, RAN3 may consider optimisations that will help avoiding frequent path switch at the AMF/UPF and frequent switching data forwarding source. This is, of course, needed only assuming that direct early forwarding is to be used.

Question 8: Please, comment if RAN3 shall analyse the solution for simultaneous data transmission from the UPF to all prepared SNs to avoid frequent path switch and switching of the forwarding source?

| Company | Yes / No | If the answer is ‘no’, please, explain |
| --- | --- | --- |
| Nokia | Yes | Answering ‘Yes’ does not mean, of course, that the solution is to be enabled. However, it should be analysed if it is feasible and if it offers benefits as compared to a classic direct early data forwarding. |
| CATT | No | We don’t think the SA should be involved |
| Huawei |  | We are open to evaluate it, but probably this solution is not the preferred one. |
| Lenovo |  | Not if the RAN node based solution does not work or is way too complicated. |
| Qualcomm |  | We are also open to evaluate this proposal. |
| ZTE | No | The data transmission from UPF to all of prepared SNs, then the duplicated data transmission will be happen. It is not good solution because duplication data transmission at NG interface. |
| Intel |  | We prefer to wait for RAN2 to progress more before delving into solutions for optimizations. |
| China Telecom | Prefer No | We think data forwarding optimization should be studied in RAN3, however the solution for simultaneous data transmission from the UPF to all prepared SNs will increase NG transmission load and is not optimal. |
| NEC |  | Open to discuss but involving UPF to duplicate data, may be not an easy thing to do. |
| NTT DOCOMO | Yes | Agree with Nokia. |
| Google |  | Agree with Intel. |
| Samsung |  | We are open to evaluate it. |
| E/// | No | This has been raised as bicasting from CN in Rel-16, which was not selected by SA2 as a possible solution. We don’t see much difference in this case. |

# Conclusion, Recommendations [if needed]

If needed

# References

- [5367]	R3-225367, Disscussion on NR-DC with selective activation (ZTE)

- [5373]	R3-225373, Decision on selective activation of the cell groups (NEC)

- [5392]	R3-225392, Data forwarding considerations for selective activation (Nokia, Nokia Shanghai Bell)

- [5418]	R3-225418, Configuration and activation of multiple cell groups in NR-DC (Qualcomm Incorporated)

- [5423]	R3-225423, Consideration on support of selective activation of cell groups (China Telecommunication)

- [5496]	R3-225496, (TP for TS 38.473) On SCG selective activation (Lenovo)

- [5649]	R3-225649, Discussion of selective activation (NTT DOCOMO INC.)

- [5665]	R3-225665, (TPs to TS 37.340, TS 38.423 BL CRs) Consideration on selective activation of SCGs (Huawei)

- [5713]	R3-225713, Considerations on selective activation of the cell groups (Samsung)

- [5745]	R3-225745, Considerations on NR-DC with Selective Activation (vivo)

- [5770]	R3-225770, Discussion on NR-DC with selective activation of the cell groups (CATT)