3GPP TSG-RAN WG3 Meeting #117bis-e	R3-226024

# E-meeting, 10 – 18 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0260 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of L1/L2 inter-cell mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2 |  |  |  |  |  | Date: |  |  | 2022-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following progress was made at RAN3 #117-e meeting. They should be captured in the BLCR to trace the research progress. Both intra- DU and intra-CU inter-DU scenarios are supported for L1/L2 mobility. RAN3 will study the signaling impacts on below use cases following to RAN2 prioritization: - Stand alone - Carrier Aggregation (Change of PCell) - NR-DC (Change of PCell at MN, Change of PScell at SN) RAN3 will aim for a single solution for network signaling design on L1/L2 based inter-cell mobility to support all agreed scenarios. The details of solution are FFS. WA: For intra-DU L1/L2 mobility, the existing F1AP procedure (e.g., F1AP UE CONTEXT MODIFICATION) is reused for handover configuration for inter-cell mobility. RAN3 focuses on the network-controlled procedure for L1/L2 based inter-cell mobility. The gNB-CU initiates the L1/L2 mobility configuration procedure. FFS on whether gNB-DU can also initiate the L1/L2 mobility configuration procedure. The configuration of candidate target cell(s) for L1/L2 mobility is initiated by the gNB-CU. Details are FFS. WA: RAN3 assumes that the UE sends the L1 measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell. All details are up to RAN1 and RAN2 discussion. FFS on how the gNB/gNB-DU detects the UE access and whether there is an F1 impact. For intra-DU L1/L2 handover, whether and how to release the source cell/prepared cells’ resources in the gNB DU is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The message flow for intra-DU L1/L2 handover procedure is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-DU L1/L2 handover is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------- the start of change -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU L1/L2 based inter-cell Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based inter-cell mobility. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based inter-cell mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 based inter-cell Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- WA: The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cells. FFS: whether parallel UE Context Modification procedure should be used rather than a candidate list in a single message.

- WA: If the request for configuring L1/L2 inter-cell mobility is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the L1/L2 inter-cell mobility command to the UE.

- FFS: How the gNB-DU detects the UE access to the target cell is up to RAN2.

- The gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message. Th target cell ID is included.

- The gNB-CU may send the UE Context Modification message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

--------------------------------------------------------- the end of change -------------------------------------------------------------