3GPP TSG-RAN WG3 #119bis-e	R3-231264

Online, 17 April – 26 April 2023

Agenda Item:	26.2

Source: 	Qualcomm Incorporated

Title: 		Discussion on Timing Synchronization Status and Reporting

Document for:	Discussion/Decision

# Introduction

The Timing Resiliency and uRLLC WI RP-230754 was approved in the RAN plenary RP#99. The WI has three objectives. In this paper we discuss Objective 1 as shown below.

RP-230754 1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

# Discussion

## Clock Quality Reporting from RAN to AMF

The Objective 1 addresses the key issue #1 on 5GS network timing synchronization status and reporting.

d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

AMF can subscribe to the clock quality information from RAN. If RAN supports reporting of the clock quality information, then RAN may provide clock quality information to AMF as shown below.

To support subscription and notification of Clock Quality information, the existing NGAP messages cannot be reused as there are no relevant messages supporting similar functionality. Hence the proposal is to introduce new Class 1 message over NGAP for AMF to subscribe to the clock quality information. In the subscription message AMF shall request for the required clock quality parameters. OAM may configure the thresholds for each timing synchronization status attribute for clock quality reporting. When the network timing synchronization status exceeds the threshold (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the RAN notifies the AMF.

Since the time synchronisation is at node level, gNB-CU shall propagate the subscription request from AMF to gNB-DU and gNB-CU-UP. gNB-CU-CP shall collect the clock quality information from gNB-DU and gNB-CU-UP and send the clock quality information to AMF.

Proposal 1: A new Class 1 message shall be introduced over NGAP for AMF to subscribe to clock quality information from gNB. The class 1 message shall contain the clock quality parameters needed. Corresponding new E1AP and F1AP Class 1 message shall be introduced to fetch the clock quality parameters from gNB-CU-UP and gNB-DU. The parameters needed for clock quality reporting control information are FFS

If the gNB supports the clock quality reporting feature and the requested clock quality parameters are available, gNB shall respond back to AMF with the supported clock quality parameters in the Class 1 Response message. If the clock quality reporting feature is not supported or none of the requested clock quality parameters are available in gNB, then gNB shall respond back to the AMF with Class 1 failure message.

Proposal 2: Based on the gNB capability, gNB may respond to the class 1 message with clock quality parameters supported in response message. If none of the requested clock quality parameters are supported, then gNB can respond to the AMF with failure message.

Once the subscription is successfully accepted by the gNB, it shall start monitoring the clock quality based on the threshold values configured by OAM for each clock quality parameter for reporting. When the threshold is met or the threshold is exceeded, then gNB sends notification to the AMF with the updated clock quality parameters. A new Class 2 message shall be introduced to notify the AMF for the change in clock quality.

Proposal 3: A new class 2 message shall be introduced over NGAP for gNB to report the change in clock quality information to AMF upon the threshold conditions met or exceeded. Corresponding new E1AP and F1AP Class 2 message shall be introduced to report the change in clock quality information from gNB-CU-UP and gNB-DU.

## Clock Quality Reporting from RAN to UE

a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3.

A time sensitive UE may subscribe via NAS for TSS information. AMF may provide the UE’s TSS subscription information during the Initial UE context setup attach procedure or in the subsequent UE context modification procedure to gNB.

Proposal 4: AMF provides clock quality reporting control information for the UE in the NGAP Initial Context Setup Request and/or NGAP UE Context Modification Request in Time Synchronisation Assistance Information IE to gNB. The parameters needed for clock quality reporting control information are FFS.

During Xn Handover, AMF can also provide the TSS subscription to the target NG-RAN in Path Switch Request message.

Proposal 5: AMF provides clock quality reporting control information for the UE in the NGAP Path Switch Request Acknowledge message in Time Synchronisation Assistance Information IE to the target NG-RAN during Handover. The parameters needed for clock quality reporting control information are FFS

In the TSS subscription information of the UE, sent to NG-RAN, AMF can indicate the clock quality detail. The clock quality detail indicates, if UE is subscribed to receive the clock quality metrics or acceptable/not acceptable indication. In the later case, AMF shall send the corresponding range of values of clock quality metrics (like an accuracy range or multiple lock states), for RAN to provide acceptable/not acceptable indication.

Proposal 6: AMF indicates to the gNB in the clock quality reporting control information for the UE, the clock quality detail level as clock quality metrics or acceptable/not acceptable indication.

Based on the clock quality reporting control information for the UE, gNB shall provide the clock quality information to the UE via RRC. To save UE’s power and to avoid frequent monitoring of clock quality information by the UE, OAM may group the few cells belonging to the same gNB which share common clock characteristics under the same Event ID.

Proposal 7: Cells within the single gNB sharing the common clock characteristics shall be grouped together by OAM to indicate common clock quality information to the UE via Event ID.

# Summary

Based on the discussion, we have following observations and proposals:

Proposal 1: A new Class 1 message shall be introduced over NGAP for AMF to subscribe to clock quality information from gNB. The class 1 message shall contain the clock quality parameters needed. Corresponding new E1AP and F1AP Class 1 message shall be introduced to fetch the clock quality parameters from gNB-CU-UP and gNB-DU. The parameters needed for clock quality reporting control information are FFS

Proposal 2: Based on the gNB capability, gNB may respond to the class 1 message with clock quality parameters supported in response message. If none of the requested clock quality parameters are supported, then gNB can respond to the AMF with failure message.

Proposal 3: A new class 2 message shall be introduced over NGAP for gNB to report the change in clock quality information to AMF upon the threshold conditions met or exceeded. Corresponding new E1AP and F1AP Class 2 message shall be introduced to report the change in clock quality information from gNB-CU-UP and gNB-DU.

Proposal 4: AMF provides clock quality reporting control information for the UE in the NGAP Initial Context Setup Request and/or NGAP UE Context Modification Request in Time Synchronisation Assistance Information IE to gNB. The parameters needed for clock quality reporting control information are FFS.

Proposal 5: AMF provides clock quality reporting control information for the UE in the NGAP Path Switch Request Acknowledge message in Time Synchronisation Assistance Information IE to the target NG-RAN during Handover. The parameters needed for clock quality reporting control information are FFS

Proposal 6: AMF indicates to the gNB in the clock quality reporting control information for the UE, the clock quality detail level as clock quality metrics or acceptable/not acceptable indication.

Proposal 7: Cells within the single gNB sharing the common clock characteristics shall be grouped together by OAM to indicate common clock quality information to the UE via Event ID.