3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231482

Online, 17th – 26th April, 2023

Title: 	Discussion on UE positioning and additional ULI for VMR

Source: 	Huawei

Agenda item:	13.1

Document Type:	Discussion

1. Introduction

In the RAN3 #118 meeting, RAN3 received an LS of VMR (mobile IAB) from SA2 [1], which informed some progress they have reached and asked questions for point #1, #2, #6, and #7. The questions are listed as follows:

- For point#1 (regarding KI#1), SA2 will work to ensure that the MBSR is able to establish a secure and trusted connection with OAM server in a serving PLMN. SA2 would like to ask RAN3 to provide information on additional parameters for mobile IAB if RAN3 has identified any.

- For point#2 (regarding KI#3), SA2 has concluded the study from SA2 perspective (as in clause 8.3 of TR 23.700-05v1.2.0). Corresponding system impacts of supporting dynamic TAC or static TAC were documented in clause 6.16.4 and 6.17.4. SA2 will align the normative specification of the work item based on RAN 2/3 feedback.

- For point#6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe.

- For point#7 (regarding KI#6), SA2 would like to clarify that additional information besides existing ULI from donor-gNB (as defined in Rel-17) is needed, so that the 5GC can understand that the existing ULI cannot be used directly. This is crucial for the support of services that rely on the cell ID to infer the UE locations, e.g. emergency services. Therefore, SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution.

RAN3 replied an LS to SA2 in the last meeting as follows [2]:

- For point#1 (regarding KI#1): RAN3 is still discussing procedures and configurations for mobile IAB and has so far not identified any new OAM configuration parameters. RAN3 will provide information on additional parameters to be configured on the mobile IAB by OAM, if any, when these discussions have been finalized.

- For point#2 (regarding KI#3): Regarding TAC, RAN3 has agreed:

Capture on stage 2 that the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. It needs to be further discussed how the mobile IAB-DU’s TAC/RANAC is changed and what Stage 3 impacts are (if any).

RAN3 will inform SA2 on further progress regarding TAC for mobile IAB, if any.

- For point#6 (regarding KI#5): RAN3 needs to conduct further discussions to converge on a solution. RAN3 will inform SA2 for any progress.

- For point#7 (regarding KI#6): RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18.

This contribution focuses on the remaining issues of point#6 and point#7.

2. Discussion

2.1 For point#6 question

1) UE ID related issue

In this issue, if LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB, the UE ID is supposed to be transmitted in both NRPPa and F1AP, since the NRPPa procedures has “mirror” signalling in F1 interface.

According to SA2, the UE ID can be the GPSI of IAB-MT. The UE’s GPSI is not visible by RAN node in previous releases, so we are not sure whether it has any security issue (e.g. privacy exposure) to allow the UE GPSI being carried in F1AP signalling, better to check with SA3 first.

5.9.8	Generic Public Subscription Identifier

Generic Public Subscription Identifier (GPSI) is needed for addressing a 3GPP subscription in different data networks outside of the 3GPP system. The 3GPP system stores within the subscription data the association between the GPSI and the corresponding SUPI.

GPSIs are public identifiers used both inside and outside of the 3GPP system.

The GPSI is either an MSISDN or an External Identifier, see TS 23.003 [19]. If MSISDN is included in the subscription data, it shall be possible that the same MSISDN value is supported in both 5GS and EPS.

NOTE:	There is no implied 1-to-1 relationship between GPSI and SUPI.

Besides, according to TS23.501, the GPSI seems not mandatory to be assigned to a UE because it is only “needed” for “addressing a 3GPP subscription in different data networks outside of the 3GPP system”, especially when the UE is an IAB-MT which may not have any requirement of communication with a data network outside 3GPP.

Using SUPI as the UE ID can be an alternative, but introducing SUPI in RAN signalling may also create privacy issues, and should be check with SA3 group also.

Based on such consideration, we suggest to further discuss on whether to use GPSI or SUPI as the UE ID of the MBSR. It may be further discussed pending confirmation from SA3.

Proposal 1：NRPPa and F1AP to include UE ID of IAB-MT. FFS on whether to use GPSI or SUPI. The potential privacy issue of introducing the GPSI/SUPI in RAN signaling should further check with SA3.

2) Mobile TRP type indication issue

According to SA2’s CR for TS 23.273 in S2-2301478 [3], TRP information exchange is performed between LMF and gNB-CU, and between gNB-CU and gNB-DU. In the TRP information, the TRP type is supposed to be indicated as the “mobile TRP”. Although the mobile TRP type may be somehow conducted implicitly (e.g. according to the consecutive TRP location report in a period), it will be clearer to adopt explicit indication, i.e., add a new value “mobile trp” in the enumerated TRP Type IE. The mobile TRP is an inherent type more than a current status, and a “mobile TRP” may not move at some time. Therefore, we propose:

Proposal 2: NRPPa and F1AP to include explicit mobile TRP indication in the IE.

3) Velocity and timestamp issue

The velocity and timestamp are related to the Option 1 in the section of “5GC-MT-LR procedure involving Mobile Base Station Relay” proposed in S2-2301478. As described in step 5-7 of the CR, SA2 has agreed to obtain the geo-coordinates of the MBSR, the velocity and the time for obtaining them via NRPPa/F1AP TRP Information Exchange procedure. The figure of 5GC-MT-LR procedure involving Mobile Base Station Relay in SA2 CR is posted as follows:

------------------------------------Start of excerpt from S2-2301478------------------------------------

Figure 1: 5GC-MT-LR procedure involving Mobile Base Station Relay

- [Optional] The location services client sends a request to the GMLC for a location for the target UE identified by an GPSI or an SUPI. The UE may or may not be served by a MBSR.

- The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF serving the target UE to request the current location of the target UE. The location procedure may be also triggered at the AMF for a 5GC-NI-LR as at step 1 in clause 6.10.1, or an event report sent by the UE for a deferred 5GC-MT-LR for periodic or triggered location as at step 25 in clause 6.3.1.

- The 5GC-MT-LR procedure as specified in clause 6.1.1 step 5-8 or clause 6.1.2 step 6-12 are performed with the difference that the LMF selection also consider LMF capability supporting MBSR involvement. If the AMF is aware that the target UE is served by a MBSR, it would select the LMF that can support the MBSR handling.

- NOTE 1: The step 4 to 11 may happen as part of step 3.LMF derives if any MBSR(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 3. The AMF serving the target UE may indicate that the serving cell is an MBSR (if applicable). When the MBSR was integrated as a TRP (IAB-DU) with a gNB, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MBSR and/or the UE-ID (GPSI) associated with MBSR. As the MBSR can be mobile the LMF may need to determine an updated location of the MBSR by either performing step 5-7 (option 1) or performing step 8-10 (option 2) if option 1 is not feasible. If several MBSRs were derived, then step 5-7 or step 8-10 are be performed for each MBSR.

Editor's note: The trigger of LMF to initiate the TRP information exchange needs further discussion for the case of MBSR’s inter-CU mobility.

Editor's note: allowing the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB needs to be coordinated with RAN WGs.

- [Conditional] The LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MBSR(s) by directing the TRP information exchange messages to the gNB associated with the Cell ID(s), for example, by setting the NRPPa TRP Information Type Item IE to "geo-coordinates". Donor-CU further send a F1 TRP INFORMATION REQUEST message to any MBSR IAB-DU.

- [Conditional] The MBSR IAB-UE initiates a 5GC-MO-LR procedure (as defined in clause 6.2) to get its location information. The MBSR IAB-UE's location info is used by the co-located MBSR IAB-DU to determine the updated location for its TRP.

- [Conditional] The MBSR IAB-DU report its updated TRP location, e.g. TRP's geo-coordinate, velocity and the time for obtaining them, to the Donor-CU, which is further forwarded to LMF.

……

------------------------------------End of excerpt from S2-2301478------------------------------------

The idea for Option 1 is described as follows: Firstly the 5GC-MT-LR procedure is performed for the UE positioning (step 1~3), and then, considering the MBSR is used as a TRP for the UE positioning (step 4), the LMF shall acquire the current location, velocity of the MBSR and the timestamp, via the TRP information exchange procedure (step 5, step 7), for further calculating the UE position. Wherein the current location for the MBSR is acquired by itself via the 5GC-MO-LR procedure for the MBSR IAB-UE, i.e., the mIAB-MT (step 6).

For the Option 1’s solution, there may be some issues which impact the feasibility of this solution and worth to be further discussed:

- In the DU migration case: When the LMF needs to perform step 5, the F1 terminating CU of the MBSR may changes. For example, when performing UE positioning in step 3, for example, the mIAB-DU’s F1 terminating CU is CU1, but when the LMF attempts to initiate TRP information exchange procedure in step 5, the mIAB-DU’s F1 terminating CU may change to CU2. If the LMF is unaware the change of F1 terminating CU, it will fail to find the new F1 terminating CU for the TRP information exchange.

- In the partial migration case: Which LMF is responsible for the UE (served by MBSR) positioning? Not sure it is still the original LMF (connects to the F1 terminating CU) to be used for the UE positioning, if UE moves to a new area with the MBSR, while the NG-RAN node in this new area connects to a different LMF. If so, it should guaranteed that the LMF is accessible for both mIAB-DU’s CU and mIAB-MT’s CU, during the MBSR’s consecutive partial migration.

Based on the observations above, we propose:

Proposal 3：RAN3 first discuss the following issues for Option 1, before capturing the potential NRPPa enhancement to include velocity and timestamp:

- Issue 1: In the DU migration case, if the mIAB-DU migrates to a new F1 terminating CU, how the LMF can find the new F1 terminating CU?

- Issue 2: In the partial migration case, how to perform UE location, if MT and DU connects to different donor? Which LMF (e.g. the LMF serve the F1 terminating donor, or the LMF serve the non-F1 terminating donor) to be used for the UE location service?

2.2 For point#7 question

For the KI#6 in SA2 LS [1], providing the mobile IAB-MT’s ULI related additional ULI in the UE’s ULI is a feasible solution. By this way, the UE’s AMF can know the UE’s location accurately referring to the location of the co-located IAB-MT of the serving IAB-DU.

Besides, a possible alternative solution is to provide IAB-MT’s ULI together with the co-located IAB-DU’s served cell IDs to the UE’s AMF in advance. Thus, when a UE report the ULI including the serving cell ID to its serving AMF, the AMF can obtain the additional ULI based on the pre-acquired serving cell ID. However, this alternative requires to transmit the MT’s ULI together with all the activated cells’ IDs under IAB-DU to the UE’s AMF, and different UE’s AMF may also be different. We don’t think this alternative has advantage for reducing the signalling overhead compared with carrying the additional ULI in each UE’s ULI report.

Proposal 4: RAN3 to confirm the mobile IAB-MT’s ULI to be included with the UE’s ULI.

For the scenario that the mobile IAB-MT and mobile IAB-DU connects to different donor CU, the additional ULI is related to the mobile IAB-MT’s serving cell (e.g., cell ID of the mobile IAB-MT), and is unknown by the F1 terminating CU initially. The additional ULI should be notified to the F1 terminating CU. Theoretically, both mIAB-MT’s CU and mIAB-DU can inform mIAB-DU’s CU the additional ULI, and considering the Xn interface may not always exist between the mIAB-MT’s CU and mIAB-DU’s CU, we slightly prefer the mIAB-DU to inform.

Proposal 5：For the scenario that the mobile IAB-MT and mobile IAB-DU connects to different donor CU, the additional ULI (e.g. cell ID of the mobile IAB-MT) will be notified to the donor CU serving the UE (i.e. the F1 terminating donor) by the mIAB-DU.

Referring to TS 38.413, the ULI of NR UE includes the following contents:

| >NR user location information |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |

Some contents may not applicable for the mobile IAB-MT. For example, the PSCell Information is only applicable for NR-DC based IAB-MT (such case is deprioritized according to previous agreements), the NID is for NPN, and NR NTN TAI Information is for NTN. The Age of location which is a timestamp of the location information will be carried in the UE’s ULI, and if the additional ULI is included in same message, there is no need to include the same timestamp again in the additional ULI. Therefore, for the additional ULI related to MBSR, we would suggest to include the NR CGI and the TAI in the additional ULI, which are also mandatory for UE’s ULI.

Proposal 6: The additional ULI includes the NR CGI and the TAI of the mobile IAB-MT’s serving cell.

3. Conclusion

This paper presents our view on the point#6 and point#7 questions for the SA2 VMR LS. The following is proposed:

For point#6 question

Proposal 1：NRPPa and F1AP to include UE ID of IAB-MT. FFS on whether to use GPSI or SUPI. The potential privacy issue of introducing the GPSI/SUPI in RAN signaling should further check with SA3.

Proposal 2: NRPPa and F1AP to include explicit mobile TRP indication in the IE.

Proposal 3：RAN3 first discuss the following issues for Option 1, before capturing the potential NRPPa enhancement to include velocity and timestamp:

- Issue 1: In the DU migration case, if the mIAB-DU migrates to a new F1 terminating CU, how the LMF can find the new F1 terminating CU?

- Issue 2: In the partial migration case, how to perform UE location, if MT and DU connects to different donor? Which LMF (e.g. the LMF serve the F1 terminating donor, or the LMF serve the non-F1 terminating donor) to be used for the UE location service?

For point#7 question

Proposal 4: RAN3 to confirm the mobile IAB-MT’s ULI to be included with the UE’s ULI.

Proposal 5：For the scenario that the mobile IAB-MT and mobile IAB-DU connects to different donor CU, the additional ULI (e.g. cell ID of the mobile IAB-MT) will be notified to the donor CU serving the UE (i.e. the F1 terminating donor) by the mIAB-DU.

Proposal 6: The additional ULI includes the NR CGI and the TAI of the mobile IAB-MT’s serving cell.

It is also proposed to:

- Agree the TP to TS 38.455 for capturing the mobile TRP type, TP to TS 38.473 for capturing the mobile TRP type and additional ULI report to donor-CU, and TP to TS 38.413 for capturing the additional ULI report to AMF in Annex 1, 2, and 3, respectively.

4. Reference

- S2-2211437, Reply LS on FS_VMR solutions review

- R3-231011, Reply LS on FS_VMR solutions review

- S2-2301478, MT-LR procedure for when a MBSR is involved in the location of a UE, SA2 Meeting # 154-AH-e, January 16-20, 2023.

Annex 1: Text Proposal for TS 38.455

-------------------------------------------------Start of change-----------------------------------------------------------

9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>NR CGI | M |  | 9.2.9 |  | - |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |
| >>SSB Information | M |  | 9.2.54 |  | - |  |
| >>SFN Initialisation Time | M |  | Relative Time 1900 9.2.36 |  | - |  |
| >>Spatial Direction Information | M |  | 9.2.45 |  | - |  |
| >>Geographical Coordinates | M |  | 9.2.46 |  | - |  |
| >>TRP type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp,…,mobile-trp) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information | M |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association | M |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information | M |  | 9.2.82 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

-------------------------------------------------End of change-----------------------------------------------------------

Annex 2: Text Proposal for TS 38.473

-------------------------------------------------Start of change-----------------------------------------------------------

9.3.1.176	TRP Information

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.3.1.197 |  | - |  |
| TRP Information Type Response List |  | 1 |  |  | - |  |
| >TRP Information Type Response Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | - |  |
| >>CHOICE TRP Information Type Response Item | M |  |  |  | - |  |
| >>>NR PCI |  |  |  |  |  |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>>NR CGI |  |  |  |  |  |  |
| >>>>NR CGI |  |  | 9.3.1.12 |  | - |  |
| >>>NR ARFCN |  |  |  |  |  |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>PRS Configuration |  |  |  |  |  |  |
| >>>>PRS Configuration | M |  | 9.3.1.177 |  | - |  |
| >>>SSB Information |  |  |  |  |  |  |
| >>>>SSB Information | M |  | 9.3.1.202 |  | - |  |
| >>>SFN Initialisation Time |  |  |  |  |  |  |
| >>>>SFN Initialisation Time | M |  | Relative Time 1900 9.3.1.183 |  | - |  |
| >>>Spatial Direction Information |  |  |  |  |  |  |
| >>>>Spatial Direction Information | M |  | 9.3.1.179 |  | - |  |
| >>>Geographical Coordinates |  |  |  |  |  |  |
| >>>>Geographical Coordinates | M |  | 9.3.1.184 |  | - |  |
| >>>TRP Type |  |  |  |  |  |  |
| >>>>TRP Type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp,…, mobile-trp) | TS 38.305 [42] | YES | reject |
| >>>On-demand PRS TRP Information |  |  |  |  |  |  |
| >>>>On-demand PRS TRP Information | M |  | 9.3.1.240 |  | YES | reject |
| >>>TRP Tx TEG Association |  |  |  |  |  |  |
| >>>>TRP Tx TEG Association | M |  | 9.3.1.252 |  | YES | reject |
| >>>TRP Beam Antenna |  |  |  |  |  |  |
| >>>>TRP Beam Antenna Information | M |  | 9.3.1.256 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

-------------------------------------------------Next change-----------------------------------------------------------

9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| Cells Status List |  | 0..1 |  | Complete list of active cells | YES | reject |
| > Cells Status Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Service Status | M |  | 9.3.1.68 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| > Dedicated SI Delivery Needed UE Item |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-DU ID | O |  | 9.3.1.9 |  | YES | reject |
| gNB-DU TNL Association To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-DU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | reject |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | - | - |
| >>TNL Association Transport Layer Address gNB-CU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | Ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| Co-located IAB-MT NR ULI | O |  | 9.3.1.x | Indicates the NR ULI for the co-located IAB-MT. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

-------------------------------------------------Next change-----------------------------------------------------------

9.3.1.x	Co-located IAB-MT NR ULI

This IE contains the NR ULI for the co-located IAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR ULI | M |  |  | Indicates the NR ULI for the co-located IAB-MT. |
| >NR CGI | M |  | 9.3.1.12 | The NR CGI of the cell, which is the serving cell of the IAB-DU’s co-located IAB-MT. |
| >TAI | M |  | 9.3.1.y | The TAI supported by the cell, which is the serving cell of the IAB-DU’s co-located IAB-MT |

-------------------------------------------------Next change-----------------------------------------------------------

9.3.1.y	TAI

This IE is used to uniquely identify a Tracking Area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.1.14 |  |
| TAC | M |  | 9.3.1.29 |  |

-------------------------------------------------End of change-----------------------------------------------------------

Annex 3: Text Proposal for TS 38.413

-------------------------------------------------Start of change-----------------------------------------------------------

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |
| Additional NR ULI | O |  |  | Indicates the NR ULI for the co-located IAB-MT of the UE’s serving IAB-DU. | - |  |
| >NR CGI | M |  | 9.3.1.7 | The NR CGI of the cell, which is the serving cell of the IAB-MT co-located with the UE’s serving IAB-DU | - |  |
| >TAI | M |  | 9.3.3.11 | The TAI supported by the cell, which is the serving cell of the IAB-MT co-located with the UE’s serving IAB-DU | - |  |

-------------------------------------------------End of change-----------------------------------------------------------