3GPP TSG-RAN WG3 Meeting #119bis-e                                                               R3-231797

17th – 26th April 2023, Online

Agenda item:	12.2.1

Source:	CMCC, CATT

Title:	(TP for AI&ML BLCR to TS38.300) Stage 2 updates on the new procedures of AI/ML for RAN

Document for:	Discussion and Decision

1	Introduction

In recent RAN3 meetings, the new procedures of AI/ML for RAN have been discussed and following agreements achieved [1]:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point.

Introduce a new Class 1 procedure for initiating the reporting of AI/ML Related Information and a Class 2 procedure for Data Reporting of AI/ML Related Information.

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

The new procedure is non-UE associated procedure. If needed, the procedure can be used to capture UE-associated information.

The response message of the new procedure for AI/ML Related Information indicates if the requested information can be provided.

Then in last RAN3#119 meeting, following further agreements achieved for the new class1 class2 procedures:

The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting.

Introduce into the agreed new request message (AI/ML INFORMATION REQUEST, the name needs further discussion), an indication that UE performance feedback is provided after handover event. Whether the indication is in implicit or explicit way needs to be further discussed.

Introduce a trigger indication in the HO request message to indicate that UE performance feedback is requested after HO completion. The details of indication need to be discussed.

The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting.

Partial reporting mechanism is supported in the agreed AI/ML information procedures (AI/ML INFORMATION REQUEST/RESPONSE/UPDATE, which name is FFS). The solutions need to be discussed.

As showed in the endorsed TP to 38.423 [2], the procedures could be used to request and report Predicted Resource Status, Predicted Number of Active UEs, Predicted RRC connections, as well as UE Performance, and the procedures are common for the three use cases. So, it makes sense to be captured in the BLCR to TS38.300. Following, the corresponding TP on the new procedures for AI/ML BLCR to TS38.300 is provided.

2	TP to 38.300

******************************Start of  Change*****************************************

X.X AI/ML for NG-RAN

X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML Related Information, e.g., Predicted Resource Status, Predicted Number of Active UEs, Predicted RRC connections, as well as UE Performance are exchanged between NG-RAN nodes with data type agnostic procedures to support AI/ML function in a request/response way.

Partial reporting is supported in the AI/ML information exchange procedures.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

******************************End of  Change*****************************************

4	Reference

[1] 	RAN3_119bis-e_agenda_20230328

[2]	R3-230978 was R3-230599_(TP to 38.423 and 38.420) AIRAN impact on Xn Interface_v1