3GPP TSG-RAN WG3 Meeting #120                                                      	R3-232854

Incheon, May 22nd – May 26th, 2023

Agenda item:	26.1

Source:	Qualcomm Incorporated

Title:	Equivalent SNPNs and non-3GPP access for SNPN

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss RAN3 related impacts to support the following objectives for eNPN

Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2]

Support for non-3GPP access for SNPN [RAN3]

# Discussion

  - Equivalent SNPNs

The equivalent SNPNs IE is introduced within NPN Mobility Information IE contained in the Mobility Restriction List IE over XnAP and NGAP

To support the NG based mobility across SNPNs, the Selected NID IE is introduced in the current Target ID IE contained in the HANDOVER REQUIRED message

To support Xn-based HO across equivalent SNPNs, there is no need to add a new selected SNPN ID in the HANDOVER REQUEST message, because the MRL can indicate the selected SNPN

FFS on whether there is a need for MN to indicate the selected NID to SN during SN addition and SN modification procedure.

In case of NR-DC, there were proposals last meeting for MN to indicate the selected NID to SN in SN ADDITION REQUEST and SN MODIFICATION REQUEST. Before we discuss this further, let’s take a step back and look at the “Selected PLMN” IE in TS 38.423:

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Currently for equivalent PLMNs, MN can indicate the serving PLMN via MRL, and can additionally indicate the selected PLMN of the SCG in the SN.

SN knows the equivalent PLMNs and the serving PLMN in the MN and therefore can decide suitable PLMN for SCG. Therefore, it is not clear why MN needs to “select” a PLMN and signal it during SN addition/modification.

Proposal 1: RAN3 should clarify why MN signals the “selected PLMN” during SN addition/modification when SN knows the equivalent PLMNs and the serving PLMN in the MN and can therefore decide suitable PLMN for SCG by itself

Proposal 2: RAN3 should clarify what is meant by “RRM purpose” when signaling the selected PLMN during SN addition/modification

Further we think we should LS SA2 to check whether MN and SN can be served by different SNPNs.

Proposal 3: LS SA2 to check whether MN and SN can be served by different SNPNs.

SN knows the equivalent SNPNs and the serving SNPN in the MN and therefore can decide suitable SNPN for SCG. Therefore, it is not clear why MN needs to “select” a SNPN and signal it during SN addition/modification.

Proposal 4: SN knows the equivalent SNPNs and the serving SNPN in the MN and therefore can decide suitable SNPN for SCG. There is no need for MN to “select” an SNPN and signal it during SN addition/modification

  - Non-3GPP access for SNPN

To support non-3GPP access for SNPN services, a non-3GPP access specific selected NID should be added in the INITIAL UE MESSAGE over NG interface

FFS on how to add selected NID in NGAP INITIAL UE MESSAGE:

- Option 1: Add the Selected NID in the top level of INITIAL UE MESSAGE

- Option 2: Add the Selected NID under NPN Access Information in INITIAL UE MESSAGE

Option 1: Add the Selected NID in the top level of INITIAL UE MESSAGE

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the selected PLMN id for the non-3GPP access. | YES | ignore |
| Selected NID | O |  | 9.3.3.42 | Indicates the selected NID for the non-3GPP access. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB node | YES | reject |
| CE-mode-B Support Indicator | O |  | 9.3.1.156 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.157 |  | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |

Option 2: Add the Selected NID under NPN Access Information in INITIAL UE MESSAGE

9.3.3.46    NPN Access Information

This IE contains information to perform access control for NPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE NPN Access Information | M |  |  |  | - |  |
| >PNI-NPN Access Information |  |  |  |  |  |  |
| >>Cell CAG List | M |  | 9.3.3.47 |  | - |  |
| >SNPN Access Information |  |  |  |  | YES | reject |
| >>Selected NID | M |  | 9.3.3.42 | Included at initial access, if the SNPN is provided via non-3GPP access, as specified in TS 23.502 [10]. | - |  |

Option 1 is simpler and aligns with the Selected PLMN identity for non-3GPP access and therefore Option 1 is preferred.

Proposal 5 To support non-3GPP access for SNPN, the Selected NID needs to be included in the top level of INITIAL UE MESSAGE over NGAP.

# Conclusion

Equivalent SNPNs

Proposal 1: RAN3 should clarify why MN signals the “selected PLMN” during SN addition/modification when SN knows the equivalent PLMNs and the serving PLMN in the MN and can therefore decide suitable PLMN for SCG by itself

Proposal 2: RAN3 should clarify what is meant by “RRM purpose” when signaling the selected PLMN during SN addition/modification

Proposal 3: LS SA2 to check whether MN and SN can be served by different SNPNs.

Proposal 4: SN knows the equivalent SNPNs and the serving SNPN in the MN and therefore can decide suitable SNPN for SCG. There is no need for MN to “select” an SNPN and signal it during SN addition/modification

Non-3GPP access for SNPN

Proposal 5 To support non-3GPP access for SNPN, the Selected NID needs to be included in the top level of INITIAL UE MESSAGE over NGAP.

# 4. References