3GPP TSG-RAN WG3 Meeting #120	R3-232870

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TPs to TS 37.340, 38.423 BL CRs) other CHO related aspects

Source: 	Huawei

Agenda item:	14.3

Document Type:	other

# 1. Introduction

On the Support of CHO in NR-DC agenda item 14.3, there are several topics need to be further discussed:

- Topic 1: per T-SN information provided from the T-MN to the S-MN in case of CHO with multiple SCGs

- Topic 2: RAN3 impacts of CHO associated CPAC configurations

- Topic 3: if and how to avoid unnecessary signalling between S-SN and T-MN/T-SN

- Topic 4: how to avoid multiple data forwarding path

The Topic 4 is discussed in [1], in this contribution, we focus on the Topic 1, 2 and 3 and provide the related TPs related to Topic 1 and 2.

# 2. Discussion

## 2.1 Topic 1: per T-SN information provided from the T-MN to the S-MN in case of CHO with multiple SCGs

RAN3 focuses on the following aspects for CHO with multiple SCGs.

- T-MN provides the PDU session admission results of different T-SN(s) in the HO procedure considering the pair of candidate T-MN and T-SN(s).

- A set of data forwarding addresses are provided from candidate T-MN to the source node.

In Rel-17 CPAC, when the MN or the S-SN initiates to prepare one T-SN, even if the T-SN prepared multiple PSCells, it is assumed that the T-SN admit same PDU sessions and QoS flows for these prepared PSCells, and data forwarding is also performed per T-SN, instead of per PSCell. And in case multiple T-SNs are prepared, the messages from different Xn interfaces will be able to provide different admission control results and data forwarding addresses from the T-SNs to the MN, and the SN Change Confirm message was also enhanced to provide to the S-SN about the different data forwarding addresses from different T-SNs.

From our understanding, to avoid unnecessary complexity of the signaling design for Rel-18 CHO with multiple SCGs, it is better to keep these assumptions also for CHO with multiple SCGs, i.e.:

- For CHO with multiple SCGs within the same T-SN, the T-SN admit same PDU sessions and QoS flows for prepared PSCells, and data forwarding is performed per T-SN;

- For CHO with multiple SCGs among different T-SNs, enhance HO Request ACK message to provide different PDU Session admission results and data forwarding addresses for each prepared T-SN.

Note that CHO is triggered per Target Cell, therefore the above assumption allows to provide PDU session admission results and data forwarding addresses in a per PCell per T-SN granularity by default.

Similar to Rel-17 CPAC, the peer node sends the list of prepared PSCells to the initiating node, we think it is also needed to provide the list of prepared PSCells in the HO Request ACK for CHO with multiple SCGs.

Proposal 1-1: For CHO with multiple SCGs within the same T-SN, the T-SN admit same PDU sessions and QoS flows for prepared PSCells, and data forwarding is performed per T-SN.

Proposal 1-2: For CHO with multiple SCGs among different T-SNs, enhance HO Request ACK message to provide different PDU Session admission results, data forwarding addresses and list of prepared PSCells for each prepared T-SN.

## 2.2 Topic 2: RAN3 impacts of CHO associated CPAC configurations

- RAN3 continue checking the potential impacts of CHO associated CPAC configurations, following progress made by RAN2 on execution conditions.

The following progress was made in RAN2#121bis-e meeting:

- For the CHO+CPC case:

When both CHO and CPC conditions are met, both CHO and CPC cell change is executed.

Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).

Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

In the preparation of CHO with multiple target SCGs, considering that it is the candidate MN who triggers the CPA/CPC preparation, the candidate MN may be suitable to suggest the candidate PSCells and provide the relevant execution conditions. To ensure these execution condition(s) are valid in the measurement configuration of the source MCG, the candidate MN should provide these trigger event parameters to the source MN, to ask the source MN to generate the MeasConfig by using these trigger event parameters for CPA/CPC, details are pending to RAN2 further progress.

Proposal 2: RAN3 consider to include a set of trigger event configurations associated to candidate PSCells in Handover Request Acknowledge once RAN2 confirms that C-MN decides the trigger events of CPAC.

## 2.3 Topic 3: if and how to avoid unnecessary signaling between S-SN and T-MN/T-SN

- Whether S-SN is aware that reconfiguration of SCGs would not have an impact on the target SCG. Additional enhancements may need to be considered if S-SN requires knowledge of all changes that affect the target SCG.

- Regarding how to avoid unnecessary signaling between S-SN and T-MN/T-SN, RAN3 will down-select a solution that solely relies on RAN3 if needed.

For this topic, there were several solutions discussed in previous meetings, but still lack of detailed clarification about the scenario/configuration which is applicable for this S-SN awareness. From our view, for the case only CHO is configured, or CHO with CPC, CHO with MR-DC, it is hard for the source side to determine if the target node needs to be contacted in case the configuration at source side is updated. For better understanding, let’s classify the RRC fields/IEs into two parts:

- Part 1: the fields/IEs which are absent in the target configuration for which the UE will use the stored source configuration, i.e. absent fields with Need M.

- Part 2: the fields/IEs which are explicitly configured by the target, in this case, the target side may or may not consider the source configuration of the same fields/IEs and the other fields/IEs (related features)

For Part 1, so far there is no requirement for the source node to interpret the target configuration, the source is not able to know whether some fields are absent or not from the target configuration. For Part 2, the source is also not able to determine whether the source configuration will impact the target configuration or not, as it does not know the implantation of the target side when determining the configured parameters by the target node.

With these considerations, in case the source SN updates some parameters (even if only the measurement configuration), we do not think the source SN is able to determine whether its own configuration updates impact the target configuration or not. We prefer to keep the current status/signalling design, and do not see the need to do the optimization as it may lead to more problems.

Proposal 3: Close the discussion on avoiding unnecessary signalling between S-SN and T-MN/T-SN.

# 3. Conclusion

- In this paper, we discussed the following topics and get the related proposals:

## Per T-SN information provided from the T-MN to the S-MN in case of CHO with multiple SCGs

Proposal 1-1: For CHO with multiple SCGs within the same T-SN, the T-SN admit same PDU sessions and QoS flows for prepared PSCells, and data forwarding is performed per T-SN.

Proposal 1-2: For CHO with multiple SCGs among different T-SNs, enhance HO Request ACK message to provide different PDU Session admission results, data forwarding addresses and list of prepared PSCells for each prepared T-SN.

- RAN3 impacts of CHO associated CPAC configurations

Proposal 2: RAN3 consider to include a set of trigger event configurations associated to candidate PSCells in Handover Request Acknowledge once RAN2 confirms that C-MN decides the trigger events of CPAC.

## If and how to avoid unnecessary signalling between S-SN and T-MN/T-SN

Proposal 3: Close the discussion on avoiding unnecessary signalling between S-SN and T-MN/T-SN.

- With these proposals, the corresponding TPs to TS 37.340 and TS 38.423 BL CRs for proposal 1-2 are provided in section 5 and 6.

# 4. Reference

- R3-232869 (TPs to TS 37.340 and 38.423 BL CRs) avoid multiple data forwarding paths

# 5. TP to TS 37.340 BL CR

### 10.19.2	MR-DC with 5GC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN. This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

Figure 10.19.2-1: Conditional Handover with Secondary Node procedure

Figure 10.19.2-1 shows an example signaling flow for Conditional Handover with Secondary Node.

NOTE 1:	For a CHO without SN change, the source SN and the target SN shown in Figure 10.19.2-1 are the same node.

NOTE 2:	For a CHO with SN addition, the source SN and steps involving the source SN in Figure 10.19.2-1 are ignored.

1.	The source MN starts the conditional handover procedure by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the (source) SN UE XnAP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message.

NOTE 3:	In case of the CHO with/without SN change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration, if configured, before step 1.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to a CHO and to enable the SN to identify requests related to the same UE.

NOTE 3a:	The target MN and other potential target MNs may trigger the SN Addition Preparation procedure to the same (target) SN.

NOTE 3b:	The source MN may initiate additional Xn Handover Preparation procedures towards the same or other target MNs. Based on each Xn Handover Preparation procedure, each target MN may decide to trigger SN Addition Preparation procedure.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

NOTE 4:	In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

NOTE 4a0:	In CHO with (multiple) SN configurations, the (candidate) SN assigns the same data forwarding addresses for multiple data forwarding requests from different target MNs. If available, the (candidate) SN indicates direct data forwarding path availability with the source node(s).

Editor’s note: Details of the indication(s) from the target SN are FFS.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the conditional handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

NOTE 4ax:	In CHO with (multiple) SN configurations, for each (candidate) SN, the candidate MN provides to the source MN about the PDU session resources admitted list and not admitted list, together with the list of prepared PSCells included in the Handover Request Acknowledge message.

4a.	The source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies conditional handover to the (source) SN, which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 4a:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the Early Status Transfer message(s) from the source SN, if any, is forwarded to the right target MN. The Xn-U Address Indication procedure may further be initiated to indicate to the (source) SN to stop already initiated early data forwarding for some SN-terminated bearers, if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers.

5.	The source MN sends an RRC reconfiguration message to the UE, including the CHO configuration, i.e. a list of RRC reconfiguration* messages and associated execution conditions, in which each RRC reconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRC reconfiguration** message received from the candidate SN in step 3.

6.	The UE applies the RRC reconfiguration message received in step 5, stores the CHO configuration and replies to the MN with an RRC reconfiguration complete message.

7/8. The UE maintains connection with the source MN and, if the UE is configured with a PSCell, with the source PSCell, after receiving CHO configuration, and starts evaluating the CHO execution condition for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN. If the stored configuration for the selected candidate cell includes an SCG configuration, the UE includes an embedded SN RRCReconfigurationComplete** message for the target SN. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

NOTE 5:	In case the target SN includes the indication of the full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE 6:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the (target) SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

12a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

12c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

12d. The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12e/f. If the target MN is configured with other candidate PCell(s) associated with other candidate SN(s) than the target SN, the target MN sends the SN Release Request message(s) to the corresponding candidate SN(s). Other candidate MN(s) send(s) the SN Release Request message(s) to other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13a. The (source) SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

13b. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR/E-UTRA resource.

14.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target MN forwards the SN Status to the target SN, if needed.

15.	If applicable, data forwarding takes place from the source side (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

16-19.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 8:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

20.	The target MN initiates the UE Context Release procedure towards the source MN.

21.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 12a.

# 6. TP to TS 38.423 BL CR

Start of the change

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

Unchanged part omitted

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the CHO SCG Information IE is included in the Conditional Handover Information Acknowledge IE within the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider a CHO with target SCG(s) is prepared for the UE.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Start of the next change

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO SCG Information | O |  |  |  | YES | ignore |
| >>Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | – |  |
| >>>PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | – |  |
| >>>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

Start of the next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

//skip unchanged part

id-Neighbour-NG-RAN-Node-List,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-CHO-SCG-Information,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

//skip unchanged part

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue		CellCapacityClassValue				OPTIONAL,

capacityValueInfo			CapacityValueInfo, -- this IE represents the IE "CapacityValue" in 9.2.2.a, it’s used to distinguish the "CapacityValue"  in 9.2.2.c

iE-Extensions				ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} }OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ControlPlaneTrafficType ::= INTEGER (1..3, ...)

CHO-SCG-CandidatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CHO-SCG-CandidatePSCells-item

CHO-SCG-CandidatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

iE-Extensions					ProtocolExtensionContainer { { CHO-SCG-CandidatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

CHO-SCG-CandidatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHO-SCG-Information ::= SEQUENCE {

cho-scg-target-sn-list				CHO-SCG-target-SN-list,

iE-Extensions						ProtocolExtensionContainer { { CHO-SCG-Information-ExtIEs} } OPTIONAL,

...

}

CHO-SCG-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHO-SCG-target-SN-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CHO-SCG-target-SN-Item

CHO-SCG-target-SN-Item ::= SEQUENCE {

target-S-NG-RANnodeID							GlobalNG-RANNode-ID,

candidate-pscells								CHO-SCG-CandidatePSCells-list,

pDUSessionResourcesAdmitted-List				PDUSessionResourcesAdmitted-List,

pDUSessionResourcesNotAdmitted-List				PDUSessionResourcesNotAdmitted-List				OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { CHO-SCG-target-SN-Item-ExtIEs} }	OPTIONAL,

...

}

CHO-SCG-target-SN-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHO-MRDC-EarlyDataForwarding ::= ENUMERATED {stop, ...}

CHO-MRDC-Indicator ::= ENUMERATED {true, ..., coordination-only }

CHOtrigger ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOinformation-Req ::= SEQUENCE {

cho-trigger						CHOtrigger,

targetNG-RANnodeUEXnAPID		NG-RANnodeUEXnAPID										OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "CHO-replace" --,

cHO-EstimatedArrivalProbability	CHO-Probability											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Req-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Req-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-Ack ::= SEQUENCE {

requestedTargetCellGlobalID		Target-CGI,

maxCHOoperations				MaxCHOpreparations											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Ack-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Ack-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-CHO-SCG-Information	CRITICALITY ignore	EXTENSION CHO-SCG-Information	PRESENCE optional },

...

}

CHOinformation-AddReq ::= SEQUENCE {

source-M-NGRAN-node-ID				GlobalNG-RANNode-ID,

source-M-NGRAN-node-UE-XnAP-ID		NG-RANnodeUEXnAPID,

cHO-EstimatedArrivalProbability		CHO-Probability													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-AddReq-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-ModReq ::= SEQUENCE {

conditionalReconfig					ENUMERATED {intra-mn-cho, ...},

cHO-EstimatedArrivalProbability		CHO-Probability													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-ModReq-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-ModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHO-Probability ::= INTEGER (1..100)

CNsubgroupID ::= INTEGER (0..7, ...)

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

Start of the next change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

//skip unchanged part

id-PositioningInformation																			ProtocolIE-ID ::= 360

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-CHO-SCG-Information																				ProtocolIE-ID ::= xxx

END

-- ASN1STOP

End of change