3GPP TSG-RAN WG3 #120                                                    R3-232900                                                                                               Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	26.1

Source:	CATT

Title:	Discussion on RAN impact for NPN enhancement in Rel-18

Document for:	Discussion

Introduction

FFS on whether there is a need for MN to indicate the selected NID to SN during SN addition and SN modification procedure.

How to include the Selected NID in INITIAL UE MESSAGE:

Option 1: Add the Selected NID in the top level of INITIAL UE MESSAGE.

Option 2: Add the Selected NID under NPN Access Information in INITIAL UE MESSAGE.

In this paper, we further analysis the detail impacts in RAN3 for above open issues.

Discussion

FFS on whether there is a need for MN to indicate the selected NID to SN during SN addition and SN modification procedure.

In last RAN3 meeting, a 38.300 CR [1] is endorsed with the following change inside,

### NR-NR Dual Connectivity within a single SNPN or across SNPNs indicated in the mobility restriction list is supported. Editor notes: It will be further checked when RAN3 has consensus on whether and how to support equivalent SNPN in NR-DC, and it may also depend on RAN2.

In legacy, SCG (re)selection at the SN is based on roaming and access restriction information (i.e., Mobility Restriction List) in SN.

TS 37.340:

11.1 Roaming and Access Restrictions The principles for conveying roaming and access restriction info for EN-DC are described in TS 36.300 [2]. For MR-DC with 5GC, SCG (re)selection at the SN is based on roaming and access restriction information in SN. If roaming and access restriction information is not available at the SN, the SN shall consider that there is no restriction for SCG (re)selection. Therefore, MN needs to convey the up-to-date roaming and access restriction information to SN via XnAP messages.

So at least for PLMN case, SCG (re)selection at the SN is based on Mobility Restriction List in SN. In our understanding, this principle is also applicable to SNPN case i.e., for SNPN case, SCG (re)selection at the SN is based on Mobility Restriction List in SN.

The equivalent SNPNs IE is introduced NPN Mobility Information IE contained in the Mobility Restriction List IE over XnAP and NGAP.

It is consider that the equivalent SNPNs IE is introduced in NPN Mobility Information IE contained in the Mobility Restriction List IE, SN can select NID among serving SNPN and equivalent SNPNs on Mobility Restriction List.

Proposal 1: NR-DC across registered SNPN and SNPNs in the equivalent SNPN list is supported in Rel-18.

Proposal 2: There is no need for MN to indicate the selected NID to SN. SN selects the NID based on Mobility Restriction List in SN.

How to include the Selected NID in INITIAL UE MESSAGE:

Option 1: Add the Selected NID in the top level of INITIAL UE MESSAGE.

Option 2: Add the Selected NID under NPN Access Information in INITIAL UE MESSAGE.

The selected NID should be introduced in Initial UE message as an optional IE. The LS [2] agreed in the previous meeting also shows this change.

  - Signaling for conveying the selected NID from non-3GPP access node to AMF in NGAP INITIAL UE MESSAGE

To align with PLMN, we propose to add the selected NID in the top of INITIAL UE MESSAGE. Currently, NPN Access Information in INITIAL UE MESSAGE is for PNI-NPN. Another option is to use user location information to identify selected NID. Considering that there is no NID in N3IWF user location information hence this solution cannot work.

9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the selected PLMN id for the non-3GPP access. | YES | ignore |
| Selected NID | O |  | NID 9.3.3.42 | Indicates the selected NID for the non-3GPP access. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB node | YES | reject |
| CE-mode-B Support Indicator | O |  | 9.3.1.156 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.157 |  | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |

Proposal 3: Introduce selected NID in the top level of Initial UE message as an optional IE in NG.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: NR-DC across registered SNPN and SNPNs in the equivalent SNPN list is supported in Rel-18.

Proposal 2: There is no need for MN to indicate the selected NID to SN. SN selects NID based on Mobility Restriction List in SN.

Proposal 3: Introduce selected NID in the top level of Initial UE message as an optional IE in NG.

# 4. Reference

[1] R3-231978, On introduction of R18 eNPN, China Telecom, Huawei, ZTE, CATT.

[2] R3-230813, Reply LS on RAN impact for NPN enhancement in Rel-18, Qualcomm.