3GPP TSG-RAN WG3 Meeting #120	R3-233073

22 -26 May 2023

Incheon, South Korea

Agenda Item:	12.2.2.2

Source:	InterDigital

Title:	Mobility Optimization Outputs and their use

Document for:	Discussion and Agreement

# 1	Introduction

In December RAN plenary approved the work item for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

The main objective of the WI is:

Specify data collection enhancements and signalling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3).

This is the follow up to the study of these topics the outcome of which is captured in 37.817.

The study item made a number of agreements on how to proceed but left a number of topics open to be resolved during the specification phase.

This contribution tries to look at the outputs of the mobility optimization model and discuss whether the output needs to go outside of the local RAN node and thus have standards impact.  And it looks at whether the target gNB needs the outputs to optimize the particular handover or is it potentially part of the AI data collection.

In past RAN3 discussion on these topics was initiated, the discussion did lead to some more clarity about some of the outputs, but many are still left open.

# 2. Discussion

Below are agreed outputs of the mobility optimization model from 37.817::

UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time) Note: Whether the UE trajectory prediction is an external output to the node hosting the Model Inference function should be discussed during the normative work phase.

Estimated arrival probability in CHO and relevant confidence interval

Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

Priority, handover execution timing, predicted resource reservation time window for CHO.

UE traffic prediction (will be used by the RAN node internally and the details are left to normative work phase)

Model output validity time will be discussed during R18 normative work per inference output.

Many of these require a little further discussion.

## 2.1	Estimated Arrival Probability in CHO

This is a parameter that already exists in the Xn Handover preparation so no standard impact

## 2.2	Priority

It is unclear whether this parameter has any meaning outside of helping the local node to put the execution of handovers in a proper order. This may need to be clarified by the proponents.

## 2.3	Handover execution timing

The concept of HO timing is useful to have an understanding of the estimated execution timing for future handovers, that would be predicted by UE trajectoryand other methods that will minimize the effect on UE QoS. It may also be useful to increase the precision of HOs, so that user experience is not compromised. Finally, it may help in optimizing data forwarding to understand when a handover would be executed.This something that would be useful for the target gNB when executing the handover preparation thus it wouldn’t be just considered AI information and included in that procedure.

## 2.4	Predicted resource reservation time window for CHO.

These related outputs are relevant to the target gNB to optimize both resource reservation when preparing handovers and to optimize data forwarding, particularly early data forwarding. In CHO, the UE is pre-configured for a HO that will happen in the future so predicting aspects these related to CHO is an obvious enhancement that perfectly fits the scope of the AI/ML work.Keeping this output local makes no sense since it is much more meaningful for the target gNB than the source gNB and it is useful at the time of the handover preparation. . It is also possible that the UE can also make use of this prediction, This aspect is FFS. .

## 2.5 	UE traffic prediction

It is clear that UE traffic prediction will be used by the local node, but the admission control in the target gNB could very much make use of the expected UE traffic that is predicted which would help scheduling resources for reception of that UE thus making it useful at the time of the handover preparation. On the other hand, the UE probably will not benefit from being sent the prediction. In fact for uplink data the UE would have a better understanding of the predicted traffic anyway.

# 3	Proposal

Based on the discussion above, the following proposal should be agreed:

Proposal 1: The following mobility optimization model outputs should be added to the Handover Preparation Procedure (HANDOVER REQUEST), exact IE structure is FFS:

handover execution timing

Predicted resource reservation time window for CHO

UE Traffic prediction

Proposal 2: The TP in R3-233076 should be agreed to be added to the baseline CR for 38.423