3GPP TSG-RAN WG3 Meeting #121bis	R3-235131

# Xiamen, China, 09 – 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0309 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR for TS 38.401) Introduction of 5G Timing Resiliency and URLLC enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Samsung, Qualcomm, CATT, Ericsson, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC-NR-Core |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The WID on NR Timing Resiliency and URLLC enhancements was approved in RP-230754. This CR is to specify the necessary functions and procedures to support the objectives.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 2 support for 5GS network timing synchronization status and reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 2 support for 5GS network timing synchronization status and reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR0972 TS 38.423 CR1049 TS 38.473 CR1168 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: Inclusion of TP agreed at RAN3#121 (R3-234598) Rev1: Resubmission to RAN3#121bis |  |  |  |  |  |  |  |  |

Start of change

## 8.x	Timing resiliency service

The following clauses describe the overall procedures for RAN Timing Synchronisation Status (TSS) reporting involving gNB-CU/gNB-DU.

### 8.x.1	RAN TSS reporting towards the CN

The signaling flow for RAN TSS reporting towards the CN is shown in Figure 8.x.1-1. This procedure is used when the TSCTSF subscribes to RAN TSS reporting at the AMF as described in TS 23.502 [x].

Figure 8.x.1-1:	RAN TSS reporting towards the CN

0.	The gNB-DU is pre-configured with thresholds for each RAN TSS attribute it supports.

NOTE 1: 	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator.

1.	The AMF requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-CU.

2.	The gNB-CU requests RAN TSS reporting from at least one gNB-DU by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

NOTE 2:	It is up to gNB-CU implementation whether to send the request to all its gNB-DUs or to a particular subset of its gNB-DUs, depending on e.g. network topology.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

4.	The gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS RESPONSE message. If the gNB-CU does not receive a successful response from at least one gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS FAILURE message and the flow stops at this step.

5. 	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 3:	The RAN TSS attributes included in the report is up to gNB-DU implementation.

6.	The gNB-CU sends the TIMING SYNCHRONISATION STATUS REPORT message to the AMF. The message contains the RAN TSS attributes received from the gNB-DU and the RAN TSS Scope IE to indicate whether the scope of the RAN TSS report is “RAN node level” or “cell list level”.

7.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

8.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

9. 	Same as step 6.

### 8.x.2	RAN TSS reporting towards the UE

The signaling flow for RAN TSS reporting towards the UE in RRC_CONNECTED state is shown in Figure 8.x.2-1.

Figure 8.x.2-1:	RAN TSS reporting towards the UE in RRC_CONNECTED state

NOTE 1:	In this signalling flow, it is assumed that RAN TSS reporting is already enabled at the gNB-DU.

1.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU, containing the Clock Quality Reporting Control Information IE within the Time Synchronization Assistance Information IE. The clock quality reporting control information indicates the clock quality detail level to provide to the UE, i.e. “metrics” or “acceptable/not acceptable indication”. If clock quality detail level equals “acceptable/not acceptable indication”, the clock quality reporting control information also contains the clock quality acceptance criteria.

NOTE 2:	The clock quality reporting control information can also be provided in the UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, or PATH SWITCH REQUEST ACKNOWLEDGE messages.

2.	The gNB-CU replies to the AMF by sending the INITIAL CONTEXT SETUP RESPONSE message.

3.	The gNB-CU sends the latest clock quality information to the UE by sending the DLInformationTransfer message. The clock quality information provided to the UE depends on the clock quality detail level received in step 1 (i.e., “metrics” or “acceptable/not acceptable indication”).

4.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

5.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

6.	Same as step 3.

End of change