3GPP TSG-RAN WG3 #121bis	R3-235207

Xiamen, China, 9 – 13 Oct 2023

Title: 	(TP for TS 37.483, TS 38.473) Discussion on MBS RAN sharing

Source: 	Samsung

Agenda item:	15.2

Document Type:	Discussion

1. Introduction

The documents discuss the open issues in E1 and F1 for the MBS RAN sharing.

2. Discussion

2.1	E1 Impact

The last RAN3 meeting we reached the following agreements:

Introduce Broadcast session transport request/response/failure procedure to support NG-RAN triggered NG-U tunnel establishment.

We had also drawn some general conclusions related to this issue in previous meetings listing in below:

- For MOCN, it is up to NG-RAN node implementation to decide how many NG-U tunnels to be setup

- Include in the MBS Session Setup or Modification Response Transfer IE an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session in case of unicast transport.

Our initial requirements for achieving these agreements came from SA2. In TS23.247, in the section 6.18 of resource sharing across broadcast MBS session during network sharing, describes that NG-RAN decides how many NG-U tunnels to be setup. In section 7.3.7 of transport change for resource sharing across broadcast MBS Sessions during network sharing describes in which scenarios the NG-RAN may decide to establish a new NG-U tunnel.

According to TS23.247, the AF creates broadcast MBS Sessions via 5GC Operators A, B and C respectively to deliver the same content and N3mb unicast transport is used from 5GC to the NG-RAN. The NG-RAN may establish only one  N3mb tunnel from the 5GC (i.e., Operator A in Figure 6.18-1) or the NG-RAN can establish the N3mb tunnels from the 5GCs of all the operators to the shared NG-RAN, as showed in the below figure 6.18-1. The NG-RAN makes decision based on operator policy in the NG-RAN node.

Figure 6.18-1: Example of Resource sharing across multiple broadcast MBS Sessions via different CNs to deliver the same content during network sharing

In case the NG-RAN establish N3mb tunnels for all operators, from the agreements made in the last meeting, the NG-RAN sends multiple E1AP BC Bearer Context Setup message to the CU-UP, one per PLMN. From the associated session ID included in the E1AP messages, the CU-UP knows the relationship between the shared MBS session. For the resource efficiency, the PDCP and MRB could be identical for the shared MBS sessions.

If only one N3mb tunnel is setup, the NG-RAN may decide to setup another N3mb tunnel. We have agreed the NG-RAN/CU-CP can send a new NGAP message to setup the new N3mb tunnel, but it is not a complete solution. There are still three issues need to be solved in order to make it a complete solution.

The first issue is how the NG-RAN can determine when to set up another NG-U tunnel (e.g. Operator B in Figure 6.18-1).

In TS23.247, the NG-RAN performs N3mb tunnel change in one of below two scenarios:

-	A broadcast MBS session, from which the shared NG-RAN receives DL data stream, is released, and no DL data stream is received in the NG-RAN from the remaining broadcast MBS session(s) with the same Associated Session ID.

-	When the shared NG-RAN fails to receive DL data stream from the CN (e.g., due to failure in the user plane), the NG-RAN attempts to get the DL data stream via another CN's user plane.

Regarding to the second scenario, the CU-UP should have a way to notify the failure in the user plane to the CU-CP. Thus, the CU-CP can attempt to setup another N3mb tunnel via another CN to keep the transmission continuity.

Proposal 1:	Indicate the failure in the user plane in BC Bearer Context Modify Required message from the gNB-CU-UP to the gNB-CU-CP.

The second issue how the NG-RAN knows the TNL@NG-RAN of another NG-U tunnel

For MOCN, it is up to NG-RAN node implementation to decide how many NG-U tunnels to be setup. For the sake of resource efficiency, it is possible that only one N3mb tunnel is established by the NG-RAN. Using the example in Figure 6.18-1. If only N3mb tunnel via CN-A is established, so far it is unclear if the CU-UP still needs to allocate the NG-U resource for CN-B or CN-C before the NG-RAN decides to setup NG-U toward CN-B or CN-C. There are three alternatives for the second issue:

Alternative 1: No TNL@RAN of another NG-U is stored in CU-CP

If the CU-CP decides not to setup the NG-U for the TMGI in CN-B and CN-C, the CU-CP doesn’t send the E1AP Bearer Context Setup Request for CN-B and CN-C. Thus, the CU-UP doesn’t know there is an associated sessions in CN-B and CN-C. When the ongoing shared session in CN-A has the failure in DL user plane, the CU-UP sends the BC Bearer Context Modify Required message (new message as proposed in proposal 1) for the shared session in CN-A, then the CU-CP decides to setup N3mb tunnel in CN-B and sends E1AP Bearer Context Setup for TMGI in CN-B. The CU-UP allocates the TNL info @ NG-RAN and includes it in the response message. After that, the CU-CP sends a NGAP message to setup another NG-U towards CN-B.

Alternative 2: No TNL@RAN of another NG-U is stored in CU-CP

Even the CU-CP decides not to setup for the TMGI in CN-B and CN-C, the CU-CP still sends the E1AP Bearer Context Setup Request message to the CU-UP for the TMGI in CN-B and CN-C. In the Request message for CN-A, the CU-CP includes Shared NG-U Not Established in the BC Bearer Context Setup Request message. Thus the CU-UP doesn’t allocate the NG-U tunnle resource for CN-B and CN-C. When the ongoing session in CN-A has problem in the NG-U, the CU-UP sends the BC Bearer Context Modify Required message (new message as proposed in proposal 1) to indicate the problem, the following steps is same as showed in alternative 1. Compare to alternative 1, alternative 2 has the same effects but have more messages exchange. therefore alternative 2 is not proposed.

Alternative 3: TNL@RAN of another NG-U are stored in CU-CP

Even the CU-CP decides not to setup for the TMGI in CN-B and CN-C, the CU-CP still sends the E1AP Bearer Context Setup Request message to the CU-UP for the TMGI in CN-B and CN-C. In the Request message for CN-A, the CU-CP includes Shared NG-U Not Established in the BC Bearer Context Setup Request message. While the CU-UP still allocate the NG-U tunnle resource for CN-B and CN-C and include the NG-U tunnel resource in the Response message. When the ongoing session in CN-A has problem, the CU-UP sends the BC Bearer Context Modify Required message (new message as proposed in proposal 1) to indicate the problem. The CU-CP already stores the TNL@NG-RAN for CN-B and CN-C in E1AP Bearer Context Setup procedure. The CU-CP can decide to setup N3mb tunnel in CN-B and sends a NGAP message to setup another NG-U towards CN-B without contacting CU-UP for CN-B again. Compare with alternative 1, alternative 3 has benefit to fast the data transmission via another CN and to reduce the data interruption, by exchanging the NG-U tunnel beforehand. Therefore it is propose to adopt this alternative.

Proposal 2:	It is proposed to select between alternative 1 and alternative 3.

Proposal 3:	Includes Shared NG-U Not Established in the E1AP BC Bearer Context Setup Request message if the gNB-CU-CP decides to not establish shared NG-U resource towards the 5GC for this bearer context.

The third issue how the UPF knows from which packet to start with when another NG-U is setup

To support data continuous reception, the gNB-CU-UP can indicate the next GTP-U SN to the gNB-CU-CP together the reporting of the user plane failure. The gNB-CU-CP can include the GTP-U SN in the BC Bearer Modification Required message when setup another NG-U tunnel to CN-B or CN-C. Then the UPF can start to send the packet with this GTP-SN. This can reduce data loss and avoid duplicate data transmission.

Proposal 4:	Agree to include expected GTP-U SN in the E1AP BC Bearer Modification Required message and in NGAP BC Session Transport Request message.

2.2	F1 impact

The impact of F1 needs to be analysed separately for MOCN case and multiple cell broadcasting case.

For MOCN, we reached the following agreements:

For MOCN scenario, only one set of shared F1-U tunnels is established and kept for MOCN scenario as long as there is one PLMN keeping the MBS service.

For MOCN, Adopt the option of Multiple F1AP contexts/messages, one per PLMN, for MOCN scenario.

FFS whether the set of F1-U tunnel(s) and NG-U tunnel are always one to one mapping or not.

We think no need to always keep the one-on-one mapping between the NG-U tunnel and F1-U tunnel. When the NG-U tunnel changes, the F1-U tunnel can remain unchanged. To achieve this, the CU-UP only needs to assign the same F1-U TNL info @ CU to the MBS services with the same Associated session ID.

Proposal 5:	For MOCN, F1-U tunnel and NG-U tunnel are not always one to one mapping. When the gNB-CU-CP decides to setup another NG-U tunnel, the F1-U tunnel needn’t be switched.

For multiple cell broadcast scenario, we reached the following agreements:

In case of RAN Sharing with multiple cell-ID broadcast, each logical gNB-DU will receive within the F1AP: BROADCAST CONTEXT SETUP REQUEST message the MBS RAN Sharing efficiency Information received from CN (if received).

For multiple cell-ID broadcast scenario, the entity controlling logical DUs decides how many F1-U tunnels to be set up. The decision of CU-CP on establishment of NG-U tunnel takes the feedback of DU on establishment of a set of F1-U tunnels into account.

From the above agreements, it implies that the NG-RAN/CU-CP can decide which NG-U should be setup. The previously reached conclusion for MOCN needs to be extended to multiple cell broadcast scenario.

Proposal 6:	For multiple cell broadcast scenario, it is up to NG-RAN node implementation to decide how many NG-U tunnels to be setup.

For the multiple cell broadcast scenario, different CU-CP may configure different PDCP configuration. The DU decides how many F1-U tunnel to be setup. If the DU decides to setup only one F1-U tunnel, for the F1-U needn’t be setup, the gNB-DU can tell the CU-CP that the F1-U tunnel is not established in the BC Context Setup Response. Thus the CU-CP can take it into account when decide whether to setup the corresponding NG-U tunnel. The CU-CP sends E1AP MC Bearer Context Release Request message to the CU-UP that does not need to establish a NU-U tunnel. As showed in below figure 2, the F1-U tunnel and NG-U tunnel are related. If DU decides to setup F1-U for operator A, then NG-U for operator A also needs to be established. F1-U and NG-U for data transmission are one-on-one mapping.

Figure 2: multiple cell broadcast scenario

Proposal 7:	For multiple cell broadcast scenario, include in the F1AP BC Context Setup Response an indication that the DU has decided to not establish shared F1-U transport resources for the broadcast MBS session in case of unicast transport.

Proposal 8:	For multiple cell broadcast scenario, F1-U tunnel and NG-U tunnel establishment are always one to one mapping.

There is an open issue for PDCP configuration for multiple cell broadcast scenario:

Identify the issue first, then discuss whether both of the options are supported

Option 1: OAM configures the MRB-PDCP-ConfigBroadcast of each MRB.

Option 2: DU makes decision on which MRB-PDCP-ConfigBroadcast it used.

For option 2: whether to provide the used MRB-PDCP-ConfigBroadcast to CU and whether CU could reject and fall back to Rel-17 behaviour.

We believe that the scenario is about whether the F1-U tunnel can be switched from one gNB-CU to another. If allowed, then there is a scenario where different gNB-CUs use the same PDCP configuration to maintain the same PDCP configuration during F1-U tunnel switching. If F1-U tunnel does not need to switch, then there is no scene, and DU selects a PDCP configuration for broadcasting. If following the decision made the NG-U tunnel, NG-U tunnel can be switch over, then as showed in figure 2, the F1-U tunnel should be one to one mapping to the NG-U tunnel, that means F-U tunnel can be switch over in some case.

Proposal 9:	For multiple cell broadcast scenario, to discuss if the F1-U tunnel can be switch over. If the F1-U tunnel can be switch over, it is proposed to agree the MRB PDCP configuration decided by the gNB-DU is provided to the gNB-CU.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	Indicate the failure in the user plane in BC Bearer Context Modify Required message from the gNB-CU-UP to the gNB-CU-CP.

Proposal 2:	It is proposed to select between alternative 1 and alternative 3.

Proposal 3:	Includes Shared NG-U Not Established in the E1AP BC Bearer Context Setup Request message if the gNB-CU-CP decides to not establish shared NG-U resource towards the 5GC for this bearer context.

Proposal 4:	Agree to include expected GTP-U SN in the E1AP BC Bearer Modification Required message and in NGAP BC Session Transport Request message.

Proposal 5:	For MOCN, F1-U tunnel and NG-U tunnel are not always one to one mapping. When the gNB-CU-CP decides to setup another NG-U tunnel, the F1-U tunnel needn’t be switched.

Proposal 6:	For multiple cell broadcast scenario, it is up to NG-RAN node implementation to decide how many NG-U tunnels to be setup.

Proposal 7:	For multiple cell broadcast scenario, include in the F1AP BC Context Setup Response an indication that the DU has decided to not establish shared F1-U transport resources for the broadcast MBS session in case of unicast transport.

Proposal 8:	For multiple cell broadcast scenario, F1-U tunnel and NG-U tunnel establishment are always one to one mapping.

Proposal 9:	For multiple cell broadcast scenario, to discuss if the F1-U tunnel can be switch over. If the F1-U tunnel can be switch over, it is proposed to agree the MRB PDCP configuration decided by the gNB-DU is provided to the gNB-CU.

4. Text Proposal to TS 37.483 against BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.6.1.1.2	Successful Operation

Figure 8.6.1.1.2-1: BC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested MBS session resources, it replies to the gNB-CU-CP with the BC BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BC BEARER CONTEXT SETUP RESPONSE message, the result of all the requested resources in the following way:

-	A list of BC MRBs which are successfully established shall be included in the BC MRB Setup Response List IE;

-	A list of BC MRBs which failed to be established shall be included in the BC MRB Failed List IE;

-	For each established BC MRB, a list of MBS QoS Flows which are successfully established shall be included in the MBS QoS Flow Setup List IE;

-	For each established BC MRB, a list of MBS QoS Flows which failed to be established shall be included in the MBS QoS Flow Failed List IE.

When the gNB-CU-UP reports the unsuccessful establishment of a BC MRB or MBS QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Requested Action for Available Shared NG-U Termination IE in the BC Bearer Context To Setup IE in the BC BEARER CONTEXT SETUP REQUEST message is set to

-	"apply available configuration" and an appropriate Shared NG-U Termination is available, the gNB-CU-UP shall apply the radio bearer configuration of the Shared NG-U Termination, and indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of the Shared NG-U Termination, if the radio bearer configuration of the Shared NG-U Termination is different than the one requested by the gNB-CU-CP.

-	"apply requested configuration" the gNB-CU-UP shall make use of an available appropriate Shared NG-U Termination if the radio bearer configuration of the Shared NG-U Termination, is the same as the one requested by the gNB-CU-CP, otherwise allocate separate resources as requested by the gNB-CU-CP and indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of the Shared NG-U Termination.

-	"apply available configuration if same as requested" the gNB-CU-UP shall make use of an available appropriate Shared NG-U Termination only if the radio bearer configuration of the Shared NG-U Termination is the same as the one requested by the gNB-CU-CP and reply with BC BEARER CONTEXT SETUP RESPONSE message.

If the Associated Session ID IE is contained in the BC BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take this information into account to determine the appropriate resources, as specified in TS 38.401 [2].

If the MBS Service Area IE is contained in the BC BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take this information into account to determine the appropriate resources, as specified in TS 38.401 [2].

If the gNB-CP-CP node decides to not establish shared NG-U resources towards the 5GC it shall include the Shared NG-U Not Established IE set to "true" in the BC Bearer Context To Setup IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.6.1.3	BC Bearer Context Modification Required

8.6.1.3.1	General

The purpose of the gNB-CU-UP initiated BC Bearer Context Modification Required procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to initiate the modification MBS session resources for a broadcast MBS session and inform the gNB-CU-CP. The procedure uses MBS-associated signalling.

8.6.1.3.2	Successful Operation

Figure 8.6.1.3.2-1: BC Bearer Context Modification Required procedure, gNB-CU-UP initiated: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BC BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies to the gNB-CU-UP with the BC BEARER CONTEXT MODIFICATION CONFIRM message.

8.6.1.3.3	Abnormal Conditions

void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3.30	BC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |  |  |
| User Plane Indicator | O |  | ENUMERATED (UP failure, ...) | Indicates user plane failure in the NG interface | YES | ignore |
| GTP-U SN | O |  | INTEGER (1..FFS, ...) | GTP-SN that the UPF should transmit | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3.26	BC Bearer Context To Setup

This IE contains MBS session resource related information used to request BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  | - |  |
| BC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.112 |  | - |  |
| BC MRB To Setup List | M |  | BC MRB Setup Configuration 9.3.1.114 |  | - |  |
| Requested Action for Available Shared NG-U Termination | O |  | 9.3.1.115 |  | - |  |
| Shared NG-U Not Established | O |  | ENUMERATED (true, ...) | Editor’s Note: whether this IE is applicable for both, unicast and multicast NG-U transport or only for unicast transport is FFS | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-VersionID,

id-MBSAreaSessionID,

id-Secondary-PDU-Session-Data-Forwarding-Information,

id-UserPlaneIndicator,

id-GTPUSN,

id-SharedNGUNotEstablished,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

maxnoofMBSSessionIDs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

##### -- BCBearerContextToSetup

BCBearerContextToSetup ::= SEQUENCE {

snssai									SNSSAI,

bcBearerContextNGU-TNLInfoat5GC			BCBearerContextNGU-TNLInfoat5GC				OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration,

requestedAction							RequestedAction4AvailNGUTermination			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SharedNGUNotEstablished						CRITICALITY ignore	EXTENSION SharedNGUNotEstablished				PRESENCE optional},

...

}

BCBearerContextNGU-TNLInfoat5GC::= CHOICE {

locationindependent				MBSNGUInformationAt5GC,

locationdependent				LocationDependentMBSNGUInformationAt5GC,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextNGU-TNLInfoat5GC-ExtIEs}}

}

BCBearerContextNGU-TNLInfoat5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupConfiguration-Item

BCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

##### -- BCBearerContextToModifyRequired

BCBearerContextToModifyRequired ::= SEQUENCE {

bcMRBToRemoveList	BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-UserPlaneIndicator			CRITICALITY ignore	EXTENSION UserPlaneIndicator		PRESENCE optional}|

{ID id-GTPUSN						CRITICALITY ignore	EXTENSION GTPUSN				PRESENCE optional},

...

...

}

##### -- BCBearerContextToModifyConfirm

BCBearerContextToModifyConfirm ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyConfirm-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyConfirm-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UserPlaneIndicator	::=	ENUMERATED	{

UP-failure,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

GTPUSN	::=	INTEGER (1..FFS, ...)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SharedNGUNotEstablished ::= ENUMERATED {

true,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PDCP-COUNT-Reset												ProtocolIE-ID ::= 184

id-MBSSessionAssociatedInfoNonSupportToSupport					ProtocolIE-ID ::= 185

id-VersionID													ProtocolIE-ID ::= 186

id-InactivityInformationRequest									ProtocolIE-ID ::= 187

id-UEInactivityInformation										ProtocolIE-ID ::= 188

id-MBSAreaSessionID												ProtocolIE-ID ::= 189

id-Secondary-PDU-Session-Data-Forwarding-Information			ProtocolIE-ID ::= 190

id-UserPlaneIndicator	 										ProtocolIE-ID ::= aa1

id-GTPUSN														ProtocolIE-ID ::= aa2

id-SharedNGUNotEstablished 										ProtocolIE-ID ::= aa3

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

5. Text Proposal to TS 38.473 against BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.14	NR MBS Procedures

8.14.1	Broadcast Context Setup

8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If the Associated Session ID IE is contained in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4]. If the gNB-DU decides to not establish shared F1-U resources towards the gNB-CU it shall include the Shared F1-U Not Established IE set to "true" in the BROADCAST CONTEXT SETUP RESPONSE message.

8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

8.14.1.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Shared F1-U Not Established | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

SharedF1UNotEstablished

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-SharedF1UNotEstablished,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

BroadcastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupResponseIEs} },

...

}

BroadcastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-Setup-List			CRITICALITY reject TYPE BroadcastMRBs-Setup-List				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetup-List	PRESENCE optional	}|

{ ID id-BroadcastAreaScope					CRITICALITY ignore TYPE BroadcastAreaScope						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics					PRESENCE optional	}|

{ ID id-SharedF1UNotEstablished				CRITICALITY ignore TYPE SharedF1UNotEstablished								PRESENCE optional},

...

}

BroadcastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Setup-ItemIEs} }

BroadcastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetup-ItemIEs} }

BroadcastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Setup-Item			CRITICALITY reject	TYPE BroadcastMRBs-Setup-Item			PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SharedF1UNotEstablished ::= ENUMERATED {

true,

...

}

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList                                 ProtocolIE-ID ::= 707

id-SharedF1UNotEstablished                          ProtocolIE-ID ::= aaa

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>