3GPP TSG-RAN WG3 Meeting #121bis	R3-235377

Xiamen, China, 9 - 13 October 2023

Agenda Item:	25.2.1

Source:	ZTE

Title:	(TP to TS 38.413) Discussion on support of PDU Set Handling

Document for:	Discussions & Approval

1	Introduction

We made the following progress at the RAN3#121 meeting.

Add the PDU Set QoS Parameters in the QoS Flow Level QoS Parameters IE over NGAP, XnAP, F1AP and E1AP. The PDU Set QoS parameters includes: - PDU Set Delay Budget (PSDB). - PDU Set Error Rate (PSER). - PDU Set Integrated Handling Information (PSIHI). PDU Set Information as below needs to be transferred over NG-U, Xn-U and F1-U: - PDU Set Sequence Number. - Indication of End PDU of the PDU Set. - PDU Sequence Number within a PDU Set. - PDU Set Size in bytes. - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. End of Data Burst information needs to be transferred over NG-U, Xn-U and F1-U. Enhance NGAP TSCAI to include the N6 Jitter information associated with DL Periodicity, and clarify TSCAI can also be used for non-GBR. Enhance XnAP/F1AP/E1AP TSCAI to include the N6 Jitter information associated with DL Periodicity.

This paper further discusses how to support PDU Set Handling for XR enhancements.

# 2	Discussion

## 2.1	Container of PDU Set information

At the last meeting, we agreed to transferred PDU Set Sequence Number, Indication of End PDU of the PDU Set, PDU Sequence Number within a PDU Set, PDU Set Size in bytes, PDU Set Importance and End of Data Burst information over NG-U, Xn-U and F1-U. The remaining issue is whether to extend the existing container or introduce a new container.

From our point of view, taking NG-U as an example, there are actually three options as below.

- Extend the PDU session container by adding the information in the DL PDU SESSION INFORMATION frame (PDU type = 0)

- Extend the PDU session container by adding the information in a new frame (PDU type = 2)

- Introduce a new container which is in the same level as the existing PDU session container

Observation 1: Taking NG-U as an example, there are three options to transfer the PDU Set information.

- Extend the PDU session container by adding the information in the DL PDU SESSION INFORMATION frame (PDU type = 0)

- Extend the PDU session container by adding the information in a new frame (PDU type = 2)

- Introduce a new container which is in the same level as the existing PDU session container

The benefit of option 1 is that it has less impact on the specification. Some companies believe that this option may bring the compatibility issue, however, in R17 we have already added the MSNP and DL MBS QFI Sequence Number in the in the DL PDU SESSION INFORMATION frame. The main drawback of this option is that some redundant information also needs to be transferred which may impact the decoding efficiency.

Observation 2: The benefit of option 1 is that it has less impact on the specification. This option will not bring the compatibility issue since in R17 the MSNP and DL MBS QFI Sequence Number have already been added in the in the DL PDU SESSION INFORMATION frame. The main drawback of this option is that some redundant information also needs to be transferred which may impact the decoding efficiency.

Option 2 is a compromise option between option 1 and option 3. However, the main drawback of this option is that an existing extension header type, i.e. PDU session container, is reused. A unknown extension header type can be easily skipped, but if the contents (i.e. PDU type =2) of a known existing extension header can not be understood, this will cause higher complexity since it needs to go back and then skip the length.

Observation 3: Option 2 is a compromise option between option 1 and option 3. However, the main drawback of this option is that an existing extension header type, i.e. PDU session container, is reused. A unknown extension header type can be easily skipped, but if the contents (i.e. PDU type =2) of a known existing extension header can not be understood, this will cause higher complexity since it needs to go back and then skip the length.

Option 3 provides better compatibility and can improve decoding efficiency if only PDU set related information is transferred. However, the main drawback of this solution to bring a relatively large impact on the specification.

Observation 4: Option 3 provides better compatibility and can improve decoding efficiency if only PDU set related information is transferred. The main drawback of this solution to bring a relatively large impact on the specification.

Among these three options, we prefer option 1 since this is a traditional way when we introduce new information for new features, and has less impact on the specification.

Proposal 1: Extend the PDU session container to add the PDU Set information in the DL PDU SESSION INFORMATION frame.

Proposal 2: Extend the RAN container to add the PDU Set information in the DL USER DATA frame.

## 2.2	QFI information

Another remaining issue regarding user plane is whether to include the QFI in the F1-U and Xn-U. Since the PDU Set Sequence Numbers in different QoS flows are not unique, it is necessary to introduce QFI information to distinguish a certain PDU Set belongs to which QoS flow, e.g. whether PDU Set 1 belongs to QoS flow 1 or QoS flow 2.

Observation 5: It is necessary to introduce QFI information to distinguish a certain PDU Set belongs to which QoS flow.

Proposal 3: Include the QFI in the F1-U and Xn-U.

## 2.3	Non-homogenous support

The non-homogenous support of PDU set based handling in NG-RAN is specified in TS 23.501 as below.

TS 23.501 By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set based handling is supported. Based on this, SMF may activate the PDU Set identification and marking in the PSA UPF. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the NG-RAN indication, the SMF may, upon completion of the handover procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters to NG-RAN and configure the PSA UPF to activate/deactivate the PDU Set identification and marking. In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

According to TS 23.501, if the SMF sends the PDU Set QoS parameter to the NG-RAN node to activate the PDU Set QoS handling for a given QoS flow, the NG-RAN node needs to provide an indication of whether the PDU Set QoS handling is supported.

Observation 6: According to TS 23.501, if the SMF sends the PDU Set QoS parameter to the NG-RAN node to activate the PDU Set QoS handling for a given QoS flow, the NG-RAN node needs to provide an indication of whether the PDU Set QoS handling is supported.

At the last meeting, some companies propose to use a RACS-like method to detect the peer node support PDU Set QoS handling. Technically, both of the solutions can work, the RACS solution is more future proof. But it is more complex to use since explicit indicators are not used and relationship information needs to be introduced to link the NGAP-ProtocolI IE-Id with a certain feature. Compared with these two solutions, we prefer to use the solution specified by SA2.

Observation 7: Both of the RACS solution and the SA2 specified solution can work.

Observation 8: The RACS solution is more future proof. But it is more complex to use since explicit indicators are not used and relationship information needs to be introduced to link the NGAP-ProtocolI IE-Id with a certain feature.

Proposal 4: Add an indication of whether the PDU Set QoS handling is supported in the PDU session resource setup response message and the PDU session resource modify response message.

Three possible cases were summarized at the last meeting as below:

Case1: NG-RAN node supports PDU set QoS parameters, and the corresponding QoS flow is setup successfully.

Case2: NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow.

Case3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow.

For case 1, the NG-RAN node can use the new indicator to indicate the QoS flow is setup successfully with PDU Set QoS handling.

Proposal 5: For case 1, the NG-RAN node can use the new indicator to indicate the QoS flow is setup successfully with PDU Set QoS handling.

For case 2 and case 3, the SMF does not care whether the NG-RAN node supports PDU set QoS handling, it only cares whether the QoS flow is setup successfully with PDU Set QoS handling. So for these two cases, the NG-RAN does not transfer the new indicator to indicate the QoS flow is setup as a legacy QoS flow.

Observation 9: The SMF does not care whether the NG-RAN node supports PDU set QoS handling, it only cares whether the QoS flow is setup successfully with PDU Set QoS handling.

Proposal 6: For case 2 and case 3, the NG-RAN does not transfer the new indicator to indicate the QoS flow is setup as a legacy QoS flow.

During Xn based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the source NG-RAN node and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the SMF. If the support indicator is not received, the SMF deactivates the PDU Set PDU Set identification and marking.

Proposal 7: During Xn based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the source NG-RAN node and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the SMF.

Proposal 8: If the support indicator is not received, the SMF deactivates the PDU Set PDU Set identification and marking.

If the QoS flow is accepted with PDU Set based handling, the target NG-RAN also need to send the new indicator of whether the PDU Set based handling is supported to the source NG-RAN node. If the support indicator is not received, the source NG-RAN node does not include the PDU Set information in the GTP-U header of the PDCP SDU forwarded to the target NG-RAN node.

Proposal 9: During Xn based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the Source NG-RAN node and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the source NG-RAN node.

Proposal 10: If the support indicator is not received, the source NG-RAN node does not include the PDU Set information in the GTP-U header of the PDCP SDU forwarded to the target NG-RAN node.

If no PDU Set QoS parameter is received from the source NG-RAN node, and if the target NG-RAN ndoe supports PDU Set based handling, the target NG-RAN node sends the indicator to the SMF to indicate that PDU Set based handling is supported in the target NG-RAN node. If there is PDU Set QoS parameter for the accepted QoS flow, after handover procedure is completed, the SMF may initiate network requested PDU Session Modification procedure to provide the PDU Set QoS parameters of the QoS flow to the NG-RAN and activate the PDU Set identification and marking.

Proposal 11: If no PDU Set QoS parameter is received from the source NG-RAN node, and if the target NG-RAN node supports PDU Set based handling, the target NG-RAN node sends the indicator to the SMF to indicate that PDU Set based handling is supported in the target NG-RAN node.

During NG based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the SMF and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the SMF. If the support indicator is not received, the SMF deactivates the PDU Set PDU Set identification and marking.

Proposal 12: During NG based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the SMF and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the SMF.

Proposal 13: If the support indicator is not received, the SMF deactivates the PDU Set PDU Set identification and marking.

If no PDU Set QoS parameter is received from the SMF, and if the target NG-RAN ndoe supports PDU Set based handling, the target NG-RAN node sends the indicator to the SMF to indicate that PDU Set based handling is supported in the target NG-RAN node. If there is PDU Set QoS parameter for the accepted QoS flow, after handover procedure is completed, the SMF may initiate network requested PDU Session Modification procedure to provide the PDU Set QoS parameters of the QoS flow to the NG-RAN and activate the PDU Set identification and marking.

Proposal 14: If no PDU Set QoS parameter is received from the SMF, and if the target NG-RAN node supports PDU Set based handling, the target NG-RAN node sends the indicator to the SMF to indicate that PDU Set based handling is supported in the target NG-RAN node.

# 3	Conclusion

Based on above analysis, we provide the following observations and proposals.

Observation 1: Taking NG-U as an example, there are three options to transfer the PDU Set information.

- Extend the PDU session container by adding the information in the DL PDU SESSION INFORMATION frame (PDU type = 0)

- Extend the PDU session container by adding the information in a new frame (PDU type = 2)

- Introduce a new container which is in the same level as the existing PDU session container

Observation 2: The benefit of option 1 is that it has less impact on the specification. This option will not bring the compatibility issue since in R17 the MSNP and DL MBS QFI Sequence Number have already been added in the in the DL PDU SESSION INFORMATION frame. The main drawback of this option is that some redundant information also needs to be transferred which may impact the decoding efficiency.

Observation 3: Option 2 is a compromise option between option 1 and option 3. However, the main drawback of this option is that an existing extension header type, i.e. PDU session container, is reused. A unknown extension header type can be easily skipped, but if the contents (i.e. PDU type =2) of a known existing extension header can not be understood, this will cause higher complexity since it needs to go back and then skip the length.

Observation 4: Option 3 provides better compatibility and can improve decoding efficiency if only PDU set related information is transferred. The main drawback of this solution to bring a relatively large impact on the specification.

Proposal 1: Extend the PDU session container to add the PDU Set information in the DL PDU SESSION INFORMATION frame.

Proposal 2: Extend the RAN container to add the PDU Set information in the DL USER DATA frame.

Observation 5: It is necessary to introduce QFI information to distinguish a certain PDU Set belongs to which QoS flow.

Proposal 3: Include the QFI in the F1-U and Xn-U.

Observation 6: According to TS 23.501, if the SMF sends the PDU Set QoS parameter to the NG-RAN node to activate the PDU Set QoS handling for a given QoS flow, the NG-RAN node needs to provide an indication of whether the PDU Set QoS handling is supported.

Observation 7: Both of the RACS solution and the SA2 specified solution can work.

Observation 8: The RACS solution is more future proof. But it is more complex to use since explicit indicators are not used and relationship information needs to be introduced to link the NGAP-ProtocolI IE-Id with a certain feature.

Proposal 4: Add an indication of whether the PDU Set QoS handling is supported in the PDU session resource setup response message and the PDU session resource modify response message.

Proposal 5: For case 1, the NG-RAN node can use the new indicator to indicate the QoS flow is setup successfully with PDU Set QoS handling.

Observation 9: The SMF does not care whether the NG-RAN node supports PDU set QoS handling, it only cares whether the QoS flow is setup successfully with PDU Set QoS handling.

Proposal 6: For case 2 and case 3, the NG-RAN does not transfer the new indicator to indicate the QoS flow is setup as a legacy QoS flow.

Proposal 7: During Xn based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the source NG-RAN node and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the SMF.

Proposal 8: If the support indicator is not received, the SMF deactivates the PDU Set PDU Set identification and marking.

Proposal 9: During Xn based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the Source NG-RAN node and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the source NG-RAN node.

Proposal 10: If the support indicator is not received, the source NG-RAN node does not include the PDU Set information in the GTP-U header of the PDCP SDU forwarded to the target NG-RAN node.

Proposal 11: If no PDU Set QoS parameter is received from the source NG-RAN node, and if the target NG-RAN node supports PDU Set based handling, the target NG-RAN node sends the indicator to the SMF to indicate that PDU Set based handling is supported in the target NG-RAN node.

Proposal 12: During NG based handover, if the target NG-RAN node receives PDU Set QoS parameter for a given QoS flow from the SMF and if the QoS flow is accepted with PDU Set based handling, the target NG-RAN node sends the new indicator of whether the PDU Set based handling is supported to the SMF.

Proposal 13: If the support indicator is not received, the SMF deactivates the PDU Set PDU Set identification and marking.

Proposal 14: If no PDU Set QoS parameter is received from the SMF, and if the target NG-RAN node supports PDU Set based handling, the target NG-RAN node sends the indicator to the SMF to indicate that PDU Set based handling is supported in the target NG-RAN node.

4	Text Proposal for BL CR TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node.

** Unchanged text skipped **

For each QoS flow which has been successfully established, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [9]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store this information and, if supported, use it for RAN part delay reporting.

For each QoS flow requested to be setup, if the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, store the received PDU Set QoS Parameters in the UE context and use it as specified in TS 23.501 [9], and shall, if supported, include the PDU Set QoS Handling Indicator IE in the PDU SESSION RESOURCE SETUP RESPONSE message.

For each QoS flow requested to be setup the NG-RAN node shall take into account the received QoS Flow Level QoS Parameters IE. For each QoS flow the NG-RAN node shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

** Unchanged text skipped **

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.3	PDU Session Resource Modify

#### 8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE MODIFY REQUEST message to the NG-RAN node.

** Unchanged text skipped **

For each PDU session included in the PDU Session Resource Modify Request List IE:

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, based on the QoS Flow Level QoS Parameters IE, the NG-RAN node may establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly. The NG-RAN node shall associate each QoS flow accepted to setup or modify which is not associated with an MBS QoS flow with a DRB of the PDU session. The associated DRB for the QoS flow accepted to modify may not change. If the PDU Set QoS Parameters IE is received, the NG-RAN node shall, if supported, store the received PDU Set QoS Parameters in the UE context and use it as specified in TS 23.501 [9], and shall, if supported, include the PDU Set QoS Handling Indicator IE in the PDU SESSION RESOURCE SETUP RESPONSE message..

-	For each QoS flow, if the Redundant QoS Flow Indicator IE is included, the NG-RAN node shall, if supported, store it and consider it for the redundant transmission as specified in TS 23.501 [9].

** Unchanged text skipped **

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration and associated security;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Context in the UE context and take it into use as defined in TS 33.501 [13];

-	if supported, store the received UE Slice Maximum Bit Rate List in the UE context and use the received UE Slice Maximum Bit Rate List for each S-NSSAI for the concerned UE as specified in TS 23.501 [9].

-	if supported, store the received PDU Set QoS parameters in the UE context and use it as specified in TS 23.501 [9].

-	if supported, include the PDU Set QoS Handling Indicator IE in the PDU SESSION RESOURCE SETUP RESPONSE message.

Upon reception of the UE History Information IE, which is included within the Source to Target Transparent Container IE of the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF. Upon reception of the PATH SWITCH REQUEST message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Path Switch Request Transfer IE to the SMF associated with the concerned PDU session.

*// skip unchanged part //*

If the 5G ProSe Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, update its ProSe authorization information for the UE accordingly. If the 5G ProSe Authorized IE includes one and more IEs set to "not authorized", the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant 5G ProSe service(s).

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported:

-	replace the previously provided 5G ProSe UE PC5 Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 QoS Parameters IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, use it as defined in TS 23.304 [47].

If the PDU Set QoS Handling Indicator IE is included in the PATH SWITCH REQUEST message, the SMF shall, if supported, use it as specified in TS 23.501 [9].

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "single RRC connected state report" and the UE is in RRC_CONNECTED state, the NG-RAN node shall, if supported, send one RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE.

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "single RRC connected state report" and the UE is in RRC_INACTIVE state, the NG-RAN node shall, if supported, send to the AMF one RRC INACTIVE TRANSITION REPORT message plus one subsequent RRC INACTIVE TRANSITION REPORT message when the RRC state transitions to RRC_CONNECTED state.

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send one RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE and subsequent RRC INACTIVE TRANSITION REPORT messages to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

Interactions with PDU Session Resource Notify procedure:

If the QoS related parameters (e.g. the CN Packet Delay Budget Downlink IE or the CN Packet Delay Budget Uplink IE) are included in the Path Switch Request Acknowledge Transfer IE of the PATH SWITCH REQUEST ACKNOWLEDGE message, but can not be succesfully accepted by the NG-RAN node, the NG-RAN node should continue to use the old values received from the source NG-RAN node, if any. The NG-RAN node shall, if supported, send the PDU SESSION RESOURCE NOTIFY message to notify the AMF.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.99	Associated QoS Flow List

This IE indicates the list of QoS flows associated with e.g. a DRB or UP TNL endpoint.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Associated QoS Flow Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >QoS Flow Mapping Indication | O |  | ENUMERATED (ul, dl, …) |  | - |  |
| >Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | ignore |
| >PDU Set QoS Handling Indicator | O |  | 9.3.1.z | This IE indicates if PDU Set QoS Handling is supported for a QoS flow. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.x	PDU Set QoS Parameters

This IE defines the PDU Set QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Packet Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.135 | PDU Set Delay Budget as specified in TS 23.501 [9]. |
| PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.81 | PDU Set Error Rate as specified in TS 23.501 [9]. |
| PDU Set Integrated Handling Information | O |  | ENUMERATED (true, false, …) | PDU Set Integrated Handling Information as specified in TS 23.501 [9]. |

9.3.1.y	N6 Jitter Information

This IE indicates the jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Information | O |  | FFS | FFS |

9.3.1.z	PDU Set QoS Handling Indicator

The PDU Set QoS Handling Indicator IE indicates if the PDU Set QoS Handling is supported for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set QoS Handling Indicator | M |  | ENUMERATED (supported, …) | Indicates that PDU Set QoS handling is supported |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.2.13	QoS Flow List with Data Forwarding

This IE is used to provide a list of QoS flows with indication if forwarding is accepted.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Item with Data Forwarding |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >Data Forwarding Accepted | O |  | 9.3.1.62 | This IE is included for the QoS flows in the PDU session to be forwarded over the PDU session forwarding tunnel. It may be included for the QoS flows in the PDU session to be forwarded over the DRB forwarding tunnel(s). | - |  |
| >Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | ignore |
| >PDU Set QoS Handling Indicator | O |  | 9.3.1.z | This IE indicates if the PDU Set QoS Handling is supported for a QoS flow | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node. | - |  |
| QoS Flow Add or Modify Response List |  | 0..1 |  |  | - |  |
| >QoS Flow Add or Modify Response Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | Ignore |
| >>PDU Set QoS Handling Indicator | O |  | 9.3.1.z | This IE indicates if the PDU Set QoS Handling is supported for a QoS flow | YES | Ignore |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. | - |  |
| QoS Flow Failed to Add or Modify List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Additional NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer corresponding to the modified UPF endpoint received in the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node for the redundant transmission. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Additional Redundant NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer for delivery of redundant DL PDUs corresponding to the modified UPF endpoint(s) received in the UL NG-U UP TNL Modify List IE of the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| Secondary RAT Usage Information | O |  | 9.3.1.114 |  | YES | ignore |
| MBS Support Indicator | O |  | 9.3.1.210 |  | YES | ignore |
| MBS Session Setup or Modify Response List | O |  | MBS Session Setup Response List 9.3.1.213 |  | YES | ignore |
| MBS Session Failed to Setup or Modify List | O |  | MBS Session Failed to Setup List 9.3.1.214 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.4.8	Path Switch Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| DL NG-U TNL Information Reused | O |  | ENUMERATED (true, …) | Indicates that DL NG-U TNL Information has been reused. | - |  |
| User Plane Security Information | O |  | 9.3.1.60 |  | - |  |
| QoS Flow Accepted List |  | 1 |  | QoS flows associated with the DL NG-U UP TNL Information IE. | - |  |
| >QoS Flow Accepted Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>PDU Set QoS Handling Indicator | O |  | 9.3.1.z | This IE indicates if the PDU Set QoS Handling is supported for a QoS flow | YES | ignore |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of redundant DL PDUs. | YES | ignore |
| Redundant DL NG-U TNL Information Reused | O |  | ENUMERATED (true, …) | Indicates that Redundant DL NG-U TNL Information has been reused. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of Redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Support Indicator | O |  | 9.3.1.210 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>