# 3GPP TSG-RAN WG3 Meeting #121	R3-234111

# 21-25 August 2023, Toulouse, France

Agenda Item:	10.2.3

Source:	CATT

Title:	Discussion on for RACH enhancements

Document for:	Discussion

- Introduction

In the RAN3#120 [1], we achieved some agreements for RACH report.

Not consider the NG and S1 forwarding of RA report.

Include the “PSCell list” optionally in addition to the “RACH Report Container” in ACCESS AND MOBILITY INDICATION over Xn.

In this contribution, we further discuss the left open issue and provide our proposals to try to close this topic.

- Discussion

2.1 RACH partitioning

There are five options are listed as below to address the open issue about RACH report information enhancements for RACH partitioning.

RACH report information enhancements:

Option 1: Include the feature priority

Option 2: Include the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition)

Option 3: Include the time between RACH access that led to the generation of a RA Report and reporting of the RA Report

Option 4: The network controls the UE to report RA information

Option5: Mobility information

We understand that the feature priorities (FeaturePriority-r17) are broadcast in SIB1and RACH partition configuration information (FeatureCombinationPreamblesList-r17 in RACH-ConfigCommon) is also a per cell configuration. They are common configurations for all UEs in one cell. However, RACH report is not a real-time report. Network may change the feature priorities and/or RACH partition configuration related information over time. Hence network may not know RACH partitioning information at the time of RACH report generated.

Option 1 and option 2 can be considered together. It avoids network to store the "old” RACH partitioning related configuration and network can optimize RACH resource for all recorded RACH report entries.

Option 3 can also acquire RACH partitioning related information while it requires network to remember the configuration for a long time. Also, UE has to calculate running time between the generations of a RACH Report and reporting of the RACH Report, it increases the complexity of UE. Companies concern that such information will not be changed frequently, it would increase Uu interface load if includes them in RACH report. While it is note that RA-InformationCommon which has some cell level information e.g., locationAndBandwidth-r16 and subcarrierSpacing-r16 are already included in RACH report.

Observation 1: We prefer option 1 and option 2 to option 3 because option 3 has to calculate running time in UE and network has to store the configuration for a long time. While UE reports cell level information is already supported in RACH report.

Option 4 can only perform RACH optimization based on the latest feature priority/RACH partition configuration. However, UE has to delete the RACH report which related to the out of date feature priority/RACH partition configuration by UE’s implementation because network may not know which RACH report entries are associated with the latest/current feature priority and RACH partition configuration. It against the principle that UE can store RACH report for 48h and it will introduce additional UE operation.

Observation 2: Option 4 introduces additional UE impact to filter out the old RACH report entries which are associated with old feature priority/RACH partition configuration. Also, the RACH optimization of the out of data configuration cannot be performed.

Option 5 is feasible but it should take RAN2’s discussion into account. In the last meeting, RAN3 sends a LS to RAN2 to ask whether option 3-like solution and option 5-like solution for SPR are feasible. RAN3 can consider SPR solution and RACH report solution together and continue wait for RAN2’s progress.

Proposal 1: RAN3 supports UE signals feature priority/RACH partition configuration to network (option 1/2). Whether also support UE reports Mobility information (option 5) should wait for RAN2 progress.

2.2 RA report forwarding in inter-MN handover case

FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report.

RAN3 identified a case that it may lack of X2/Xn interface between third node (MN that retrieves the RA Report) and PScell. In our understanding, the implementation can address part of this issue and we do not see a strong requirement to address issue of lack of interface between nodes for following considerations.

- MN that retrieves the RA Report can store SN NR container and send it to relevant SN when this MN connects to the SN associated to the PSCells, and the storage time in MN that retrieves the RA Report implementation.

- MN that retrieves the RA Report can read the UE history information. It can know which MN (PCell) that the SN associated to the PSCells connects to when the SN RACH report entry is generated.

- MN that retrieves the RA Report can read the neighbor cell information. It can know currently which MN (PCell) that the SN associated to the PSCells connects to.

- The same issue also exist in R17 that there may be no Xn interface between gNB that retrieves the RA Report and gNBs associated to the cell in RA Report.

Proposal 2: Do not see a strong requirement to address the case of lack of interface between nodes for RACH report forwarding in inter-MN handover case.

- Conclusion

Observation 1: We prefer option 1 and option 2 to option 3 because option 3 has to calculate running time in UE and network has to store the configuration for a long time. While UE reports cell level information is already supported in RACH report.

Observation 2: Option 4 introduces additional UE impact to filter out the old RACH report entries which are associated with old feature priority/RACH partition configuration. Also, the RACH optimization of the out of data configuration cannot be performed.

Proposal 1: RAN3 supports UE signals feature priority/RACH partition configuration to network (option 1/2). Whether also support UE reports Mobility information (option 5) should wait for RAN2 progress.

Proposal 2: Do not see a strong requirement to address the case of lack of interface between nodes for RACH report forwarding in inter-MN handover case.

- Reference

[1] RAN3#120, Chair Notes.

- Annex

# 3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

# 21-25 August 2023, Toulouse, France

Title:	[Draft] LS on RACH enhancement

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name: 		Liu Ziqiao

E-mail Address:	liuziqiao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

RACH partitioning

RAN3 see benefits of enhancing the RACH report to include feature priorities and feature combination preambles for RACH partitioning. RAN3 kindly ask RAN2 to introduce such enhancement in RACH report and provide feedback if it is not acceptable from RAN2 perceptive. In addition, whether to support “mobility information” solution for RACH partitioning is also discussed in RAN3 but RAN3 think it should base on the conclusion of R3-233380 in RAN2.

SN RA Report for EN-DC and (NG)EN-DC

RAN3 do not see the requirement of enhancing specification to address the case of lack of Xn/X2 interfaces between the MN that retrieves the RA Report and SN associated to the PSCells. It means that there is no further RAN2 impact identified by RAN3 about this topic.

- 2. Actions:

RAN3 respectfully asks RAN2 to take above into account and to provide feedback if needed.

3. Date of next TSG RAN WG3 meetings:

TSG-RAN3 Meeting #121bis                          09 Oct - 13 Oct 2023	Xiamen

TSG-RAN3 Meeting #122                              13 Nov - 18 Nov 2023	Chicago