3GPP TSG-RAN WG3 Meeting #121	R3-234145

Toulouse, France, 21st – 25th August, 2023

Agenda item:	13.5

Source: 	Huawei

Title: 	Discussion on the IAB-node de-authorization handling

Document Type:	Discussion

1. Introduction

RAN3 received an LS from SA2 in the last meeting [1], which requested RAN3 to answer some questions related to the IAB-node de-authorization handling. The key part of the LS is given as follows:

SA2 would like to check whether this is a correct understanding:

- The gNB attempts to perform handover of all the UEs served by the MBSR

- When the step 1 completes, the gNB releases the F1-C

Q1: is the understanding above correct?

Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

In this contribution, we provide our views on the SA2’s questions, and give the draft reply LS in the Annex.

2. Discussion

We think SA2’s understanding is helpful to reduce the impact to the served UEs caused by the IAB de-authorization. If the MT’s CU and the DU’s CU is same, there is no obstacles to orderly perform the UE handover and release the F1-C. If the MT’s CU and the DU’s CU are different, to be UE friendly, the DU’s CU should be informed the current authorization result, and then also to orderly handover the UEs and release the F1-C, which is explained in our paper of AI 13.2. We agree to SA2’s understanding which is beneficial, but it is still not mandatory. Thus, it is observed:

Observation 1: For the not authorized case, it is suggested but not mandatory that the donor CU attempts to perform handover of all UEs served by mobile IAB and then release the F1 connection towards the mobile IAB node.

After the DU’s CU handover all the UEs served by the mobile IAB-DU and releases F1 connection to the IAB-DU, the MT’s CU shall release the BH resources (including the default BH configuration) which were used for the mobile IAB in its topology. If the mobile IAB node is authorized again, the MT’s CU will know the updated authorization results from AMF, and can provide the default BAP configuration to the mobile IAB-MT, for the consequent F1 setup initiated from the mobile IAB-DU. From the mobile IAB-node’s perspective, it will initiate the F1 setup procedure only when it is provided with the default BH configuration. So, there is no motivation to indicate the DU to not try F1 connection again when perform F1 release.

Observation 2: If the mobile IAB node is authorized, the IAB-donor-CU can provide the default BH configuration to the mobile IAB-MT via RRC message, after obtaining such default BH configuration, the mobile IAB-node can initiate the F1 setup procedure.

Observation 3: There is no motivation to indicate the DU to not try F1 connection again when perform F1 release.

Based on the observations above, when the default BH configuration is received by the mobile IAB-node, it can determine it is authorized and initiate the F1 setup procedure. It is proposed:

Proposal: After receiving the default BAP configuration, the mobile IAB-node can initiate the F1 setup procedure towards the IAB-donor-CU.

3. Conclusion

This paper presents our view on the IAB-node de-authorization handling for the SA2 LS. The following observations and proposal are given:

Observation 1: For the not authorized case, it is suggested but not mandatory that the donor CU attempts to perform handover of all UEs served by mobile IAB and then release the F1 connection towards the mobile IAB node.

Observation 2: If the mobile IAB node is authorized, the IAB-donor-CU can provide the default BH configuration to the mobile IAB-MT via RRC message, after obtaining such default BH configuration, the mobile IAB-node can initiate the F1 setup procedure.

Observation 3: There is no motivation to indicate the DU to not try F1 connection again when perform F1 release.

Proposal: After receiving the default BAP configuration, the mobile IAB-node can initiate the F1 setup procedure towards the IAB-donor-CU.

It is also proposed:

- Agree the LS back in Annex.

4. Reference

- R3-233730, LS on IAB-node De-authorization handling.

3GPP TSG-RAN3 Meeting #121	R3-xxxxxx

Toulouse, France, 21st – 25th August 2023

Title:	[DRAFT] Reply LS on the IAB-node de-authorization handling

Response to:	S2-2308158/R3-233730

Release:	Rel-18

Work Item:	VMR

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	-

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 would like to thank SA2 for their LS on the IAB-node de-authorization handling (S2-2308158/R3-233730).

SA2 has the following request to RAN3:

SA2 would like to check whether this is a correct understanding:

- The gNB attempts to perform handover of all the UEs served by the MBSR

- When the step 1 completes, the gNB releases the F1-C

Q1: is the understanding above correct?

Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

Regarding the request by SA2, RAN3 replies:

For Q1: The understanding is correct. For the not authorized case, the donor CU can attempt to handover all UEs served by mobile IAB and then release the F1 connection towards the mobile IAB node.

For Q2: The mobile IAB node can initiate the F1 setup procedure again if receiving new default BAP configuration. There is no motivation to indicate the DU to not try F1 connection again when perform F1 release.

RAN3 asks SA2 to take the above feedback into account.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	#121bis – October 9 to 13, 2023      Xiamen, China

TSG-RAN3 Meeting	#122 – November 13 to 17, 2023     Chicago, USA