3GPP TSG-RAN3 Meeting #121	R3-234255

# Toulouse, France, Aug 21st – 25th 2023

Agenda item:	23.2.2

Source:	Xiaomi

Title:	Discussion on LPHAP positioning enhancement

Document for:	Discussion and Decision

1	Introduction

In this contribution, we’d like to discuss the following aspects on LPHAP positioning according to the previous discussion and RAN2 progress.

Area-specific SRS configuration

Area-specific SRS re-configuration

pre-configuration SRS

2	Discussion

2.1 Area-specific SRS configuration

For Area specific SRS, RAN2 already has agreement and send the following LS [1] to RAN3

For the SRS configuration with validity area, RAN2 has agreed on the following:

RAN2 consider that the LMF should determine the area-specific SRS configuration.  Details are up to RAN3.

In our view, the SRS configuration for a validity area (i.e. “area SRS” for short) means that all the gNBs in the area needs to reserve the SRS resource for the UE. It’s not appropriate to let one single gNB to decide SRS resource for other gNBs, LMF is more appropriate than a single gNB to make the area specific SRS configuration, besides, RAN2 already agreed that the LMF should determine the area specific SRS configuration.

Before LMF make the decision, LMF needs to know the available SRS resources for multiple gNBs, so that it can determine the area-specific SRS for a validity area, to make sure all the gNBs in this area have corresponding SRS resource to be reserved.

One may say that LMF can obtain the available SRS resource from multiple gNBs by OAM, this means that the multiple SRS resources should be statically reserved in multiple gNBs, gNB cannot use those resources, which makes the resources wasted and it’s not flexible, in our understanding, radio resource such as SRS can be dynamic scheduled by gNB, it’s more efficient to let the gNB notify the available SRS resource by signalling according to its resource status, thus signalling procedure between LMF and gNB to exchange available SRS resource is needed and beneficial. And the procedure to support LMF obtains available SRS from different gNBs can be similar to TRP information exchange procedure, the aim is to collect available SRS resource per cell, so the procedure can be non-UE associated, a new procedure or TRP information exchange can be used for this purpose.

On the other hand, after LMF determine the area specific SRS, LMF will recommend the serving gNB of the UE, the serving gNB will make the final decision and send UE and LMF the final configuration, this procedure can be similar to positioning information exchange procedure, and this procedure can be reused.

At last, the LMF notifies the gNBs in the area to reserve the SRS resource, in our views, the gNBs in the area not only needs to reserve the resource, but also needs to be ready to perform the measurement, it is possible that the UE is far away from e.g. gNB1 (one of the gNBs in the area), but gNB1 needs to know how to perform the measurement in case the UE moves in to the gNB1’s coverage, with this in mind, we think anyway the measurement procedure should be used, if the area specific SRS configuration is in the measurement request, the gNB receives the measurement request should reserve the resource and get ready to perform the SRS measurement and report to LMF if any measurement report, and don’t report to LMF if no measurement report.

Based on the above analysis, the possible stage2 procedure can be as below.

Figure 1 LMF and NG-RAN node coordination for area specific SRS configuration

Step 1, LMF collect available SRS resources from multiple NG-RAN nodes.

Step 3, if LMF determine the area specific SRS based on the available SRS resource, LMF sends the suggested area SRS to the serving NG-RAN node of UE, the serving NG-RAN node can make the final decision.

Step 4-5, if the serving NG-RAN node send the area SRS configuration to UE, it should notify LMF about it.

Step 6, LMF send measurement request including the area SRS configuration to the involved NG-RAN nodes. The involved gNB will reserve the resource and get ready to perform the measurement.

Proposal 1, RAN3 confirms that the LMF should determine the area-specific SRS configuration and the serving gNB should make the final decision.

Proposal 2, the positioning information exchange procedure can be used as baseline for area-specific SRS configuration.

Proposal 3, LMF needs to collect available SRS resource from the gNBs in a specific area to determine the area specific SRS.

Proposal 4, RAN3 discuss either to use new procedure or TRP information exchange procedure to collect available SRS resource for LMF.

2.2 Area-specific SRS re-configuration

According to the previous discussion, the area-specific SRS configuration needs to be re-configured when UE leaves the validity area.

In this case, LMF needs to know the UE moves out of the area and the new serving cell ID, so that the LMF can initiate new positioning information procedure with the new serving gNB to send new area specific SRS configuration to UE.

From RAN3 perspective, we need to consider the scenario that the new serving gNB is not the anchor gNB.

The following is one possible stage 2 procedure.

Figure 2 Area-specific SRS re-configuration

In the above procedures, there are 3 phases:

Phase 1, UE resumes to a new serving gNB and new serving gNB retrieve the UE context if needed, in this phase, the existing UE context retrieve procedure can be reused,

Phase 2, LMF knows UE moves out the previously configurated validity area and the current serving cell ID of the UE.

Phase 3, LMF initiate new procedure with the new serving gNB to send new SRS configuration for a new validity area, in this phase, the normal area specific SRS configuration can be reused.

For phase 2, how the LMF knows UE’s new cell serving ID and further knows that UE moves out of the previously configured area is an issue, we think there are three possible options:

- option A, before path switch, the anchor gNB notifies the LMF the new serving gNB of the UE.

- option B, during path switch, the AMF knows that the cell is changed, and AMF can notify LMF the new serving gNB of UE, this option needs to check with SA2.

- option C, after path switch, the serving gNB can notify the LMF, this can rely on R17 enhancement for inactive UE positioning, i.e. the positioning information from anchor gNB.

Observation 1, UE context retrieve procedure can be re-used if UE leaves the validity area and send request to the serving gNB to obtain the new configuration.

Observation 2, the normal area specific SRS configuration can be re-used for re-configuring UE with SRS configuration for a new validity area.

Proposal 5, RAN3 discuss and down-select the following options to notify LMF the serving cell of the UE is changed.

- option A, the anchor gNB notifies the LMF.

- option B, the AMF notifies the LMF.

- option C, the serving gNB notifies the LMF.

2.3 Pre-configuration SRS

Pre-configuration SRS is discussed in RAN2, in our understanding, the pre-configuration SRS is obtained by UE before the actual positioning measurement, if needed, UE select at least one SRS resource in the preconfigured SRS according to its radio situation, but how the UE obtains the preconfigured SRS is pending to RAN2 discussion.

Agreement:

RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also.  FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request.  This agreement does not imply that the UE will be allowed to transmit autonomously.

And according to RAN2 agreement above, if the UE decide to select the preconfigured SRS for positioning, it will indicate the serving gNB the request for preconfigured SRS via at lease msg3/msgA, in case the serving gNB is different from UE’s anchor gNB, the serving gNB can obtains the positioning info from the anchor gNB via existing procedure for R17 inactive positioning, the serving gNB decide the detail SRS configuration according to the UE report, and send it to UE via RRC release message and to LMF via NRPPa positioning update message. In CU-DU split architecture, a preconfigured SRS ID may be introduced over F1AP, to let the gNB-DU to assign the detail SRS configuration according to the preconfigured SRS ID the UE reports.

Figure 3 Pre-configuration SRS

The above flow is our understanding on pre-configuration SRS, we think as some SRS information is determined by the serving gNB according to the radio conditions of the UE, the pre-configuration SRS may not include all the SRS information, so the full SRS configuration should be decided and provided by the serving gNB after UE request to activate the selected SRS. The positioning information exchange procedure can be re-used for this purpose.

It is foreseen that there may be F1/NRPPa impacts to support pre-configuration SRS, but the details on how to pre-configure the SRS, the behaviours of UE and gNB are not clear right now, which are pending to RAN2 progress.

Observation 3, the pre-configured SRS information may not include all the SRS configuration.

Observation 4, the serving gNB will provide the full SRS configuration after UE send the request to activate the selected SRS.

Proposal 6, RAN3 discuss the possible stage 2 call flow to support pre-configuration SRS.

3	Conclusion

In this contribution, we had the following observations and proposals.

Area-specific SRS configuration

Proposal 1, RAN3 confirms that the LMF should determine the area-specific SRS configuration and the serving gNB should make the final decision.

Proposal 2, the positioning information exchange procedure can be used as baseline for area-specific SRS configuration.

Proposal 3, LMF needs to collect available SRS resource from the gNBs in a specific area to determine the area specific SRS.

Proposal 4, RAN3 discuss either to use new procedure or TRP information exchange procedure to collect available SRS resource for LMF.

Area-specific SRS re-configuration

Observation 1, UE context retrieve procedure can be re-used if UE leaves the validity area and send request to the serving gNB to obtain the new configuration.

Observation 2, the normal area specific SRS configuration can be re-used for re-configuring UE with SRS configuration for a new validity area.

Proposal 5, RAN3 discuss and down-select the following options to notify LMF the serving cell of the UE is changed.

- option A, the anchor gNB notifies the LMF.

- option B, the AMF notifies the LMF.

- option C, the serving gNB notifies the LMF.

pre-configuration SRS

Observation 3, the pre-configured SRS information may not include all the SRS configuration.

Observation 4, the serving gNB will provide the full SRS configuration after UE send the request to activate the selected SRS.

Proposal 6, RAN3 discuss the possible stage 2 call flow to support pre-configuration SRS.

4	References

[1] R2-2306841 LS on LPHAP