3GPP TSG-RAN WG3 Meeting #121	R3-234423

Toulouse, France, 21 – 25 Aug 2023

Agenda Item:	11.3

Source:	ZTE, China Unicom

Title:	(TP to BL CR of 37.340) QoE in NR-DC

Document for:	Discussions & Approval

1	Introduction

This paper provides TP to BL CR of 37.340 based on the discussion in R3-234421.

2	TP to BL CR of 37.340

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

13	Other aspects

<unchaned text omitted>

13.x	Application Layer Measurement Collection

13.x.1	Overview

The QoE Measurement Collection function as described in TS 38.300 [3] is extended to address NR-DC operation. The requirements on the gNB provided in TS 38.300 [3] apply to the MN, together with additional requirements on MN and SN provided in following sub-clauses.

13.x.2	QoE Measurement Configuration

13.x.2.1	QoE Measurement Collection Activation and Reporting

For a UE in NR-DC, the MN and the SN may coordinate QoE measurement collection activation and reporting as follows:

For signalling-based QoE activation and management-based QoE activation received by the MN from the AMF/OAM, the MN sends the configuration to the UE using SRB1 and notifies the SN about the configuration.

For management-based QoE activation received directly by the SN from the OAM, the SN may perform UE selection. For a selected UE, the SN indicates to the MN QoE reference of the management-based QoE session and whether it is going to receive the corresponding reports via the MN (using SRB4) or using SRB5. The MN determines whether the MN or the SN sends the QoE configuration to the UE and notifies the SN accordingly.

For all the QoE configuration activation, it is MN allocates the measurement application layer id and notifies to the SN.

The network indicates to the UE whether to send QoE reports via SRB4 or SRB5, per QoE reference. The SRB for QoE reporting can be changed during the application session. The command for changing the SRB used for reporting may be sent to the UE by the node that configured that specific QoE configuration.

The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement configuration.

If the MN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should indicate to the SN the QoE reference, the MCE IP address and the measConfigAppLayerId.

If the SN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the MN, then, if the SN decides that the MN forwards the reports directly to the MCE, the SN should indicate to the MN the QoE reference and the MCE IP address.

13.x.3 Reporting leg switch

13.x.3.1 QoE reporting leg switch in overload

When the node that receives QoE reports experiences overload, it could indicate to the peer node, as a request to switch the QoE reporting leg to the other network node. If the request is approved, the node that configures QoE to UE may send an RRC message to indicate the switch of reporting leg to the UE.

An example of MN in overload is shown below.

Figure 13.x.3.1-1 An example of the procedures for QoE reporting leg switch

- The QoE reports are sent to the MN and forwarded to the MCE, after the QoE configuration has been activated.

- The MN experiences overload.

- The MN sends the SN addition request or SN modification request message to the SN, including a request to change the reporting leg to the SN.

- The SN sends SN addition response or SN modification response message to the MN, including a response to the reporting leg switch request.

- If the SN approves the request for switching the reporting leg, the node that configures the UE with QoE may send an RRC message to the UE, indicating the reporting leg.

NOTE: The node that configures the UE with QoE might be MN or SN.

- The UE sends the QoE reports to the other node via the other reporting leg.

NOTE: If the reporting leg switch request is rejected, a pause indication shall be sent to the UE.

13.x.3.2 RAN visible QoE reporting leg switch

The reporting leg of RAN visible might be changed during the session, as described in section 21.x1 in 38.300[3].

When the node that configures RAN visible QoE determines that the bearer(s) for the application session data flow(s) may be provided by the other node, it should send an indication to the other node, as an inquiry about whether to change the reporting leg of RAN visible QoE. In the response message, the node that provide bearer(s) may reject to receive the RAN visible QoE reports, or to request to receive the RAN visible QoE reports directly. The recommended RAN visible QoE configuration parameters may also be provided in the response.

An example is shown below,where it is the MN configures RAN visible QoE to UE:

Figure 13.x.3.1-2 An example of the procedures for RAN visible QoE reporting leg switch

0a.  The MN sends RAN visible QoE configuration to the UE in an RRC message.

0b.  The UE sends RAN visible QoE report(s) to the MN.

- The MN determines that some bearer(s) for the application session are provided by the SN.

- The MN sends the SN modification request message to the SN, including the RAN visible QoE session indication.

- The SN sends the SN modification response message to the MN, with the response to the RAN visible QoE session indication. The response may include a rejection to receive RAN visible QoE reports, or a request to receive the RAN visible QoE reports directly, and may also include some recommended RAN visible QoE configuration parameters.

- If the MN receives the response with a request to receive the RAN visible QoE reports directly, the MN sends an RRC message to the UE with the RAN visible QoE reporting indication.

- The UE sends the RAN visible QoE reports directly to the SN.

NOTE: If the SRB5 is not configured, the RAN visible QoE reports can be sent on the SRB4 from the UE via the MN to the SN.

- After the SN is indicated that it provides the bearers for application session, the SN may send RAN visible QoE configuration to the UE direcly.

- The UE could send the corresponding RAN visible QoE reports to the SN directly.

13.x.4	QoE Measurement Continuity for Mobility

Editor’s note: Content for this clause is FFS. QMC continuity during mobility in NR-DC should be discussed after the baseline solution for QMC in NR-DC is in place.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>