3GPP TSG-RAN WG3 #121						               R3-23xxxx

21th – 25th Aug 2023

Toulouse, France

Agenda Item:	3

Source: 	RAN3 Chair

Title: 	Agenda

Document for:	Approval

# Agenda

| Tdoc | Title |  | Comments |  |
| --- | --- | --- | --- | --- |
| # 1. Opening of the meeting |  |  |  |  |
| # 2. Reminders |  |  |  |  |
| ## 2.1. IPR Declaration https://www.3gpp.org/3gpp-calendar/89-call-for-ipr-meetings |  |  |  |  |
| I draw your attention to your obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP. Delegates are asked to take note that they are thereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become, essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (See: http://ipr.etsi.org/). |  |  |  |  |
| ## 2.2. Statement of Antitrust Compliance https://www.3gpp.org/about-3gpp/legal-matters/21-3gpp-calendar/1616-statement-of-antitrust-compliance |  |  |  |  |
| I also draw your attention to the fact that 3GPP activities are subject to all applicable antitrust and competition laws and that compliance with said laws is therefore required of any participant of this TSG/WG meeting including the Chair and Vice Chairs. In case of question I recommend that you contact your legal counsel. The leadership shall conduct the present meeting with impartiality and in the interests of 3GPP. Furthermore, I would like to remind you that timely submission of work items in advance of TSG/WG meetings is important to allow for full and fair consideration of such matters. |  |  |  |  |
| ## 2.3. Responsible IT Behavior http://www.3gpp.org/ftp/PCG/PCG_27/DOCS/PCG27_13r1.zip |  |  |  |  |
| We all share meeting IT resources with one another. Delegates should restrict their IT usage to things which are essential for the meeting, and they: 1. shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. 2. shall not engage in non-work-related activities that consume excessive bandwidth or cause significant network performance degradation. And most importantly: 1. DON’T place your WiFi device in ad-hoc mode; 2. DON’T set up a personal hotspot in the meeting room; 3. DO try 802.11a if your device supports it; 4. DON’T manually allocate an IP address; 5. DON’T stream video, play online games, or download huge files; 6. DON’T use packet probing software (e.g., packet sniffers or port scanners) which clogs the local network. |  |  |  |  |
| ## 2.4. Additional reminders |  |  |  |  |
| 1. All agreed CRs must be provided during the meeting week, that is, BEFORE the end of the meeting. In order to continue with the principle of “agreed unseen” CRs, please make sure that all such CRs are uploaded in time and that they contain exactly the agreed changes. 2. During physical meetings, prefer face-to-face offline discussion to e-mail discussion. 3. Come-Backs (CB), server, reflector and e-mail discussions: When a CB is set up, e.g.: CB: # 1_Name - topics of the offline discussion (Company Owner - moderator) Rev in R3-xxxxxx Summary of offline disc R3-xxxxxy - Create a folder in “Inbox/Drafts/1_Name” with the assigned CB number (1) and name; - Upload all drafts, corrections, revisions, etc. in the same folder “Inbox/Drafts/1_Name”; - Avoid sending drafts via e-mail or on the reflector! - When sending e-mails, do not attach any document, and please minimize e-mail discussion (e.g. it is enough to announce start of discussion, availability of drafts on server, support for a document, discussion conclusion). - It is highly beneficial if the summary of offline discussion contains proposals for “official” group conclusions, e.g. “propose to agree R3-xxxxxx”, “propose to agree that….”, “no agreement”, “to be continued”, etc. 3bis. For e-meetings, the above also applies for e-mail discussions set up by the Chair before the meeting, e.g.: CB # 2_E-mail_Name - open-ended topics of the e-mail discussion (Company Owner - moderator) Summary of offline disc R3-xxxxxx …etc. 4. To encourage the use of pCRs, if there are discussion papers and pCRs from the same company on the same topic, only the pCRs will be treated. 5. Papers submitted to the wrong AI will not be treated. 6. When subsections are available, please do not submit papers to the “top level” AI. If you think none of the available subsections fits your contribution, then it should go to the “Others” subsection. Any papers submitted to the “top level” AIs should not expected to be treated. 7. To save time, incoming LSs which have no action for RAN3 will not be treated unless they are flagged to the Chair before the start of the meeting. 8. QUOTAS – Each company may submit up to a certain number of contributions to the Agenda Item where this number appears. This number applies to the sum of the Tdocs submitted to all the sub-Agenda Items. If e.g. QUOTA: 5 appears in AI 10.x, a company may submit up to 5 contributions to AI 10.x in any combination: e.g. up to 4 to 10.x.1.1 and up to 1 to 10.x.1.2, or up to 3 to 10.x.1.1 and up to 2 to 10.x.1.2, and so on. Please see also at the end of this document. Quota rules are to be maintained R3-221096 (revised from R3-200133) and continue to be the basis for working with quotas in RAN3. Some suggestions for better RAN3 meetings can also be found here. RAN3#121 Tdoc submission deadline: 7am UTC of the 11th of Aug RAN3#121 meeting registration deadline: 7am UTC of the 14th of Aug (align with 3GU) |  |  |  |  |
| # 3. Approval of the Agenda |  |  |  |  |
| R3-233701 | RAN3#121 Meeting Agenda (RAN3 Chair) |  | Agenda Approved Any changes on meeting minutes after meeting should be highlighted and distributed over RAN3 email reflector, and any changes require consensus. |  |
| # 4. Approval of the minutes from previous meetings |  |  |  |  |
| R3-233702 | RAN3#120 Meeting Report (ETSI-MCC) |  | Report Approved |  |
| # 5. Documents for immediate consideration Recording of GoToWebinar/GotoMeeting sessions of the present meeting is strictly prohibited. No individual or entity - including the speakers and/or the authors -may electronically record any portion of the meeting without prior written consent of the Chair and all the meeting participants. Recording of voice or video at meetings is not used in 3GPP; this applies also to e-Meeting. |  |  |  |  |
| R3-233703 | Guidelines for RAN3 Meetings (RAN3 Chair, RAN3 Vice-Chairs) |  | Discussion Endorsed |  |
| # 6. Organizational topics RAN3 election for 1 VC to be held: Votes (3gpp.org) |  |  |  |  |
| # 7. General, protocol principles and issues RAN3 Work Plan and Working Procedures: TR 30.531 MCC allocates protocol IE IDs, checking with Rapporteurs during CR implementation phase Rapporteurs to update specifications with ASN.1 comments related to conditional IEs LS reply to CT4 on IANA port allocation agreed in R3-212800 Reply LS on Tracking IANA assignment requests in R3-230802 |  |  |  |  |
| R3-234022 | TR 30.531 v1.47.0 Work Plan and Working Procedures - RAN WG3 (ETSI-MCC) |  | draft TR - Change Jaemin’s rapporteurship company from Intel to LG Rev in R3-234494 Endorsed unseen |  |
| # 8. Incoming LSs |  |  |  |  |
| ## 8.1. New Incoming LSs |  |  |  |  |
| PEI |  |  |  |  |
| R3-233727 | Reply LS on the use of PEI during an emergency PDU session (SA2(Ericsson)) |  | LS in R17 noted |  |
| R3-233843 | Conclusion on the use of PEI during emergency PDU session (Nokia, Nokia Shanghai Bell) |  | Discussion Resp in R3-234491 Nok: RAN2 is not going to discuss RAN paging at all. E///, CATT: Do not see benefits on reply LS and CR, no further discussion is needed ZTE: Share view as E///. CR is not correct, according to SA2/CT1 specs, RAN does not to know which PDU session is emergency or not. SS: Fine to agree CR. RAN based paging needs more inputs from RAN2. HW: It’s a bit early to have conclusion. Qualcomm: There is need to have CR which has already been covered in SA2 spec. noted |  |
| R3-233844 | Response LS on the use of PEI during an emergency PDU session (Nokia, Nokia Shanghai Bell) |  | LS out To: SA2 CC: noted |  |
| R3-233845 | Correction of Paging with PEI (Nokia, Nokia Shanghai Bell) |  | draftCR noted |  |
| New Attribute |  |  |  |  |
| R3-233738 | Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA (SA5(China telecom)) |  | LS in R18 |  |
| R3-233997 | Discussion on the attribute "isENDCAllowed" (China Telecom,Huawei,ZTE) |  | discussion |  |
| R3-233996 | Reply LS on introduction of a new attribute "Only Resource Coordination" to support source coordination between LTE and NR SA (China Telecom,Huawei) |  | LS out To: SA5 CC: |  |
| R3-234410 | Consideration on on introduction new attribute to support source coordination between LTE and NR SA (ZTE) |  | discussion |  |
| R3-234411 | [Draft]Response LS on introduction new attribute to support source coordination between LTE and NR SA (ZTE) |  | other |  |
| R3-234287 | Discussion on the on introduction of a new attribute “Only Resource Coordination” (Ericsson) |  | discussion |  |
| What’s the drawback of reusing “isENDCAllowed”? Nok: Fine to reuse “isENDCAllowed” attribute HW: Slight to have the new attribute, if we reuse the old one, then other procedures will be triggered without any benefits. ZTE: New attribute is preferred with clear purpose. E///: Even the new attribute is used, the basic interface setup procedure needs to be supported. The impact on LTE should be minimized. CT: The evaluation has already been done in last year. Only one company does not agree with new attribute. CATT: If the “isENDCAllowed” attribute is reused, other procedures used by EN-DC is still allowed which is not the intention. CB: # 1_NewAttribute - Clarify the reason why “isENDCAllowed” or "Only Resource Coordination" is agreed in RAN3 - Reply LS to SA5 (moderator - ZTE) Summary of offline disc R3-234495 |  |  |  |  |
| MBS Packet Loss |  |  |  |  |
| R3-233728 | Reply to LS on addressing packet loss during multicast MBS delivery (SA2(Ericsson)) |  | LS in R17, cc ZTE: Tends to agree that it might be useful. For P1 and P2, needs more discussion. It’s beneficial for RAN3 to reply the LS considering the latency issue. QC: Observation3 is unclear E///, Nok: For intra-CU split case, there is no interest to exposure it to SA2/SA6. No LS back and involvement between RAN2 and SA related WGs. SS: Clarify in stage2 is beneficial Nok, Lenovo, CATT: Take a look on this and back on next meeting CB: # 2_MBSReplyLS - Only check whether reply LS is needed or not? (moderator - HW) |  |
| R3-234373 | Consideration on packet loss during multicast MBS delivery (Huawei) |  | discussion |  |
| R3-234374 | [DRAFT] Reply to LS on addressing packet loss during multicast MBS delivery (Huawei) |  | LS out To: SA2, RAN2, SA6 CC: Rev in R3-234611 |  |
| R3-234375 | Clarification on gNB moving UEs to RRC_Inactive for active multicast session (Huawei) |  | draftCR |  |
| DTLS for SCTP |  |  |  |  |
| R3-233836 | DTLS for SCTP next steps and request for input (IETF_TSVWG) |  | LS in |  |
| R3-233736 | Reply LS on SCTP-AUTH and DTLS (SA3(Ericsson)) |  | LS in |  |
| R3-234492 | Reply LS on DTLS for SCTP (SA3(Ericsson)) |  | LSin |  |
| R3-234194 | DTLS for SCTP: Summary and Further Discussion (Ericsson LM) |  | discussion |  |
| R3-234195 | [DRAFT] Reply LS on DTLS for SCTP next steps and request for input (Ericsson LM) |  | LS out To: IETF TSVWG CC: SA3, SA2, CT1, CT4 - Update to simple version to reply to message size question Rev in R3-234497 |  |
| R3-234412 | Consideration on DTLS for SCTP (ZTE) |  | discussion |  |
| R3-234413 | [Draft]Reply LS on DTLS for SCTP next steps and request for input (ZTE) |  | other |  |
| R3-234469 | Discussion on DTLS for SCTP next steps - message size (Huawei) |  | discussion |  |
| R3-234470 | [Draft] Reply LS on DTLS for SCTP next steps and request for input (Huawei) |  | LS out To: IETF_TSVWG CC: SA3 |  |
| E///: The implementation should not be discussed in 3GPP Nok: Simple reply to message size question, RAN3 does not have the capability to answer the second question HW: If there is implementation issue in RAN node which should be discussed in SA3 and IETF |  |  |  |  |
| cc |  |  |  |  |
| R3-233812 | LS on information on actions taken by SG15 on inclusive language (RAN(Huawei)) |  | LS in noted |  |
| R3-233710 | LS on Rel-18 higher-layers parameter list (RAN1(Ericsson)) |  | LS in R18 noted |  |
| R3-234493 | LS on the user consent for trace reporting (SA3(Ericsson)) |  | LSin R18 Noted To be continued in AI8.3 in next meeting... |  |
| R3-233704 | Reply LS on Network Triggered Service Request for a UE in Suspend State (CT4(Ericsson)) |  | LS in R17, cc noted |  |
| R3-233707 | Reply LS on 1-symbol PRS (RAN1(ZTE)) |  | LS in R18, cc noted |  |
| R3-233724 | Reply LS on 3GPP work on Energy Efficiency (SA1(Nokia)) |  | LS in R18, cc noted |  |
| R3-233737 | LS on 3GPP work on Energy Efficiency (SA4(Qualcomm)) |  | LS in R18, cc noted |  |
| ## 8.2. LSin received during the meeting |  |  |  |  |
| ## R3-234532 |  | Support of Time Sensitive Networking (TSN) enabled Transport Network (TN) (CT4(Nokia)) |  | LS in R18, cc noted |
| ## R3-234560 |  | Reply LS on applicability of pre-configured measurement gaps for RedCap UE(SA2(Qualcomm)) |  | LS in R17 |
| ## 8.3. Left over LSs / pending actions |  |  |  |  |
| # 9. Corrections to Rel-17 or earlier releases [TU: 0.5 (0.5, 0.5, 0.5)] (shared with AI 31) Only essential corrections are allowed for frozen releases, e.g., R15, R16, R17. Corrections related to E1 AP, any mirror CR to TS37.48x should go for REL-17 Cat. A CR with proper WI code, and fill the “Other core specifications” field to show the corresponding REL-15/16 Cat. F CR with its CR number together with the following notes in the “Other comments” field in the coversheet: This Cat. A CR to TS 37.48x is a mirror CR of previous release of TS 38.46x. No REL-17 CR to TS 38.46x is needed as TS 38.46x REL-17 is an empty pointer specification to TS 37.48x. |  |  |  |  |
| ## 9.1. LTE QUOTA: 1 |  |  |  |  |
| R3-233917 | MDT user consent update in LTE and EN-DC (Huawei, Deutsche Telekom, Nokia, Nokia shanghai Bell, Ericsson) |  | CR1916r, TS 36.413 v17.5.0, Rel-17, Cat. F ZTE: Changes are questionable, the motivation is not clear HW: User consent is sensitive for operators E///: QC: There is another IE in LTE which is different with NR. Do we need to change the management based MDT allowed IE at the same time or not? CB: # 3_MDTUserConsent - Check the motivation - The issue on the management based MDT allowed IE? (moderator - HW) Summary of offline disc R3-234498 |  |
| R3-233918 | MDT user consent update in EN-DC (Huawei, Deutsche Telekom, Nokia, Nokia shanghai Bell, Ericsson) |  | CR1754r, TS 36.423 v17.5.0, Rel-17, Cat. F |  |
| R3-234471 | Further discussion on the presence of QoE area scope (ZTE, China Telecom, China Unicom, Ericsson) |  | Discussion HW: Fine with X2AP CRs, but S1AP CRs are not needed Nok: How to deal with the misalignment between stage2 and stage3 CB: # 4_QoEAreaScope - Check whether there is any stage2 impact? - Check whether S1AP CRs are needed? - Check BC way is feasible/preferable or not? - The order of PLMN ID and TAC? (moderator - ZTE) Summary of offline disc R3-234502 |  |
| R3-234472 | Correction on QoE area scope tabular reference and presence (ZTE, Huawei, China Telecom, China Unicom, Ericsson) |  | CR1758r, TS 36.423 v15.13.0, Rel-15, Cat. F |  |
| R3-234473 | Correction on QoE area scope tabular reference and presence (ZTE, Huawei, China Telecom, China Unicom, Ericsson) |  | CR1759r, TS 36.423 v16.11.0, Rel-16, Cat. A |  |
| R3-234474 | Correction on QoE area scope tabular reference and presence (ZTE, Huawei, China Telecom, China Unicom, Ericsson) |  | CR1760r, TS 36.423 v17.5.0, Rel-17, Cat. A |  |
| R3-234475 | Correction on presence of the QoE Area Scope IE (ZTE, China Telecom, China Unicom, Ericsson) |  | CR1917r, TS 36.413 v15.11.0, Rel-15, Cat. F |  |
| R3-234476 | Correction on presence of the QoE Area Scope IE (ZTE, China Telecom, China Unicom, Ericsson) |  | CR1918r, TS 36.413 v16.13.0, Rel-16, Cat. A |  |
| R3-234477 | Correction on presence of the QoE Area Scope IE (ZTE, China Telecom, China Unicom, Ericsson) |  | CR1919r, TS 36.413 v17.5.0, Rel-17, Cat. A |  |
| ## 9.2. NR QUOTA: 3 (was 4) Including critical issues for R17 |  |  |  |  |
| Routing ID |  |  |  |  |
| R3-233705 | Reply LS on the maximum length of Routing ID (CT4(Huawei)) |  | LS in |  |
| R3-233825 | Clarification on maximum length of Routing ID (NTT DOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Huawei) |  | CR0953r5, TS 38.413 v16.13.0, Rel-16, Cat. F - The maximum length is 16 octets, referring to the length of a Universally Unique Identifier (UUID) version 4 as defined in section 4.4in IETF RFC 4122 [X] - Add ZTE as co-source Rev in R3-234503 Agreed unseen |  |
| R3-233826 | Clarification on maximum length of Routing ID (NTT DOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Huawei) |  | CR0954r5, TS 38.413 v17.5.0, Rel-17, Cat. A - The maximum length is 16 octets, referring to the length of a Universally Unique Identifier (UUID) version 4 as defined in section 4.4in IETF RFC 4122 [X] - Add ZTE as co-source Rev in R3-234504 Agreed unseen |  |
| R3-234064 | Further clarification on maximum length of Routing ID (ZTE, China Telecom, China Unicom) |  | discussion |  |
| R3-234065 | Correction on maximum length of Routing ID (ZTE, China Telecom, China Unicom) |  | CR1015r, TS 38.413 v16.13.0, Rel-16, Cat. F |  |
| R3-234066 | Correction on maximum length of Routing ID (ZTE, China Telecom, China Unicom) |  | CR1016r, TS 38.413 v17.5.0, Rel-17, Cat. A |  |
| R3-234067 | [DRAFT] Response to Reply LS on the maximum length of Routing ID (ZTE) |  | LS out To: CT4 CC: SA2 noted |  |
| ASN.1 Issue |  |  |  |  |
| R3-234456 | Misalignment between tabular and ASN.1 for CHO Information IE (Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, China Telecom) |  | CR1755r, TS 36.423 v16.11.0, Rel-16, Cat. F HW, ZTE: Why do not we update the tabular without changes ASN.1, change it to BC CR SS: Share view as E///, if the IE is set as ignore, what’s behavior in the receiving node? Nok: For resp message, ignore is fine, but for the request, if it is set as ignore, the feature that CHO HO revert to legacy HO has not been discussed - In the request message, change the IE criticality as “reject” in ASN.1 - In the response message, keep the IE criticality as “ignore” in ASN.1 - Change the IE criticality in ASN.1 can be regarded as BC Rev in R3-234509 |  |
| R3-234457 | Misalignment between tabular and ASN.1 for CHO Information IE (Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, China Telecom) |  | CR1756r, TS 36.423 v17.5.0, Rel-17, Cat. A - In the request message, change the IE criticality as “reject” in ASN.1 - In the response message, keep the IE criticality as “ignore” in ASN.1 - Change the IE criticality in ASN.1 can be regarded as BC Rev in R3-234510 |  |
| R3-234463 | Correction on misalignment between tabular and ASN.1 for positioning_Option1 (ZTE, Xiaomi, CATT) |  | CR0110r, TS 38.455 v17.5.0, Rel-17, Cat. F Merged to rapporteur’s updates |  |
| R3-234464 | Correction on misalignment between tabular and ASN.1 for positioning_Option1 (ZTE, Xiaomi, CATT) |  | CR1216r, TS 38.473 v17.5.0, Rel-17, Cat. F Merged to rapporteur’s updates |  |
| R3-234465 | Correction on misalignment between tabular and ASN.1 for positioning_Option2 (ZTE, Xiaomi, CATT) |  | CR0111r, TS 38.455 v17.5.0, Rel-17, Cat. F |  |
| R3-234466 | Correction on misalignment between tabular and ASN.1 for positioning_Option2 (ZTE, Xiaomi, CATT) |  | CR1217r, TS 38.473 v17.5.0, Rel-17, Cat. F |  |
| R3-234214 | PRS CONFIGURATION REQUEST Correction (Huawei, Nokia, Nokia Shanghai Bell, CMCC, Ericsson) |  | CR1206r, TS 38.473 v17.5.0, Rel-17, Cat. F NBC CR - Update the coversheet with co-source companies - Check WI code Rev in R3-234511 |  |
| R3-234092 | Discussion on misalignment of TAI between ASN.1 and tabular in QoE (Huawei, ZTE, InterDigital, China Unicom, Ericsson) |  | discussion |  |
| R3-234093 | Correction to misalignment of TAI between ASN.1 and tabular in QoE (Huawei, ZTE, InterDigital, China Unicom, Ericsson) |  | CR1083r, TS 38.423 v17.5.0, Rel-17, Cat. F Agreed |  |
| R3-234122 | Correction on SL relay F1AP ASN.1 IE semantics description (Philips International B.V.) |  | CR1201r, TS 38.473 v17.5.0, Rel-17, Cat. F HW: the SL-PathSwitchConfig IE is included in CellGroupConfig IE noted |  |
| R3-234242 | ASN.1 and tabular alignment for Broadcast related messages (CATT,Nokia, Nokia Shanghai Bell, Ericsson, ZTE,China Telecom,CBN) |  | CR1018r, TS 38.413 v17.5.0, Rel-17, Cat. F HW: Prefer to keep ASN.1. Why we need to differentiate the behavior towards response message and failure message CATT: Same principle for multicast and broadcast Nok：Support this CR Agreed BC CR |  |
| R3-234080 | Correction on the sharedNGU-MulticastTNLInformation (Huawei, CBN, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, CATT, Ericsson, Orange, ZTE, BT) |  | CR1017r, TS 38.413 v17.5.0, Rel-17, Cat. F NBC CR Agreed |  |
| R3-234081 | Correction on MBS Session Forwarding Address (Huawei, CBN, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, CATT, Ericsson, Orange, ZTE, BT) |  | CR0073r, TS 37.483 v17.5.0, Rel-17, Cat. F NBC CR Agreed |  |
| R3-233897 | Correction of Mobility Information (Huawei, Ericsson, Qualcomm, BT, Deutsche Telekom, Vodafone) |  | CR1011r, TS 38.413 v16.13.0, Rel-16, Cat. F - Follow BC way - The old IE is not used in the semantic description Rev in R3-234512 Agreed unseen |  |
| R3-233898 | Correction of Mobility Information (Huawei, Ericsson, Qualcomm, BT, Deutsche Telekom, Vodafone) |  | CR1012r, TS 38.413 v17.5.0, Rel-17, Cat. A - Follow BC way - The old IE is not used in the semantic description - Fix duplication in ASN.1 - Update the coversheet Rev in R3-234513 Agreed unseen |  |
| R3-234198 | Correction of Served Cell Specific Info Request (Huawei, Deutsche Telekom, BT) |  | CR1084r, TS 38.423 v17.5.0, Rel-17, Cat. F E///: Remove it from ASN.1 Agreed |  |
| MBS |  |  |  |  |
| R3-233846 | Correction of MBS Distribution procedure (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, Huawei, CATT, ZTE) |  | Discussion SS, E///: Remove last change. It’s implementation. |  |
| R3-233847 | Correction of MBS Distribution procedure (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, Huawei, CATT, ZTE) |  | CR1191r, TS 38.473 v17.5.0, Rel-17, Cat. F - Remove the last change Rev in R3-234516 Agreed unseen |  |
| R3-234335 | Correction of MBS Neighbour cells (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, CATT, Ericsson) |  | Discussion HW: Do not share the same view, MBS Broadcast Neighbor Cell List is not change as the same frequency as mtch-neighbourCell, it’s per cell information not per session, can be further discussed in R18 Nok: This is the solution with less impact ZTE: No strong motivation to change the current IE structure CATT: Ack the design in R17 is not perfect CB: # 5_MBSNeighbourcell - Check whether there is any issue introduced by new design? - Take 4209 and 4231 into account (moderator - Nok) Summary of offline disc R3-234517 |  |
| R3-234336 | Correction of MBS Neighbour cells (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, CATT, Ericsson) |  | CR1212r, TS 38.473 v17.5.0, Rel-17, Cat. F |  |
| R3-234209 | Correction of presence of MBS CU to DU RRC Information IE in the BROADCAST CONTEXT MODIFICATION REQUEST message (ZTE, Ericsson, CATT, Lenovo, CMCC) |  | CR1204r, TS 38.473 v17.5.0, Rel-17, Cat. F Whether MBS CU to DU RRC Information IE should be optional or mandatory? |  |
| R3-234210 | Correction on condition of successful MBS Broadcast Context Setup (ZTE, Huawei, CATT) |  | CR1205r, TS 38.473 v17.5.0, Rel-17, Cat. F Nok: Support the CR, add Nok as co-source - Add Nok as co-source Rev in R3-234518 Agreed unseen |  |
| R3-234230 | Missing stage 2 specification for applicability of shared NG-U termination for Broadcast MBS Sessions (Ericsson, AT&T, CATT, Nokia, Nokia Shanghai Bell, ZTE, China Unicom) |  | draftCR Lenovo, QC: When UE moves to another cell, there is no need to guarantee the PDCP Count continuity ZTE: This the agreement achieved last year. The first bullet does not help for broadcast, remove the first bullet noted |  |
| R3-234231 | Correction of control of mtch-neighbourCell content (Ericsson, AT&T, CATT, China Unicom, ZTE) |  | CR1207r, TS 38.473 v17.5.0, Rel-17, Cat. F SS: Not sure whether this indication is needed. Without the IE included, it means there is no change on mtch-neighbourCell |  |
| R3-234243 | Data forwarding handling of MRB in split target gNB (CATT,China Telecom,CBN) |  | Discussion Nok: Same discussion in 3850, same problem with different solution, and different view on p3 HW, SS: Need offline CB: # 6_MRBinSplitCase - Check the proposals and solutions - Capture agreements if any (moderator - CATT) Summary of offline disc R3-234519 |  |
| R3-234244 | Correction to F1AP on Data forwarding handling of MRB (CATT, China Telecom, CBN) |  | CR1209r, TS 38.473 v17.5.0, Rel-17, Cat. F |  |
| R3-234245 | Correction to E1AP on Data forwarding handling of MRB (CATT, ZTE, Ericsson, China Telecom, CBN) |  | CR0075r, TS 37.483 v17.5.0, Rel-17, Cat. F |  |
| R3-233850 | Correction of F1 Context Reference (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, Huawei) |  | discussion |  |
| R3-233851 | Correction of F1 Context Reference (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, Huawei) |  | CR1192r, TS 38.473 v17.5.0, Rel-17, Cat. F |  |
| R3-233848 | Correction of Location Dependent Service (Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, Huawei, CATT, ZTE) |  | Discussion E///: Agree with the changes |  |
| R3-233849 | Correction of Location Dependent Service (Nokia, Nokia Shanghai Bell, Orange, qcomm, HW, CATT, ZTE) |  | CR0069r, TS 37.483 v17.5.0, Rel-17, Cat. F - Update the procedure and message names in the coversheet - Add the Criticality in the tabular Rev in R3-234520 Agreed unseen |  |
| Others |  |  |  |  |
| R3-233817 | Clarification on R17 MO SDT termination (ZTE, China Telecom, Huawei, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Lenovo, CATT, Ericsson, Xiaomi) |  | draftCR - Check the wording - Move Note4 between step5 and step6 - Update the “Clarification” to “correction” in the title - Editorial updates if needed - Remove changes over changes Rev in R3-234514 |  |
| R3-233895 | Downlink SRB1 forwarding for remote UE’s RRC Reestablishment/Resume procedure (Huawei, Interdigital, Nokia, Nokia Shanghai Bell, LG Electronics, NEC, ZTE, CATT, CMCC, China Telecom, Samsung) |  | Discussion E///: Question on the scenario, good to have, needs more description on the scenario SS: Another scenario has identified, support this CR |  |
| R3-233896 | Mapping of SRB1 for the remote UE (Huawei, Interdigital, Nokia, Nokia Shanghai Bell, LG Electronics, NEC, ZTE, CATT, CMCC, China Telecom, Samsung) |  | CR1193r, TS 38.473 v17.5.0, Rel-17, Cat. F - Try to avoid using slash in the procedure text - Add description on the scenario identified? Rev in R3-234521 |  |
| R3-233884 | Corrections to TS 38.401 for SL relay (ZTE Corporation, CAICT, China Telecom, Philips, CATT) |  | CR0299r, TS 38.401 v17.5.0, Rel-17, Cat. F - Remove the bullet: This CR has an impact under functional point of view. Rev in R3-234522 Agreed unseen |  |
| R3-233829 | UDC Parameters over E1 (Nokia, Nokia Shanghai Bell, China Telecom, ZTE, CATT, Huawei, Ericsson) |  | CR0067r, TS 37.483 v17.5.0, Rel-17, Cat. F Agreed |  |
| R3-233915 | Correction on E-UTRA - NR Cell Resource Coordination (Huawei, Orange, China Telecom, Deutsche Telekom, Nokia, Nokia shanghai Bell, ZTE, Ericsson) |  | CR1753r, TS 36.423 v17.5.0, Rel-17, Cat. F Agreed |  |
| R3-233916 | Correction on E-UTRA - NR Cell Resource Coordination (Huawei, Orange, China Telecom, Deutsche Telekom, Nokia, Nokia shanghai Bell, ZTE, Ericsson) |  | CR1071r, TS 38.423 v17.5.0, Rel-17, Cat. F - Update the field in “Consequences if not approved:” Rev in R3-234523 Agreed unseen |  |
| R3-234051 | Discussion on inter-node message for CU-DU split scenario (Samsung) |  | discussion |  |
| R3-234052 | Correction to TS 38.473 on inter-node message for CU-DU split scenario (Samsung, ZTE, CATT, Nokia, Nokia Shanghai Bell, Ericsson) |  | CR1196r, TS 38.473 v16.14.0, Rel-16, Cat. F Agreed |  |
| R3-234053 | Correction to TS 38.473 on inter-node message for CU-DU split scenario (Samsung, ZTE, CATT, Nokia, Nokia Shanghai Bell, Ericsson) |  | CR1197r, TS 38.473 v17.5.0, Rel-17, Cat. A Agreed |  |
| R3-233919 | Correction on the local NG-RAN Node Identifier on Xn (Huawei, Nokia, Nokia shanghai Bell, ZTE) |  | CR1072r, TS 38.423 v17.5.0, Rel-17, Cat. F - Update the coversheet to remove NBC to BC, and also describe that there is no RAN2 impact - Add the same TEI identifier in the title as we introduced before - Update the “Clauses affected:” - Add E/// as co-source Rev in R3-234524 |  |
| R3-233957 | Correction of QoS Flow Mapping Indication IE in PDU Session Resource Modification Required Info – SN terminated (Huawei, Deutsche Telekom, CATT, CMCC, Vodafone) |  | CR1073r, TS 38.423 v15.17.0, Rel-15, Cat. F Agreed |  |
| R3-233958 | Correction of QoS Flow Mapping Indication IE in PDU Session Resource Modification Required Info – SN terminated (Huawei, Deutsche Telekom, CATT, CMCC, Vodafone) |  | CR1074r, TS 38.423 v16.14.0, Rel-16, Cat. A Agreed |  |
| R3-233959 | Correction of QoS Flow Mapping Indication IE in PDU Session Resource Modification Required Info – SN terminated (Huawei, Deutsche Telekom, CATT, CMCC, Vodafone) |  | CR1075r, TS 38.423 v17.5.0, Rel-17, Cat. A Agreed |  |
| R3-233864 | Partial QoS Flow release (NEC) |  | discussion |  |
| R3-233865 | Correction for Partial QoS Flow release (NEC) |  | CR1010r, TS 38.413 v17.5.0, Rel-17, Cat. F Way2 is the common understanding the IE is mandatory which does not need any procedure text noted |  |
| R3-233955 | Discussion on Avioding unnecessary setup of DRB(s) in indirect data forwarding (Samsung) |  | Discussion E///: It’s optimization and not critical Noted To be continued... |  |
| R3-233956 | Avoiding unnecessary setup of DRB(s) in indirect data forwarding (Samsung) |  | CR0071r, TS 37.483 v17.5.0, Rel-17, Cat. F noted |  |
| R3-233971 | Transferring of IAB authorized info during inter-CU topology adaptation and backhaul RLF recovery procedure (ZTE, Lenovo, Xiaomi) |  | Discussion HW: Support p2, not agree with P1. It’s not proper to combine the the IAB Node Indication IE with authorization ZTE: Whether to introduce the authorization indication over Xn explicitly? QC: Fine to have explicit indication over Xn Ho procedures E///: The AMF can inform NG-RAN node when the authorization has changed Nok: NGAP CR is needed |  |
| R3-233972 | Transferring of IAB authorized info during inter-CU topology adaptation and backhaul RLF recovery procedure (ZTE, Lenovo, Xiaomi, CATT) |  | CR1014r, TS 38.413 v17.5.0, Rel-17, Cat. F Agreed |  |
| R3-233973 | Transferring of IAB authorized info during inter-CU topology adaptation and backhaul RLF recovery procedure (ZTE, Lenovo, Xiaomi, CATT) |  | CR1078r, TS 38.423 v17.5.0, Rel-17, Cat. F noted |  |
| R3-234151 | Correction on IAB bar configuration (Huawei, Nokia, Nokia Shanghai Bell, Lenovo, Xiaomi, CATT, Ericsson, Qualcomm, Samsung, ZTE) |  | CR1174r2, TS 38.473 v16.14.0, Rel-16, Cat. F - the codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. Rev in R3-234525 Agreed unseen |  |
| R3-234152 | Correction on IAB bar configuration (Huawei, Nokia, Nokia Shanghai Bell, Lenovo, Xiaomi, CATT, Ericsson, Qualcomm, Samsung, ZTE) |  | CR1175r2, TS 38.473 v17.5.0, Rel-17, Cat. A - the codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. Rev in R3-234526 Agreed unseen |  |
| R3-234148 | Discussion on the BH info configuration for DRBs support CA based duplication (Huawei, Nokia, Nokia Shanghai Bell, CATT, Lenovo, Samsung) |  | discussion |  |
| R3-234149 | Configuration of BH information for DRBs support CA based duplication (Huawei, Nokia, Nokia Shanghai Bell, CATT, Lenovo, Samsung) |  | CR1202r, TS 38.473 v16.14.0, Rel-16, Cat. F Agreed |  |
| R3-234150 | Configuration of BH information for DRBs support CA based duplication (Huawei, Nokia, Nokia Shanghai Bell, CATT, Lenovo, Samsung) |  | CR1203r, TS 38.473 v17.5.0, Rel-17, Cat. A Agreed |  |
| R3-233960 | Correction of Additional PDCP Duplication TNL List (Huawei, Deutsche Telekom, Ericsson) |  | CR1076r, TS 38.423 v16.14.0, Rel-16, Cat. F Rev in R3-234608 CB: # 7_TNLAddress - Check the issue and the IEs (moderator - HW) Summary of offline disc R3-234527 |  |
| R3-233961 | Correction of Additional PDCP Duplication TNL List (Huawei, Deutsche Telekom, Ericsson) |  | CR1077r, TS 38.423 v17.5.0, Rel-17, Cat. A Rev in R3-234609 |  |
| R3-234077 | Correction of data forwarding for split PDU session (Huawei, Nokia, Noia Shanghai Bell, Qualcomm Incorporated, CMCC) |  | CR0722r, TS 38.463 v15.12.0, Rel-15, Cat. F ZTE: No changes compared with the contributions submitted in last meeting. Only R16 and R17 is needed. CATT: OK with CR, comments on the procedure text E///: The CR might be needed |  |
| R3-234078 | Correction of data forwarding for split PDU session (Huawei, Nokia, Noia Shanghai Bell, Qualcomm Incorporated, CMCC) |  | CR0723r, TS 38.463 v16.14.0, Rel-16, Cat. A - Check the wording of procedure text - Change to Cat.F, and clarify the scenario in the coversheet, check the WI code - Add Orange, ZTE, E/// as co-source, correct Nokia name Rev in R3-234528 |  |
| R3-234079 | Correction of data forwarding for split PDU session (Huawei, Nokia, Noia Shanghai Bell, Qualcomm Incorporated, CMCC) |  | CR0072r, TS 37.483 v17.5.0, Rel-17, Cat. A - Check the wording of procedure text - Clarify the scenario in the coversheet, check the WI code - Add Orange, ZTE, E/// as co-source, correct Nok name Rev in R3-234529 |  |
| R3-234215 | Positioning Information Transfer (Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, CMCC) |  | draftCR Endorsed |  |
| R3-234216 | Correction to missing NRPPa procedures (Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, CMCC) |  | draftCR E///: Comment on why includes “Measurement Preconfiguration” Endorsed |  |
| R3-234260 | Correction of Positioning SIType List (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, ZTE, CATT) |  | CR1210r, TS 38.473 v16.14.0, Rel-16, Cat. F Agreed |  |
| R3-234261 | Correction of Positioning SIType List (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, ZTE, CATT) |  | CR1211r, TS 38.473 v17.5.0, Rel-17, Cat. A Agreed |  |
| R3-234262 | Correction on Positioning SI handling over F1 (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, ZTE, CATT) |  | CR0115r, TS 38.470 v16.10.0, Rel-16, Cat. F Agreed |  |
| R3-234263 | Correction on Positioning SI handling over F1 (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, ZTE, CATT) |  | CR0116r, TS 38.470 v17.5.0, Rel-17, Cat. A Agreed |  |
| R3-234226 | Correction concerning procedural text for "Unsuccessful Operation" specified in “Successful Operation” sections (Ericsson, AT&T, China Unicom, Deutsche Telekom, Interdigital) |  | CR1002r1, TS 38.413 v15.13.0, Rel-15, Cat. F HW: No agreements made in last meeting ZTE: Share view as HW, the change in 8.2.2.3 is needed CATT: Ok to have this CR Nok: Should not open the discussion again, leave it as it is noted |  |
| R3-234227 | Correction concerning procedural text for "Unsuccessful Operation" specified in “Successful Operation” sections (Ericsson, AT&T, China Unicom, Deutsche Telekom, Interdigital) |  | CR1003r1, TS 38.413 v16.13.0, Rel-16, Cat. A noted |  |
| R3-234228 | Correction concerning procedural text for "Unsuccessful Operation" specified in “Successful Operation” sections (Ericsson, AT&T, China Unicom, Deutsche Telekom, Interdigital) |  | CR1004r1, TS 38.413 v17.5.0, Rel-17, Cat. A noted |  |
| R3-234082 | Correction on Fast MCG Recovery via SRB3 (Huawei, Deutsche Telekom, BT) |  | CR1081r, TS 38.423 v16.14.0, Rel-16, Cat. F ZTE: Agree with the intention, no consensus on BC or NBC, BC way if preferred E///: It’s BC, question on the usage of “Release fast MCG recovery via SRB3” IE noted |  |
| R3-234083 | Correction on Fast MCG Recovery via SRB3 (Huawei, Deutsche Telekom, BT) |  | CR1082r, TS 38.423 v17.5.0, Rel-17, Cat. A noted |  |
| R3-234222 | Altitude correction for the NTN TRP (Huawei, Thales, Ericsson) |  | Discussion CATT: This is not correction. Non-3GPP based solution can be used. ZTE: Same question as CATT, this issue is not in the scope of R17 Nok: Similar concern as CATT, ZTE, maybe this is the issue to be discussed in R18 QC: In R17, this cannot be the direction noted |  |
| R3-234223 | NTN Access Point Position (Huawei, Thales, Ericsson) |  | CR0106r, TS 38.455 v17.5.0, Rel-17, Cat. F noted |  |
| R3-234224 | NTN Access Point Position (Huawei, Thales, Ericsson) |  | CR0120r, TS 36.455 v17.1.0, Rel-17, Cat. F noted |  |
| R3-234225 | LS on Altitude for the Access Point (Huawei, Thales, Ericsson) |  | LS out To: SA2 CC: CT1, CT4, SA5, RAN2, RAN1 noted |  |
| R3-234445 | Discussion on exchanging AMF Set information over Xn (CMCC, Huawei, ZTE, CATT) |  | discussion |  |
| R3-234060 | Clarification on procedural description of Resource Status Reporting Initiation procedure over Xn (ZTE, China Telecom, China Unicom) |  | CR1079r, TS 38.423 v16.14.0, Rel-16, Cat. F HW: Try to understand the motivation Nok: The added part does not change anything E///: The simplest way is to stop the current measurement in the corresponding cell and start another measurement The common understanding is that stop the current measurement in the corresponding cell and start another measurement is the simplest way. noted |  |
| R3-234061 | Clarification on procedural description of Resource Status Reporting Initiation procedure over Xn (ZTE, China Telecom, China Unicom) |  | CR1080r, TS 38.423 v17.5.0, Rel-17, Cat. A noted |  |
| R3-234062 | Clarification on procedural description of Resource Status Reporting Initiation procedure over F1 (ZTE, China Telecom, China Unicom) |  | CR1199r, TS 38.473 v16.14.0, Rel-16, Cat. F noted |  |
| R3-234063 | Clarification on procedural description of Resource Status Reporting Initiation procedure over F1 (ZTE, China Telecom, China Unicom) |  | CR1200r, TS 38.473 v17.5.0, Rel-17, Cat. A noted |  |
| R3-234054 | Correction on SI delivery to RedCap UE during handover (ZTE, China Telecom, CMCC) |  | CR1198r, TS 38.473 v17.5.0, Rel-17, Cat. F Waiting for reply from RAN2 |  |
| R3-233920 | Adding stage 2 text for MLB (Huawei, CMCC, China Telecom) |  | CR0300r, TS 38.401 v16.10.0, Rel-16, Cat. F ZTE: Provided the CR in previous meeting which was noted with the comment that it is not necessary noted |  |
| R3-233921 | Adding stage 2 text for MLB (Huawei, CMCC, China Telecom) |  | CR0301r, TS 38.401 v17.5.0, Rel-17, Cat. A noted |  |
| R3-234156 | Correction on mobility restriction list for MR-DC with 5GC (Huawei, Nokia, Nokia Shanghai Bell, Deutsche Telekom) |  | draftCR ZTE: The CR is not essential E///: up-to-date is kept, but the newly added “intact” word is still ambiguous Rev in R3-234530 |  |
| R3-234157 | Correction on mobility restriction list for MR-DC with 5GC (Huawei, Nokia, Nokia Shanghai Bell, Deutsche Telekom) |  | draftCR Rev in R3-234610 |  |
| R3-234240 | Clarification on the presence of Broadcast PLMN Identity Info List NR IE (CATT, ZTE) |  | CR1087r, TS 38.423 v16.14.0, Rel-16, Cat. F Nok: Further work on the wording noted |  |
| R3-234241 | Clarification on the presence of Broadcast PLMN Identity Info List NR IE (CATT, ZTE) |  | CR1088r, TS 38.423 v17.5.0, Rel-17, Cat. A noted |  |
| R3-233833 | Correction to provide the slice availability for SN change procedure (Nokia, Nokia Shanghai Bell) |  | CR1070r, TS 38.423 v17.5.0, Rel-17, Cat. F SS: It’s not proper to transfer cell level information via UE associated procedure E///: Whether the case that no direct interface between source SN and target SN exists? noted |  |
| R3-233839 | Inactive Time Signaling over E1 for Mobility (Nokia, Nokia Shanghai Bell) |  | CR0068r, TS 37.483 v17.5.0, Rel-17, Cat. F E///: There is something missing to align with RRC HW: Similar view as E///, why we do not just refer to RAN2 spec, and what’s the usage of the the Inactive Time IE from CP to UP ZTE: The CP can send the correct time which takes the time received from RRC into account CATT: Ack the issue, same question as ZTE CB: # 8_InactiveTimeOver E1 - Check the comments above - Identify the issue (moderator - Nok) Summary of offline disc R3-234531 |  |
| R3-234042 | QoE Reference in QOE INFORMATION TRANSFER message (Ericsson) |  | CR1195r, TS 38.473 v17.5.0, Rel-17, Cat. F |  |
| R3-233866 | Correction for change of gNB-CU-UP (NEC) |  | CR0296r, TS 38.401 v15.9.0, Rel-15, Cat. F |  |
| R3-233867 | Correction for change of gNB-CU-UP (NEC) |  | CR0297r, TS 38.401 v16.10.0, Rel-16, Cat. A |  |
| R3-233868 | Correction for change of gNB-CU-UP (NEC) |  | CR0298r, TS 38.401 v17.5.0, Rel-17, Cat. A |  |
| R3-234353 | Correction on wrong abnormal description for conditional mobility in F1AP UE Context Modification procedure (LG Electronics Inc.) |  | discussion |  |
| R3-234354 | Rel-16 Correction in the UE Context Modification procedure abnormal description for conditional mobility modification (LG Electronics Inc.) |  | CR1214r, TS 38.473 v16.14.0, Rel-16, Cat. F |  |
| R3-234355 | Rel-17 Correction in the UE Context Modification procedure abnormal description for conditional mobility modification (LG Electronics Inc.) |  | CR1215r, TS 38.473 v17.5.0, Rel-17, Cat. A |  |
| R3-234324 | Solving Misalignment in UE Context Release procedure (Ericsson) |  | CR1019r, TS 38.413 v16.13.0, Rel-16, Cat. F Rev in R3-234499 |  |
| R3-234325 | Solving Misalignment in UE Context Release procedure (Ericsson) |  | CR1020r, TS 38.413 v17.5.0, Rel-17, Cat. A Rev in R3-234500 |  |
| R3-234334 | Correction of I-RNTI (Nokia, Nokia Shanghai Bell) |  | draftCR |  |
| R3-234356 | Clarification on UE radio capability encoding over NGAP-R16 (CATT) |  | CR1022r, TS 38.413 v16.13.0, Rel-16, Cat. F |  |
| R3-234357 | Clarification on UE radio capability encoding over NGAP-R17 (CATT) |  | CR1023r, TS 38.413 v17.5.0, Rel-17, Cat. A |  |
| R3-234405 | Correction on non-SDT or oversize SDT data arrival during on-going SDT session (China Telecom) |  | CR0306r, TS 38.401 v17.5.0, Rel-17, Cat. F Rev in R3-234554 |  |
| R3-234330 | Restriction of NIA0 algorithm in gNB-CU-UP (Ericsson) |  | CR0709r2, TS 38.463 v15.12.0, Rel-15, Cat. F |  |
| R3-234331 | Restriction of NIA0 algorithm in gNB-CU-UP (Ericsson) |  | CR0710r2, TS 38.463 v16.14.0, Rel-16, Cat. A |  |
| R3-234332 | Restriction of NIA0 algorithm in gNB-CU-UP (Ericsson) |  | CR0053r2, TS 37.483 v17.5.0, Rel-17, Cat. A |  |
| R3-234123 | Correction on F1AP message name (Philips International B.V.) |  | CR0303r, TS 38.401 v15.9.0, Rel-15, Cat. F |  |
| R3-234126 | Correction on F1AP message name (Philips International B.V.) |  | CR0304r, TS 38.401 v16.10.0, Rel-16, Cat. A |  |
| R3-234136 | Correction on F1AP message name (Philips International B.V.) |  | CR0305r, TS 38.401 v17.5.0, Rel-17, Cat. A |  |
| R3-233894 | Clarification of the RAN UE ID usage in stage-2 (Ericsson, Orange, Deutsche Telekom, AT&T) |  | CR0295r1, TS 38.401 v17.5.0, Rel-17, Cat. F Late contribution |  |
| ## 9.3. R17 Rapporteur Corrections Spec rapporteurs work on Cat.F CRs with NewRAT-Core and TEI17 as WI code to reflect the RRC container reference rules in R3-226777 Quota free |  |  |  |  |
| R3-234232 | Further Rapporteur's Corrections (Ericsson) |  | CR1086r, TS 38.423 v17.5.0, Rel-17, Cat. F |  |
| R3-234264 | NRPPa Rapporteur Corrections (Rapporteur (Ericsson)) |  | CR0107r, TS 38.455 v17.5.0, Rel-17, Cat. D |  |
| R3-234333 | Rapporteur’s correction of misplaced procedural text (Ericsson) |  | CR0076r, TS 37.483 v17.5.0, Rel-17, Cat. F |  |
| R3-234458 | X2AP rapporteur corrections (Ericsson) |  | CR1757r, TS 36.423 v17.5.0, Rel-17, Cat. D |  |
| R3-234076 | LPPa Rapporteur Update (Ericsson LM) |  | CR0119r, TS 36.455 v17.1.0, Rel-17, Cat. D |  |
| # 10. Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI (RAN3-led) WID [NR_ENDC_SON_MDT_enh2-Core]: RP-231157 (target: RAN #102) [TU: 1 (1, 1, 1)] QUOTA: 6 |  |  |  |  |
| ## 10.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233748 | (BLCR to 38.413) for MDT (Ericsson) |  | CR0990r2, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233757 | (BLCR to 38.423) for MDT (Huawei) |  | CR1050r2, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233758 | (BLCR to 38.423) Addition of SON features enhancement (Samsung) |  | CR0934r7, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233772 | (BLCR to 36.300) Addition of SON features enhancement (Lenovo) |  | draftCR Endorsed as BL CR |  |
| R3-233775 | (BLCR to 37.340) Addition of SON Rel.18 features (Nokia, Nokia Shanghai Bell) |  | draftCR Endorsed as BL CR |  |
| R3-233781 | (BLCR to 38.300) Addition of SON features enhancement (CMCC) |  | draftCR Endorsed as BLCR |  |
| R3-233790 | (BLCR to 38.401) Addition of SON features enhancement (ZTE) |  | CR0282r3, TS 38.401 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233794 | (BLCR to 38.413) for SON (Ericsson) |  | CR0964r4, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233802 | (BLCR to 38.470) Addition of SON features enhancement (CMCC) |  | CR0114r1, TS 38.470 v17.5.0, Rel-18, Cat. B - Update the spec version to 17.5.0 Rev in R3-234538 Endorsed as BL CR unseen |  |
| R3-233805 | (BLCR to 38.473) Addition of SON features enhancement (Huawei) |  | CR1105r5, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-234372 | (BL CR to 36.423) Addition of SON features enhancement (CATT) |  | CR1747r3, TS 36.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-234431 | Update of work Plan for Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI (CMCC) |  | Work Plan noted |  |
| R3-233740 | (BL CR to 36.423) Addition of SON features enhancement (CATT) |  | CR1747r2, TS 36.423 v17.5.0, Rel-18, Cat. B withdrawn |  |
| 10.2. Support of SON/MDT Enhancements In cooperation with RAN2 |  |  |  |  |
| 10.2.1. SHR and SPR SHR for intra-system inter-RAT, HO from NR to LTE will be treated first SPR SHR(Successful Handover Report) SHR for intra-system inter-RAT, HO from NR to LTE will be treated first SPCR(Successful PScell change report) SPCR for NR-DC, including: - SN- and MN-initiated classic PSCell change / CPC - intra-SN classic PSCell change / CPC - classic Addition / CPA - HO with SN change are not prohibited, but possibly addressed once the basic solution for SPCR is known. SPR as abbreviation for Successful PScell Change Report feature. For SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold). For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis. For intra-SN classic PSCell change/CPC, the source SN decides SPR triggers of T310 and T312 and performs root cause analysis. If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility. If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations. In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization. To assist in the forwarding of SPR, UE may include the following in SPR - CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed) - WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism) Reuse ACCESS AND MOBILTY INDICATION to forward SPR over XnAP and F1AP and use Uplink/Downlink RAN Configuration Transfer for forwarding SPR over NGAP To identify the UE context in the old source SN/old target SN when SPR is forwarded by old MN for SPR optimization, old MN identifies the UE context and sends the stored respective SN Mobility Information together with SPR to the old source SN/old target SN In case the SHR collected during an intra-NR HO is retrieved in a NR node different from source/target NR node, the receiving node performs initial analysis (identifies the node(s) to which the SHR is to be forwarded) and forwards the SHR to the corresponding node(s) which generates the SHR trigger condition that triggered the SHR (i.e., Option 3 is agreed) If the trigger is T312/T310, the objective of SPR is to - optimize PSCell change configuration and associated mobility thresholds - optimize lower layer issues of source PSCell (e.g., optimize T310/T312 timer values) Further, T310/T312 related SPR triggers can also be optimized to ensure UE doesn’t unnecessarily collect SPR or only rarely collects SPR Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN - MN is responsible to optimize PSCell change configuration and associated mobility thresholds - Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values) In case of SN initiated PSCell change, - Source SN may send the SN Mobility Information to MN via SN Change Required message (already supported by existing specifications) - Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message - If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP In case of MN initiated PSCell change, - Source SN may send the SN Mobility Information to MN via SN Release Request Acknowledge - Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message - If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP Inter-RAT SHR T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE. RAN3 thinks that at least the following parameters can be useful for optimizing inter-RAT successful handover from NR to LTE. LS RAN2 to confirm and request support. Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision. Source NR cell information Target LTE cell information Measurement results for source, target and neighbours Cause to indicate which inter-RAT SHR triggering condition was met UE location Information RAN3 thinks that all CHO related information in intra-NR SHR (e.g., time from CHO configuration to execution) is not applicable for inter-RAT SHR. RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18. Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18. WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell. If a different NR node (different from source NR node) retrieves the SHR collected during an inter-RAT HO (NR to LTE), reuse ACCESS AND MOBILITY INDICATION message (over XnAP and F1AP) and Uplink/Downlink RAN configuration transfer procedures (over NGAP) to forward the SHR to the source NR node No further discussion in RAN3 on above RACH related information. Support collection of SHR during successful inter-RAT HO (LTE to NR) for T304 trigger without any LTE impacts in R18, if the following principles are used. Send LS to RAN2 to confirm the first 4 bullets: - Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand - UE stores this SHR configuration in NR format - If T304 trigger is met, UE records SHR in NR format - UE reports this SHR to only an gNB (either the target gNB or another gNB) - gNB retrieving this SHR can forward this SHR to the target gNB for SHR optimizations The SHR collected during inter-RAT HO (LTE to NR) should include at least Source LTE cell and Target NR cell (assuming RAN2 confirms no LTE impacts based on the principles in Proposal 4) Successful PSCell Change Report The following information can be included as part of SPCR (parallel discussion happening in RAN2 as well, no need to LS RAN2 if already agreed in RAN2) Source PSCell information, in case of PSCell change/CPC Target PSCell information SPCR cause Latest measurement results Location information of the UE Time elapsed between the CPAC execution and reception of CPAC configuration, in case of CPAC LS RAN2 to check the reporting of SPCR (delayed or immediate). Ask RAN2 whether the SPCR can be stored at the UE and sent later or is sent immediately after the successful PSCell change or addition. T310 of SCG and T312 of SCG are not considered as SPCR triggers for classic PSCell addition or CPA (since there is no source SN undergoing RLF). Send LS to RAN2 to check which node (MN or SN) retrieves the SPCR from the UE, and which node may send the configuration to the UE. RAN3#120: SPR: If SPR available indication via SN RRCReconfigComplete is received by SN, SN should inform MN that an SPR is available at the UE e.g.., a new IE can be added in S-NODE MODIFICATION REQUIRED message. Turn the following WA to agreement: The triggers for SPR should be represented in terms of percentage values (similar to SHR). Continue the discussion based on R3-233335 |  |  |  |  |
| R3-233718 | Reply LS on SHR and SPR (RAN2(Huawei)) |  | LS in noted |  |
| R3-233877 | Discussion on remaining issue of inter-RAT SHR (NEC) |  | discussion |  |
| R3-234414 | (TPs for SON BLCRs for TS 38.423)Inter-RAT SHR and SPR (ZTE) |  | other |  |
| R3-233922 | Successful Handover Report and Successful PSCell Change Report (Qualcomm Incorporated) |  | discussion |  |
| R3-233899 | SHR and SPR (Huawei) |  | discussion |  |
| R3-233931 | (TP for SON BLCR for 38.423 and 38.300) SON enhancement for SHR (Samsung) |  | other |  |
| R3-233932 | (TP for SON BLCR for 37.340) SON enhancement for SPR (Samsung) |  | other |  |
| R3-234108 | (TP for SON 37.340 and 38.300) Discussion on SON enhancement for SHR and SPR (CATT) |  | other |  |
| R3-234172 | SON enhancements for SHR and SPR (Lenovo) |  | discussion |  |
| R3-234310 | (TP for SON BL CR for TS 38.423) Successful PSCell Report (SPR) for SON rel-18 (Ericsson) |  | other |  |
| R3-234316 | (TPs for SON BLCR for TS 38.423)Inter-RAT SHR and SPR (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234432 | Discussion on SON enhancement for Inter-RAT SHR (CMCC, Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234433 | Discussion on SON enhancement for SPR (CMCC) |  | discussion |  |
| Inter-RAT SHR: The correlation of inter-RAT SHR with RLF report can be up to network implementation? SS: 3 options on the table, opt1 can be taken as the solution in R18, no RAN2 impact. ZTE: UE assistance based solution needs to be discussed, all the solutions can be evaluated in R19. Lenovo: Only opt1 and opt2 can be discussed in RAN3 CMCC: there still have time to discuss the solution, exclude opt3 HW: Network solution is possible E///: Agree with ZTE, do not need any enhancements in co-relation Nok: Agree with CMCC CATT: Can consider UE assisted solution which needs to be checked in RAN2 QC: Reusing the solution in R17, which means opt1 Try to find the proceed way in R18: Either Reusing the solution in R17, opt1-1” Correlation at source gNB based on target C-RNTI” can be further discussed in R18, and check whether LS to RAN2 is needed. Or Postpone correlate of inter-RAT SHR and RLF in Rel-19. SPR: If the trigger is T304, the objective of SPR is to optimize RACH access issues of target PSCell (e.g., optimize RA resource)? CATT: including both RA issue and HO configuration ZTE: The trigger is provided by the target, the target uses this report to optimize RACH as what we had in R17. It’s over-engineering to let both the source node and target node to get SPR report. Nok: Same view as CATT SS: Share view as ZTE, there is no mechanism to use this in source node E///, HW: Which node needs to receive this SPR, our understanding is it can be used by both nodes Which node needs to receive this SPR in case the trigger is T304 and the benefits? Identify the standard impact if any. Which node decides trigger of T312/310 for MN-initiated classic PSCell change/CPC? Option 2: Source SN node decides the T310/T312 triggers and performs root cause analysis Option 3: MN decides the T310/T312 triggers and performs root cause analysis, and whether and what information from SN as input needs to be further discussed CMCC: Why SN information is needed? SS: Cannot agree to remove Opt2 Down-select Opt2 and Opt3 CB: # SONMDT1_SHRSPR - Discuss the open issues as above - Capture agreements and open issue, no new issues are allowed to be raised (moderator - QC) Summary of offline disc R3-234539 [DRAFT] Reply LS on SHR and SPR in R3-234612 |  |  |  |  |
| 10.2.2. MRO MRO for CPC and CPA based on the R17 NR-DC MRO solution MRO for the fast MCG recovery MRO for inter-system handover for voice fallback MRO for MR-DC SCG failure MRO for CPC and CPA: MRO for CPC and CPA based on the R17 NR-DC MRO solution Not consider too late CPA. CPA Execution to wrong PSCell will be considered, e.g. UE receives CPA configuration and CPA execution condition is satisfied, CPA execution fails or an SCG failure occurs shortly after a successful CPA execution; a suitable PSCell different with target PSCell is found based on the measurements reported from the UE. Too Late CPC Execution, Too Early CPC Execution and CPC Execution to wrong PSCell will be considered: - Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different with source PSCell is found based on the measurements reported for the UE. - Too Early CPC Execution: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; source PSCell is still the suitable PSCell based on the measurements reported from the UE. - CPC Execution to wrong PSCell: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE. For MRO for CPAC, deprioritize Case i/ii/iii/iv: - Case i: mixed scenarios of legacy PA and CPA, i.e. UE receives CPA configuration, a legacy PSCell addition is performed but fails, or a legacy PSCell addition is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell addition. - Case ii: mixed scenarios of legacy PC and CPC, i.e. UE receives CPC configuration, a legacy PSCell change is performed but fails, or a legacy PSCell change is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell change. - Case iii: MCG RLF or handover failure or CHO execution failure before CPA/CPC execution. - Case iv: CHO-CPC coexistence scenarios with low priority. Too Early CPA Execution will be considered. Naming for too early CPA execution? No need to rename. Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node. Information available in the network nodes should not be included in the SCGFailureInformation. Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN. For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events. Definitions of MRO events for CPAC will be introduced in TS 37.340 in a new chapter (based on [1226], CPA needs to be added). During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell. MRO for the fast MCG recovery: SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure) the signalling delay is longer than the time the UE waits for the response (T316 expired); other problems are not precluded if legacy MRO mechanism cannot cope with it. It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx). Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure. It is beneficial for the UE to report at least PSCell where SCG failure happened, the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure, SCG was deactivated or other cases that SCG is not available), and also if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx). Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed. Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running). MRO for inter-system handover for voice fallback: Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback: - Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell. - Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR. Deprioritize Case 5 for MRO enhancements for inter-system inter-RAT handover for voice fallback: - Case 5: the UE successfully performs inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, but the handover is about to failure. Deprioritize MRO enhancements for redirection for voice fallback. Introduce stage 2 descriptions of failure type definition for inter-system inter-RAT HO from NR to E-UTRA for voice fallback. The RLF Report needs to indicate that the last failed inter-system inter-RAT HO was triggered due to voice fallback. Stage-2 description of the detection mechanism will be introduced in TS 38.300 (based on R3-231585). MRO for MR-DC SCG failure: Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios. Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for NE-DC SCG failure, and necessary updates can be added on top of it if needed. Take Stage 2 descriptions of PSCell change failure in TS38.300 as baseline for NE-DC SCG failure, and necessary updates can be added on top of it if needed. Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for NGEN-DC SCG failure, and necessary updates can be added on top of it if needed. Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC SCG failure, and necessary updates can be added on top of it if needed. For MRO for MR-DC SCG failure, deprioritize dual failure case (i.e. both MCG failure and SCG failure occur). RAN2 deprioritized the NEDC/ENDC scenarios for SCG failure information report. MRO for MR-DC SCG failure is regarded in RAN3 as low priority pending to RAN2 progress in R18. RAN3#120: MRO for the fast MCG recovery: Agree to define case c in R18. Solution can be further discussed. Whether the network based solution or UE based solution should be used for Case c? Whether the MRO for fast MCG recovery needs to support pre-R18 UEs? MRO for the voice fall-back: RAN2 has decided to introduce the indication of voice fallback in the LTE RLF report. WA: Define a new handover report type in the Inter-system HO Report over S1 and NG. MRO for CPAC: Whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN? |  |  |  |  |
| R3-233933 | (TP for SON BLCR for 38.423 and 37.340) SON enhancements for CPAC and MCG failure recovery (Samsung) |  | other |  |
| R3-233934 | (TP for SON BLCR for 38.413) MRO for inter-system handover for voice fallback (Samsung) |  | other |  |
| R3-233923 | MRO enhancements for CPAC, voice fallback and fast MCG recovery (Qualcomm Incorporated) |  | discussion |  |
| R3-233900 | (TPs for SON BLCRs for TS 38.423) MRO on CPAC (Huawei) |  | other |  |
| R3-233901 | (TPs for SON BLCRs for TS 38.413) MRO on fast MCG recovery and voice fallback (Huawei) |  | other |  |
| R3-234055 | (TP for SON BL CR TS38.413) MRO enhancements in Rel-18 (ZTE) |  | other |  |
| R3-234109 | (TP for SON for 37.340 and 38.423) Discussion on MRO enhancement for CPAC (CATT) |  | other |  |
| R3-234110 | (TP for SON 38.413) Discussion on MRO enhancement for fast MCG recovery and voice fallback (CATT) |  | Other To be further discussed during offline discussion |  |
| R3-234173 | MRO for CPAC (Lenovo) |  | discussion |  |
| R3-234174 | MRO for fast MCG recovery and voice fallback (Lenovo) |  | discussion |  |
| R3-234311 | (TP for SON BL CR for TS 37.340) SON enhancements for MRO (Ericsson) |  | other |  |
| R3-233878 | Discussion on remaining issue of MRO (NEC) |  | discussion |  |
| R3-234323 | (TPs for SON BLCR for TS 38.423) Further discussion on the MRO for CPAC and for the fast MCG recovery (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234434 | Discussion on MRO for CPAC (CMCC) |  | discussion |  |
| R3-234175 | Update of the MRO scenarios (Lenovo, Qualcomm Incorporated, Huawei) |  | discussion |  |
| MRO for CPAC: Include CPC candidate cell list and CPC execution condition(s) to the message from the MN to the source SN. UHI infor for CPAC? HW: We do not see the need, can be discussed with low priority MRO for Fast MCG Failure Recovery: Solution for case c needs more inputs from RAN2. CMCC: RAN2 discussed the issue, we can wait for RAN2 progress - MRO for voice fall back: Convert the WA into agreement “Define a new handover report type in the Inter-system HO Report over S1 and NG”. The new Handover Report Type can be called “Inter-system Mobility Failure for Voice fall back” and can further include source NR cell ID, failed LTE cell ID and UE RLF Report container? And introduce re-establishment E-UTRA cell in HO report? ZTE: Align the name with stage2 as Inter-system Mobility Failure during Voice fall back HW: Need more time to discuss the solution CB: # SONMDT2_MRO - Check any open issues left? - - Check the details of TP for MRO for voice fall back in R3-234110 - - Provide TPs on MRO for CPAC - - Capture agreements and open issues (moderator - SS) Summary of offline disc R3-234540 |  |  |  |  |
| 10.2.3. RACH Enhancements RACH optimization for feature or feature combinations involving RACH partitioning (SDT, RedCap, Coverage Enhancement, network slicing, …) RACH report retrieval SN RACH report in MR-DC RACH partitioning: RA report is enhanced to include feature combination related information. RACH report retrieval: - WA: RAN3 works on the network based solution for RACH report retrieval, i.e., gNB-DU indicates to gNB-CU about RACH occurrence - WA: SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN. - Send LS to RAN2 with RAN3 assumption and ask RAN2 to provide feedback on UE based solution SN RACH report in MR-DC RAN3 has supported EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered in RAN3. If RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, UE should report the PScell identity outside the RACH report to help the network forward the report to the correct node. RAN3 supports a network-based solution for RACH report retrieval over F1AP based on an indication from the gNB-DU to the gNB-CU of successful RACH procedures which are not known to the gNB-CU (e.g., when RACH is triggered due to beam failure recovery, no PUCCH resource available, UL sync issue). Define a new class-2 F1AP message (e.g., RACH INDICATION) to indicate certain RACH occurrence(s) from gNB-DU to gNB-CU. SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN Define a new class-2 message (e.g., RACH INDICATION) over Xn so that the S-NG-RAN can inform M-NG-RAN that one or more RACH reports are available at the UE. The new Xn message should be non-UE associated. Using non-UE associated sigalling for RACH Indication over F1. RAN3#120: Not consider the NG and S1 forwarding of RA report. Include the “PSCell list” optionally in addition to the “RACH Report Container” in ACCESS AND MOBILITY INDICATION over Xn. FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report. LS to RAN2 on RA Report? Try to close this topic |  |  |  |  |
| R3-233715 | Reply LS on RACH enhancement (RAN2(ZTE)) |  | LS in |  |
| R3-233924 | RACH optimization enhancements (Qualcomm Incorporated) |  | discussion |  |
| R3-234312 | RACH Optimization enhancement (Ericsson) |  | discussion |  |
| R3-233935 | Discussion on SON for RACH (Samsung) |  | discussion |  |
| R3-234068 | (TPs for SON BLCR for TS 36.300, TS38.300 and TS 38.473): Remaining issues for RACH optimisation (Huawei) |  | other |  |
| R3-234069 | (TP for SON BLCR for TS 38.420): Introduction of RACH Indication (Huawei, China Telecom, CMCC) |  | other |  |
| R3-234111 | Discussion on RACH enhancement (CATT) |  | discussion |  |
| R3-234131 | Discussion on RACH optimization (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234176 | Discussion on SN RACH enhancements (Lenovo) |  | discussion |  |
| R3-234415 | (TPs for SON BLCRs for TS36.423 TS 38.423 TS 38.473 TS 37.340)RACH enhancements (ZTE) |  | other |  |
| R3-234416 | [Draft]Reply LS on Reply LS on RACH enhancement (ZTE) |  | other |  |
| SN RA Reports forwarding: It can be left to gNB implementation how to forward the SN RA Reports in case there is no X2/Xn connectivity between the node retrieving the RA Report from the UE and the PSCells indicated by UE in the RA Report? UE logs the PCell CGI of the MN and reports them to the NG-RAN along with the PSCell information and the NR RA report container as part of the LTE RA report? The MN that retrieves the SN RA Report may forward it to the source MN indicated in the UHI and enable SN RA report forwarding between eNBs by the Access and Mobility Indication procedure on X2 interface? Nok: Should not over engineering, enhancements of neighbour information needs further discussion SS: Agree not to over engineering It can be left to gNB implementation whether and how to forward the SN RA Reports in case there is no X2/Xn connectivity between the node retrieving the RA Report from the UE and the node serving the PSCells indicated by UE in the RA Report. ## RA Report Enhancements: Option 1: Include the feature priority Option 2: Include the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition) Option 3: Include the time between RACH access that led to the generation of a RA Report and reporting of the RA Report Option 4: The network controls the UE to report RA information Option 5: “Configuration Information” solution LS to RAN2? Check the feasibility and preference. QC: Opt1 and Opt2 are under the scope of RAN2 and not preferred. Opt3 can be regarded as compromised solution. E///: Why do not we send Opt1, 2 and 3 to RAN2 to check the feasibility in RAN2? Nok: Opt2 is not needed. Opt4 together with Opt3. It would be good if RAN3 can make the decision on the solution. HW: Fine to go for Opt3. E///’s proposal is also fine. CATT: Support Opt1 and Opt2 together. Lenovo: Opt3 is acceptable ZTE: All options can be checked by RAN2 SS: Opt1, 2 and Opt3 can be sent to RAN2 to check the feasibility. RAN3 works on the LS to RAN2 with Opt 2 and Opt 3 with analysis in RAN3 and check the feasibility and preference, FFS on Opt1. CB: # SONMDT3_RACH - Work on the LS to RAN2 - Check the details of TPs (moderator - HW) Summary of offline disc R3-234541 |  |  |  |  |
| 10.2.4. SON/MDT Enhancements for Non-Public Networks support of Signaling based MDT and Management based MDT for NPNs support both immediate MDT and logged MDT for NPN user consent handling for NPNs, in particular SNPNs area scope for NPNs support of NPNs in RLF Report and other UE reports used for SON and MDT RAN3 sends an LS to SA3 (cc SA5, RAN2) for user consent of NPN including the following aspects: 1: For PNI-NPN, whether existing user consent for management-based MDT (i.e., Management Based MDT PLMN List IE) can also apply for MDT in PNI-NPNs (no need of CAG-ID in user consent). 2: For SNPN, whether user consent for SNPN should include a list of SNPNs (PLMNs + NIDs) where management based MDT is allowed to take place. Introduce a CAG list for MDT area scope. The use cases RAN3 should support are: Use Case 1: Enhanced area scope information should allow collection of MDT measurements in specific PNI-NPNs, i.e. MDT measurements should be collected only within specific CAGs. Use Case 2: Enhanced area scope information should allow collection of MDT measurements both in specific PNI-NPNs (i.e. in specific CAGs) and in public network areas (e.g. specific PN cells, TAIs, etc.). Agree to the addition of a CAG list inside and outside the current choice structure for the MDT Area Scope. RAN3 to focus on the following use case for SNPN and to continue discussions on how to address MDT Area Scope for specific cells or TAs of an SNPN: Use Case 3: Enable collection of MDT measurements in the SNPN where the UE is registered. Wait for SA3 LS on MDT user consent for NPN. How to introduce NID or PNI-NPN ID into SON/MDT reports depends on RAN2’s decision. It is up to configuration and operator’s policies whether PN and NPN information can be included in the UE History Information. RAN3#120: Set the range of maxnoofCAGforMDT to 256 Add separate SNPN Cell Based MDT and SNPN TAI Based MDT area scope No need to introduce user consent of SNPN over interfaces. Add multi-SNPN MDT area scope. No need for SNPN-wide area scope? Try to close this topic |  |  |  |  |
| R3-234070 | Remaining issues for MDT in NPN (Huawei, Deutsche Telekom, CMCC) |  | discussion |  |
| R3-234313 | (TP for SON to BLCR for TS38.413, TS38.423, TS38.370) SON enhancements for Non-public networks (Ericsson) |  | other |  |
| R3-234417 | (TPs for MDT BLCRs for TS38.413 TS 37.320)MDT support in NPN (ZTE) |  | other |  |
| R3-234071 | (TP for MDT BL CR for TS 38.413): Remaining issues for MDT in NPN (Huawei, Deutsche Telekom, CMCC) |  | other |  |
| R3-234072 | (TP for MDT BLCR for TS 37.320): MDT support in NPN (Huawei, Orange, China Telecom, CMCC) |  | other |  |
| R3-234132 | Introduction of MDT enhancements to support Non-Public Networks (Nokia, Nokia Shanghai Bell) |  | draftCR Rev in R3-234607 |  |
| R3-234197 | (TP for MDT BL CR for TS 38.423): Remaining issues for MDT in NPN (Huawei, Deutsche Telekom, CMCC) |  | other |  |
| R3-234199 | MDT enhancement for NPN (CATT) |  | other |  |
| R3-234314 | Reply LS on user consent of Non-public Network (Ericsson) |  | LS out To: SA3, SA5 CC: RAN2 |  |
| The maximum number of SNPNs in the MDT SNPN list should be 16. QC: Send LS to RAN2 to check the value. No need for SNPN-wide area scope. HW, E///: This is agreeable Introduce a PNI-NPN PLMN wide? E///, Nok, SS, ZTE: This is a bit similar with SNPN case, do not understand the intention CATT: It depends on the meaning of PLMN wide IE. HW: It’s useful in the case that the number of PNI-NPN exceeds 250 ZTE: The current PLMN Wide is enough. Add NPN identifiers in the Area Scope of Neighbour Cells IE to enable operators to configure whether it is allowed/not-allowed to collect data in neighbouring NPN cells? Nok: For SNPN, we do not see the point. For PNI-NPN, do not see the use case at all. HW: Why only some UEs need to be configured with information? Send an LS reply to SA3 and SA5 to inform them that RAN3 expects SA5 to apply changes to TS32.422 to describe that user consent for MDT is not applicable to SNPNs as per inputs in S3-231399? Impact on UE history information? CB: # SONMDT4_NPN - Capture the agreements into TP - Check other open issues, try to close this topic in this meeting - LS to RAN2 about the progress? - LS to SA5 on SA3 user consent agreement? (moderator - ZTE) Summary of offline disc R3-234542 |  |  |  |  |
| 10.2.5. SON for NR-U NR-U for MRO: Add to RLF report indications concerning Measured RSSI and HOF due to consistent LBT failure. Keep existing failure type definition and detection to indicate RLF or HOF or PSCell change failure due to consistent LBT failure. Add indications of consistent LBT failures in RA report. Enhancements of RLF reports and RA reports are beneficial to separate mobility related errors from the LBT-related ones. RLF Report and RA report can be enhanced to include information concerning the LBT failures in RA procedures, the granularity and implementation details needs to be further discussed based on progress in RAN2. NR-U for MLB: Exchange over Xn of Energy Detection Threshold for UL, and Channel Occupancy Time in UL is supported. For XnAP, add in the RESOURCE STATUS UPDATE message a Channel Occupancy Time Percentage UL IE and an Energy Detection Threshold UL IE as sub-IEs of NR-U Channel Item IE. Corresponding TP is in R3-226040. Rename the existing Channel occupancy time percentage DL IE as Channel Occupancy Time Percentage DL both in F1AP and XnAP from R17. DU shall report the UL COT to CU in resource status report procedure when it is requested by CU, how to obtained this information by DU is up to implementation. The granularity of UL COT reporting in F1 interface should be NR-U channel level. WA: a gNB sends in resource status reporting via Xn an EDT UL that reflects at least the maximum EDT UL configured for the UEs. The presence of COT percentage UL in F1 is optional. RAN3#120: NR-U for MRO There is no need for the UE to report the average of the applied EDT UL in RLF report. FFS on which information on EDT in UL to report, if any. NR-U for MLB The EDT in UL included in the XnAP RESOURCE STATUS UPDATE message reflects the average of the configured maximum EDT UL. TP for XnAP is in R3-233491. FFS on the introduction in F1 of EDT in UL in RESOURCE STATUS UPDATE message. WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message. |  |  |  |  |
| R3-233925 | SON enhancements for NR-U (Qualcomm Incorporated) |  | discussion |  |
| R3-234315 | (TP for SON BL CR for TS 38.423, TS 38.473) NR-U enhancements for MRO and MLB (Ericsson) |  | other |  |
| R3-233936 | Discussion on SON for NR-U (Samsung) |  | other |  |
| R3-233862 | Further discussion on LBT waiting time for correct classification of MRO events and optimisation of the EDT (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234056 | Further discussion on NR-U optimizations (ZTE) |  | other |  |
| R3-234073 | (TP for MDT BL CR for TS 38.300): SON for NR-U (Huawei) |  | other |  |
| R3-234112 | (TP for SON for TS 38.473) Discussion on SON enhancement for NR-U (CATT) |  | other |  |
| R3-234177 | (TP for SON BLCR to TS38.423) Discussion on MRO for NR-U (Lenovo) |  | Other Move to 10.2.5 |  |
| MLB for NR-U: Convert the following WA into agreement: WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message. There is no need to transfer the UL EDT in resource status update message via F1 interface. E///, CATT: Propose to have it as optional IE ZTE, SS, HW: CU is able to decode this infor by itself MRO for NR-U: Continue the discussion on RLF MRO report optimization: - actual UL EDT - actual minimum UL EDT - lowest detected power QC: RAN2 is discussing on this. SS, Nok: RAN3 can give some analysis information to RAN2 Downlink LBT issue during HO exaction, whether the failure needs to be distinguished. UE based solution and network based solution Lenovo: Prefer UE based solution SS, ZTE: Not convinced about the scenario, which is the corner case. There is no enhancements on this case. CATT: Both solutions can be discussed, the failure needs to be distinguished CB: # SONMDT5_NRU - Capture the agreements in TP - Discuss the open issues as above - Capture agreements and open issues (moderator - E///) Summary of offline disc R3-234543 (TP for SON BL CR for 38.423) NR-U enhancements for MLB in R3-234544 (TP for SON BL CR for 38.473) NR-U enhancements for MLB in R3-234545 |  |  |  |  |
| 10.2.6. MDT Enhancements Check RAN2 progress on logged MDT override It is RAN3’s understanding that the Rel-17 feature of signaling based logged MDT override protection applies only during Intra-NR reselection and applies to Intra-5GS (gNB–>gNB). Whether to support cross-RAT logged MDT reporting (i.e., whether the NR node needs to retrieve LTE logged MDT report) for signaling based logged MDT override protection is pending on RAN2 progress. Wait for RAN2’s progress on cross-RAT logged MDT reporting before discussing whether any enhancements are needed for NG-RAN to forward the LTE logged MDT reports to the correct TCE. RAN3 confirms that the scenarios for inter-RAT signalling based logged MDT protection includes the following: Scenario 1: Inter-system inter-RAT: EPC –> 5GC Scenario 2: Intra-system Inter-RAT and intra-5GC: LTE –> NR RAN3#120: There is no requirement that NR signalling based logged MDT has a lower priority than E-UTRAN signalling based logged MDT. Check RAN2 progress. |  |  |  |  |
| R3-234133 | Further discussion on signalling based logged MDT override protection (Nokia, Nokia Shanghai Bell) |  | Discussion Proposal 1: Support configuration of priority to E-UTRAN s-based logged MDT per target NR cell or gNB. Proposal 2: No need for error signalling/new cause on NGAP in case of failed activation of NR MDT. Proposal 3: If priority of E-UTRAN s-based logged MDT has been configured as per proposal 1: If the UE indicates sigLogMeasConfigAvailable set to true in NR RRCSetupComplete or RRCResumeComplete messages, while not indicating availability of Logged MDT data, the gNB shall not configure the UE with s-based or m-based logged MDT. ZTE: P1 is not align with agreements in RAN2 HW: There is no requirement to have this priority infor E///: OAM awares the priority and can decide whether to send s-based logged MDT ZTE: What E/// said needs the co-ordination between 4G OAM and 5G OAM. noted |  |
| R3-234418 | (TPs for MDT BLCRs for TS 38.413)MDT Enhancements (ZTE) |  | Other A new cause value introduced in NGAP to inform Core network and OAM that NR Signalling based logged MDT failed due to protection of LTE signalling based logged MDT. E///: It’s not a good assumption that there is no coordination between 2 systems at all noted |  |
| 11. Enhancement on NR QoE WI (RAN3-led) WID [NR_QoE_enh-Core]: RP-223488 (target: RAN #102) [TU: 1 (1, 1, 1)] QUOTA: 4 |  |  |  |  |
| ## 11.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233762 | QoE in NR-DC (Ericsson) |  | CR1069r1, TS 38.423 v17.5.0, Rel-18, Cat. B |  |
| R3-233777 | (BLCR to 37.340) QMC enhancements for NR-DC (Nokia, Nokia Shanghai Bell, ZTE, China Unicom, Orange, Ericsson) |  | draftCR |  |
| R3-233785 | Support of Enhancement of NR QoE (China Unicom) |  | draftCR |  |
| R3-233792 | (BLCR) Enhancement on NR QoE (Samsung) |  | CR0267r5, TS 38.401 v17.5.0, Rel-18, Cat. B |  |
| R3-233795 | (BLCR to 38.413) Introduction of R18 QoE measurement (Huawei, China Telecom, China Unicom) |  | CR0984r3, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233807 | Enhancement on NR QoE (ZTE) |  | CR1070r6, TS 38.473 v17.5.0, Rel-18, Cat. B |  |
| R3-234389 | (BLCR of 38.300 to merge RAN2 CR) Support of Enhancement of NR QoE (China Unicom) |  | draftCR |  |
| R3-234385 | Update Workplan for Rel-18 NR QoE Enhancement (China Unicom) |  | Work Plan |  |
| ## 11.2. Support for New Service Type and RRC_INACTIVE/RRC_IDLE states Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2]. - Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains. Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. Both signalling based and management based QoE measurements in RRC INACTIVE/IDLE mode shall be supported in Rel-18. UE handles area scope checking for QoE measurements in RRC INACTIVE/IDLE mode. Whether UE AS layer or UE APP layer handle the area scope is to be discussed based on RAN2 progress. Support MBS broadcast service INACTIVE/IDLE QoE first. UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE. If the UE receives the configuration in RRC connected state, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast service for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline. Whether UE can only report the INACTIVE/IDLE QoE reports to gNB when the UE has entered to the RRC_CONNECTED due to other reasons is pending to RAN2 discussion. RAN3 discuss the alignment between logged MDT and MBS QoE when basic solution for MBS QoE has been defined first. RAN3 continues to discuss how to handle the QoE reports sent at new gNB when UE was in RRC_IDLE. OAM should have the flexibility to collect QoE only in high mobility scenarios and/or in HSDN cells instead of collecting blindly. No enhancements on paging for the purpose of configuring UE with legacy QoE measurement for the RRC_IDLE/INACTIVE UEs. Legacy paging only for legacy QoE purpose is up to implementation. Use the same set of parameters in QMC configuration for all RRC states. RAN3 assumes that there is no need to request QoE measurements per UE RRC state. WA: MBS service area can be expressed by QoE area scope IE, FFS on whether any enhancements of this IE are needed. The RVQoE discussion on supporting AR/MR and MBS specific metrics may only be triggered based on progress of SA4. The RRC state info when UE collects the uploaded QoE data shall not be reported in QoE report for MBS BC. MBS MC can be discussed later. MBS BC QoE measurements can proceed after the UE switches from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED. RAN3 to discuss which configuration information related to QoE measurement needs to be available in the new gNB. At least the following QoE configuration related information for MBS broadcast service should be available in the new gNB: - QoE reference - Measurement Collection Entity Information, the detail information can be further discussed RAN3 shall discuss which of other QoE configuration info for MBS BC QoE shall be available in the new gNB. AR/MR will be supported as new service type and take R17 legacy QoE mechanism as baseline, pending on SA4’s further progress in R18. Configuration container need not to be provided to the new gNB for MBS broadcast service. RRC level ID (measConfigAppLayerID) for MBS broadcast service should be available in the new gNB. For MBS QoE, an M-based QoE configuration shall not overwrite the S-based QoE configuration stored at the UE by the new gNB. QoE measurement type (s-based or m-based measurement) for MBS broadcast service should be available in the gNB serving the UE after the transition from RRC_IDLE to RRC_CONNECTED. RAN3 first focus on supporting the following scenario QMC: - QoE measurement collection and reporting when the UE is in HSDN cells For confining the QoE measurements to HSDN cells, RAN3 to choose between the HSDN-wide indication, existing area scope and other possible enhancements if needed. For supporting QMC in high mobility scenarios, RAN3 to determine the meaning of “high mobility”. RAN3#120: LS to SA4, SA5 on QoE measurement collection for application sessions delivered via MBS broadcast or multicast in R3-233457 Any other issue to be solved in R18? |  |  |  |  |
| R3-233711 | LS on area scope for QoE measurements (RAN2(Samsung)) |  | LS in |  |
| R3-234094 | Further discussion on the support of MBS QoE (Huawei) |  | discussion |  |
| R3-234043 | QoE and RVQoE Measurement Collection for Sessions Carried via MBS (Ericsson) |  | discussion |  |
| R3-234478 | Discussion on INACTIVE/IDLE QoE and high mobility QoE (ZTE, China Telecom, CMCC) |  | discussion |  |
| R3-233926 | QMC for applications carried over MBS broadcast and multicast (Qualcomm Incorporated) |  | discussion |  |
| R3-234248 | QoE configuration and reporting in RRC_INACTIVE RRC_IDLE states (Xiaomi) |  | discussion |  |
| R3-234029 | Remaining issues on QMC for MBS and RRC state (Samsung) |  | discussion |  |
| R3-234030 | [draft] Reply LS on area scope for QoE measurements (Samsung) |  | LS out To: RAN2 CC: SA4, SA5 |  |
| R3-234095 | [draft] Reply LS to Reply LS on QoE measurements in RRC IDLE/INACTIVE states (Huawei) |  | LS out To: SA5, RAN2 CC: SA4 |  |
| R3-234096 | [draft] Reply LS on area scope for QoE measurements (Huawei) |  | LS out To: SA5,SA4,RAN2 CC: |  |
| R3-234479 | Draft reply LS to RAN2 on area scope for QoE measurements (ZTE) |  | other |  |
| R3-234127 | Continued discussion on solutions for QMC in RRC_IDLE (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234178 | Discussion on QoE measurement in RRC_INACTIVE and RRC_IDLE states (Lenovo) |  | discussion |  |
| R3-234179 | (TP to 38.423 & 38.420) Support of QoE measurement in RRC_INACTIVE (Lenovo) |  | other |  |
| R3-234390 | Discussion on NR QoE in RRC_INACTIVE/RRC_IDLE states (CATT) |  | discussion |  |
| R3-234401 | Discussion on QoE measurement in RRC_INACTIVE and RRC_IDLE states (China Unicom) |  | discussion |  |
| R3-233927 | QoE enhancements for high mobility scenarios (Qualcomm Incorporated) |  | discussion |  |
| R3-234044 | QMC in HSDN Cells and High Mobility Scenarios (Ericsson) |  | discussion |  |
| ## 11.3. Support QoE for NR-DC Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2]. - Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity. - Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios. - Specify the QoE measurement continuity in mobility scenarios in NR-DC. - Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC. MN is responsible to configure the s-based QoE to UE. For M-based QoE configuration in NR-DC, coordination between MN and SN is needed. If the M-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE. If the M-based QoE configuration is received only by the SN, whether the MN or the SN performs UE selection and sends the QoE configuration to the UE needs to be further discussed. QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session. If QoE reports are received by the SN, SN can forward the QoE reports to MCE directly. RAN3 should discuss and clarify the scenarios for QoE reporting transmitted over SN. Which SRB can be used for QoE reporting in SN depend on RAN2. WA: MN and SN can generate RVQoE configurations. MN and SN should coordinate about configuring a dual-connected UE with RVQoE measurements. The details of the coordination are FFS. WA: UE can send RVQoE report to MN, MN then forward the RVQoE report to SN if needed, and vice versa. In DC, the UE switches the reporting leg based on indication from network, FFS on implicit or explicit way. RAN3 should discuss which node can command the UE to switch the reporting leg. If a node has configured the UE with QoE measurements, and the other node is receiving the QoE reports from the UE and forwarding them directly to the MCE, then: The node that has configured the UE with QoE measurements should indicate the QoE reference to the node that receives the reports and forwards them directly to MCE. The MN can generate an RVQoE configuration for a UE. The SN can generate an RVQoE configuration for a UE. FFS whether MN can modify the SN generated RVQoE configuration The MN can send an RVQoE configuration to the UE. The MN can receive RVQoE reports directly from the UE. The SN can receive RVQoE reports directly from the UE. Turn the following WA into an agreement: “UE can send RVQoE report to the MN, the MN then forward the RVQoE report to the SN if needed, and vice versa”. Agree to ensure that the RVQoE report is sent to the node(s) that provide the bearer(s) associated to the corresponding RVQoE measurement result in the RVQoE report. The coordination between the MN and the SN should support at least the following (details to be further discussed): - Initiation by either the MN or the SN for m-QoE, by the MN for s-QoE. - Coordination for configuring the UE. - Coordination for establishing the SRB for receiving QoE/RVQoE reports. - Indication about switching the reporting leg. In case of management-based QoE, the MN decides which node to perform the QoE measurement configuration, FFS which node (MN or SN) performs UE selection. When MN configures a UE with m-based QoE, it may indicate to SN: the QoE Reference, the MCE IP address. If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure. The MN should inform the SN that a UE is configured with an m-based QoE measurement. For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether: The MN shall send the configuration information to the UE, or The SN should send the configuration to the UE directly, or The SN should send the configuration information to the UE via the MN (inside a container). The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN. To determine which node(s) provide the bearers carrying an application session, a node can configure RVQoE measurements at a UE in NR-DC: - For the first RVQoE configuration, it is blindly configured by MN or SN. - From the PDU session ID and QFI in the first RVQoE report this node determines which node(s) provide the bearer(s) associated to the corresponding application session. - After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified. If a node receives an RVQoE report from a UE in NR-DC, and determines that the bearers for the application session are also or only provided by the peer node, this node can send the received RVQoE report to the peer node. Turn WA to agreement: SN can send an RVQoE configuration to the UE. The information used by the SN to express to the MN its interest in configuring a UE with an m-based QoE and RVQoE measurement configuration, shall contain the QoE reference. Support the following scenarios for m-based QoE configuration received in the SN: The SN wants to configure the UE by using SRB3. The SN wants to configure the UE, by sending the configuration in a transparent container to the MN, which then sends it to the UE via SRB1. Discuss which parameters the SN needs to indicate to the MN, to express its interest in configuring a UE with an m-based QoE measurement and the corresponding RVQoE measurement. The MN and the SN coordinate the RRC ID allocation for m-based QoE measurements to be configured at a UE, on a per-QoE reference basis. When the MN approves that the SN configures the UE with a certain m-based QoE configuration, the MN assigns an RRC ID for this m-based QoE configuration and indicates it to the SN. When SN indicates its interest in configuring m-based QoE a measurement to a UE: The SN can indicate to the MN that the reports are to be sent via the SRB5. The SN can request the use of the SRB4 for reporting, which the MN can confirm or reject. The network can explicitly instruct a UE in NR-DC to switch the reporting leg. The leg switching command can be sent to the UE by the node that configured that specific QoE configuration. The node that currently receives the QoE reports via the Uu can send a request to the peer node, asking that the QoE reporting leg is switched to the peer node. The leg switch for QoE reporting needs to be approved by both nodes serving the UE. If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID. If the MN is asked by the SN to forward to the MCE the QoE reports pertaining to a measurement configured by the SN, the SN should indicate to the MN the QoE Reference and the MCE IP Address. As the baseline, QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over the same leg. WA: QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different legs. If the SRB5 is not configured, the RVQoE reports can be sent on the SRB4 from the UE via the MN to the SN. For UEs in NR-DC, the node that configured the UE with a QoE measurement configuration can generate the corresponding RVQoE measurement configuration. The node that has initially configured a UE in NR-DC with an RVQoE configuration can modify and release the RVQoE configuration as long as this node serves the UE. The SN can send an RVQoE configuration directly to UE via SRB3 or in a transparent container to the MN, which then sends it to the UE via SRB1. Consider the QoE measurement reporting for NR-DC in following scenarios: SCG failure scenario. SN release scenario. RAN overload scenario. QMC continuity during mobility in NR-DC should be discussed after the baseline solution for QMC in NR-DC is in place. RAN3#120: When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5 or SRB4, whether it is transparently via SRB4 needs further discussion. RAN3 understanding is that the UE can be explicitly instructed whether to report over SRB5 or SRB4. When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit. In the response, the non-RVQoE-configuring node: - Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports. - Can indicate whether it prefers to receive the RVQoE reports directly from the UE. - Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration. In line with the Rel-17 principles, if the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RVQoE measurements. At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN. If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed. Finalize the basic signaling flow and stage2 and stage3 details |  |  |  |  |
| R3-234421 | Discussion on QoE in NR-DC (ZTE, China Unicom, China Telcom) |  | discussion |  |
| R3-234422 | (TP to BL CR of 38.423) QoE in NR-DC (ZTE, China Unicom, China Telecom) |  | Other Rev in R3-234591 |  |
| R3-234423 | (TP to BL CR of 37.340) QoE in NR-DC (ZTE, China Unicom) |  | other |  |
| R3-233928 | Support for QoE in NR-DC (Qualcomm Incorporated) |  | discussion |  |
| R3-234097 | Further discussion on the support for QoE in NR-DC (Huawei) |  | discussion |  |
| R3-234045 | (TPs for QoE BL CR for TS 37.340 and TS 38.300) QoE and RVQoE Measurements and Reporting in NR-DC Scenarios (Ericsson) |  | other |  |
| R3-234031 | Remaining issues on support of NR-DC (Samsung) |  | discussion |  |
| R3-234098 | (TP for 38.423) on QoE measurement enhancements (Huawei) |  | other |  |
| R3-234128 | Further discussion on QoE/RVQoE measurement report continuity and RAN overload in NR-DC (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234180 | Discussion on QoE measurement in NR-DC (Lenovo) |  | discussion |  |
| R3-234249 | Discussion on QoE configuration and reporting in NR-DC (Xiaomi) |  | discussion |  |
| R3-234391 | Discussion on Support for legacy QoE in NR-DC (CATT) |  | discussion |  |
| R3-234392 | Discussion on Support for RV-QoE in NR-DC (CATT) |  | discussion |  |
| R3-234402 | Discussion on QoE measurement in NR-DC (China Unicom) |  | discussion |  |
| R3-234606 | Summary of offline on QoE in NR-DC (ZTE) |  | discussion |  |
| R3-234403 | (TP to BL CR of 38.423) QoE in NR-DC (China Unicom) |  | Discussion withdrawn |  |
| ## 11.4. Others Left-over features from Rel-17, as well as the enhancements of existing features which are not included in Rel-17 normative phase, should be supported in Rel-18 if consensus on benefits are reached [RAN3, RAN2]. - Specify per-slice QoE measurement configuration enhancement. - Specify RAN visible QoE enhancements for QoE value, RAN visible QoE trigger event, RAN visible QoE Report over F1. - Specify QoE reporting handling enhancement for overload scenario. Including other RAN2 led objectives Introduce the slice scope information in the configuration container, and send LS out to SA4. Definition of RVQoE value needs cooperation with SA4. UE should include QoS flow information in the RVQoE report to RAN. QoS flow information should be introduced as an explicit IE in the RAN visible QoE report over F1. QoS flow ID(s) should be included in the RAN visible QoE report collected at the UE. DRB ID(s) should be transmitted over F1 as the QoS flow information in the RVQoE report. RAN3 checks with SA4 on whether RVQoE value can reflect the overall situation of the experience of an ongoing service, with multiple QoE metrics taken into account, not limited to only RVQoE metrics. RVQoE value is used by the RAN node for radio resource optimization, and can save on uplink RRC signaling, compared with transferring multiple QoE metrics (not only RAN visible QoE metrics). In this release, slice information (e.g. S-NSSAI) is not included in RVQoE report. Introduce buffer level as a threshold-based trigger for RVQoE reporting. Do not introduce the threshold-based trigger for reporting playout delay for media startup. The final list of topics that are to be discussed in Rel-18: RVQoE value (pending SA4 reply). Assistance information for handling of QoE reporting upon RAN overload. DU activation/deactivation/pause/resume of RVQoE reporting over F1. DU participation in assembling the RVQoE configuration. Event-based RVQoE reporting trigger. - RAN3 to await the further progress of SA4 and ITU-T before proceeding further on the topic of RVQoE value. In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE. Introduce the deactivation of RAN visible QoE information transfer via F1. No need to introduce pause/resume mechanism in Release 18. The assistance information can be introduced only when the clear definition is defined. Triggers for RVQoE reporting Radio-related event triggers for RVQoE reporting is not supported in Rel-18. If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the application session or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time. Discuss whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two thresholds. RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options: Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports) Option 2: Periodically based on a gNB configured reporting periodicity Option 3: A certain number of times based on gNB configured report amount Support QoE during Intra-5GC Inter-RAT handover: WA: For HO from LTE/5GC to NR, there is no impacts to RAN3. RAN3#120: Threshold-based triggers for RAN visible QoE The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed. DU participation Turn WA to agreement: A class-2 procedure is used for DU to deactivate the RVQoE reporting over F1AP. UE associated or non-UE associated? Support QoE during Intra-5GC Inter-RAT handover: WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep. Focus on above open issues |  |  |  |  |
| R3-233813 | Reply LS on buffer level threshold-based RVQoE reporting (SA4(Apple)) |  | LS in |  |
| R3-234130 | LS-related discussions (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-233929 | Threshold based reporting for buffer level (Qualcomm Incorporated) |  | discussion |  |
| R3-234129 | (TP for BLCR to TS 38.473) Discussion on report deactivation in F1 interface (Nokia, Nokia Shanghai Bell, ZTE, Xiaomi) |  | other |  |
| R3-234425 | (TPs for BL CR of TS38.401&38.470) Deactivation of RVQoE reporting over F1AP (ZTE, Nokia, Nokia Shanghai Bell, China Unicom, China Telecom) |  | other |  |
| R3-234424 | Discussion on other issues (ZTE, China Unicom, China Telecom, CMCC) |  | discussion |  |
| R3-234099 | Further discussion on the support of R17 left-over features and inter RAT handover (Huawei, China Unicom, China Telecom) |  | discussion |  |
| R3-234181 | Discussion on QoE measurement during intra-5GC inter-RAT handover (Lenovo) |  | discussion |  |
| R3-234250 | Discussion on R17 leftovers (Xiaomi) |  | discussion |  |
| R3-234393 | Discussion on Left-over issues (CATT) |  | discussion |  |
| R3-234404 | NR QoE continuity during intra-5GC inter-RAT handover (China Unicom) |  | discussion |  |
| R3-234046 | (TP for QoE BL CR for TS 38.473) Enhancements of Rel-17 QoE and RVQoE Features (Ericsson) |  | other |  |
| R3-234559 | Offline discussion on QoE AI11.4 (Ericsson) |  | Discussion |  |
| 12. AI/ML for NG-RAN WI (RAN3-led) WID [NR_AIML_NGRAN-Core]: RP-231159 (target: RAN #100) [TU: 2 (2, 2, 2)] QUOTA: 6 |  |  |  |  |
| ## 12.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233756 | (BLCR to 38.423) for AI/ML for NG-RAN (Ericsson, Nokia, Nokia Shanghai Bell) |  | CR0959r7, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233780 | Draft CR to 38.300 on AI/ML for NG-RAN (CMCC, ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, CATT, Samsung, Lenovo, Intel Corporation) |  | draftCR Endorsed as BL CR |  |
| R3-233789 | CR to TS 38.401 for addition of AI/ML-RAN feature in the case of split architecture (ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Samsung, Intel Corporation, CMCC) |  | CR0265r7, TS 38.401 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-234444 | Work plan for Rel-18 AI/ML for NG-RAN (CMCC, Ericsson) |  | Work Plan noted |  |
| 12.2. Data Collection Enhancements and Signaling Support Normative work is based on the conclusions captured in TR37.817. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3) Note: On security impacts, coordination with SA3 when needed. On OAM aspects, coordination with SA5 when needed. Note: Specify new UE measurements when needed if any. Note: Specify MDT procedure enhancements when needed if any. |  |  |  |  |
| 12.2.1. Stage2 Related e.g., functional Framework for RAN Intelligence Not to capture the flow charts right now, can be considered after the standard impacts are identified. Both non-split architecture and split architecture are in scope. Focus on the non-split architecture first. Split architecture should be specified after the work of non-split architecture. The training/inference function location is referred to TR37.817. Capture the Abbreviations of AI/ML in TS38.300. Capture the general introduction of AI/ML in TS38.300. Focus on Xn interface first. Start from SA and then consider DC. Procedures used for AI/ML support in the NG-RAN shall be use case agnostic. The cases of i) Model Training and Model Inference at the NG-RAN and ii) Model Training at OAM and Model Inference at the RAN, make use of the same procedures, with the exception that procedures for exchange of training data and feedback data (which is not related to model performance feedback) will be different for i) and ii). An LS to SA5 concerning procedures to signal training and feedback data from RAN to OAM can be sent when the details of such procedures are fully agreed in RAN3 Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures) Cell based UE Trajectory Prediction is transferred via existing HO signalling messages, it’s FFS on whether other way to transfer the cell based UE Trajectory Prediction information is needed. Signalling describing the capability to support specific information predictions used for AI/ML is not pursued in this release Signalling describing the capability to supports specific AI/ML use cases is not pursued in this release AI/ML capability exchange in NG-RAN can be achieved by means of procedures for AI/ML information request, AI/ML information response and AI/ML Information Request Failure WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”. Solutions for AI/ML information exchange over the NG interface are not considered as part of Rel-18. Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated. The requested prediction time is configured in the AI/ML INFORMATION REQUEST for one-time reporting. Requested prediction time: time in the future for which the prediction information is requested in the AI/ML INFORMATION REQUEST. RAN3#120: The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed. FFS on the encoding and exact values of the specific time point. Down-select the options: - INTEGER TYPE - ENUMERATED TYPE FFS on the benefits of the predicted information over a period of time. The specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future, can be configured for both one-time reporting and periodic reporting. Time information for periodic reporting? |  |  |  |  |
| R3-233874 | AI/ML predicting accuracy (NEC) |  | discussion |  |
| R3-233911 | Discussion on Prediction Time and Accuracy (Intel Corporation) |  | discussion |  |
| R3-233937 | Discussion on time information in AI/ML information reporting (Samsung) |  | discussion |  |
| R3-233995 | Discussion on Xn Enhancements for NG-RAN AI/ML (Qualcomm Incorporated) |  | discussion |  |
| R3-234203 | (TP to 38.423) Discussion on the duration for which the prediction is (CATT) |  | discussion |  |
| R3-234291 | Open points on requested prediction time (Ericsson, InterDigital, Deutsche Telekom) |  | discussion |  |
| R3-234292 | (TP for AI/ML on BLCR to TS38.423) Requested prediction time (Ericsson, InterDigital, Deutsche Telekom) |  | other |  |
| R3-234328 | Characteristics of the procedures for exchanging AI/ML-related information (Ericsson, InterDigital) |  | discussion |  |
| R3-234340 | (TP for TS 38.423) Timing Information and Confidence Discussion (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234376 | (TP to 38.423 & 38.300) Further discussion on Timing Information (ZTE) |  | other |  |
| R3-234386 | Discussion on open issues related to predicted time information (LG Electronics Inc.) |  | discussion |  |
| R3-234387 | (TP for NR_AIML_NGRAN-Core BL CR for TS 38.423) Discussion on open issues related to predicted time information (LG Electronics Inc.) |  | other |  |
| R3-234441 | Discussion on AIML time information (CMCC) |  | discussion |  |
| R3-234186 | Discussion on the time information in the request message (Lenovo) |  | Discussion Move to 12.2.1 |  |
| R3-234187 | Discussion on prediction accuracy (Lenovo) |  | Discussion Move to 12.2.1 |  |
| R3-234278 | Further discussions on common issues and Stage 2 updates on the introduction of RAN AI/ML (Huawei) |  | discussion |  |
| R3-234279 | (TP for AI/ML BL CR for TS 38.300) Stage 2 updates on the introduction of RAN AI/ML (Huawei, CATT, China Unicom) |  | other |  |
| R3-234290 | (TP for AI/ML BLCR to TS38.300) Characteristics of the procedures for exchanging AI/ML-related information (Ericsson, InterDigital) |  | other |  |
| R3-234288 | Discussion on AI/ML Procedures Names (Ericsson, InterDigital, Nokia, Nokia Shanghai Bell, Deutsche Telekom, AT&T, CATT, Orange) |  | discussion |  |
| R3-234289 | (TP for AI/ML BLCR to TS38.423) Procedure Name (Ericsson, InterDigital, Nokia, Nokia Shanghai Bell, Deutsche Telekom, AT&T, CATT, Orange) |  | other |  |
| R3-234185 | (TP for TS38.423 BLCR) On the naming of the new procedures (Lenovo) |  | other |  |
| R3-234002 | Discussion on validity time and prediction accuracy (China Telecommunication) |  | Discussion Late contribution |  |
| Time information: The encoding of Requested Prediction time as Integer with maximum value as 60 seconds with extensible IE structure. Integer or enumerated? HW: Should go for Integer, which is future proof and more flexibility E///, Interdigital, DT: It’s not clear on the flexibility of “Integer” type, would like to suggest enumerated and extensible. Try to limit the choices. Nok: Agree with HW, there will be a long list if it needs to be extended in future ZTE, CATT, CMCC, SS: Agree with HW, which is more flexible if takes the requirements of AI model into account QC: Prefer to enumerated type Lenovo: Both work NEC: Try to find the middle way, e.g., defining different ranges as enumerated Whether to include time interval in the request message, for one-time reporting and periodic reporting? E///: It can be achieved by requesting in the time which the prediction is needed. Nok: Support E/// view. The benefits are not convinced. CATT: Time interval can be used as optional IE, which provides the flexibility in the source node. ZTE: Similar like legacy resource status procedure, the target node need to understand the time interval to generate the prediction result Accuracy information: There is no need to transfer the prediction accuracy information over Xn in R18. The requesting node may ask for providing prediction only the prediction reach a certain level of accuracy determined by the predicting node? HW, ZTE: How to define the level of accuracy? CB: # AIRAN1_Timeinfor - Continue the discussion on above open issue - Capture agreements in TP (moderator - ZTE) Summary of offline disc R3-234547 |  |  |  |  |
| 12.2.2. Stage3 Related Specify data collection enhancements and signaling support Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information. The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure. The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point. Introduce a new Class 1 procedure for initiating the reporting of AI/ML Related Information and a Class 2 procedure for Data Reporting of AI/ML Related Information. Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting. The new procedure is non-UE associated procedure. If needed, the procedure can be used to capture UE-associated information. The response message of the new procedure for AI/ML Related Information indicates if the requested information can be provided. How to indicate validity time (e.g., implicitly with a new prediction when the previous prediction becomes invalid, explicitly with every prediction in the AI/ML output or by the request to the prediction) shall be discussed on a case by case basis. |  |  |  |  |
| 12.2.2.1. LB and Xn procedures AI/ML based Load Balancing: The following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn - UE performance (e.g, UL/DL throughput, packet delay, packet loss) Predicted Resource Status Information reported in the new procedure for AI/ML Related Information can be predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections. The request in the new Class 1 procedure for initiating the reporting of AI/ML Related Information can include an ID assigned by the requesting NG-RAN node to request for reporting, which includes - the reporting parameters - list of cells to report - reporting periodicity The response in the new Class 1 procedure for initiating the reporting of AI/ML Related Information can include an ID assigned by the responding NG-RAN node which includes the confirmation on the reporting parameters requested. The message in the Class 2 procedure for Data Reporting of AI/ML Related Information can include the corresponding IDs assigned by the NG-RAN nodes, reports result. The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting. Introduce into the agreed new request message (AI/ML INFORMATION REQUEST, the name needs further discussion), an indication that UE performance feedback is provided after handover event. Whether the indication is in implicit or explicit way needs to be further discussed. Introduce a trigger indication in the HO request message to indicate that UE performance feedback is requested after HO completion. The details of indication need to be discussed. The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting. Partial reporting mechanism is supported in the agreed AI/ML information procedures (AI/ML INFORMATION REQUEST/RESPONSE/UPDATE, which name is FFS). The solutions need to be discussed. No additional explicit indication is required in the AI/ML INFORMATION REQUEST message that UE performance feedback is provided after handover if UE performance feedback is only considered as feedback. Introduce the pair Measurement ID (e.g., NG-RAN node1 Measurement ID and NG-RAN node2 Measurement ID) in the HO request message, to establish relationship with the AI/ML INFORMATION REQUEST message. Any additional information to be added can be further discussed. A list of UE performance feedbacks is introduced into the AI/ML INFORMATION UPDATE message. UE performance feedback can be reported through one-time reporting or periodic reporting. Introduce the failed measurement in the response message to indicate partial reporting result. The successful measurement list and failure cause need to be further discussed. Stop the discussion on Predicted TNL capacity indicator, predicted slice available capacity, predicted composite available capacity in R18. RAN3#120: Node level and cell level UE performance collection is not supported in R18. The Source NG-RAN node UE XnAP ID reference IE within the UE Associated Info Result Item IE should be included in the update message. The Failed Report Characteristics shall be split into two fields: - Failed Report Characteristics for per cell measurements, reported with per cell granularity - Failed Report Characteristics for per node measurements, reported with per node granularity Introduce a Cause IE for the Measurement Failed Report Characteristics per cell and Measurement Failed Report Characteristics per node. The group discussed and had the common understanding as below: Cause value measurement not supported -> is covered by legacy cause “Measurement not Supported For The Object,” Cause value measurement temporarily not available -> is covered by legacy cause “Measurement Temporarily not Available,” It is FFS whether any more cause values should be added to the legacy list of causes. There is consensus on the benefits of enabling the requesting node to optimize the measurement request by indicating whether requested measurements can be reported in full or in part. It needs to be further discussed whether such enhancements can be introduced in Rel18. It needs to be further discussed whether and how any further granularity needs to be introduced for the selection of measurements to be reported as part of the UE Performance Feedback. The problem of how to signal to the reporting node time configurations for the UE Performance Feedback measurement reporting is acknowledged. Time configuration for UE Performance Feedback measurement reporting consists at least of: - Time duration of the UE Performance Feedback measurement collection (the time duration starting at handover execution and including the last UE Performance Feedback report) - Whether, within such time duration, measurements are reported periodically or one time - In case of periodic reporting, the period of UE Performance Feedback measurements (whether existing of new IE) Work on stage3 details |  |  |  |  |
| R3-233912 | Discussion on Partial Reporting and UE Performance Feedback (Intel Corporation) |  | discussion |  |
| R3-234329 | Partial reporting of AI-ML assistance data (Ericsson) |  | discussion |  |
| R3-234294 | (TP for AI/ML to BLCR for TS38.423) Partial reporting of AI-ML assistance data (Ericsson) |  | other |  |
| R3-234377 | (TP to 38.423&38.420) Further discussion on remaining issues on procedures for AI (ZTE) |  | other |  |
| R3-233938 | Discussion on Xn impact of LB (Samsung) |  | other |  |
| R3-234280 | (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Load Balancing use case (Huawei) |  | other |  |
| R3-234293 | (TP for AI/ML to BLCR for TS38.423) UE Performance Feedback Configuration (Ericsson, InterDigital, Orange, Deutsche Telekom) |  | other |  |
| R3-234318 | UE Performance Feedback Configuration (Ericsson, InterDigital, Orange, Deutsche Telekom) |  | discussion |  |
| R3-234341 | (TP for TS 38.423) Discussion on AI/ML Load Balancing (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234440 | Discussion on AI/ML remaining issues (CMCC) |  | discussion |  |
| R3-233875 | AI/ML Load Balancing (NEC) |  | discussion |  |
| R3-234188 | Left issues related to UE performance feedback collection (Lenovo) |  | discussion |  |
| R3-234204 | (TP to 38.423) Discussion on partial success and post-handover measurement (CATT) |  | discussion |  |
| Partial Response: - Opt1: Add a flag in the request message - Opt2: Introduce new cause values in the failure measurement objectives in the response message, e.g., “partial measurement not available” - Op3: Add priority indication for one or more requested measurement objectives in the request message HW: Disagree with Opt1. Open to Opt3. Nok: The flag cannot solve the issue sufficiently. For Opt2, rely on OAM configuration which means there are not so many failure cases. Opt3 is a reasonable enhancement. CATT: Opt3 is kind of optimization, which does not need to be supported in R18 QC: Have further look at Opt2. E///: Opt1 comes from LTE, what we can achieve now is Opt1. Opt3 is an optimization on top of Opt1. ZTE: Agree with E///. Try to draw conclusion on which option can be accepted in R18. UE Performance feedback: Do not support the finer granularity of UE performance feedback in R18. E///: It should be supported depending on different UE. Introducing the index in the request message. Nok: Index solution brings complexity. How can it be configured via non-UE associated message while to be used with UE associated purpose? ZTE: It’s the target node collects the UE performance information. Similar like resource status report procedure, it’s not needed to let the source node to restrict the report of UE performance information. CMCC: The current method can be used as well. UE performance feedback can be reported both periodically and one-time, how to define the reporting window? QC: Why do we need this reporting window? CATT: The reporting window may be different on different UEs. CB: # AIRAN2_LB - Continue the discussion on the above open issues - Check the details of stage3 CRs - Capture agreements and open issues (moderator – E///) Summary of offline disc R3-234548 |  |  |  |  |
| 12.2.2.2. ME and Xn procedures AI/ML based mobility optimization: The following information should be specified as a start point on the basis of TR37.817: - UE performance (e.g., UL/DL throughput, packet delay, packet loss) - Predicted resource status information over Xn Predicted cell-granularity UE trajectory can be exchanged over Xn for AI/ML based mobility optimization. Support the following UE performance information to be sent for feedback purposes: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate. Cell-based UE Trajectory prediction has the same structure as UE History Information IE. Cell-based UE Trajectory prediction is provided as a list of cells into the future, each of which is indicated together with an expected time of stay into the cell. Single UE – Transfer of UE Trajectory Prediction UE Trajectory Prediction is transferred to the target gNB via the Handover Request. There is no need to include predicted RRC state in the cells in the predicted UE Trajectory in this release. There is no need to include beam index information in the predicted UE Trajectory in this release. Predicted UE Trajectory conveyed in the Handover Request can span across multiple NG-RAN nodes. In Rel_18, RAN3 will not pursue enhancements for one gNB to request UE trajectory from more than one hop gNBs. RAN3#120: The presence of predicted time UE stays in cell shall be optional. In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node, and the source NGRAN node can collect measured UE trajectory (in the form of UE History Information) related to one or multiple cells within the single next hop target NGRAN node after handing over the concerned UE(s). Take the procedure for UE performance feedback report as the baseline to also support the measured UE trajectory collection by the source gNB. The details on how to request the measured UE trajectory collection needs to be further discussed. Work on stage3 details |  |  |  |  |
| R3-234378 | (TP to 38.423) Discussion on left issues of AI based mobility optimization (ZTE) |  | other |  |
| R3-234388 | Discussion on open issue related to measured UE trajectory collection (LG Electronics Inc.) |  | discussion |  |
| R3-234400 | (TP for NR_AIML_NGRAN-Core BL CR for TS 38.423) Discussion on open issue related to measured UE trajectory collection (LG Electronics Inc.) |  | other |  |
| R3-234189 | (TP for TS38.423 BLCR) Left issues related to UE trajectory collection by the source gNB (Lenovo, Intel Corporation, ZTE) |  | other |  |
| R3-234281 | (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Mobility Enhancements use case (Huawei) |  | other |  |
| R3-234435 | Discussion on Measured UE Trajectory Collection (CMCC) |  | discussion |  |
| R3-234436 | (TP for TS 38.423)Measured UE Trajectory Collection (CMCC) |  | other |  |
| R3-233913 | Discussion on mobility optimization (Intel Corporation) |  | discussion |  |
| R3-233939 | Discussion on Xn impact of ME (Samsung) |  | other |  |
| R3-233974 | Mobility Optimization Outputs (InterDigital ) |  | discussion |  |
| R3-233975 | [TP for BL CR 38.423] Handling Additional Mobility Outputs (InterDigital) |  | other |  |
| R3-233994 | Discussion on UE Trajectory Feedback for NG-RAN AI/ML (Qualcomm Incorporated) |  | discussion |  |
| R3-234208 | (TP for 38.423) Support of AI/ML based mobility optimization (CATT) |  | discussion |  |
| R3-234295 | Cell based UE trajectory prediction configuration (Ericsson, InterDigital, Orange, Deutsche Telekom) |  | discussion |  |
| R3-234296 | (TP for AIML BLCR for TS 38.423) Cell based UE trajectory prediction exchange (Ericsson, InterDigital, Deutsche Telekom) |  | other |  |
| R3-234342 | AI/ML Mobility Optimization (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234343 | (TP for TS 38.423) AI/ML Mobility Use Case (Nokia, Nokia Shanghai Bell) |  | other |  |
| How to support the actual UE trajectory feedback: The measurement object to request the report of the UE Trajectory IE should be introduced in the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message. The AI/ML Measurement ID in HANDOVER REQUEST from source gNB is reused to trigger the target node feedback the UE trajectory information to the source gNB. AI/ML INFORMATION UPDATE from target node is used to convey the UE trajectory information. Time window for actual UE trajectory feedback? Whether the switch of UE state can be used as the trigger to send the UE Trajectory back to the source node? The UE Trajectory IE should contain the cell the UE visited, including the target cell, and the information related to the UE trajectory measured in the visited cell? Whether to reuse UE history information? How to trigger the predicted UE Trajectory report? CB: # AIRAN3_ME - Capture the agreements to TP - Continue the discussion on the open issues above - Capture agreements and open issues (moderator - CMCC) Summary of offline disc R3-234573 |  |  |  |  |
| 12.2.2.3. ES and Xn procedures AI/ML based Energy Saving: RAN3 focus on the cell-level energy saving strategy as a start point, to avoid overlapped discussion for network energy saving SI. Predicted resource status information of neighbouring NG-RAN node(s) generated by the current NG-RAN node is internally used, and no standard impacts. Regarding AI/ML based Energy Saving, the following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn - UE performance (e.g, UL/DL throughput, packet delay, packet loss) Current Energy Efficiency metric can be exchanged between RAN nodes for the energy saving use case. Energy Efficiency constitutes a metric that reflects the energy consumption of a cell or a node. It is FFS what the granularity and exact coding of this metric is. The "Energy Efficiency" metric should be measurable, produced and interpretable by the RAN. It’s the common understanding that AI/ML based energy saving aims to optimize the overall energy efficiency of the coverage of a gNB and its neighbours. Start with per node granularity EE and Per cell granularity EE could be considered if it is feasible. WA: Take the EE defined in SA5 as the baseline for the energy efficiency of a gNB. Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs. Adopt the below Option-3a and exchange Energy Cost (EC) upon request over the Xn interface. The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level. WA: If the Energy Cost is encoded as an index (0, ..Max), representing energy consumption on a linear scale, it is agreed that the OAM configures rules to a NG-RAN node to determine how to normalize the values of the EC. The rules shall be the same at least for all neighboring NG-RAN nodes within the area where a request on EC reporting is triggered by a source NG-RAN node. It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18. WA: Use the already introduced AI/ML Information Reporting Initiation (Class 1 – AI/ML INFORMATION REQUEST/RESPONSE) procedure to signal to the target NG-RAN node a description of the “additional load”. Use the AI/ML Information Reporting (Class 2 – AI/ML INFORMATION UPDATE) procedure to allow the target NG-RAN node to report the estimation of the Energy Cost (name of the procedures to be further discussed) It is agreed to include the measured Energy Cost in the AI/ML Information Reporting Initiation and AI/ML Information Reporting procedures (name of the procedures to be further discussed) RAN3#120: EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy. FFS on whether the inferred EC should be introduced in R18. FFS on whether the additional load needs to be introduced in R18. Work on stage3 details |  |  |  |  |
| R3-233914 | Discussion on energy efficiency (Intel Corporation) |  | discussion |  |
| R3-234282 | (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Energy Saving use case (Huawei) |  | other |  |
| R3-234297 | (TP for AI/ML BLCR to TS38.423) AI/ML Network Energy Saving (Ericsson) |  | other |  |
| R3-234317 | AI/ML Network Energy Saving (Ericsson) |  | discussion |  |
| R3-234379 | (TP to 38.423) Further discussion on AIML based energy saving (ZTE) |  | other |  |
| R3-234442 | Further Discussion on Inferred EC for AI/ML for NG-RAN Energy Saving (CMCC) |  | discussion |  |
| R3-234443 | (TP for AI/ML BLCR to TS38.423) AI/ML Network Energy Saving (CMCC) |  | other |  |
| R3-233940 | Discussion on Xn impact of ES (Samsung, NTT DOCOMO INC.) |  | discussion |  |
| R3-233941 | TP to 38.423 for predicted energy saving strategy exchanging procedure for AI/ML for NG-RAN (Samsung, Lenovo, Xiaomi, NTT DOCOMO INC., ZTE) |  | other |  |
| R3-233993 | Discussion on ES Enhancements for NG-RAN AI/ML (Qualcomm Incorporated) |  | discussion |  |
| R3-234190 | Discussion on issues related to AI based network energy saving (Lenovo) |  | discussion |  |
| R3-234205 | Discussion on the EC metric (CATT) |  | discussion |  |
| R3-234344 | Discussion on AI/ML Energy Saving (Nokia, Nokia Shanghai Bell, Deutsche Telekom, Orange) |  | discussion |  |
| R3-234345 | (TP for TS 38.423) AI/ML Energy Saving (Nokia, Nokia Shanghai Bell, Deutsche Telekom, Orange) |  | other |  |
| R3-234371 | Further discussion on NG-RAN AIML energy saving (NTT DOCOMO INC..) |  | discussion |  |
| The definition and signalling over RAN interfaces of the “Additional Load” as well as inferred EC are not pursued in Rel-18? Accept the definition and signalling over RAN interfaces of inferred EC based on actual node level EC in Rel-18? Stop inferred EC discussion in R18? Nok: Without this, there is no AI/ML ES in R18. CMCC: Keep the discussion on inferred EC in R18, but leave the Additional Load out of the scope ZTE: It’s difficult to give clear understanding on inferred EC in R18. Define the Energy Cost IE as an INTEGER (0..10000,…), it can be revisited based on reply from SA5. HW: Wait for reply LS from SA5 CB: # AIRAN4_ES - Try to converge on the way how to treat “inferred EC” in R18 - Capture agreements to TP and the details of measured EC, e.g., the procedure text for measured EC - Capture agreements and open issues (moderator - HW) Summary of offline disc R3-234549 |  |  |  |  |
| 12.2.2.4. Other interfaces Identify the potential stage3 impact over NG, F1, E1… RAN3 will focus on non-split architecture use cases and procedures first and discuss split architecture use cases and procedures when completion for the non-split architecture use cases and procedures is achieved. UE traffic metric takes the data volume for a UE as the starting point. The cell-level UE trajectory prediction function is located in gNB CU-CP. The location for resource status prediction in split architecture: For current resource status input data from gNB DU, the resource status prediction function is located in gNB CU-CP. For current resource status input data from gNB CU-UP, the resource status prediction function is located in gNB CU-CP. Focus on the necessity support over F1, E1 based on agreements achieved over Xn. |  |  |  |  |
| R3-233942 | Discussion on E1/F1 impact of AI/ML for NG-RAN (Samsung) |  | discussion |  |
| R3-233943 | (CR to 37.483) for AI/ML for NG-RAN (Samsung) |  | CR0070r, TS 37.483 v17.5.0, Rel-18, Cat. B |  |
| R3-234191 | (TP for TS37.480 TS38.470) Discussion on E1 F1 interface impact (Lenovo) |  | other |  |
| R3-234207 | (TP to 38.473 & 37.483) Discussion on E1AP and F1AP impacts (CATT) |  | other |  |
| R3-234283 | (TP for AI/ML BL CR for TS 37.483 and 38.473) On the impact of RAN AI/ML over E1 and F1 interfaces (Huawei) |  | other |  |
| R3-234380 | (TP to 38.473&37.483) Consideration on impact of F1E1 impacts (ZTE) |  | other |  |
| F1 and E1 should be enhanced to support the energy cost reporting from gNB DU/gNB CU-UP to gNB CU-CP? Introduces new F1AP procedure for the DU to provide Energy Cost per DU as requested by CU, e.g., DATA COLLECTION REQUEST/RESPONSE/REPORT? Nok: Too early to make decision, waiting for reply from SA5 HW: Whether this EC infor can be done via current E1/F1 signalling ZTE: Discuss how to support EC over F1 first SS: Start with measured EC, need progress on this topic CATT: DU provides the measured EC to CU. QC: Prefer to discuss this in R19. Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed. UE traffic/data volume information transmission? SS, ZTE, CATT: Yes Nok, E///, HW: No |  |  |  |  |
| ## 12.3. Others Potential MDT enhancement related issues as follows, need more time to discuss the details and potential standard impacts, coordination with RAN2/SA5 if needed: - enhance the mdt procedure to solve the issue how to support the consecutive ai/ml data collection for the certain time-series ai/ml model. - how the source ng-ran node obtains logged ue trajectory information when ue enters rrc connected state and reports to the new ng-ran node. - how to enable a more granular selection of ues based on enhanced mdt configuration information in management based MDT - how to map ai/ml feedback information to ai/ml actions and report them over MDT Study the scenarios, issues and solutions to support the continuous data collection within a period for AI/ML via MDT. WA: There are the benefits in enabling UEs to continuously collect MDT measurement across RRC states and to provide to the network such continuous time series of data for the purpose of AI/ML Training/Retraining. The evaluation of gains vs impacts related to the use cases in R18 coming from potential solutions needs to be further discussed. The existing MDT framework is used as baseline for data collection from the UE. Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive). RAN3#120: Common understanding in RAN3: For signaling based MDT, TCE could identify the UE via UE identifier For management based MDT, it is not possible for TCE to identify the UE with UE identifier that can lead to the unique UE identity since m-based MDT is intentionally made anonymous. Identify the gap between current MDT mechanism and the requirement, LS to other group if needed. |  |  |  |  |
| R3-234346 | Support of continuous MDT measurement collection (Nokia, Nokia Shanghai Bell) |  | CR1021r, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-234347 | Continuous MDT Evaluation (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234298 | Continuous MDT proposition (Ericsson, AT&T, InterDigital, Deutsche Telekom) |  | discussion |  |
| R3-234299 | [DRAFT] LS on Continuous MDT (Ericsson, AT&T, InterDigital, Deutsche Telekom) |  | LS out To: SA5 CC: RAN2 |  |
| R3-234300 | Improved UE selection granularity (Ericsson, AT&T, InterDigital, Deutsche Telekom) |  | discussion |  |
| R3-234301 | [DRAFT] LS on Improved UE selection granularity (Ericsson, AT&T, InterDigital, Deutsche Telekom) |  | LS out To: SA5 CC: RAN2 |  |
| R3-234381 | Discussion on MDT enhancement for continuous AI-ML related information (ZTE, Lenovo, Samsung) |  | discussion |  |
| R3-234382 | (TP to TS38.413) MDT Enhancements for continuous data collection (ZTE, Lenovo, Samsung) |  | other |  |
| R3-234383 | [DRAFT] LS on the MDT enhancement to support continuous AI-ML related information reporting from UE (ZTE) |  | LS out To: RAN2, SA5 CC: |  |
| R3-233989 | Discussion on MDT enhancement for NG-RAN AI/ML (Qualcomm Incorporated) |  | discussion |  |
| R3-234206 | Continuous MDT supporting for AI (CATT) |  | other |  |
| R3-234284 | Further discussions on MDT enhancements (Huawei) |  | discussion |  |
| On the top of the solutions proposed in previous meetings: RAN3 to discuss the two options to enable TCE correlate different logs pertaining to the same UE, namely: Option 1: Use a random number allocated at the CN per UE upon registration. Option 2: Include the TR and TRSR of the Logged MDT session in MDT records for Immediate MDT data sent to the TCE. ZTE, SS: First discuss whether to enhance signaling based MDT to minimize the impact HW: Prioritize management based MDT, solution needs to be checked CB: # AIRAN5_MDT - Check the solutions proposed by companies in R18 (moderator - Nok) Summary of offline disc R3-234550 |  |  |  |  |
| 13. Mobile IAB for NR WI (RAN3-led) WID [NR_mobile_IAB-Core]: RP-222671 (target: RAN #102) [TU: 1 ( 1, 1.5, 1.5)] QUOTA: 4 |  |  |  |  |
| ## 13.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233750 | (BLCR to 38.413) Support for mobile IAB (Nokia, Nokia Shanghai Bell) |  | CR0988r2, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233788 | Introduction of Mobile TRP (Ericsson, Xiaomi, Qualcomm Inc., CATT, Nokia, Nokia Shanghai Bell, ZTE) |  | draftCR Endorsed as BL CR |  |
| R3-233801 | Support for mobile TRP Location Information (Xiaomi, Ericsson, Qualcomm Inc., CATT, Nokia, Nokia Shanghai Bell, ZTE, Huawei) |  | CR0101r5, TS 38.455 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233806 | Support of mobile TRP location information (Ericsson, Xiaomi, Qualcomm, CATT, Nokia, Nokia Shanghai Bell, ZTE, Huawei) |  | CR1176r5, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233889 | Updated workplan for Rel-18 mobile IAB (Qualcomm Inc. (Rapporteur)) |  | Work Plan noted |  |
| R3-233890 | (Draft Reply LSs) Discussion on Reply LSs to RAN2 and SA2 (Qualcomm Inc.) |  | other |  |
| R3-234546 | Offline discussion on Mobile IAB (Qualcomm Inc.) |  | Discussion |  |
| ## 13.2. Support IAB-node mobility Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. As already supported in Rel17, a mobile IAB-MT and its co-located mobile IAB-DU may be served by different donor CUs. The mobile IAB donor that the co-located IAB-DU connects to may remain unchanged after the IAB-MT HO. RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT stays connected to the same donor before and after the mobile IAB-DU migration. RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT executes inter-donor migration. When IP connectivity between target IAB-donor DU and source IAB-donor CU is available, and when Xn connectivity between source and target donor CU is available, the Rel-17 partial migration is used as baseline for supporting the F1 transport migration and inter-donor routing when an mobile IAB-DU and its co-located mobile IAB-MT are connected to different donor CUs. The mobile IAB-node may perform multiple consecutive partial migrations without inter-donor migration of its mobile IAB-DU. RAN3 to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability. Mobility of dual-connected mobile IAB nodes is down prioritized in Rel18. Rel17 mechanisms support intra donor CU migration of mobile IAB. For DU migration cases, to execute the handover of the served UEs, the mobile IAB-node concurrently supports two logical mobile IAB-DUs, which have F1AP associations with the source CU and the target CU, respectively. The UEs connected to the mobile IAB-node are handed over from the cell of the logical mobile IAB-DU (i.e., the source logical mobile IAB-DU) that has an F1AP association with the source CU to the cell of the logical mobile IAB-DU (i.e., the target logical mobile IAB-DU) that has an F1AP association with the target CU. RAN3 to discuss whether a mobile IAB node may be configured with multiple configurations, each corresponding to a different target donor, that can be activated upon fulfillment of certain condition(s). The details of the configurations are FFS. mIAB-DU migration and mIAB-MT handover can be executed independently from each other. Details on the scenarios need to be further discussed. For partial migration of mIAB-node, the inter-donor HO of mIAB-MT is decided and triggered by the donor CU serving the mIAB-MT. The donor CU serving the mIAB-DU decides whether to execute mIAB-DU migration or inter-donor F1 transport migration for the mIAB-DU. For inter-donor partial migration, the donor CU serving the mIAB-DU is informed about the mIAB-MT HO. FFS on signalling details concerning the indication. WA: The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed • FFS: the TNL address of the target donor CU for the mIAB-MT HO. For partial migration of mIAB-node, the donor CU serving the mIAB-DU and the target donor CU for the mIAB-MT HO can directly exchange Xn IAB Transport Migration messages, in case direct Xn connectivity exists (or is established) between the two donor CUs. Focus first on the scenarios where Xn and IP connectivity are available between the source and target donors for IAB-MT HO and mIAB-DU migration. RAN3 to discuss support of mIAB-node mobility over NGAP. Which type of migration needs to be further discussed. To hand over the UEs between the logical mIAB-DUs, the source donor CU for mIAB-DU migration should be notified about the cell IDs served by the second (target) logical mIAB-DU. The source donor CU for mIAB-DU migration should be informed that the second logical mIAB-DU has successfully established an F1 connection towards the target CU. Details are FFS. WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. This WA is subject to validation that the impact involved is affordable. The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed. In case the donor of the mIAB-DU decides the F1AP setup for DU migration, the donor of the mIAB-DU triggers via F1 signalling the IAB node to perform the F1 Setup procedure for the DU migration. An OAM based solution is not excluded. For the establishment of Xn, the mIAB-DU’s donor CU can obtain the TNL address of the mIAB-MT’s target donor CU via legacy mechanisms. The info sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU does not include the target donor CU’s TNL address. The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO. The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. The trigger for F1 setup between the mobile IAB-node’s second logical DU and its donor CU may be based on OAM or pre-configuration. When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the information of target logical mIAB-DU’s CU (e.g. IP address, gNB-ID). The IAB-node can inform the source logical mIAB-DU’s CU via F1AP about the successful F1 Setup with the target logical mIAB-DU’s CU, and it can include the IDs of the cells activated by the target logical mIAB-DU’s CU. In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU. The HO of UEs from the source logical mIAB-DU´s CU to the target logical mIAB-DU´s CU should happen after the completion of the F1 setup. When to trigger the HO is up to source logical mIAB-DU´s CU implementation. After all UEs have been handed over, the source logical mIAB-DU’s F1AP association can be released by the source logical mIAB-DU or by the source logical mIAB-DU’s CU. Target donor CU selection for mIAB-DU migration and triggering conditions for F1 setup can be up to source CU implementation (unless it is justified that this is not possible) or based on OAM configuration at the source CU. RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available. For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF. Turn into an agreement the WA stating that: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. Capture the mIAB-MT HO and mIAB-DU migration as separate procedures in TS 38.401. When triggering the F1 setup from the mIAB node to the target CU for mIAB-DU migration, the source CU can indicate to the mIAB-DU: - The gNB-ID of mIAB-DU’s target CU. - Optionally, the IP address(es) of mIAB-DU’s target CU and SeGW. The mIAB-node may obtain the IP address of target CU for mIAB-DU migration and the IP address of its SeGW from the OAM. Down select between the following two options for providing the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU to mIAB-DU’s target CU: - Option A: XnAP signalling from the mIAB-DU’s source CU. - Option B: F1AP signalling from the target logical mIAB-DU. For Option B, discuss whether and how the mIAB-DU can obtain the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU. As a baseline: The target CU for mIAB-DU migration learns the traffic profile of the UE traffic from Handover Preparation procedures for individual UEs. The “Non-F1-Terminating IAB-donor UE XnAP ID” in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message sent from mIAB-DU’s target CU to the mIAB-MT’s CU is generated by the mIAB-MT’s CU. For the upstream data handling at the BAP of mobile IAB MT, the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection. For the downstream data handling arriving at the mobile IAB-node, the upper layers (e.g., IP layer) can differentiate the data to different logical DUs based on upper-layer header information. For consecutive partial migration, after the mIAB-MT HO is completed, the mIAB-DU’s CU sends the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the mIAB-MT’s target CU, including the UE XnAP ID assigned to the mIAB-MT by the mIAB-MT’s target CU as the “Non-F1-Terminating IAB-donor UE XnAP ID”. RAN3#120: Include “No PDU Session Indication” in the NGAP HANDOVER REQUEST message. WA: legacy NG HO Required message with “dummy” PDU information is used. It needs to be further discussed if any specification changes are needed to describe the use of such dummy information. If specification changes is needed, consider whether an explicit indication is worth adding. After mIAB-DU migration, the BH RLC and BAP routing configurations used in the non-F1 terminating topology for TMM with the mIAB-DU’s source CU may be released. WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU. Whether the information on the DU’s CU can also be configured by the MT’s CU. During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU. How to pass the UE XnAP ID and NCGI or gNB-ID in case the IAB node knows the information of the DU’s CU by other means than OAM, if agreed. It is FFS whether both gNB-ID and CGI needs to be provided to the mIAB-DU’s CU It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP. It is FFS whether the gNB-ID of the MT´s CU is passed from the IAB node to the IAB DU´s CU via F1AP. Option 1: Use existing procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU Option 2: Use new procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU The DU’s CU can initiate the Xn TM Management Procedure pertaining to an mIAB-MT even though it has never had an RRC connection with this IAB-MT. RAN3 to introduce NGAP inter-gNB information exchange procedure(s) for exchange of IAB TMM messages and IAB Resource Coordination messages RAN3 to decide whether this procedure is generic, i.e., it can carry any information, or IAB-specific, i.e., limited to IAB-specific purposes. |  |  |  |  |
| R3-233893 | Open issues for mobile IAB topology adaptation (Qualcomm Inc.) |  | discussion |  |
| R3-234035 | (TP for mIAB BL CR for TS 38.401) Migration Procedure for Mobile IAB-Nodes (Ericsson) |  | other |  |
| R3-233837 | Discussion on IAB-node mobility (CANON Research Centre France) |  | discussion |  |
| R3-233881 | (TP for Mobile IAB BL CR TS38.401) Discussion on IAB-node mobility and integration (Fujitsu) |  | other |  |
| R3-233892 | (TP to TS 38.401) BL ST2 procedures for mobile IAB (Qualcomm Inc.) |  | other |  |
| R3-233967 | (TP for Mobile IAB BL CR for TS 38.413 38.423 38.473) Inter-donor migration in mobile IAB scenario (ZTE) |  | Other Rev in R3-234593 |  |
| R3-234009 | (TP for Mobile IAB TS38.413 BL CR) Discussion on Support IAB-node mobility (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234047 | Discussion on DU migration (Samsung) |  | discussion |  |
| R3-234048 | Discussion on multiple partial migration (Samsung) |  | discussion |  |
| R3-234141 | (TP for NR_mobile_IAB BL CRs for TS 38.401/ 38.473) Support of mobility for mobile IAB (Huawei) |  | Other Rev in R3-234594 |  |
| R3-234169 | Discussion on mobile IAB-node inter-donor topology adaptation (Lenovo) |  | discussion |  |
| R3-234252 | Discussion on IAB-node mobility (Xiaomi) |  | discussion |  |
| R3-234259 | Discussion on XnAP and F1AP enhancements for mobile IAB (MITRE Corporation) |  | discussion |  |
| R3-234305 | (TPs to TS38.473/TS38.413 BL CR) Consideration on support IAB-node mobility (CATT) |  | other |  |
| Assign CB for ST2 procedure of Xn-based consecutive mIAB-MT migration, using R3-233892 as baseline. The details of the BLCR are to be discussed further New TP for TS38.401 in R3-234592 ZTE: Clarifications on the Xn connectivity QC: assumption is that Xn is available Ericsson: The text in the proposal is clear enough. “Sequential MIAB-MT migration” should be clarified. QC: maybe we should discuss the meaning of “sequential” in a CB? In next meeting, RAN3 to establish baseline ST2 procedure of NG-based mIAB-MT migration. Ericsson: the word consecutive is not necessary as it has not been discussed Huawei: NG based HO procedures are normally captured in SA2 specifications Nokia: This is to be discussed at the next meeting, hence let´s leave the discussion for then Ericsson: we should not question if NG based MT migration is needed as the MT will eventually move to a new AMF area. In next meeting, RAN3 to establish baseline ST2 procedure for DU migration in presence of Xn including agreements of RAN3#121. Huawei: The proposal should be that RAN3 only focusses on the scenario that ensures Xn connection between source IAB-DU´s CU and target IAB-DU´s CU and the Xn connection between the MT CU and the DU´s CU QC: HW´s proposal narrows down the scope. The scope should be wider for the discussion at the next meeting ZTE: We already agreed to focus first on the scenario where there is Xn connectivity, the actual text is fine. Down-select between the following two options for providing the gNB-ID of the mIAB-MT’s CU to the target mIAB-DU’s CU: 1. Option A: XnAP signalling from the source logical mIAB-DU’s CU. 2. Option B: F1AP signalling from the target logical mIAB-DU. For Option B, the mIAB-MT obtains the gNB-ID from SIB of the MT’s CU. Show of hands result: Option A: Huawei, Mitre, Xiaomi, Samsung Option B: Nokia, ZTE, Fujitsu, Qualcomm, Ericsson, Canon, Lenovo For the case of DU migration, F1AP signalling from the target logical mIAB-DU is selected for providing the gNB-ID of the mIAB-MT’s CU to the target mIAB-DU’s CU. Option B will be considered as baseline for transferring of remaining information i.e. the ID of the IAB MT. The relevance of the use case of overlapping MT migration and DU migration and, if relevance is confirmed, how to address it is to be continued. Xiaomi: Two types of trigger are on the table, one is OAM and the other is CU based. Which trigger are we talking about here QC: we are covering both triggering cases Fujitsu: We should consider gNB-UID and XnUE AP ID together Mitre: we agree that we should consider gNB-ID and XnUE AP ID as a package. They should be combined together in the signalling Ericsson: Agree with QC. We should first take the gNB-ID discussion. Xn AP IDs have more complexity hidden Samsung: There is an ongoing discussion on how to define the IAB MT ID. We could have an FFS on the definition of the IAB MT ID QC: We identified that for the MT ID we get the problem of multiple IAB nodes migrating at the same time Huawei: the problem if we decouple the IAB MT ID and the gNB-ID of the mIAB-MT’s CU is that we may end up in different ways to transfer these IDs. Samsung: agree with the amended text including the IAB MT ID QC, ZTE, Ericsson: disagree, the text should include only the mIAB-MT’s CU Mitre: Option B has technical shortfalls concerning concurrent MT migration and DU migration Ericsson: We cannot consider all scenarios possible. Option B covers the relevant scenarios ZTE: Agree with Ericsson. The use case from Mitre is not to be considered as reference QC: Agree with ZTE. The use case has not been discussed yet. In the next meeting, RAN3 to decide which ID assigned by the MT CU for the IAB MT should be used for the TMM procedure and how this ID can be mapped to a corresponding IAB node. ZTE: One option could be not to use the XnAP IDs Ericsson: we have already agreed that Option B is used to transfer the MT ID Samsung: which ID should be used for the IAB MT? Suggest to place an FFS on the ID Huawei: Agree that the issue is which ID we use to identify the IAB MT. Place an FFS on this point ZTE: Agree with SS and Huawei. In the next meeting, RAN3 to decide whether the target CU can indicate how much resources it can offer for the incoming DU-migration during or before the DU’s F1 Setup procedure based on information related to the UE traffic carried by the target DU, and what this information should contain. Ericsson: there could be a partial acceptance, hence the formulation needs to account for that too. Nokia: Disagree to the sentence above. The issue described above has never been discussed in light of the agreements taken at this meeting. We should not prioritise this issue. Qualcomm: Agree to a certain extend that the issue can be postponed. Huawei: Agree with Nokia Samsung: It is important to ensure to guarantee UE access to the network via the IAM node. Lenovo: Agree that this issue is a low priority issue and we use the HO migration procedure as baseline. Canon: Agree with Samsung to discuss how to reject the migration procedure One F1AP procedure is used for the triggering of DU migration and a separate procedure is used for the reporting of the outcome of the F1 Setup procedure. The first procedure is only used for CU triggered DU migration and the second procedure is used for both CU triggered and OAM triggered DU migration. Show of hands for Option 1: In favour: Nokia, Ericsson, Samsung, Mitre, Canon, Xiaomi, Huawei, Lenovo, CATT, Qualcomm Against: Fujitsu ZTE: We may not use separate procedures. We could use the response message of the Class 1 procedure to report the outcome. Fujitsu: We should use separate procedures for OAM based triggering and for CU based triggering. Nokia: The issue with the approach from ZTE is that there is already a timer configured for the existing Class 1 procedure to be used. We do not want to modify the configuration of existing procedure and rather use a new one. Huawei: Support the proposal from Qualcomm. Both triggering methods can make use of the outcomes signalled by the separate procedures. RAN3 to decide in this meeting whether existing or new procedures are used for the triggering of DU migration and/or for the reporting of the outcome. Option 1: reuse existing procedures Option 2: use new procedures Show of hands result: Option 1: Qualcomm, ZTE, Nokia, CATT, Canon Option 2: Samsung, Fujitsu, Mitre, Xiaomi, Huawei, Lenovo, Ericsson Qualcomm can accept option 2 New class 2 non UE associated procedures are used for the triggering of F1 setup for the purpose of DU migration and/or for the reporting of the outcome of the F1 Setup procedure for the target logical DU. CATT: Prefer to use existing procedures, as these procedures can fulfil the purpose already Ericsson: There is no suitable existing Class 2 procedure that can be used for notification purposes ZTE: Use an existing Class 1 procedure. CU/DU update procedures seem to be described in a way that allows their reuse Huawei: Agree with Ericsson, introduce two new Class 2 procedures Nokia: Reuse existing Class 1 procedure Canon: support ZTE and Nokia Qualcomm: Existing procedures Samsung: New procedure. Functionalities covered by current procedures do not cover the functionality we need here. The following WA is turned into an agreement “As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs. This agreement does not relate to the configuration sharing between two logical collocated mIAB-DUs.” It is agreed that a Mobile-IAB indication is included in the NGAP Initial UE message. Huawei: Remove “based on SA2´s decision” RAN3 to discuss if mobile-IAB-authorized indication to be included in the HO Request message for the mIAB-MT. Ericsson: we have included in Rel17 a Mobile IAB authorization indication in Path Switch message. If AMF is able to inform the RAN with a Mobile IAB authorization status, why should we signal it over inter RAN interfaces Xiaomi: Similar question as Ericsson. ZTE: the purpose of signalling the info is to let the target donor determine whether to assign BAP configurations to the mIAB node. Given that the BAP configuration is assigned during HO preparation, the info is needed in the HO message Nokia: If we only use the Path Switch Request Ack then you need to trigger an RRC reconfiguration to transfer BAP configuration related information Lenovo: Agree that the information needs to be added in the HO Request. CATT: Agree with ZTE. Ericsson: We have another message in NGAP to send authorisation indication. Hence the AMF can signal the information at every HO. Is there any requirement about not sending BAP configuration if the mobile IAB node is not authorised? Huawei: we clarify that the HO request is the Xn HO Request. QC: the WA is about optimising the speed in configuring the UE with BAP configuration information Samsung: We should apply this solution also in case of MT migration. IF we do not include the information in HO Request, then traffic will stop Agree to introduce a Mobile-IAB-authorization status indication in the Path Switch Request Ack for the mIAB-MT. R3-233967 is revised in R3-234593 Agree that the mIAB-DU’s CU to be informed about the mIAB-authorized status by mIAB-MT´s CU when IAB MT and IAB DU connect to different IAB donor CUs. Ericsson: how should we inform about this information? The information should be signalled over Xn from the MT´s CU because the DU´s CU has nothing to do with the AMF of the MT Xiaomi: The DU´s CU only needs to be informed about de-authorisation information. Ericsson: information of authorized/non authorized status is needed to avoid F1 setup rejections. In case the mIAB-DU’s CU obtains “non-authorized” indication for the mIAB-node, it performs an orderly F1 release. Nokia: we have this already and can agree to this RAN3 to discuss how to capture in stage2 the handling of OAM connectivity as the mIAB node moves between areas of different OAM systems. CB: # MobileIAB - Agree to New TP to TS38.401 R3-234592 - Agree to TP to NGAP in R3-234593, revision of R3-23 3967 - R3-234141 is revised in R3-234594. Can the TP to TS38.473 to introduce two new class 2 procedures in R3-234594 be agreed? - New TP to TS38.473 in R3-234595 to introduce the gNB-ID of the MT CU in F1 message. Can R3-234595 be agreed? (moderator - QC) New TP to TS38.401 in R3-234592 New TP to TS38.473 in R3-234595 |  |  |  |  |
| ## 13.3. Mobility Enhancements Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. For group mobility enhancement, RAN3 to discuss the benefit and whether to support signaling of information related to multiple UE contexts in a single message, during e.g. the handover preparation, path switch, and context release procedures. The donor CU should know that the IAB node is “mobile”. RAN3 to discuss whether the target IAB-donor should know the migrating IAB-node is “mobile IAB-node” from the source IAB-donor. RAN3 to discuss whether to support means to identify onboard UEs. After baseline procedures have been established, RAN3 to discuss the benefit and whether to support signaling of information related to multiple UE contexts in a single message for UE handover preparation, path switch, and context release procedures. As the baseline, F1 establishment and configuration of the new logical DU follows legacy procedures. RAN3 to discuss whether and which information can be shared between two logical DUs in case of IAB-DU migration. RAN3 to discuss which of the OAM-configured and network-configured parameters may be pre-configured at a mobile IAB-node, after a baseline procedure for IAB-DU migration is developed. Source donor CU of mobile IAB-MT informs the target donor CU of mobile IAB-MT that the migrating node is a mobile IAB-node, via explicit indication in XnAP HO Request message. RAN3 to discuss whether source donor should know whether the target cell belongs to a mIAB-Node. The NCGI of the mobile IAB-DU cell is changed when the F1-terminating donor CU of the mobile IAB-DU is changed. RAN3 to further discuss the following options for TAC/RANAC issue: - Option 1: The TAC/RANAC for the mobile IAB cell can be changed in order to reflect the physical location when the mobile IAB-node moves. - Option 2: Using static TAC/RANAC for mobile IAB when it moves. Involvement of SA2 may be needed Dynamic TACs: Static TAC solution is not pursued. RAN3 assumes that dynamic TAC solution should be supported. RAN3 to continue discussions on impacts (if any) of dynamic TAC solutions on RAN3 specs Send an LS to RAN2 (include SA2 in To) informing RAN2 of the decisions taken by RAN3 With respect to mIAB-DU migration and partial migration, RAN3 to discuss how the mobile IAB-DU’s parameters are (re-)configured. Capture on stage 2 that the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. It needs to be further discussed how the mobile IAB-DU’s TAC/RANAC is changed and what Stage 3 impacts are (if any). The BH RLC CH(s), BAP address and default BAP configuration configured on the mIAB-MT can be used for delivering the F1 traffic of both logical mIAB DUs. Non-F1 traffic to be further discussed. RAN3#120: WA: As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs. This agreement does not relate to the configuration sharing between two logical collocated mIAB-DUs It is FFS whether such mapping information is needed for all activated cells. The mIAB-DU’s NCGI is configured by OAM, and, e.g. to avoid CGI collision, it may be re-configured by the donor CU via F1 based on a list of NCGIs that has been configured on this donor CU by OAM or by pre-configuration. This should not affect the existing procedure of configuring NCGI of cells served by a stationary DU via OAM. The underlying assumption is that the DU´s OAM has visibility on the result of the CU-based CGI re-configuration. It needs to be further discussed how to ensure that such observability is supported. RAN3 to send an LS to SA5 including the content of the agreement. Explain the status quo in RAN3 concerning the use cases discussed. Ask SA5 how to ensure that OAM has visibility over the CGI reconfiguration decisions, as well as on feasibility and feedback of the solution and requirements agreed. To be further discussed whether any further questions need to be posed to SA5 Use cases requiring CGI re-configuration independent of CGI collision events needs to be further discussed and proven. If no other use cases than CGI collisions can be identified, the proposal above applies only to CGI collision avoidance Discussions on enhancements to legacy HO procedures to be taken at RAN3#121. If no consensus on such enhancements, the following will be agreed: For DU migration, the following baseline is enough for target CU being aware of the QoS profile for each UE traffic: The target CU for mIAB-DU migration learns the traffic profile of the UE traffic from Handover Preparation procedures for individual UEs. No further enhancements are needed. |  |  |  |  |
| R3-234010 | Discussion on IAB configuration and Mobility Enhancements (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-233824 | Discussion on mobility enhancements for mobile-IAB (KT Corp.) |  | discussion |  |
| R3-233968 | (TP for Mobile IAB BL CR for TS 38.473) Enhancements to IAB node migration in mobile IAB scenario (ZTE) |  | other |  |
| R3-234036 | IAB-Node Mobility Enhancements (Ericsson) |  | discussion |  |
| R3-234049 | Discussion on mobility enhancements (Samsung) |  | discussion |  |
| R3-234142 | (TP for NR_mobile_IAB BL CR for TS 38.423): Mobility enhancement for mobile IAB (Huawei) |  | other |  |
| R3-234170 | Mobility enhancements for mobile IAB-node and its served UE (Lenovo) |  | discussion |  |
| R3-234253 | Discussion on mobility enhancement (Xiaomi) |  | discussion |  |
| R3-234306 | Enhancements for mobility IAB (CATT) |  | discussion |  |
| 13.4. Mitigation of interference Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2] PCI space partitioning via OAM configuration can be used in some cases for avoidance of PCI collisions. From RAN3 perspective, existing mechanism can be used for PCI collision detection in mobile IAB scenario. Further enhancement is FFS. RAN3 to discuss whether mobile IAB needs any enhancements to the existing mechanisms for PCI collision avoidance and/or optimization. From RAN3 perspective, no enhancements are needed for RACH collision avoidance unless requested by other WGs. PCI Space Partitioning is performed by OAM and up to implementation. As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message. PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively. PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node. How to avoid PCI collision in the scenario with Xn between IAB-DU’s donor and IAB-MT’s donor. How to avoid PCI collision in the scenario without Xn between IAB-DU’s donor and IAB-MT’s donor if the scenario is supported. Whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report. |  |  |  |  |
| R3-234011 | Mobile IAB interference mitigation (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-233969 | Discussion on PCI collision avoidance for mobile IAB (ZTE) |  | discussion |  |
| R3-234037 | PCI Collision Avoidance for Mobile IAB-Nodes (Ericsson) |  | discussion |  |
| R3-234050 | Discussion on mitigation of interference (Samsung) |  | discussion |  |
| R3-233838 | PCI Collision Avoidance with Mobile IAB (CANON Research Centre France) |  | discussion |  |
| R3-234171 | PCI collision mitigation of mobile IAB-node mobility (Lenovo) |  | discussion |  |
| R3-234307 | Discussion on interference mitigation (CATT) |  | discussion |  |
| 13.5. Others Interactions with SA2: Reply LS on FS_VMR solutions review to SA2 agreed in R3-231011 With respect to mobile IAB, for issues concerning the control of UE access to MBSR using CAG function no enhancement is needed and no replies are foreseen from RAN3 on this matter. NGAP Initial Context Setup Request, UE Context Modification Request and HO Request to include an IE with code points “mobile-IAB authorized”, “mobile-IAB not-authorized”. Introduce a new IAB-MT User Location Information IE into the existing User Location Information NGAP IE. Mobile IAB indication over NGAP: The IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5. RAN3 to decide whether to support NGAP indication to AMF for mobile IAB-authorization based on progress in SA2. Mobile IAB-node involved positioning: LMF obtains an updated location and velocity information and location time stamp of the mobile TRP by performing either option 1 or option 2 during UE positioning. This statement remains valid unless stated otherwise by SA2. WA: if Network Assisted procedure is used (i.e. UL related positioning is performed), LMF may obtain an updated location and velocity information of the mobile TRP in UL measurement result related message(s) in NRPPa and F1AP. It needs to be further discussed on which message(s) is used and whether time stamp for the location of mobile TRP is needed, according to TS23.273 section 6.1.4. RAN3 agrees to introduce a new TRP type for mobile TRP in NRPPa/F1AP. if option 1 is supported, the NRPPa/F1AP TRP information exchange procedure can be used to trigger MO-LR procedure of IAB-MT to obtain the location and velocity information of mobile TRP as well as the timestamp. if GPSI is available, mobile IAB-MT’s UE ID (i.e. GPSI) can be included in NRPPa/F1AP TRP information response message so that LMF can perform MT-LR procedure to obtain mobile TRP’s location. Checking the security issue of GPSI with SA3 is out of RAN3 scope. The issue for the case that “LMF that performs the location estimation of the mobile TRP can be different than the LMF that performs the location estimation of the target UE” is not in RAN3 scope. Additional ULI: RAN3 agrees that the IAB-DU’s donor-CU includes at least the NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP. RAN3 to select between the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID in case IAB-MT and IAB-DU are connected to different IAB-donors - Option A, the cell ID of IAB-MT’s serving cell is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU. - Option B, the cell ID of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU. Try to close this topic |  |  |  |  |
| R3-233713 | LS on UE RACH-less handover for mobile IAB (RAN2(Huawei)) |  | LS in |  |
| R3-233717 | LS on CAG solution for mobile IAB (RAN2(Ericsson)) |  | LS in |  |
| R3-233730 | LS On IAB-node De-authorization handling (SA2(Ericsson)) |  | LS in |  |
| R3-234039 | Discussion and Draft Reply LS on Mobile IAB-node De-authorization Handling (Ericsson) |  | LS out To: SA2 CC: |  |
| R3-234012 | Discussion on incoming LS (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234143 | (TP for NR_mobile_IAB BL CR for TS 38.455): Positioning enhancement on uplink E-CID measurement for mobile TRP (Huawei, Ericsson, Xiaomi) |  | other |  |
| R3-234144 | (TP for NR_mobile_IAB BL CR for TS 38.473): Positioning enhancement on uplink E-CID measurement for mobile TRP (Huawei, Ericsson, Xiaomi) |  | other |  |
| R3-234455 | Enhancements for IAB-node mobility and onboard UEs (AT&T) |  | Discussion Move to 13.5 |  |
| R3-233891 | Draft Reply LS on VMR (Qualcomm Inc.) |  | LS out To: SA2 CC: |  |
| R3-233970 | (TP for Mobile IAB BL CR TS38.473) Enhancements on positioning and additional ULI for mobile IAB (ZTE, Xiaomi) |  | other |  |
| R3-234038 | (TP for mIAB BL CR for TS 38.470) Discussion of SA2 FS_VMR Solutions (Ericsson) |  | other |  |
| R3-234145 | Discussion on IAB-node De-authorization handling (Huawei) |  | discussion |  |
| R3-234146 | Support of RACH-less HO for mobile IAB (Huawei) |  | discussion |  |
| R3-234147 | [draft] Reply LS on UE RACH-less handover for mobile IAB (Huawei) |  | LS out To: RAN2 CC: |  |
| R3-234251 | (TP for TS 38.423 support of de-authorization for mobile IAB) Discussion on authorization and additional ULI (Xiaomi) |  | other |  |
| R3-234168 | Discussion on IAB-node de-authorization handling (Lenovo) |  | Discussion Move to 13.5 |  |
| 14. Further NR mobility enhancements WI WID [NR_Mob_enh2-Core]: RP-231475 (target: RAN #102) [TU: 1 (1, 1, 1)] QUOTA: 4 |  |  |  |  |
| ## 14.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233743 | (BLCR to 38.401) for L1L2Mob (Huawei, Ericsson, Nokia, Nokia Shanghai Bell) |  | CR0260r11, TS 38.401 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233759 | (BL CR to TS 38.423) Introduction of SCG Selective Activation (Huawei) |  | CR1051r2, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233766 | (BLCR to 38.473) Additions for L1/L2 triggered mobility (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, Intel Corporation) |  | CR1037r8, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233776 | (BLCR to 37.340) Introduction of CHO with SCG(s) (CATT) |  | draftCR - Update spec version ->17.5.0 and revision number -> blank Rev in R3-234563 Endorsed as BL CR unseen |  |
| ## 14.2. Signaling Support for L1/L2 based Inter-Cell Mobility To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized Both intra- DU and intra-CU inter-DU scenarios are supported for L1/L2 mobility. RAN3 will study the signaling impacts on below use cases following to RAN2 prioritization: - Stand alone - Carrier Aggregation (Change of PCell) - NR-DC (Change of PCell at MN, Change of PScell at SN) RAN3 will aim for a single solution for network signaling design on L1/L2 based inter-cell mobility to support all agreed scenarios. The details of solution are FFS. WA: For intra-DU L1/L2 mobility, the existing F1AP procedure (e.g., F1AP UE CONTEXT MODIFICATION) is reused for handover configuration for inter-cell mobility. RAN3 focuses on the network-controlled procedure for L1/L2 based inter-cell mobility. The gNB-CU initiates the L1/L2 mobility configuration procedure. The configuration of candidate target cell(s) for L1/L2 mobility is initiated by the gNB-CU. WA: RAN3 assumes that the UE sends the L1 measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell. All details are up to RAN1 and RAN2 discussion. During L1/L2 handover configuration, the gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request procedure. The gNB-DU may accept the target cells of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message(s). gNB-DU may accept all or part of the target candidate cells. gNB-DU initiated L1/L2 handover configuration is not allowed. The UE sends the lower-layer measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell. The following previous agreements for intra-DU case are confirmed to be also applicable for inter-DU case: 1.Both intra- DU and intra-CU inter-DU scenarios are supported for L1/L2 mobility. 2.RAN3 will study the signaling impacts on below use cases following to RAN2 prioritization: - Stand alone - Carrier Aggregation (Change of PCell) - NR-DC (Change of PCell at MN, Change of PScell at SN) 3.RAN3 will aim for a single solution for network signaling design on L1/L2 based inter-cell mobility to support all agreed scenarios. The details of solution are FFS. 5.RAN3 focuses on the network-controlled procedure for L1/L2 based inter-cell mobility. 7.The configuration of candidate target cell(s) for L1/L2 mobility is initiated by the gNB-CU. Details are FFS. For inter-DU inter-cell mobility, the UE Context Setup procedure is reused for handover configuration. CU suggest the candidate cell(s) to DU, “gNB-DU can suggest candidate cells after the gNB-CU initiates the L1/L2 inter-cell mobility configuration” is with low priority. CU can update the suggested candidate cells. For intra-DU case, the gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message. For inter-DU case, the target gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message. RAN3 works on the same signaling procedure for both initial cell switch and subsequent cell switch for intra-DU L1/L2 handover. During execution phase, it is up to the gNB-DU implementation when will the gNB-DU signal to the CU. This does not mean that the gNB-DU is “allowed” to signal to the gNB-CU before LTM command is sent to the UE. For intra-DU LTM, the gNB-CU assigns a new UL GTP TEID for each DRB and provides it to the gNB-DU via UE Context Modification Request message(s). The gNB-DU assigns the new DL GTP TEIDs per DRB per candidate cell (whether it should be per candidate cell needs to be further discussed) and provides them back to the gNB-CU in UE Context Modification Response message(s). For inter-DU LTM, the gNB-CU assigns a new UL GTP TEID for each DRB and provides it to the target gNB-DU via UE Context Setup Request message(s). The target gNB-DU assigns the new DL GTP TEIDs per DRB per candidate cell (whether it should be per candidate cell needs to be further discussed) and provides them back to the gNB-CU in UE Context Setup Response message(s). Intra-CU UP case: CU will start data transmission after LTM cells switch signaling from DU including target cell ID. The CU requests the candidate DU to provide RACH resource per candidate cell for TA acquisition in inter-DU case. For intra-DU LTM, the gNB-DU sends a DDDS frame about unsuccessfully transmitted downlink data to the gNB-CU after LTM cell switch if RLC reestablishment is configured. For inter-DU LTM, the DDDS should be sent from source gNB-DU to CU-UP when the LTM cell switch command is sent. Then the CU-UP can start forwarding the unsuccessfully transmitted data to target gNB-DU. The gNB-CU may modify or release L1/2 Triggered Mobility (LTM) candidate cells in the gNB-DU. The (candidate) gNB-DU may cancel already configured L1/2 Triggered Mobility (LTM) candidate cells and notify to the CU. The gNB-CU may use the UE Context Modification procedure to modify or release the prepared resources of candidate cells in the (candidate) gNB-DU and use the UE Context Release procedure to release the UE context in the (candidate) gNB-DU. For inter-CU-UP LTM, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission. For inter-CU-UP LTM, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP. In case of gNB-CU-UP change, the gNB-CU triggers the source gNB-CU-UP to start data forwarding after receiving LTM cells switch signalling from DU. For inter-CU-UP LTM, Path switch procedure is performed towards the core network after detecting the UE has accessed to the target cell. For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs for which RLC is not re-established. For intra-CU inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access (following legacy). The (candidate) gNB-DU may use the UE Context Modification Required message to release the candidate cells, and the gNB-CU shall not reject. The (candidate) gNB-DU may use the UE Context Modification Required procedure to request to cancel the prepared resources of a subset of candidate cells in it and use the UE Context Release Request procedure to request to release all candidate cells in it. RAN3#120: RAN3 agrees option2 and supports for multiple messages for LTM candidate cell configuration. In case of CP UP separation, once CU-CP receives LTM cell switch signaling from (source)DU, CU-CP initiates E1 bearer context modification to the CU UP including DL tunnel ID per DRB for target cell, for data transmission. For LTM with UP change, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission. Notification of LTM triggering to gNB-CUs: WA: option 1 is adopted with only class 2 procedure. If more information to source DU in response message is needed, we go for option 2. E1 aspects for inter-DU LTM: For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP. For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure towards the core network. TA acquisition: CU can request RACH resources for early TA acquisition together with the LTM candidate cell configuration to a Candidate DU in the UE Context Setup Request or UE Context Modification Request messages (for inter-DU LTM and intra-DU LTM respectively). If the Candidate DU accepts the RACH resource request for early TA acquisition, the Candidate DU responds the CU with RACH configuration in the UE Context Setup Response or UE Context Modification Response messages. DDDS: For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs RLC is not re-established. For both intra-DU and inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID, if new UL TEID being assigned, to CU-UP after target gNB-DU detects the UE access. Work on stage2 and stage3 details |  |  |  |  |
| R3-233709 | LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM (RAN1(Fujitsu/Mediatek/CATT)) |  | LS in noted |  |
| R3-233719 | LS on Early TA and RACH-less (RAN2(Ericsson)) |  | LS in noted |  |
| R3-233720 | LS on L1 measurements for LTM (RAN2(Ericsson)) |  | LS in noted |  |
| R3-234074 | (TP for L1L2Mob BLCR for TS 38.401): Discussion on RAN3 impacts from the incoming LSs (Huawei) |  | other |  |
| R3-233905 | (TP for LTM BL CR to TS 38.401) Solutions for LTM (Ericsson) |  | other |  |
| R3-233827 | TP (BL CR TS 38.401) L1/2 Triggered Mobility (LTM) Procedures (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234165 | Discussion on L1L2 based inter-cell mobility (Lenovo) |  | discussion |  |
| R3-234459 | (TP for LTM BL CR to TS 38.473) Discussion on L1L2 triggered mobility (ZTE) |  | other |  |
| R3-234460 | TP for LTM BL CR to TS 38.401 (ZTE) |  | other |  |
| R3-233984 | Signalling Support for LTM (Qualcomm Incorporated) |  | discussion |  |
| R3-233840 | (TP to Mob_enh2 BL CR TS38.401) Discussion on L1/L2 based Inter-cell Mobility (Samsung Beijing) |  | other |  |
| R3-234447 | Discussion on L1L2 based Inter-Cell Mobility (CMCC) |  | discussion |  |
| R3-234446 | (TP to TS 38.401) L1/L2 based Inter-Cell Mobility (CMCC) |  | other |  |
| R3-234348 | Discussions on stage-3 and inter-DU LTM (LG Electronics Inc.) |  | discussion |  |
| R3-233869 | Rel-18 LTM issues (NEC) |  | discussion |  |
| R3-234368 | Further discussion on LTM (NTT DOCOMO INC..) |  | discussion |  |
| R3-233828 | TP (BL CR TS 38.473) Additional Implications to L1/2 Triggered Mobility (LTM) (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234075 | (TP for L1L2Mob BLCR for TS 38.473): L1/L2 Mobility (Huawei) |  | other |  |
| R3-233906 | (TP for LTM BL CR to TS 38.473) F1AP impacts for LTM (Ericsson) |  | other |  |
| R3-233870 | (TP to TS 38.473 on LTM) co-existence between LTM and L3 mobility (NEC) |  | other |  |
| R3-233886 | (TP for L1L2Mob BLCR for TS 38.401) Reference configuration and Target Configuration ID in LTM (Google Inc.) |  | other |  |
| R3-233887 | (TP for L1L2Mob BLCR for TS 38.473) Reference configuration and Target Configuration ID in LTM (Google Inc.) |  | other |  |
| R3-233888 | (TP for L1L2Mob BL CR for TS 38.473) UE Context identification (Google Inc.) |  | other |  |
| R3-234040 | Resource management at gNB-DU for LTM (Rakuten Symphony) |  | discussion |  |
| R3-234041 | Intra gNB CU-UP relocation during LTM (Rakuten Symphony) |  | discussion |  |
| R3-234166 | (TP to TS 38.401 & TS 38.470) Support of L1L2 based inter-cell mobility (Lenovo) |  | other |  |
| R3-234201 | (TP for BLCR of 38.401)Further consideration about LTM execution procedure (CATT) |  | other |  |
| R3-234202 | (TP for 38.473 BLCR) Further consideration about candidate cell configuration (CATT) |  | other |  |
| R3-234349 | (TP for LTM BL CR for TS 38.473) (LG Electronics Inc.) |  | other |  |
| Execution of L1/L2 triggered mobility serving cell change Notification of LTM triggering to gNB-CUs Turn the WA to agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch. L1 measurement configuration and reporting no-UE associated procedure or UE associated procedure: Samsung: prefer no-UE associated procedure. LG: slightly prefer no-UE associated procedure. E///: RS configuration issue, discuss the content first. QC: UE associated procedure for RS configuration. ZTE: first identify the procedure and the detail of message. CMCC: prefer UE associated procedure. Nok: fine with UE associated procedure. HW: RS configuration from C-DU to S-DU via CU. Leave the UE associated procedure or no- UE associated procedure FFS. Samsung: pospont the discussion in RAN3. RS configuration sends from C-DU to S-DU via CU or CU request. Need to discuss detail. For inter-gNB-DU LTM, the gNB-CU uses the UE Context Setup/UE Context Modification procedure to request the candidate gNB-DUs to provide the RS configuration of candidate cells during LTM configuration. Samsung: prefer no-UE associated procedure. Proposal 2:The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations. The gNB-CU signals the final LTM configuration to the involved Source gNB-DU and Candidate gNB-DU(s) via the UE Context Modification procedure. TCI state configuration The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM. LG, CATT: agree. After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE. CATT: all candidate Dus is related with subsequence LTM. Selected beam transfer The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU. FFS for the detail of selected beam information. For the part of CU to target DU is FFS of which procedure can be used. ZTE: wait for RAN2 progress. For RACH-less LTM, the selected beam is included as a separate IE in the DU to CU RRC Information in the UE Context Modification Required message from the source DU to the CU, and in the CU to DU RRC Information in the UE Context Modification Request message from the CU to the target DU TA management RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue. TA establishment triggering TA acquisition The CU requests the candidate DU to provide the TA for RACH-less LTM, and sends the TA to the source DU. CU shall send the TA value to the source DU. HW: clarify of the motivation and there is other solution. E///: list all the possible solutions and offline discussion. Samsung: rely on offline. TA value in LTM cell switch RAN3 to introduce new Class 2 messages to signal the TA Value and preamble related information from Candidate gNB-DU to gNB-CU, and from gNB-CU to Source gNB-DU. TA acquisition failure and fallback solutions If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU). Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side) TA validity and time alignment timer Capture in Stage 2 the RAN3 agreement that gNB-CU requests RACH resources for TA acquisition from Candidate gNB-DU(s) using existing UE Context Setup and/or UE Context Modification procedures. Early DL sync Proposal 6:In inter-DU LTM, the gNB-CU provides the RRC RACH configuration and CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure. Proposal 7:To introduce new class 2 non-UE associated F1 signalling to deliver the TA information and the CFRA resource information from the candidate gNB-DU to the source gNB-DU via gNB-CU. Proposal 10: RAN3 agrees that the TA validity is checked by the source gNB-DU based on implementation. E1 aspect HO collision Proposal 11:Flexible priority is applied to address the HO collision between L3 handover and LTM. - Case 1, L3 handover triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has higher priority than LTM. - Case 2, LTM triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has higher priority than L3 handover, - Case 3, LTM and L3 handover are triggered almost simultaneously (cross signalling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has higher priority. Subsequent LTM Reference configuration CB: # MobilityEhn1_L1L2Mobility - discuss on the principles for the above issues, and capture the agreements - Update the corresponding TPs. - list all the open issues. (moderator - HW) Summary of offline disc R3-234574 |  |  |  |  |
| ## 14.3. Support CHO in NR-DC For CHO including target MCG and target SCG in NR-DC [RAN3]: - to specify data forwarding optimizations; and - to specify, if needed, a solution to avoid unnecessary signaling exchange between source MN and target SN. To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline In Rel.18, RAN3 will continue the work on the CHO with SCG at the target. The scope will be limited to the data forwarding optimizations. Regarding CHO with multiple SCGs at the target, RAN3 will wait for the progress in RAN2 before starting signalling design. At the next meeting, RAN3 will open discussion on the data forwarding aspects. WA: RAN3 agrees to create a separate chapter in TS 37.340 related to CHO with DC, and to do it as part of the work on the Rel.18 Mobility Enhancements. There is a need to discuss the avoidance of unnecessary signaling between MN and target SN for CHO + MR-DC. Early Data Forwarding optimizations with involvement of the target SCG(s) in Rel-18 will be supported. Focus on optimizing duplicated data forwarding scenario. There is no issue to identify the same target candidate SN by the source in case direct data forwarding is used on all the forwarding paths/target MNs. WA: both direct and indirect data forwarding will be supported. Direct data forwarding is supported by current specification, FFS on further signaling enhancement. Optimization on indirect data forwarding is by network implementation. RAN3 acknowledges unnecessary signaling exchange between MN and the target SN would cause inefficiency and extra latency for CHO + NR-DC, the solution is FFS. WA: In CHO with (multiple) SCG configuration, the (candidate) SN can acknowledge whether it has direct data forwarding path with source SN. If existed, it can assign the same data forwarding address for multiple data forwarding paths, otherwise, it is up to the candidate SN implementation. Confirm the early data forwarding for CHO with multiple SCGs is a new problem. Data forwarding optimizations should not impact legacy HO mechanism as the fundamental basis. Data forwarding optimizations focus on how to avoid multiple data forwarding paths. RAN3 focuses on the following aspects for CHO with multiple SCGs. - T-MN provides the PDU session admission results of different T-SN(s) in the HO procedure considering the pair of candidate T-MN and T-SN(s). - A set of data forwarding addresses are provided from candidate T-MN to the source node. RAN3#120: Regarding avoiding duplication of the data forwarding: RAN3 confirms the problem that is to be solved is avoiding that the single T-SN receives the same data from multiple T-MNs prepared for CHO (assuming there are multiple T-MNs prepared with the same T-SN). RAN3 agrees to enable the T-SN to let the T-MN know if it has direct path available to S-MN (or to both, S-SN and S-MN). It is FFS if any enhancements for the case with a single T-MN are needed. Regarding avoiding unnecessary CHO cancellation: RAN3 agrees to enable the T-MN to inform the S-MN if the CHO is prepared with full or delta configuration. Work on stage2 and stage3 details |  |  |  |  |
| R3-234085 | (TPs for TS 37.340, 38.423 BLCRs) Other CHO related aspects (Huawei) |  | other |  |
| R3-233982 | CHO with multiple candidate SCGs (Qualcomm Incorporated) |  | discussion |  |
| R3-233907 | (TP to TS 37.340 and 38.423 BL CRs) CHO with candidate SCG(s) (Ericsson) |  | other |  |
| R3-234084 | (TPs to TS 37.340 and 38.423 BL CRs) avoid multiple data forwarding paths (Huawei, Samsung, Lenovo, Qualcomm Incorporated) |  | other |  |
| R3-233834 | [TPs to TS38423, TS37483 and TS37340, CHO with NRDC] Continuation of the discussions on enhancements for CHO with MR-DC (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-233818 | (TP for CHO with NR-DC to TS 38.423, TS37.340): Early data forwarding optimization for CHO with SCG procedure (ZTE) |  | other |  |
| R3-233954 | (TP to BLCR TS38.423) Considerations on CHO in NR-DC (Samsung) |  | other |  |
| R3-234167 | Discussion on CHO in NR-DC (Lenovo) |  | discussion |  |
| R3-234350 | Discussion on signaling support for CHO with SCGs (TP for TS 38.423) (LG Electronics Inc.) |  | discussion |  |
| R3-234394 | TP to BLCR for 38.423 on CHO with SCG and multiple SCGs (CATT) |  | other |  |
| Topic 1: per T-SN information provided from the T-MN to the S-MN in case of CHO with multiple SCGs Following the legacy procedures in R17, data forwarding is per-node other than per-cell. For CHO with multiple SCGs within the same T-SN, and PDU session/DRB data forwarding is performed per T-SN SN instead of per PSCell. Samsung: there is another alt solution. CATT: the T-SN admit same PDU sessions and QoS flows for prepared PSCells. For CHO with multiple SCGs among different T-SNs, enhance HO Request ACK message to provide different PDU Session admission results, data forwarding addresses and list of prepared PSCells for each prepared T-SN. Proposal 1-3: S-MN indicates the Number of PSCells To Prepare IE to the target MN in HO REQUEST message. Proposal 1-4: T-MN indicates the Number of PSCells To Prepare IE to the target SN in S-NODE ADDITION REQUEST message. Topic 2: RAN3 impacts of CHO associated CPAC configurations Proposal 2: Include in the HANDOVER REQUEST ACKNOWLEDGE message about the prepared candidate PSCells and parameters of the associated execution conditions. Topic 3: if and how to avoid unnecessary signaling between S-SN and T-MN/T-SN Including existing RRC Config Indication IE in XnAP HANDOVER REQUEST ACKNOWLEDGE message. Topic 4: avoid multiple data forwarding paths Proposal 1:Keep current mechanism of direct data forwarding for normal handover and conditional handover in Rel-18. Proposal 2:The editor's note in the BL CR for direct data forwarding can be removed. Proposal 1: in CHO with SCG, reuse the existing Source NG-RAN Node ID IE in the S-NODE ADDITION REQUEST message as the S-SN ID. Proposal 2: in CHO with SCG, reuse the existing Direct Data Forwarding Path Indication IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate the direct forwarding path is available between the T-SN and S-SN. Proposal 3: in CHO with SCG, introduce a new IE in SN Additional Request ACK to indicate to the target MN about the direct path availability between target SN and source MN. Proposal 4: in CHO with SCG, introduce a new IE in XnAP: HO Request ACK to indicate to the source MN about the direct path availability between target SN and source SN. i.Add the RRC Config Indication IE to the S-NODE MODIFICATION REQUEST and HANDOVER REQUEST ACKNOLEDGE messages to indicate that full or delta configuration is applied at the target side. ii.Add one indicator to the S-NODE MODIFICATION REQUIRED message to indicate whether the source SCG reconfiguration has impact on the target SCG or not once the delta configuration is applied. Procedure for CHO with multiple candidate SCGs Proposal 1. Source MN indicates to a candidate MN in Handover Req the maximum number of candidate target PSCells that the candidate MN can suggest to the candidate SNs to prepare. Candidate MN indicates to a candidate SN in SN Addition Request the maximum number of candidate target PSCells that the candidate SN can prepare. Proposal 2. The parameters of the CPA/CPC execution conditions are provided by a candidate MN to the source MN in Handover Req Ack, and are provided outside the Target NG-RAN node to Source NG-RAN node Transparent Container in Handover Req Ack. The CPA/CPC execution condition parameters are provided for each candidate target PSCell. Data forwarding optimizations and other signalling aspects Proposal 3. In the case of CHO with candidate SCG or CHO with multiple candidate SCGs, when direct data forwarding from source MN or source SN to a candidate SN is possible, the candidate SN informs a candidate MN that TEIDs allocated for data forwarding are shared with other candidate MNs. Only the candidate MN which receives the TEIDs from the candidate SN forwards it to the source MN. Proposal 4. In CHO with multiple candidate SCGs, a candidate MN includes in Handover Req Ack a separate list of accepted PDU sessions for each candidate SN with which it performs CPC or CPA preparation. Thus, the Handover Req Ack has a separate list of accepted PDU sessions for each pair of candidate target PCell and candidate SN. This has Xn specification impact. Avoiding duplication of forwarded data Additional problems CHO with multiple SCG request CB: # MobilityEhn2_CHO - focusing on the agreements for the above issues. - capture the agreements into TP is possible. (moderator - SS) Summary of offline disc R3-234575 |  |  |  |  |
| ## 14.4. Others RAN2 led To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] RAN3 considers SCG selective activation is prioritized in the Rel-18 work. It can be revisited based on RAN2 progress. WA: RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signaling aspects. It can be revisited based on RAN2 progress. From RAN3 point of view, Rel-16/Rel-17 CPAC procedures are considered as start point for the Rel-18 work. The following scenarios are depending on RAN2 progress. SCG failure handling enhancements to enable PSCell addition and PSCell change after SCG failure. Signaling support for inclusion of CPC configuration within a CPC or CPA configuration, in case CPC/CPA configuration is supported within CHO configuration. WA: A primary focus of the objective is to enable subsequent cell changes by keeping conditional reconfigurations after a cell change. RAN3 to pursue study of the Xn/F1 signaling changes required to support this objective. RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signaling aspects. It can be revisited based on RAN2 progress. [last meeting’s WA turned into agreement] WA: RAN3 will work to enable both indirect and direct early data forwarding in Selective Activation. At this moment, RAN3 does not foresee any scenarios where direct forwarding is not feasible/desired. WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation. WA: Enhance signalling for Selective Activation. Enhance XnAP and F1AP signaling to support NR Selective Activation. Introduce a new indicator to the S-NODE ADDITION REQUEST message over Xn to indicate that the request is for Selective Activation. RAN3 assumes that a UE can be configured to keep a conditional configuration for CPA after CPA execution. The kept CPA conditional configuration is used for subsequent CPC (but with different triggering conditions). This can be revisited based on RAN2 progress. RAN3 should further analyze the impacts if RAN2 decides to support activation/deactivation of candidate PSCell evaluation after the first time SCG selective activation configuration. WA: Add a new indication as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation. Note: This WA may be revisited based on RAN2 progress. For inter-SN SCG selective activation, after CPC execution, the MN needs to notify the source SN and the selected SN of the cell change. Reuse the following messages to update/modify/cancel the prepared candidate PSCells for SCG Selective Activation: - SN Modification Request/ SN Modification Request Acknowledge - SN Modification Required/ SN Modification Confirm - Conditional PSCell Change Cancel - SN Change Required/ SN Change Confirm - SN Release Request / SN Release Request Acknowledge RAN3 eliminates the option for UPF-based data forwarding thus assuming that the number of PSCell prepared for Selective Activation will be limited and the serving PSCell will not change too often. Reuse the Xn-U Address Indication message and the Early Status Transfer message to support early data forwarding for SCG Selective Activation. RAN3#120: Add a new IE as a sub IE of the Conditional PSCell Change Information Required IE in the S-NODE CHANGE REQUIRED message to indicate that the request is for SCG Selective Activation. It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container). Note: This can be revisited based on RAN2 progress. Add a new IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation. It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container). Note: This can be revisited based on RAN2 progress. Introduce a new indicator for selective activation in the S-NODE MODIFICATION REQUEST message to indicate that the request is for SCG selective activation. FFS on the scenarios. Introduce a new indicator for selective activation in the Xn-U address indication message to indicate that the request is for SCG selective activation. FFS on the scenarios. |  |  |  |  |
| R3-233983 | SCG Selective Activation in NR-DC (Qualcomm Incorporated) |  | discussion |  |
| R3-233908 | (TP to TS 38.423 BL CR) Subsequent CPAC (Ericsson) |  | other |  |
| R3-234461 | (TP for TS 38.423 BL CR) Discussion on support of subsequent CPAC (ZTE) |  | other |  |
| R3-233835 | Considerations on the indication of Selective Activation and collateral problems that must be addressed to enable the feature. (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234086 | (TP to TS 38.423 BLCR) Consideration on Subsequent CPAC (Huawei) |  | other |  |
| R3-234462 | (Subsequent CPAC BL CR to TS 37.340) Introduction of subsequent CPAC (ZTE, China Telecom, Huawei, China Unicom) |  | draftCR |  |
| R3-233871 | Discussion on Selective Activation of the cell of groups (NEC) |  | discussion |  |
| R3-233872 | (TP to TS38.423 BL CR) Selective Activation of the cell of groups (NEC) |  | other |  |
| R3-233885 | (TPs for SCG Selective Activation BLCR for TS 38.423 & TS 38.473) Reference SCG configuration (Google Inc.) |  | other |  |
| R3-234192 | (TP for TS 38.473) On Subsequent CPAC (Lenovo) |  | other |  |
| R3-234369 | Security issue on selective activation (NTT DOCOMO INC..) |  | discussion |  |
| R3-234384 | Discussion on selective activation of the cell groups (Samsung) |  | discussion |  |
| R3-234395 | Discussion on NR-DC with selective activation of the cell groups (CATT) |  | discussion |  |
| R3-234000 | Discussion on subsequent CPAC procedures (China Telecommunication) |  | Discussion Late contribution |  |
| R3-234001 | (TP to BL CRs of 38.423/38.473) On support of subsequent CPAC procedures (China Telecommunication) |  | Other Late contribution |  |
| RAN3 follows RAN2 and uses the same name “subsequent CPAC” or “S-CPAC” for the selective activation. ZTE,HW : prefer subsequent CPAC. RAN3 shall address first preparing S-CPAC, execution of the initial access and subsequent SN change. Additional aspects, like e.g. update of the set of prepared SNs (deleting or adding an SN) should be considered once basic functionality is ready. Configuration of Subsequent CPAC RAN3 should wait for RAN2 progress regarding the optionality of the reference SCG configuration before further discussing the details of the subsequent CPAC indication introduced in the SN change required message or the SN addition request message. NEC: RAN2 has agreement. The information of other candidate PSCells in other candidate target SNs should be provided in the SN Addition Request message/SN Modification Request message from the MN to the candidate target SNs, if RAN2 agrees. QC: wait for RAN2. E///: need to consider whether to consider this is RAN3 issue. HW: clarification on details of the information. Proposal 2: The other candidate PSCells and corresponding execution conditions are sent from the candidate target SNs to the MN via SN ADDITION REQUEST ACK message. Such information could be within an RRC container, which is pending RAN2’s discussion. Keep conditional reconfigurations Proposal 3:For SN-initiated inter-SN CPC, the target SN decides which conditional configurations can be reused for subsequent CPC. Proposal 4:Similarly for MN-initiated CPA, the target SN decides which conditional configurations can be reused for subsequent CPC. Proposal 5:Enhance the SN Addition procedure by including an indicator for kept configurations. The specific details are still to be determined (FFS). Proposal 6:After execution, MN can inform the target SN about which conditional configurations the UE has stored and other related information. Applicable conditional configurations Reference configurations Partial success for the Selective Activation Signalling support for data forwarding Proposal 3-1: To support early indirect data forwarding with MN serving as an anchor node, MN must signal data forwarding TEID to the target SNs during preparation. Proposal 3-1: To support early direct data forwarding (i.e. direct tunnel between target SN and other prepared targets), SN Modification procedure must be enhanced in order to signal the TEIDs of other prepared target SNs. Coordination of selective activation scenarios Delayed path switch CB: # MobilityEhn3_S-CPAC - working on the agreements of S-CPAC of RAN3 work. - working on the TPs if possible. - capture the open issues. (moderator - ZTE) Summary of offline disc R3-234576 |  |  |  |  |
| ## 15. Enhancements of NR Multicast and Broadcast Services WI WID [NR_MBS_enh-Core]: RP-213568 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 2 |  |  |  |  |
| ## 15.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233741 | Support of MBS enhancement (Ericsson, Nokia, Nokia Shanghai Bell) |  | CR1068r1, TS 38.423 v17.5.0, Rel-18, Cat. B |  |
| R3-233749 | Support of MBS enhancement (CATT, Nokia, Nokia Shanghai Bell) |  | CR1007r1, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233765 | (BLCR to 38.473) Support of MBS enhancement (Samsung, Nokia, Nokia Shanghai Bell) |  | CR1189r1, TS 38.473 v17.5.0, Rel-18, Cat. B |  |
| R3-233782 | (BL CR to TS 38.300) Introduction of NR MBS enhancements (Nokia, Nokia Shanghai Bell, Huawei, ZTE, Ericsson, Lenovo) |  | draftCR |  |
| R3-233791 | (BLCR to 38.401) Introduction of NR MBS enhancements (Huawei, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson, Lenovo) |  | CR0281r3, TS 38.401 v17.5.0, Rel-18, Cat. B |  |
| R3-233803 | (BLCR to 38.470) Multicast Reception for RRC_INACTIVE state UEs (Lenovo, Huawei, Nokia, Nokia Shanghai Bell) |  | CR0111r2, TS 38.470 v17.5.0, Rel-18, Cat. B |  |
| R3-234615 | Summary of unofficial offline Discussion on Rel-18 MBS (CATT) |  | Discussion |  |
| 15.2. Support for MBS reception in RAN sharing scenarios Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3] Incoming LS from RAN#97e in RP-222678 TSG RAN suggests RAN3 to focus on the work on the broadcast service for resource efficiency improvement for MBS reception in RAN sharing scenario, and to further coordinate with SA2 on the applicability of the solution to multicast service when needed. NG-RAN shall be able to identify the MBS session signaling from different operators’ 5GCs aim at the same MBS session. The detail information is pending to SA2. The same PTM radio resource can be allocated in a shared cell for transmission of the same MBS service provided by different operators. The solution provided by RAN3 work on protocal in RAN sharing scenario should not have impact on Pre Rel-18 UE. RAN3 believes that Solution(s) which assume MOCN RAN nodes can identify the same MBS service based on the information provided by 5GC should be supported. The following principles should be considered when discussing solutions on which information should be provided from 5GC: Principle1: The solution provided by RAN3 for RAN sharing should not have impact on Rel-17 UE and Rel-17 gNB. Principle3: The identity providing a reference to the same MBS service should not depend on the momentarily participating operators considering of the possibility for sharing operators leaving or entering the common ongoing session from time to time, that’s to say the solution should be robust to cover the cases that the shared PLMNs start and stop the MBS session at the same time and start and stop the MBS session at the different time. Principle4: It could not be assumed that MB-SMF/AF/MBSF is aware which NG-RAN node or which cell within a NG-RAN node is shared since currently NG-RAN node only inform AMF of the supported PLMN and no coordination with MB-SMF/AF/MBSF. RAN3 think that a solution based on information received from 5GC is desired. Solutions 2,7,24 and 29 can work, while solutions 2, 7 with majority support in RAN3. Solution 24 brings configuration efforts which may have flexibility and scalability issue in case MBS services are dynamically added or removed. It is up to the NG-RAN node implementation on how to handle different QoS parameters for the same service from different PLMNs in case different QoS parameters for the same service are received. Wait for feedback from SA2 on solution down-selection. For local MBS service, cell granularity shared area decision according to overlapped area. For location dependent MBS service, the NG-RAN node should associate the relevant shared area corresponding to area session ID, FFS on how to handle different area session IDs allocated from different PLMNs, and whether and how to handle different service areas associated with the area session IDs. F1 impact: The gNB-CU provides the MBS RAN Sharing efficiency Information received from CN (if received) to the gNB-DU in F1AP: BROADCAST CONTEXT SETUP REQUEST message. The name and details of "MBS RAN Sharing efficiency Information" are FFS. "MBS RAN sharing efficiency information" == "information enabling the gNB to identify the MBS sessions among which resource efficiency for MBS reception in RAN sharing scenarios can be applied" In case of RAN Sharing with multiple cell-ID broadcast, each logical gNB-DU will receive within the F1AP: BROADCAST CONTEXT SETUP REQUEST message the MBS RAN Sharing efficiency Information received from CN (if received). Agree Option4 to support shared NG-U tunnel. For MOCN, it is up to NG-RAN node implementation to decide how many NG-U tunnels to be setup. For MOCN, it is up to the NG-RAN node implementation on how to handle different S-NSSAI received for the same shared service from different PLMNs (i.e. same Associated Session ID). It is agreed to transfer the Associated Session ID together the MBS Session ID. WA: The Associated Session ID is per TMGI per Area Session ID. Therefore, it is transferred outside the N2 SM container. WA: Introduce an explicit indication to 5GC in case that NG-U resources are not setup. Support, for MOCN, sharing of F1-U resources among multiple broadcast MBS sessions with the same associated session ID. WA: In case of RAN sharing with multiple Cell ID broadcast, the entity controlling the logical DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH. RAN3#120: Add Associated Session ID IE in the same level of MBS Session ID IE in related NG/F1 messages. FFS on E1. Include in the MBS Session Setup or Modification Response Transfer IE an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session in case of unicast transport. Define a new class 1 procedure to allow the gNB/CU-CP requesting the setup of shared NG-U resources during an ongoing broadcast MBS session for unicast transport. Turn the following WA to agreement with removal of the text in the bracket: In case of location dependent broadcast services, the gNB deduces identical broadcast content from the MBS Associated Session ID and the MBS Service Area information provided by the participating 5GCs. The CU-CP sends the MBS Service Area to DU and DU determines whether to apply RAN sharing optimization over the radio. FFS on the determination between CU and DU for MOCN case. FFS on where to apply the RAN sharing optimization. FFS on how DU determines whether RAN sharing enhancement applies to the corresponding cells, i.e. based on which information and based on which behavior. FFS whether an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session also applies to multicast transport. FFS on how many MBS session context procedure instances should be triggered in gNB-DU to establish the related MBS session contexts. It could be further discussed based on the whole picture of Broadcast context setup procedure which includes NG,F1 and E1. FFS on how many F1-U tunnels should be established for multiple cell-ID broadcast cases. PDCP configuration selection at gNB-DU for RAN sharing with multiple cell-ID broadcast case: Open issue 1: Which option should be used by DU to decide the MRB-PDCP-ConfigBroadcast? Option 1: OAM configure the MRB-PDCP-ConfigBroadcast of each MRB to CU or DU. Option 2: DU applied the MRB-PDCP-ConfigBroadcast of the first CU or the second CU. FFS whether DU needs to provide the used MRB-PDCP-ConfigBroadcast to CU. Any action in RAN3 needed on OAM based solution? |  |  |  |  |
| R3-233948 | (TP for BLCR TS37.483) Discussion on MBS RAN sharing (Samsung) |  | other |  |
| R3-233852 | (TP for TS 38.470, 37.483, 38.473) Resolution of RAN sharing open points (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-233979 | Support of MBS in RAN sharing scenarios (Qualcomm Incorporated) |  | discussion |  |
| R3-234089 | (TPs to MBS BL CRs) MBS reception in RAN sharing scenario (Huawei, CBN) |  | other |  |
| R3-234211 | (TP to TS 38.413, 38.473) Network sharing for MBS Broadcast (ZTE) |  | discussion |  |
| R3-234233 | Further thoughts on MBS reception for RAN sharing scenarios (Ericsson) |  | other |  |
| R3-234247 | Discussion on efficient MBS reception in RAN sharing scenario (CATT,CMCC,CBN) |  | other |  |
| R3-234184 | Remaining issues of supporting MBS reception in RAN Sharing (Lenovo) |  | discussion |  |
| R3-233978 | Stage-2 CR for Introducing MBS RAN Sharing for OAM based solution. (Qualcomm Incorporated, Huawei, Nokia, Nokia Shanghai Bell) |  | draftCR |  |
| 15.3. Support for RRC_INACTIVE state RAN2 led Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] It is the common understanding that the following information, among others, may be taken into account by the gnb when deciding to enable ues receiving multicast in rrc_inactive state: a) the capability of ue (of whether support the mode “multicast over rrc inactive”); b) the rel-17 multicast context, e.g. the qos parameters not associated to any specific ue; c) parameters available at the local gnb without enhancement on interfaces, e.g. cell load. RAN3 can discuss the mobility taken into account the progress in RAN2 and coordinate with RAN2. The gNB decides whether a UE is configured to receive multicast data in RRC_INACTIVE. The gNB may take at least the following information into account based: 5QI, PER, ARP, and expected UE Activity Behaviour, information locally available at the gNB and other. The QoS requirements apply to the provision of the multicast session, independently from the strategy a gNB applies to achieve their fulfillment. NG-RAN signaling supports service continuity for UEs receiving multicast session data in RRC_INACTIVE, i.e., a UE is able to continue multicast reception without RRC state transitioning after cell reselection in RRC_INACTIVE state if the configuration of the new cell is available for the UE. FFS impacts to network interface. During an active multicast session, the gNB-DU shall keep the PTM transmission when delivering respective multicast data to RRC_INACTIVE UEs. Detailed F1AP design is pending on RAN2 decision for PTM configuration delivery method and further RAN3 discussions. Support a per UE per MBS session indication from CN to RAN. Additional protocol function for multicast MCCH configuration is expected to be included in F1 Multicast Context management procedures, based on RAN2 progress on MCCH matters. RAN3 acknowledged the new SIB defined in RAN2 and how to introduce the new SIB over F1 needs to be further discussed. XnAP signalling for exchange of neighbour cells’ PTM configuration is not supported. No enhancement to enable network to be aware of the distribution of UEs receiving multicast in RRC_INACTIVE is agreed. RAN3#120: Agree the following statement in the stage 2 and stage 3 spec: Stage 2 (38.300): As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS for a UE Assistance Information associated with a multicast MBS session, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE is expected to require dedicated resources very frequently. Based on this information, the gNB may decide the RRC state of the UE. in Stage 3 (NGAP/XnAP), the definition is as follows: This IE provide MBS Assistance Information as specified in TS 38.300 [8] and TS 23.247 [x]. No LS is sent to SA2 on the MBS Assistance Information definition related aspect. Check progress in other WGs, and continue work on stage2/3 |  |  |  |  |
| R3-234212 | (TP to TS 38.413, 38.473) Multicast reception in RRC_INACTIVE (ZTE) |  | discussion |  |
| R3-234234 | Further thoughts on support for RRC_INACTIVE (Ericsson) |  | other |  |
| R3-233853 | (TP for TS 38.300, TS 38.423) Resolution of open points for Reception in RRC Inactive State (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-233949 | (TP for BLCR TS38.473) Discussion on MBS reception by inactive state UE (Samsung) |  | other |  |
| R3-233980 | Enhancements to support Multicast reception by UEs in RRC_INACTIVE state (Qualcomm Incorporated) |  | discussion |  |
| R3-234090 | (TPs to MBS BL CRs) Multicast Reception for RRC_INACTIVE state UEs (Huawei, CBN) |  | other |  |
| R3-234183 | Discussion on multicast reception in RRC_INACTIVE (Lenovo) |  | discussion |  |
| R3-234246 | Discussion on Multicast over Inactive (CATT,CBN) |  | other |  |
| R3-234450 | Multicast Reception in RRC_INACTIVE state (CMCC) |  | discussion |  |
| 16. NR Sidelink Relay Enhancements WI WID [NR_SL_relay_enh-Core]: RP-223501 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 2 |  |  |  |  |
| ## 16.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233745 | (BLCR to 38.401) Introduction of NR SL relay enhancements (LG Electronics) |  | CR0285r4, TS 38.401 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233754 | (BLCR to 38.413) Support for NR Sidelink Relay Enhancements (Nokia, Nokia Shanghai Bell, Qualcomm, Ericsson, CMCC, ZTE, Samsung, LG Electronics, Huawei) |  | CR0928r5, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233763 | (BLCR to 38.423) Support NR Sidelink Relay Enhancements (Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, ZTE, Samsung, Huawei, CATT, LG Electronics, CMCC) |  | CR0967r5, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233769 | (BLCR to 38.473) Support for NR Sidelink Relay Enhancements (Huawei, CMCC, LGE, CATT, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE) |  | CR1123r5, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233787 | Support of SL relay enhancement in TS38.300 (CMCC, CATT, Samsung, China Telecom, Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, ZTE) |  | draftCR - Update the editable figure and change the version. Rev in R3-234577 Endorsed as BL CR unseen |  |
| 16.2. Support Relay and Remote UE Authorization Check SA2 progress Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3] RAN3 waits for RAN2 progress on E2E PC5 QoS split for U2U relay. WA: Support U2U relay in CU-DU split architecture, FFS on the enhancements NG-RAN receives the multi-path authorization from the AMF. The multi-path authorization can be added in the 5G ProSe Authorized IE if it is needed. Check progress in other WGs |  |  |  |  |
| R3-233716 | Reply LS on ProSe Authorization information related to UE-to-UE Relay operation (RAN2(ZTE)) |  | LS in noted |  |
| R3-233725 | Reply LS to SA2 on authorization for multi-path Scenario 2 (SA2(LGE)) |  | LS in noted |  |
| R3-233882 | Authorization for U2U relay (ZTE) |  | Other RAN3 needs to enhance “5G ProSe authorised” information in Xn/F1/NG to include authorization for L2 U2U relay operation. |  |
| 16.3. Support Service Continuity Enhancements RAN2 led Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]: Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”) Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”) Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”) Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”) Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations. Reuse the existing network procedures to support single-hop L2 U2N Relay in Rel-18. Source gNB decides to trigger path switching for the L2 U2N remote UE. Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch. RAN3 focuses on the XnAP and possible F1AP impacts to support the basic scenarios. Regarding the support of lossless data delivery during path switch, RAN3 would wait for RAN2’s progress first. For direct/indirect to indirect path switching, enhance Xn: HANDOVER REQUEST to include at least the Remote UE L2 ID and Relay UE L2 ID. For inter-gNB path switching scenarios, RAN3 should specify mechanisms to support service continuity for L2 U2N relays in NG based handovers as well after supporting service continuity for L2 U2N relays in Xn based handovers, If there is some conclusion from SA2, and then to support NG based HO. Source gNB selects the target path type (direct or indirect). Focus on the following two ways for the future discussion, - Way1: to go for Op1, and Op2 can be further discussed. - Way2: accept Op2, or at least as a compromise. No more discussion on Op3 in RAN3. During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message. At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message. For XnAP HANDOVER REQUEST message: - Add a new IE containing a list (up to 32) of candidate Relay UE IDs. - No need to introduce new IE for remote UE ID. - The UE Context information IE is needed for remote UE. Common Understanding is source gNB can initiate parallel Xn handover preparation to multiple target gNBs, and the XnAP HANDOVER REQUEST message sent to a target gNB only include candidate Relay UEs of same cell of the target gNB. So no need to have any restriction on how source gNB select candidate target Relay UE. RAN3 will not further discuss - source gNB provides the Measurement results for a list of candidate relay UEs to target gNB - target gNB page Relay UE to transition it to RRC CONNECTED. - target gNB can select a candidate relay UE not included in the list provided by source gNB. For NGAP, add a new IE containing a list (up to 32) of candidate Relay UE IDs in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE. For TS38.401, add the call flow for inter-CU direct to indirect path switch (inter-CU D2I). RAN3#120: During inter-gNB path switching, source gNB can signal the serving cell of the relay UE to target gNB via existing IE Target Cell Global ID. RAN3 do not pursue the two optimizations in R18: - The list of candidate Relay UEs is an ordered list, e.g. based on the Remote UE’s measurement report on candidate Relay UEs. - Target gNB include the selected target Relay UE in the XnAP HANDOVER REQUEST ACKNOWLEDGE message. Down-select between D4 and D5 in next meeting. The criticality of the candidate relay UE info list IE should be “reject” in the XnAP HANDOVER REQUEST message and the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE over NGAP. FFS whether move step 7 before step 5 in inter-gNB d2i/i2i path switch procedures. |  |  |  |  |
| R3-233902 | (TP for BLCR 38.413, 38.423) Inter-gNB mobility (Huawei) |  | other |  |
| R3-233909 | (TP for SL Relay BL CR to TS 38.423) Inter-gNB Service Continuity for L2 U2N Relay (Ericsson) |  | other |  |
| R3-234013 | (TP for TS38.300 BL CR) Discussion on Support Service Continuity Enhancements (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234026 | Another issue on DL lossless delivery (Samsung) |  | discussion |  |
| R3-234453 | Remaining open issues for service continuity enhancement (LG Electronics) |  | other |  |
| R3-234113 | (TP for SL relay 38.300, 38.413, 38.423 and 38.473) Discussion on Support Service Continuity Enhancements (CATT) |  | other |  |
| R3-234438 | Discussion on downlink lossless delivery (CMCC) |  | discussion |  |
| R3-233879 | Discussion on remaining issue for service continuity (NEC) |  | discussion |  |
| R3-234437 | (TP for SL relay to TS 38.300) Considerations on service continuity (CMCC, Ericsson, CATT, Huawei, China Telecom, NEC, Samsung) |  | discussion |  |
| R3-233998 | Discussion on service continuity and U2U authorization (China Telecommunication) |  | Discussion Late contribution |  |
| DL lossless data delivery: • Solution-D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based) • Solution-D5: Proactive Data forwarding from source gNB to target gNB Solution-D4: HW, ZTE Solution-D5: HW, NEC, CATT, E///,Nok, CMCC, LG, Samsung. NEC: D4 introduces unnecessary message. CATT: If D5 is workable, D4 is not needed. RAN3 agrees on proactive Data forwarding from source gNB to target gNB. Path Switch procedures: - check and align with Rel-17 and Rel-18 TS 38.401 BL CR. To align with Rel-17 and Rel-18 TS38.401 BL CR, update TS38.300 BL CR that RRCReconfiguration to Relay UE should be performed before the HANDOVER REQUEST ACKNOWLEDGE message. In inter-gNB d2i/i2i path switch procedures, the order of Step 5 and 7 is up to the gNB implementation. It is proposed to move Step 7 before Step 5 in the figures for inter-gNB d2i/i2i path switch procedures and add the NOTE “The order of Step 5 and 6 is up to the gNB implementation.” It is proposed to add a following NOTE for the i2d/i2i path switching procedure. - In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data may be acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB. The target gNB retransmits unsuccessfully delivered DL data to the L2 U2N Remote UE. CB: # SLRely1_DLlosslessdelivery - check all the TPs and merge. - capture the agreements with D5. - check and align with Rel-17 and Rel-18 TS 38.401 BL CR. - capture the open issues, and related with CU/DU and CP/UP split architecture. (moderator - LG) Summary of offline disc R3-234578 |  |  |  |  |
| 16.4. Multi-path Support RAN2 led Specify mechanisms to support the following multi-path scenarios [RAN2, RAN3]: A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2). Note 3A: The mechanisms to support scenario 1 and scenario 2 are specified based on the assumptions and restrictions agreed in study phase. Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline. Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress. From RAN3 perspective, multi-path scenario should be supported in Rel-18. Both intra-DU and inter-DU cases will be supported under the same gNB. RAN3 waits for the RAN2 progress on how to define control plane and user plane scenarios for multi-path support. RAN3 waits for the RAN2 progress on whether and how to define the Primary path in multi-path support. Addition of direct/indirect path are supported as follows: - Add direct path, after the establishment of the indirect path. - Add indirect path, after the establishment of the direct path. - This does not imply the exclusion of any other path addition possibility. RAN3 will study the signaling impact on the direct or indirect path change under the same gNB for a UE connected via multi-path. The other mobility scenarios can be further considered based on RAN2 decision. The following use cases are not supported in Rel-18. - Configure two indirect paths - More than two paths - Inter-gNB multi-path support For Scenario 1, the responsibility of gNB-CU and gNB-DU in Rel-17 SL relay can be reused as a baseline. For the responsibility of gNB-CU and gNB-DU in Scenario 2, the RAN3 waits for RAN2’s progress on protocol stack for Scenario 2. For the multi-path support, the gNB-CU takes the responsibility to decide the addition/modification/release of the path. For intra-DU and inter-DU cases, the UE Context Setup / Modification procedure can be reused to configure the 2nd path with possible enhancements. The details will be discussed based on RAN2 progress. The RAN3 waits for the RAN2 progress on whether the gNB-DU knows the path information of each configured path. The gNB-CU is responsible to determine the data split among two paths for a DRB for both intra-DU and inter-DU cases. For intra-DU case, two F1-U tunnels are setup between CU and DU for a split DRB. Previous RAN3 agreement is updated as follows: For Scenario 1, addition of direct/indirect path are supported as follows: Add direct path, after the establishment of the indirect path. Add indirect path, after the establishment of the direct path. For Scenario 2, addition of direct/indirect path are supported as follows: Add indirect path, after the establishment of the direct path. Whether to add direct path, after the establishment of the indirect path is pending to RAN2 decision. For Scenario 2, interface between UEs are non-3GPP defined. Therefore, in the UE context setup/modification procedure, the PC5 Relay RLC channel configurations are not needed for remote UE and relay UE. From RAN3 point of view, the Multi-path study phase is completed, and the Multi-path can move to normative work phase. Add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401. The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision. For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB. The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition. RAN3 further wait for RAN2 progress before removing two Editor’s notes in BL CR to TS 38.401. For intra-DU case, the gNB-CU should inform the gNB-DU about the path to be added, released or modified. The Relay UE L2 ID for indirect path addition and the PCell ID for direct path addition should be provided to the gNB-DU. Whether to inform the gNB-DU of other information needs to be further discussed. The radio bearer type and channels mapping to be added can be provided to the gNB-DU by Rel-17 U2N relay signaling design. Add the intra-DU path addition procedure in the BL CR to TS 38.401. For Scenario 2, the responsibility for gNB-CU and gNB-DU are defined as follows: - gNB-CU’s responsibility: - Remote UE and relay UE context maintenance - Remote UE bearer mapping - Relaying Uu RLC channel management - gNB-DU’s responsibility: - Determine the RLC/MAC/PHY Configuration for the Uu Relay RLC channels of relay UE - The responsibility for gNB-CU and gNB-DU can be further enhanced based on RAN2 progress. WA: For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE. RAN3#120: WA: For indirect path addition, the Remote UE Local ID IE and the T420-like timer are sent to the gNB-DU. The details of T420-like timer are pending to RAN2. RAN3 reuses the existing F1AP signaling to achieve the operations for ‘Path to be modified’ by means of modifying the configurations associated to the path(s). |  |  |  |  |
| R3-233712 | LS to RAN3 on mode 1 scheduling in inter-DU multi-path case (RAN2(NEC)) |  | LS in For Scenario-1, mode-1 scheduling for remote UE is supported at least for intra-DU case, with the SR/BSR and grant sent on the direct path; whether it is supported for inter-DU case is up to R3, but R2 do not intend to make specification changes to support this case, and for specification purposes RAN2 intend to model it as a single MAC entity at the UE. noted |  |
| R3-233880 | Discussion on mode 1 RA for U2N remote UE under multi-path relay (NEC, TCL, ZTE, OPPO, LG Electronics, CMCC, Samsung) |  | discussion |  |
| R3-233910 | (TP for SL Relay to TS 38.470) Multi-path for Sidelink Relay (Ericsson) |  | other |  |
| R3-234014 | discussion on the support for multi-path (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-233903 | (TP for BLCR 38.413, 38.423, 38.473) Multi-path relay and U2U relay (Huawei) |  | other |  |
| R3-233883 | Discussion on multi-path relay support (ZTE) |  | other |  |
| R3-234454 | Further consideration on U2U relay and multi-path support (LG Electronics) |  | other |  |
| R3-234027 | Remaining issues on multipath (Samsung) |  | discussion |  |
| R3-234114 | (TP for SL relay 38.401 and 38.473) Discussion on Multi-path Support for SL relay (CATT) |  | other |  |
| R3-234439 | (TP for SL relay to TS 38.401) Considerations on multi-path (CMCC) |  | discussion |  |
| R3-234196 | Draft BLCR to 38.470 for SL relay Enhancement (ZTE, CAICT, China Telecom, CATT, LG Electronics) |  | draftCR |  |
| R3-234008 | (TP to TS 38.473) Discussion on multi-path for sidelink relay (China Telecommunication) |  | Other Late contribution |  |
| WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay. E///: need more time to check offline. It should be the gNB-DU who has a direct link with Remote UE that handles the mode-1 resource allocation. RAN3 has no issue supporting mode-1 resource allocation for inter-DU scenarios and would continue evaluating the details. Same functionalities of gNB-CU and gNB-DU are applicable for Scenario-1. Agree to introduce MP functions for the gNB-DU to F1AP stage-2 specification. Proposal 1-1: Enhance F1AP to capture the RB duplication in multi-path. Proposal 1-2: RAN3 Study the three options on how to inform a gNB-DU on a specific path to be released, when both direct path and indirect path use same gNB-DU. - Option a: The path is identified by an ENUMERATED IE with value “direct” and “indirect”. - Option b: The direct path is identified by PCell ID, and indirect path is identified by Relay UE ID - Option c: The path is identified by a path ID Proposal 1-3: gNB-CU inform gNB-DU on how to send the RRC message via direct path, or indirect path, or duplication over both paths CB: # SLRelay2_MP - Introduce MP functions for the gNB-DU to F1AP stage-2 specification. - Discuss whether to reply RAN2 LS. - Check TP for authorization. - Capture all the open issues. (moderator - E///) Summary of offline disc R3-234579 |  |  |  |  |
| 17. NR NTN enhancements WI WID [NR_NTN_enh-Core]: RP-231484 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 2 |  |  |  |  |
| ## 17.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233751 | NGAP BLCR on NTN Functionality (Nokia, Nokia Shanghai Bell, Ericsson, Thales, ZTE, Omnispace, TTP, CATT, Hughes Network Systems, Huawei, Lockheed Martin, Intelsat, ESA, Samsung, Qualcomm Incorporated) |  | CR1008r1, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233784 | (BLCR to 38.300) Stage 2 BL CR for NR NTN (Ericsson, CATT, Thales, Huawei, Samsung, ZTE, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated) |  | draftCR Rev in R3-234508 |  |
| R3-233799 | XnAP BLCR on NTN Functionality (Huawei, Ericsson, Thales, ZTE, Omnispace, TTP, Nokia, Nokia Shanghai Bell, CATT, Hughes, EchoStar, CMCC) |  | CR0933r5, TS 38.423 v17.5.0, Rel-18, Cat. B |  |
| R3-233842 | R18 WI NR-NTN-enh work plan at RAN1, 2 and 3 (THALES) |  | Work Plan |  |
| ## 17.2. Support Mobility and Service Continuity Enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2, RAN3, RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3] In Rel-18, mobility enhancement based on NG and Xn can be discussed in WI based on technical issues to be solved Enhancements for the support of CHO over NG for NTN-NTN hand-over should be discussed in this WI. Time based CHO should be supported. The target gNB is able to uniquely identify the target cell based on the target cell information received from the source gNB. Start time, duration are added in the signaling of time-based CHO. The exchange of NTN Cell Coverage Stop Time between gNBs may be further discussed in future RAN3 meetings. There is no need to exchange the cell coverage stop time in the signaling of time-based CHO parameters. Agree to add time information for time-based CHO, which includes a start time T1 and time duration T2, in Xn Handover Request message, taking R3-225580 as the starting point. There is no need to exchange a ‘Hard or Soft Feeder link Switch over indication’ via XN Setup procedure and Configuration Update procedure. The earth moving cell scenario described in Section 3.1 of R3-226859 is valid. Turn WA to agreement: The Uu cell ID is used as target Cell ID in both NG and Xn handover signaling. Introducing time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact. Confirm to add the handover window start and duration IEs to the NGAP Source NG-RAN Node to Target NG-RAN Node Transparent Container IE. Confirm to enhance the early data forwarding with data discarding for NG HO. RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS38.413. RAN3#120: Change the WA to the agreement: Uu Cell ID should be used in Xn Setup and Configuration Update procedures. WA: Do not exchange multiple TACs over Xn for NTN. How to describe which TAC should be used in semantic description? When time-based trigger condition is used, the source NG-RAN node should consider the time indicated to the UE to decide when start the early data forwarding to the target NG-RAN node. |  |  |  |  |
| R3-233721 | LS on common signalling in (C)HO (RAN2(OPPO)) |  | LS in |  |
| R3-234358 | Consideration on support of common signalling in (C)HO (CATT,OPPO) |  | discussion |  |
| R3-234359 | Discussion on mobility leftover issues (CATT) |  | discussion |  |
| R3-234360 | (TP to BL CR for TS 38.423) Correction to time based CHO (CATT, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Huawei) |  | other |  |
| R3-234015 | (TP for TS38.300 BL CR) Discussion on Support Mobility and Service Continuity Enhancements (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234028 | Remaining issue on service continuity enhancement for NTN (Samsung) |  | discussion |  |
| R3-234057 | Further discussion on mobility issue for NR NTN (ZTE) |  | other |  |
| R3-234100 | Further discussion on multiple TACs (Huawei) |  | discussion |  |
| R3-234101 | Discussion on common signalling in CHO (Huawei) |  | other |  |
| R3-234160 | Common Signaling in (C)HO (Ericsson LM) |  | discussion |  |
| R3-234158 | Time Margin for CHO in NR NTN (Ericsson, Thales, ESA) |  | discussion |  |
| R3-233876 | Further discussion on TAC exchange over Xn for NR NTN (NEC) |  | discussion |  |
| R3-234370 | Xn enhancement for NR NTN mobility (NTT DOCOMO INC..) |  | discussion |  |
| R3-233987 | [Draft] Reply LS on Common Signaling in (C)HO (Qualcomm Incorporated) |  | LS out To: RAN2 CC: |  |
| R3-234058 | [DRAFT] Reply LS on common signalling in (C)HO (ZTE) |  | LS out To: RAN2 CC: |  |
| R3-234159 | Time Margin for CHO in NR NTN - XnAP Impact (Ericsson, ESA, Thales) |  | other |  |
| R3-234161 | [DRAFT] Reply LS on Common Signaling in (C)HO (Ericsson LM) |  | LS out To: RAN2 CC: |  |
| R3-233999 | Discussion on NTN Service Continuity Enhancements (China Telecommunication) |  | Discussion Late contribution |  |
| 17.3. Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4: Multiple satellite in view by the UE may be considered if time allows Note 5: The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6: The enhancements should take into account the mirror-image ambiguity Note 7: Network verified UE location is an optional UE feature The verification is performed in the CN. If the reported UE location is not correct, the CN will take necessary action and Rel-17 behavior can be kept as baseline. RAN3 wait for RAN1/2 progress on the specific position method to be used for verification. RAN3 is not affected by UE location reporting No additional RAN3 impact if UE location is not correct |  |  |  |  |
| R3-234102 | Further discussion on network verified UE location (Huawei, Ericsson, CATT, Samsung) |  | discussion |  |
| R3-233988 | Discussion on TRP Information for NR NTN (Qualcomm Incorporated) |  | discussion |  |
| R3-234016 | Discussion on UE location verification (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234361 | Consideration on OAM requirements for UE location verification (CATT,Ericsson, Huawei,Samsung) |  | discussion |  |
| R3-234103 | (TP to 38.300) OAM Requirements for UE Location Verification (Huawei, Ericsson, CATT) |  | other |  |
| R3-234124 | OAM Requirements for UE Location Verification (Ericsson, CATT, Huawei) |  | draftCR |  |
| R3-234125 | [DRAFT] Reply LS on Latency impact for NTN verified UE location (Ericsson, CATT, Huawei) |  | LS out To: SA2, RAN2, SA1 CC: RAN1, RAN |  |
| ## 18. IoT NTN Enhancements WI WID [IoT_NTN_enh-Core]: RP-231407 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 1 (was 2) |  |  |  |  |
| ## 18.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233771 | (BLCR to 36.300) IoT NTN enhancements (ZTE, Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson) |  | draftCR |  |
| R3-233773 | (BLCR) Support for IoT NTN enhancements (Nokia, Nokia Shanghai Bell, CATT, Thales, Ericsson, Huawei, ZTE) |  | CR1895r8, TS 36.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233774 | (BLCR) X2AP CR on IoT NTN Functionality (Huawei, CATT, Nokia, Nokia Shanghai Bell, Qualcomm, ZTE, CMCC, Ericsson, Samsung) |  | CR1734r5, TS 36.423 v17.5.0, Rel-18, Cat. B |  |
| ## 18.2. Support discontinuous coverage Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3]. There is no need to provide the ephemeris info over S1. Whether MME shall be aware of the coverage should be decided by SA2. Paging enhancement for power saving should wait for the progress of RAN2 or SA2. The mobility management enhancements for discontinuous coverage should pending to the progress in RAN2. The new cause value “Release due to discontinuous coverage” is applicable for the UE Context Release Request procedure. There is no need to enhance the existing S1AP procedure to support the tracking area reported by RAN before AN release. RAN3 relies on the fact that the procedure will be UE context release complete procedure without failure. The time-based CHO over X2 should be supported. The Uu cell ID is used as the target cell ID in both S1 and X2 handover signalling. There is no need to consider the 5GC for discontinuous coverage issue in Rel-18 IoT NTN WI. Regarding the duplicated issues, IoT NTN shall wait for the corresponding progress in NR NTN. RAN3#120: Uu Cell ID is used to be exchanged via X2 Setup and eNB Configuration Update procedure. |  |  |  |  |
| R3-234059 | Further discussion on discontinuous coverage issue for IoT NTN (ZTE) |  | other |  |
| R3-234200 | (TP for BL CR IoT NTN TS36.423) Correction for Time based CHO (CATT, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Huawei) |  | discussion |  |
| R3-234219 | (TP to BL CRs, 36.300, 36.423) Alignment with NR NTN (Huawei, Ericsson) |  | other |  |
| R3-234017 | (TP for BL CR 36.300 and 36.413) on support X2-CHO and S1-HO with time-based trigger condition (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234105 | Time-Based HO and IoT NTN - Stage 2 Impacts (Ericsson, CATT, ESA, Huawei) |  | other |  |
| R3-234106 | Time-Based HO for IoT NTN - S1AP Impacts (Ericsson, Huawei, CATT, ESA) |  | other |  |
| R3-234107 | Time Margin for CHO in IoT NTN - X2AP Impact (Ericsson, Inmarsat, ESA) |  | other |  |
| 19. NR support for UAV WI WID [NR_UAV-Core]: RP-230782 (target: RAN #102) [TU: 0, 0, 0.5] QUOTA: |  |  |  |  |
| ## 19.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233810 | Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO INC, Qualcomm Incorporated, Intel Corporation, Samsung, NEC, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT) |  | CR0618r11, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233811 | NR support for UAV over Xn (Huawei, China Unicom, China Telecom, CATT, Intel Corporation, ZTE, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell, Samsung) |  | CR0951r5, TS 38.423 v17.5.0, Rel-18, Cat. B |  |
| R3-233809 | (BLCR) Draft CR to 38.300 on NR support for UAV (Nokia, Nokia Shanghai Bell, Huawei, Intel Corporation, ZTE, Ericsson, Samsung, CATT, Qualcomm, Deutsche Telekom, NEC) |  | draftCR Withdrawn |  |
| ## 19.2. Support Subscription-based Aerial-UE Identification Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. Agreed to introduce Aerial UE Subscription Information IE over NGAP in INITIAL CONTEXT SETUP, UE CONTEXT MODIFICATION, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE messages. The Aerial UE Subscription Information IE is based on LTE format with codepoints ENUMERATED (allowed, not allowed, …) Introduce Aerial UE Subscription Information IE over XnAP. Whether additional codepoints are needed for Aerial UAV Subscription Information IE is subject to SA2 and RAN2 discussions. RAN3 will not initiate the discussion on Inter-RAT and NR-DC support. RAN3#120: Capture the UAV Flightpath Information IE in at least stage 3 TPs for NGAP and XnAP (TS 38.413 and TS 38.423). The UAV Flightpath Information IE shall be added into the at least following messages: NGAP HANDOVER REQUIRED/HANDOVER REQUEST (in Source NG-RAN Node to Target NG-RAN Node Transparent Container IE) XnAP HANDOVER REQUEST UAV Flightpath Information IE is OCT STRING and refers to RRC. This IE may be re-checked by RAN3 if RAN2 has further progress on this IE design, The discussion on A2X supporting is postponed until RAN3 has formal TU for UAV Work Item. Whether the UAV Flightpath Information IE shall be added into XnAP RETRIEVE UE CONTEXT RESPONSE message, to be continued in R18 UAV WI. |  |  |  |  |
| # 20. NR MT-SDT WI WID [NR_MT_SDT-Core]: RP-213583 (target: RAN #101) [TU: 0.5 (0.5)] QUOTA: 2 |  |  |  |  |
| ## 20.1. General Time plan, skeletons, BLs |  |  |  |  |
| R3-233742 | (BL CR to TS 37.483) Introduction of MT-SDT (Nokia, Nokia Shanghai Bell, ZTE, Ericsson, Intel Corporation, China Telecom, Huawei, Lenovo, LG Electronics) |  | CR0054r5, TS 37.483 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233744 | (BLCR to 38.401) Introduction on MT-SDT (Huawei, ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Intel Corporation, China Telecom, Lenovo, LG Electronics) |  | CR0284r4, TS 38.401 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233760 | (BLCR to 38.423) Introduction on MT-SDT (Ericsson, ZTE, Intel Corporation, Nokia, Nokia Shanghai Bell, China Telecom, Huawei, Lenovo) |  | CR1010r4, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233767 | (BLCR to 38.473) Introduction on MT-SDT (China Telecom, Intel Corporation, ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Huawei) |  | CR1140r5, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233778 | (BLCR to 37.480) Introduction on MT-SDT (LG Electronics, Nokia, Nokia Shanghai Bell) |  | CR0003r1, TS 37.480 v17.1.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233783 | (BLCR to 38.300) Introduction on MT-SDT (ZTE, CATT, Ericsson, China Mobile, China Telecom, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Google, LG Electronics) |  | draftCR Endorsed as BL CR |  |
| R3-233796 | (BLCR to 38.420) Introduction on MT-SDT (Lenovo, CATT, ZTE, Nokia, Nokia Shanghai Bell) |  | CR0034r3, TS 38.420 v17.2.0, Rel-18, Cat. B Endorsed as BL CR |  |
| ## 20.2. Support for Paging-Triggered SDT Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI. MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling. Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging. Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. Agree to reusing existing IE (i.e., SDT Support Request) within the XnAP Retrieve Context Request message when the UE resumes for MT-SDT, and there is no RAN3 standard impact. Include an MT-SDT Information Request IE as optional IE in the E1AP: BEARER CONTEXT SETUP REQUEST/ MODIFICATION message to request the report of MT-SDT Information for bearers configured as SDT bearers. For the issue on DL non-SDT data arrives during the ongoing MT-SDT procedure, RAN3 waits for RAN2 on whether any signaling enhancements are needed. RAN3 acknowledges the case that DL non-SDT data arrives at the last serving gNB following the MT-SDT paging procedure before receiving UE Context Retrieval Request message. In XnAP: RAN Paging message includes the MT-SDT Data Size IE. The encoding and the name of MT-SDT information IE in E1AP DL DATA NOTIFICATION message include MT-SDT Data Size IE (Mandatory, INTEGER (1…96000, …). The encoding and the name of MT-SDT information IE in F1AP: Paging message include MT-SDT indicator IE (Mandatory). RAN3#120: In E1AP: MT-SDT indicator IE within MT-SDT information IE is not needed. In XnAP: MT-SDT data size calculation includes total of both SDT signalling and SDT user plane data. Try to capture it into the TP. In XnAP: Both MT-SDT indicator IE and MT-SDT Data Size IE are “Mandatory”. In F1AP: MT-SDT indicator IE is “Mandatory”. In F1AP: gNB-CU makes the MT-SDT decision, gNB-DU shall follow the decision. When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method. When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method. FFS on how to calculate MT-SDT Data Size for SDT DL data packets. Complete the stage3 details |  |  |  |  |
| R3-233819 | Discussion on MT-SDT (ZTE, Qualcomm Incorporated) |  | discussion |  |
| R3-233820 | (TP to 37.483, 38.401) Leftove issues for MT-SDT (ZTE) |  | other |  |
| R3-233832 | Remaining Issues on Supporting of MT-SDT (China Telecom) |  | discussion |  |
| R3-233854 | Completion of MT-SDT Open Points (Nokia, Nokia Shanghai Bell, Orange) |  | discussion |  |
| R3-233855 | (TP for TS 37.483) Completion of MT-SDT Open Points (Nokia, Nokia Shanghai Bell, Orange) |  | other |  |
| R3-233976 | Signaling enhancements to enable MT-SDT for RRC_INACTIVE UEs. (Qualcomm Incorporated) |  | discussion |  |
| R3-233977 | Draft LS on MT-SDT signalling optimization for partial context transfer. (Qualcomm Incorporated) |  | LS out To: RAN2 CC: |  |
| R3-234087 | (TP to TS 37.480 BL CR) Consideration on remaining issues for MT-SDT (Huawei) |  | other |  |
| R3-234088 | (TP to TS 38.401 BL CR) non-SDT or oversize SDT data arrival (Huawei, Qualcomm Incorporated) |  | other |  |
| R3-234182 | Support for Paging-Triggered SDT (Lenovo) |  | discussion |  |
| R3-234265 | Discussion on MT-SDT Open issues (Ericsson, Qualcomm Inc, ZTE, Lenovo) |  | discussion |  |
| R3-234266 | (TP to TS 37.483 BL CR) MT-SDT Data Size configuration (Ericsson, Qualcomm Inc, ZTE, Lenovo) |  | other |  |
| R3-234267 | (TP to TS 38.401 BL CR): Correction of MT-SDT DL Data Notification (Ericsson) |  | other |  |
| R3-234268 | (TP to TS 38.300 BL CR): Miscellaneous fixes (Ericsson) |  | other |  |
| R3-234351 | Discussions for open issues related to DL notification and data volume (LG Electronics Inc.) |  | discussion |  |
| R3-234352 | Necessary corrections for stage-2 BL CRs (TPs for NR_MT_SDT BL CR for TS 38.300 and TS 38.401) (LG Electronics Inc.) |  | discussion |  |
| R3-234362 | Discussion on leftover issues for MT-SDT (CATT) |  | discussion |  |
| R3-234363 | (TP to BL CR for TS 38.401) Handling of non-SDT or oversized SDT data (CATT,China Telecom) |  | other |  |
| R3-234489 | Remaining issues on MT-SDT (Samsung) |  | discussion |  |
| When a DL non-SDT data is coming, the gNB-CU-UP shall send DL DATA NOTIFICATION message, adding an explicit indicator to indicate that DL non-SDT data arrives. To be continued... When a large size of DL DT data is coming, the gNB-CU-UP shall send DL DATA NOTIFICATION message to indicate that large size of DL SDT data arrives. FFS on reusing the existing IE or new one. CATT, Nok, CT: Reusing the current DL DATA NOTIFICATION message is enough. What’s the behavior in the receiving node towards different cases？ HW: Agree p1 QC: Whether to send QFI or not? ZTE: In R18, the indication can be used to differentiate the non-SDT data and large size SDT data Lenovo: R18 is different with R17. Nok: If this is non-SDT data, then UE needs to be sent to active mode, while for large size SDT data, UE may need to be sent to active mode. An explicit indicator is needed, whether it is used to indicate non-SDT data and large size SDT data. DL SDT Data Size threshold shall be introduced in E1: Bearer Context Setup/ Modification message. CATT, CT: It’s not necessary to be sent, which can be configured by OAM, whether the threshold is node level or UE level E///: In the multiple connected UP case, OAM based solution seems not feasible Nok: Only CU-CP knows how to configure the threshold per UE, which is related with radio condition Proposal 4: When deciding to stop the ongoing SDT procedure and to move UE into RRC inactive mode, the gNB-CU-CP shall send Bearer Context Modification message, using a new codepoint (e.g., SuspendforSDT) in Bearer Context Status Change IE to suspend SDT DRB. Proposal 5: When deciding to stop the ongoing SDT procedure and to move UE into RRC connected mode, the gNB-CU-CP shall send Bearer Context Modification message, reusing existing codepoint (resume) in Bearer Context Status Change IE to resume all DRB. (TP to 38.401) Introduction on MT-SDT in R3-234583 (TP to 37.480) Introduction on MT-SDT in R3-234584 (TP to 37.483) Introduction on MT-SDT in R3-234585 (TP to 38.401) Leftover issues for MT-SDT in R3-234586 (TP to 38.300) Leftover issues for MT-SDT in R3-234587 (TP to 38.483) Leftover issues for MT-SDT in R3-234588 [Draft] LS on MT-SDT signalling optimization for partial context transfer in R3-234589 CB: # R18SDT_Solution - Continue the discussion on open issues left - Provide TPs based on the agreements (moderator - ZTE) Summary of offline disc R3-234534 |  |  |  |  |
| ## 21. NR Redcap Enhancement WI WID [NR_redcap_enh-Core]: RP-223544 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 2 |  |  |  |  |
| ## 21.1. General ## Time plan, skeletons, BL CRs |  |  |  |  |
| R3-233753 | (BLCR to 38.413) Introduction of NR Redcap Enhancement (Huawei, Nokia, Nokia Shanghai Bell, ZTE, Ericsson) |  | CR0989r2, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233786 | (BL CR to 38.300) introduction of NR Redcap Enhancement (Nokia, Nokia Shanghai Bell, CATT, Ericsson, ZTE) |  | DraftCR Endorsed as BL CR |  |
| R3-233793 | (BLCR to 38.410) Introduction of NR Redcap Enhancement (CATT, Nokia, Nokia Shanghai Bell, ZTE) |  | CR0044r2, TS 38.410 v17.1.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233800 | (BLCR to 38.423) Introduction of RedCap enhancement (Ericsson, Nokia, Nokia Shanghai Bell, Huawei, ZTE) |  | CR1052r1, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233808 | (BLCR to 38.473) Introduction on NR Redcap enhancement (ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Huawei) |  | CR1169r1, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-234271 | WI work plan for Rel-18 RedCap (Ericsson (Rapporteur)) |  | Work Plan noted |  |
| 21.2. Support Enhanced eDRX in RRC_INACTIVE Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Introduce a new CN-based MT communication handling IE ENUMERATED (Supported,…) in the Core Network Assistance Information for RRC INACTIVE IE in NGAP Introduce a new class 1 MT Communication Handling procedure in NGAP for RAN requesting CN to perform data buffering and for notifying of UE RRC state transition. The procedure contains the following messages: - MT COMMUNICATION HANDLING REQUEST - MT COMMUNICATION HANDLING RESPONSE - MT COMMUNICATION HANDLING FAILURE The MT COMMUNICATION HANDLING REQUEST message contains: - AMF UE NGAP ID (M) - RAN UE NGAP ID (M) - FFS on RRC state (M) as defined in 9.3.1.92 - NR Paging eDRX Cycle for RRC INACTIVE (conditionally present if RRC state is set to ‘inactive’) encoded as ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …). - FFS on any other information (pending on RAN2/SA2 progress or SDT) Introduce a new DL DATA NOTIFICATION class 2 message for AMF requesting RAN Paging Extend the NR Paging eDRX Information for RRC INACTIVE IE XnAP 9.2.3.162: - Add new codepoints in the NR Paging eDRX Cycle Inactive IE: ENUMERATED (hfquarter, hfhalf, hf1, …,hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024) - add the NR Paging Time Window IE ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, - s32,…) Extend the NR Paging eDRX Information for RRC INACTIVE IE F1AP 9.3.1.259: - add new codepoints in the NR Paging eDRX Cycle Inactive IE: ENUMERATED (hfquarter, hfhalf, hf1, …,hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024) - add the NR Paging Time Window IE ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32,…) RAN3#120: RAN3 agrees that AMF sends the assistance information to NG-RAN node to decide the paging priority. RRC State indication is not needed in the MT COMMUNICATION HANDLING REQUEST message when indicating of UE connection resume. FFS on signalling a UE Reachability indication to AMF Add a CHOICE structure in the MT COMMUNICATION HANDLING REQUEST: - Activate HLCOM - NR RAN Paging eDRX Cycle for RRC INACTIVE (M) - NR RAN PTW length (M) (FFS if should be always provided by the RAN) - NR RAN PTW start H_SFN (FFS on the IE details) - Deactivate HLCOM - Deactivate HLCOM indication PPI, ARP, PDU session ID and QFI are included in the DL DATA NOTIFICATION message. FFS on 5QI. Send LS to SA2 to ask if feasible to provide DL Data size to NG-RAN during the DL DATA NOTIFICATION for the purpose of MT-SDT paging decision. - Details to be further discussed. |  |  |  |  |
| R3-233726 | LS reply to RAN3 progress on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502 (SA2(Huawei)) |  | LS in noted |  |
| R3-234272 | (TP to TS 38.413) Correction on DL Data Notification procedure name (Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Huawei, Qualcomm Inc., CATT) |  | Other Rev in R3-234564 |  |
| R3-234273 | (TPs to TSes 38.413/38.423/38.473 eRedCap BL CRs) Addition of long eDRX Paging Information for RRC_INACTIVE (Ericsson) |  | other |  |
| R3-234274 | Discussion on eRedCap Indication with TPs and LS to SA2 (Ericsson) |  | other |  |
| R3-233821 | Discussion on NR Redcap enhancement (ZTE) |  | discussion |  |
| R3-233822 | (TP to 38.413, 38.423, 38.473, 38.300) Introduction on NR Redcap (ZTE) |  | Other Rev in R3-234567 |  |
| R3-233856 | (TP for TS 38.413 and TS 38.410) Support of Extended eDRX for RRC Inactive over NG (Nokia, Nokia Shanghai Bell) |  | Other Rev in R3-234566 |  |
| R3-233857 | (TP for TS 38.470, TS 38.473 and TS 38.423) Support for R18 eRedcap (Nokia, Nokia Shanghai Bell) |  | Other Rev in R3-234572 |  |
| R3-233985 | Discussion on RRC Inactive with long eDRX for RedCap (Qualcomm Incorporated) |  | Discussion Rev in R3-234568 |  |
| R3-234115 | (TP for 38.413 and 38.410 for eRedCap) Discussion on MT Communication Handling (CATT) |  | Other Rev in R3-234565 |  |
| R3-234116 | (TP for eRedCap for 38.413, 38.423, 38.473) Discussion on Rel-18 long eDRX for RRC_INACTIVE (CATT) |  | Other Rev in R3-234570 |  |
| R3-234137 | (TP to TS 38.413 and 38.423) Further discussion on long eDRX support for RRC_INACTIVE UE (Huawei) |  | Other Rev in R3-234569 |  |
| R3-234138 | (TP to TS 38.473) Support of R18 eRedCap UE access (Huawei) |  | other |  |
| R3-234139 | (TP to TS 38.423) Support of R18 eRedCap UE access (Huawei, China Unicom, China Telecommunication) |  | other |  |
| R3-234140 | [Draft] Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502 (Huawei) |  | LS out To: SA2 CC: RAN2, CT4 Rev in R3-234571 |  |
| The following issues to be discussed offline: - Rename the DL Data Notification procedure as RAN Paging Request? - eDRX alignment with RAN2 agreement, and other issues in NGAP BL CR - eRedcap barring indication CB: # R18Redcap - Discuss the open issues above - Provide TPs based on agreements (moderator - E///) Summary of offline disc R3-234535 |  |  |  |  |
| ## 22. NR Network-Controlled Repeaters WI WID [NR_netcon_repeater-Core]: RP-230175 (target: RAN #102) QUOTA: |  |  |  |  |
| ## 22.1. General Time plan, skeletons, BL CRs |  |  |  |  |
| 22.2. Support Network-Controlled Repeater Management Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. The NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface. The discussion on RAN impact on validation function is pending to SA3 reply LS. gNB-CU knows whether the connected gNB-DU supports NCR based on OAM configuration. Down selection on all solutions which takes the feedback from SA3 and SA5 into account can be discussed in next RAN3 meeting. The NCR-OAM connectivity requirement should be supported; further details can be discussed. Exclude the solution 2. OAM-NCR connectivity can be provided via PDU session. gNB-DU needs to know the authorization status of NCR. Take Solution 3 as the basis for NCR management. The NCR may be configured with a list of allowed and/or forbidden cells. RAN validation is not supported in R18. The NG-RAN node selects an appropriate AMF for a NCR accesses to the network with the assumption that the NCR indication is received from NCR in MSG5. The gNB-CU shall indicate the NCR authorization info explicitly to gNB-DU once a NCR device is authorized by UE context setup request message and UE context modification request message. RAN3 has completed the work in RAN3. |  |  |  |  |
| ## 23. NR Positioning WI WID [NR_pos_enh2-Core]: RP-231460 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 2 |  |  |  |  |
| ## 23.1. General Time plan, skeletons |  |  |  |  |
| R3-233752 | Support of NR Positioning Enhancements (ZTE, CATT, Huawei, Nokia, Nokia Shanghai Bell, Ericsson) |  | CR0991r2, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233761 | Support of NR Positioning Enhancements (Huawei, CATT, ZTE, Nokia, Nokia Shanghai Bell, Ericsson) |  | CR1061r2, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233768 | Support of NR Positioning Enhancements (Ericsson, CATT, Huawei, ZTE, Nokia, Nokia Shanghai Bell) |  | CR1180r2, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-234590 | Summary of Positioning offline discussion (CATT) |  | Discussion |  |
| ## 23.2. Support Enhancements on NR Positioning Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4] Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3]. Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104. Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. Define extensions of signalling, protocol, and procedure for NR positioning enhancement, as needed for the above objectives. [RAN3] |  |  |  |  |
| 23.2.1. Sidelink Positioning SL Positioning/Ranging authorization is provided over in the NGAP, XnAP and F1AP, in the following messages: NG: - INITIAL CONTEXT SETUP REQUEST - UE CONTEXT MODIFICATION REQUEST - HANDOVER REQUEST - PATH SWITCH REQUEST ACKNOWLEDGE Xn: - HANDOVER REQUEST - RETRIEVE UE CONTEXT RESPONSE F1: - UE CONTEXT SETUP REQUEST - UE CONTEXT MODIFICATION REQUEST RAN3#120: RSPP/SLPP transport QoS parameters shall be provided from AMF to the NG-RAN for scheduled resource allocation mode resource management. No need to provide the Ranging/SL Positioning QoS parameters from AMF to NG-RAN. Authorization information for UE Type/Role wait the progress in other group ## SL-PRS configuration from LMF wait the progress in other group |  |  |  |  |
| R3-234032 | Discussion on sidelink positioning and others (Samsung) |  | discussion |  |
| R3-234254 | Discussion on sidelink positioning (Xiaomi) |  | discussion |  |
| R3-234409 | (TP to 38.413, 38.423, 38.473,38.455) Discussion on Sidelink positioning (ZTE) |  | other |  |
| R3-234451 | Discussion on NR SL Positioning (CMCC) |  | Discussion Move to 23.2.1 |  |
| R3-234269 | Discussion on RAN3 impacts to support SL Positioning, CPP and other topics with TP for CPP support (Ericsson) |  | discussion |  |
| R3-234364 | Discussion on SL Positioning BW aggregation and CPP (CATT) |  | discussion |  |
| R3-234217 | Discussion on Carrier Phase Positioning and Sidelink Positioning (Huawei) |  | Other Move to 23.2.1 Rev in R3-234496 |  |
| R3-234327 | (TP for TS 38.455 BL CR) Positioning accuracy enhancements (Nokia, Nokia Shanghai Bell) |  | Other Move to 23.2.1 |  |
| R3-234256 | (TP for BL CR to TS 38.455) Support of SRS bandwidth aggregation (Xiaomi, Samsung, ZTE) |  | Other Move to 23.2.1 |  |
| Whether and how to include the UE types in the Ranging/Sidelink Positioning authorized information. RAN3 impact on SL-PRS allocation need more progress from RAN1/RAN2. |  |  |  |  |
| 23.2.2. LPHAP RAN3#120: LPHAP: Call flow with details on how to determine SRS validating area and which node allocates SRS resources in validating area. |  |  |  |  |
| R3-233714 | LS on LPHAP (RAN2(Huawei)) |  | LS in Move to 23.2.2 |  |
| R3-234218 | Discussion on LPHAP (Huawei) |  | other |  |
| R3-234255 | Discussion on LPHAP positioning enhancement. (Xiaomi) |  | discussion |  |
| R3-234270 | Discussion on RAN3 impacts to support LPHAP with TP (Ericsson) |  | discussion |  |
| R3-234285 | Enhancements for LPHAP (Qualcomm Incorporated) |  | discussion |  |
| R3-234326 | Coordination of SRS resources within a validity area (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234033 | Discussion on LPHAP (Samsung) |  | discussion |  |
| R3-234467 | (TP to 38.455, 38.473)Discussion on LPHAP impacts and SRS bandwith aggregation (ZTE) |  | other |  |
| R3-234365 | (TP to BL CR for TS 38.305) Support of cross-cell SRS configuration (CATT) |  | other |  |
| R3-234468 | Discussion on area-specific SRS configuration (vivo) |  | discussion |  |
| WA: LMF provides the validity area as a list of cells, and SRS recommendation to the serving gNB. The serving gNB replies with a single SRS configuration (as in legacy positioning information exchange procedure). Xiaomi: concerned about the list of cells and SRS information. The relation between them is not clear. The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information. ZTE: does it mean we reuse the existing IE for SRS configuration? FFS on whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF. |  |  |  |  |
| 23.2.3. Others No consensus on the WA made by RAN2 on LMF based Integrity and RAN3 involvement. Check progress in other WGs, identify RAN3 impact on the related topics if any. |  |  |  |  |
| R3-233706 | LS reply on the RAT-dependent positioning integrity (RAN1(Interdigital)) |  | LS in Move to 23.2.3 |  |
| R3-233722 | LS on reporting granularity for timing related positioning measurements (RAN4(Huawei)) |  | LS in Move to 23.2.3 |  |
| R3-234286 | Integrity of NR Positioning Technologies (Qualcomm Incorporated) |  | discussion |  |
| Agreed to work on the NRPPa and F1 TPs to support the additional reporting granularity corresponding to k=-1 and k=-2 - New TP for TS38455 for NRPPa – R3-234596 - New TP for TS38.473 – R3-234597 - Add a note in the TPs indicating that the values of k and the value range of the granularity factor are FFS Carrier Phase Positioning: Work on the TPs for NRPPa and F1AP to capture UL RSCP and SRS time window(s) related things, ASN.1 could be provided later, FFS should be added for the IE details. New TP for TS38.455 for NRPPa – R3-234602 New TP for TS38.473 – R3-234603 SRS Bandwidth Aggregation: Work on TPs for NRPPa and F1AP, make FFS to the new IEs. New TP to TS38.455 for NRPPa – R3-234604 New TP for TS38.473 – R3-234605 Whether Positioning activation/deactivation procedure should be involved to be further discussed the next meeting. PRS bandwidth aggregation: More progress of RAN1/ RAN2 is expected for PRS band aggregation from RAN3 perspective. CB: # R18Positioning - Work on the TPs allocated online (moderator - CATT) New TP for TS38455 for NRPPa in R3-234596 New TP for TS38.473 in R3-234597 New TP for TS38.455 for NRPPa in R3-234602 New TP for TS38.473 in R3-234603 New TP to TS38.455 for NRPPa in R3-234604 New TP for TS38.473 in R3-234605 |  |  |  |  |
| 24. NR Network Energy Savings WI WID [Netw_Energy_NR-Core]: RP-230566 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 2 |  |  |  |  |
| ## 24.1. General # Time plan, skeletons |  |  |  |  |
| R3-233798 | (BLCR to 38.423) Network energy saving techniques (Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, CATT, Intel) |  | CR1018r4, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233804 | (BLCR to 38.473) Introduction of Network Energy Saving (Ericsson, Huawei, Samsung, ZTE, CATT, Intel) |  | CR1129r6, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233990 | WI Work plan for R18 network energy savings (Huawei) |  | Work Plan noted |  |
| 24.2. Support Network Energy Savings Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3] Support beam level activation over Xn and F1. Inter-node beam activation: For inter-node beam activation, the XnAP CELL ACTIVATION procedure, and the F1AP GNB-CU CONFIGURATION UPDATE procedure are reused. - Over Xn interface, the CELL ACTIVATION REQUEST message may include the SSB beam list that is requested to be activated, and the CELL ACTIVATION RESPONSE message may include SSB beam list that are activated. When the receiving NG-RAN node cannot activate any of the SSB beams, it should respond with the CELL ACTIVATION FAILURE message with an appropriate cause value. - Over F1 interface, the GNB-CU CONFIGURATION UPDATE message may include the SSB beam list that is requested to be activated, and the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE may include SSB beam list that are activated. In case the gNB-DU cannot activate any of the requested SSB beams, it should respond with the GNB-CU CONFIGURATION UPDATE FAILURE message with an appropriate cause value. Enhancements on restricting paging in a limited area: It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique. The paging enhancement technique is applicable for UEs in RRC inactive state. Introduce the recommended SSB beam list in the F1AP paging message. Introduce a list of last few served SSB beam s/recommended SSB beam list as paging assistance information to the gNB-CU in UE CONTEXT RELEASE COMPLETE message over F1AP. Send a LS to RAN2 asking feedback containing the agreements for RRC inactive UE, and to SA2 asking feedback for RRC idle UE. The above agreements can be revisited based on reply from RAN2. RAN3#120: Cell DTX/DRX: WA: Support the exchange of the Cell DTX/DRX configuration over Xn. The detail infor of the Cell DTX/DRX configuration is pending to RAN1 and RAN2. Resolve the FFS(es) in the BLCR including the criticality of the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message, the maximum number of the SSB areas etc. Check progress in other WGs. |  |  |  |  |
| R3-233708 | Reply LS on Cell DTX/DRX activation/deactivation (RAN1(Huawei/Intel)) |  | LS in noted |  |
| R3-233729 | Reply LS on the enhancements to restricting paging in a limited area (SA2(Qualcomm)) |  | LS in noted |  |
| R3-233992 | (TP to Netw_Energy_NR BLCR for TS 38.473, 38.413, 38.470 and 38.300) Network energy saving techniques (Huawei) |  | other |  |
| R3-234426 | left issues on network energy saving (ZTE) |  | discussion |  |
| R3-234134 | (TP to TS 38.423) Support of network energy saving techniques (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-233930 | Inter-node beam activation and paging enhancements (Qualcomm Incorporated) |  | discussion |  |
| R3-233944 | Discussion on network energy saving (Samsung) |  | discussion |  |
| R3-233945 | Introduction of Network Energy Saving for Paging IDLE UE (Samsung) |  | CR1013r, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233991 | (TP to Netw_Energy_NR BLCR for TS 38.423) Network energy saving techniques (Huawei) |  | other |  |
| R3-234135 | (TP to TS 38.473) Beam deactivation decision and signalling for energy saving (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234193 | Discussion on NES related issues (Lenovo) |  | discussion |  |
| R3-234308 | Discussion on Cell DTX/DRX for NES (CATT) |  | discussion |  |
| R3-234309 | (TPs to TS38.423/TS38.473 BL CR) Discussion on new cause and paging enhancement for NES (CATT) |  | other |  |
| R3-234319 | Introduction of Network Energy Saving (Ericsson) |  | discussion |  |
| R3-234320 | Text Proposal on F1AP: Introduction of Network Energy Saving (Ericsson) |  | other |  |
| R3-234427 | TPs to BL CRs for network energy saving (ZTE) |  | other |  |
| Paging enhancements: To support CN paging enhancement, the recommended SSB list for Paging could be transferred over NG in the UE context release message? Work on reply LS to SA2. QC, Nok: RAN2 saw issues, wait for RAN2 progress. ZTE, SS: Can go for WA. - Cause value for SSB activation failure: Introduce a new cause value “SSB not Available” or more specifically “SSB activation not available” in XnAP and F1AP to indicate that none of the requested SSB beam(s) can be successfully activated? - SS: Prefer to use “SSB activation not available” - QC: It is not clear SSB activation is failed E///: Prefer SSB not Available Introduce a new cause value “SSB not Available” in XnAP and F1AP to indicate that none of the requested SSB beam(s) can be successfully activated. CB: # R18ES - Work on the reply LS to SA2 - Check RAN2 progress on paging enhancements - Capture agreements to TP (moderator - HW) Summary of offline disc R3-234614 |  |  |  |  |
| 25. XR Enhancements for NR WID [NR_XR_enh-Core]: RP-230786 (target: RAN #102) [TU: 0.5 (0.5, 0.5, 0.5)] QUOTA: 3 |  |  |  |  |
| ## 25.1. General ## Time plan, skeletons |  |  |  |  |
| R3-234018 | Work Plan for Rel-18 on XR Enhancements for NR (Nokia, Qualcomm, Ericsson) |  | Work Plan noted |  |
| Endorse the below BL CR assignment: BL CR owner. 38.300 (RAN3 part); Qualcomm. 38.413; Nokia. 38.415; ZTE. 38.423; Ericsson. 38.425; CMCC. 38.473; Huawei. 37.483; Samsung. The following BL CRs will be assigned after RAN3 agree CR is needed for the related specifications: BL CR owner. 38.401; China Telecom. 37.340; Lenovo. |
| ## 25.2. Support Enhancements on NR XR Discard operation of PDU Sets for DL and UL (RAN2, RAN3) Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3); |  |  |  |  |
| R3-233731 | LS reply on TSCAI for XR (SA2(VIVO)) |  | LS in noted |  |
| R3-233732 | Reply LS on the N6 PDU Set Identification (SA2(OPPO)) |  | LS in noted |  |
| R3-233733 | Reply LS on RAN information exposure for XRM (SA2(China mobile)) |  | LS in noted |  |
| R3-233734 | LS Reply on Design of RTP Header Extension for PDU Set handling (SA2(Huawei)) |  | LS in noted |  |
| R3-233735 | Non-homogeneous deployment of PDU Set based handling (SA2(Qualcomm)) |  | LS in noted |  |
| R3-234019 | Discussion on support for NR XR (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234021 | Discussion on multiple LSs (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234482 | (TP to TS 38.413) Discussion on support of XR enhancements (ZTE) |  | other |  |
| R3-234154 | (TP for NR_XR_enh BL CRs) Discussion on the enhancement for XR awareness (Huawei, Qualcomm Inc.) |  | other |  |
| R3-234153 | Discussion on discard operation for XR (Huawei, Qualcomm Inc.) |  | discussion |  |
| R3-234155 | (TP for NR_XR_enh BL CR of TS 38.413) Discussion on incoming LS from other WGs (Huawei, Qualcomm Inc.) |  | other |  |
| R3-234275 | Discussion on XRM support (Ericsson) |  | discussion |  |
| R3-234302 | Support for L4S in NG-RAN (Ericsson, Deutsche Telekom) |  | other |  |
| R3-234162 | Discussion on PDU Set QoS Parameters and PDU Set Information (Lenovo) |  | discussion |  |
| R3-234163 | Discussion on PDU Set based Discard (Lenovo) |  | discussion |  |
| R3-234164 | Discussion on ECN marking for L4S (Lenovo) |  | discussion |  |
| R3-233950 | Discussion on the support of enhancement for NR XR (Samsung) |  | discussion |  |
| R3-234119 | Discussion on Support Enhancements on NR XR (CATT) |  | discussion |  |
| R3-234120 | Discussion on XR LSin from SA2 (CATT) |  | discussion |  |
| R3-234257 | Discussion on enhancement for XR (Xiaomi) |  | discussion |  |
| R3-234406 | Disucssion on Support of XR Enhancement in Split Architecture (China Telecom) |  | discussion |  |
| R3-234452 | Discussion on non-homogeneous deployment of PDU Set based handling (CMCC) |  | discussion |  |
| R3-233951 | [TP for BLCR TS37.483] Introducing enhancement for NR XR (Samsung) |  | other |  |
| R3-233952 | [TP for BLCR TS38.413, TS38.423] Introducing enhancement for NR XR mobility (Samsung) |  | other |  |
| R3-233981 | TP for Stage-2 TS 38.300 for XR Enhancements (Qualcomm Incorporated) |  | other |  |
| R3-234020 | (TP for TS38.413) Support for NR XR (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234121 | (TP for XR for 38.413, 37.483 and 38.473) Support Enhancements on XR awareness (CATT) |  | discussion |  |
| R3-234258 | (TPs for TS 38.300 and TS 38.413) Non-homogenous support of PDU set based handling (Xiaomi) |  | other |  |
| R3-234276 | Introduction of XRM enhancement (Ericsson) |  | CR1089r, TS 38.423 v17.5.0, Rel-18, Cat. B |  |
| R3-234408 | (BLCR to 38.401) Introduction of XR enhancements (China Telecom) |  | CR0307r, TS 38.401 v17.5.0, Rel-18, Cat. B |  |
| R3-234483 | (XR BL CR to TS 38.415) Introduction of XR enhancements (ZTE) |  | CR0036r, TS 38.415 v17.0.0, Rel-18, Cat. B |  |
| R3-234484 | (TP to TS 38.423) Support of XR enhancements (ZTE) |  | other |  |
| R3-234407 | BLCR to 38.401 on support of XR Enhancements (China Telecom) |  | draftCR withdrawn |  |
| PDU set information handling: Add the PDU Set QoS Parameters in the QoS Flow Level QoS Parameters IE over NGAP, XnAP, F1AP and E1AP. The PDU Set QoS parameters includes: - PDU Set Delay Budget (PSDB). - PDU Set Error Rate (PSER). - PDU Set Integrated Handling Information (PSIHI). Xiaomi: All the parameters need to be transmitted over all interfaces? QC, Lenovo, HW, ZTE: All the parameters should be sent. For NGAP, add the feedback to inform the SMF that a QoS flow is successfully established with PDU Set based QoS Handling. For F1AP, add the feedback to inform the gNB-CU that a QoS flow is successfully established with PDU Set based QoS Handling. Target NG-RAN node provides to SMF with an indication of whether target NG-RAN node supports PDU Set based handling in the Path Switch Request Transfer IE. Lenovo: There are different cases, bullet3 refer to the case that from non-support NG-RAN node to support NG-RAN node. E///: Using RACS similar solution rather than indication. Nok: Start with initial setup case, and mobility case can be discussed later. During the PDU session setup/modification procedure, NG-RAN node needs to provide the feedback whether the QoS flow with PDU set QoS parameters is setup successfully. ZTE: Agree with Nok. RACS can be further checked. Continue to discuss the solution based on the possible cases as below: Case1: NG-RAN node supports PDU set QoS parameters, and the corresponding QoS flow is setup successfully Case2: NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow Case3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow PDU Set Information as below needs to be transferred over NG-U, Xn-U and F1-U: - PDU Set Sequence Number. - Indication of End PDU of the PDU Set. - PDU Sequence Number within a PDU Set. - PDU Set Size in bytes. - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. End of Data Burst information needs to be transferred over NG-U, Xn-U and F1-U. Whether to create a new “PDU Set Container” or reusing “PDU Session Container"? TSCAI enhancements: Enhance NGAP TSCAI to include the N6 Jitter information associated with DL Periodicity, and clarify TSCAI can also be used for non-GBR. Enhance XnAP/F1AP/E1AP TSCAI to include the N6 Jitter information associated with DL Periodicity. ECN: PDU set data discard： CB: # R18XR - Capture the agreements to TP - Discuss the open issues as above - Capture agreements and open issues (moderator - Nok) Summary of offline disc R3-234613 |  |  |  |  |
| ## 26. Basket for Late R18 Items ## [TU: 1 (1, 1, 0.5)] ## Includes R18 NPN, URLLC and Slice WIs |  |  |  |  |
| ## 26.1. eNPN WI WID [eNPN_Ph2-NGRAN-Core]: RP-231185 (target: RAN #101) Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2] Support for non-3GPP access for SNPN [RAN3] QUOTA: 2 The equivalent SNPNs IE is introduced NPN Mobility Information IE contained in the Mobility Restriction List IE over XnAP and NGAP. To support the NG based mobility across SNPNs, the Selected NID IE is introduced in the current Target ID IE contained in the HANDOVER REQUIRED message. To support Xn-based HO across equivalent SNPNs, there is no need to add a new selected SNPN ID in the HANDOVER REQUEST message, because the MRL can indicate the selected SNPN. To support non-3GPP access for SNPN services, a non-3GPP access specific selected NID should be added in the INITIAL UE MESSAGE over NG interface. RAN3#120: # Agree to add Selected NID IE in the top level of the INITIAL UE MESSAGE message. Inclusion of Selected NID in S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST in XnAP? Complete the stage3 details |  |  |  |  |
| R3-233739 | (BLCR to 29.413) Support of the enhanced NPN phase 2 (Huawei, China Telecom, Nokia, Nokia Shanghai Bell, NEC, LG Electronics, ZTE, Samsung, Ericsson) |  | CR0017r3, TS 29.413 v17.3.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233746 | Support of the enhanced NPN phase 2 (ZTE, Nokia, Nokia Shanghai Bell, Huawei, NEC, LG Electronics, Samsung) |  | CR0985r4, TS 38.413 v17.5.0, Rel-18, Cat. B Rev in R3-234533 Endorsed as BL CR |  |
| R3-233779 | (BLCR to 38.300) On introduction of R18 eNPN (China Telecom, Huawei, ZTE, CATT, Nokia, Nokia Shanghai Bell, LG Electronics, Samsung, NEC, Ericsson) |  | draftCR Endorsed as BL CR |  |
| R3-233830 | Introduction of equivalent SNPNs (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, LG Electronics, Samsung, ZTE, NEC) |  | CR0978r5, TS 38.423 v17.5.0, Rel-18, Cat. B Rev in R3-234501 Endorsed as BL CR |  |
| R3-234007 | Work Plan for Further Enhancement for Private Network Support for NG-RAN WI (China Telecommunication) |  | Work Plan Late contribution noted |  |
| R3-233858 | Indicating selected SNPN over Xn for Further NPN Enhancements (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-233859 | (TP for TS 38.423) Selected SNPN over Xn for further NPN Enhancements (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-234235 | Remaining topics for Rel-18 eNPN work (Ericsson) |  | discussion |  |
| R3-233873 | eNPN impact on RAN specifications (NEC) |  | discussion |  |
| R3-233953 | Discussion on Enhanced support of NPN (Samsung) |  | discussion |  |
| R3-233965 | (TP to eNPN_Ph2 BLCR for TS 38.413) Support of the enhanced NPN phase 2 (Huawei) |  | other |  |
| R3-233966 | (TP to eNPN_Ph2 BLCR for TS 38.423) Support of the enhanced NPN phase 2 (Huawei) |  | Other Rev in R3-234581 |  |
| R3-234117 | Discussion on RAN impact for NPN enhancement in Rel-18 (CATT) |  | discussion |  |
| R3-234118 | [TP to 38.423 for eNPN] RAN impact for NPN enhancement in Rel-18 (CATT) |  | other |  |
| R3-234236 | [TP for BL CR 38.413] Editorials for the NGAP BL CR (Ericsson) |  | Other Rev in R3-234582 |  |
| R3-234480 | left issues on RAN support for eNPN phase 2 (ZTE) |  | discussion |  |
| R3-234481 | TPs to BL CRs for eNPN phase 2 (ZTE) |  | other |  |
| R3-234487 | Remaining open issue in eNPN (LG Electronics) |  | discussion |  |
| R3-234003 | Discussion on remaining issues to support eNPN (China Telecommunication) |  | Discussion Late contribution |  |
| R3-234004 | (TP to BL CRs of 38.423) On support of eNPN (China Telecommunication) |  | Other Late contribution |  |
| R3-233797 | Introduction of equivalent SNPNs (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, LG Electronics, Samsung, ZTE, NEC) |  | CR0978r4, TS 38.423 v17.5.0, Rel-18, Cat. B withdrawn |  |
| Inclusion of Selected NID in S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST in XnAP? HW, ZTE, CT, Nok: If MN and SN can support different NID lists, then the selected NID is needed. NEC, E///, SS: No need to introduce new IE Check the above mentioned editorial changes for the NGAP BL CR as outlined in the TP in provided in R3-234236. Discuss the criticality to be assigned in NGAP and XnAP for the Equivalent SNPNs IE. CB: # R18NPN_Solution - Inclusion of Selected NID in S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST in XnAP? - Check the TPs, and criticality issue (moderator - CT) Summary of offline disc R3-234551 |  |  |  |  |
| ## 26.2. Timing Resiliency and URLLC WI WID [TRS_URLLC-NR-Core]: RP-231106 (target: RAN #102) 5GS network timing synchronization status and reporting [RAN3, RAN2] Interworking with TSN network deployed in the transport network [RAN3]- This objective has dependency on the progress of SA2 and CT4 Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2] QUOTA: 2 It’s the common understanding that RAN Timing Synchronization Status Information can be the same or different between “groups of cells within a single gNB” (e.g., cells served by different gNB-DUs). RAN3#120: In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF. In F1AP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB-DU to report RAN TSS to the gNB-CU. WA: When Clock Quality Detail Level IE has value “clock quality metrics”, all clock quality metrics supported by the gNB implementation are delivered to the UE (may be revisited based on SA2 agreements). Is proactive RAN feedback applicable/relevant after the initial establishment of the TSC QoS flow? Check progress in other WGs |  |  |  |  |
| R3-233723 | Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s) (SA1(Nokia)) |  | LS in noted |  |
| R3-233755 | (BLCR to 38.413) Introduction of 5G Timing Resiliency and URLLC enhancements (Huawei, China Unicom, Nokia, Nokia Shanghai Bell, Samsung, Ericsson, ZTE, CATT) |  | CR0972r4, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233764 | (BL CR for TS 38.423) Introduction of 5G Timing Resiliency and URLLC enhancements (Ericsson, Huawei, Nokia, Nokia Shanghai Bell, Samsung, ZTE, CATT) |  | CR1049r5, TS 38.423 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233770 | (BLCR to 38.473) Introduction of 5G Timing Resiliency and URLLC enhancements (ZTE, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung) |  | CR1168r3, TS 38.473 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233814 | Work plan for Timing Resiliency and URLLC enhancements (Nokia (rapporteur)) |  | Work Plan Conclude on the Stage 2 and the encoding of the RAN TSS attributes. noted |  |
| R3-233816 | (TP for TS 38.413 BL CR) Further stage 3 details for timing resiliency (Nokia, Nokia Shanghai Bell) |  | other |  |
| R3-233815 | (TP for TS 38.401 BL CR) Stage 2 for timing resiliency and URLLC (Nokia, Nokia Shanghai Bell, Samsung, Qualcomm, CATT) |  | Other Rev in R3-234598 (TP for TS 38.401 BL CR) TSS reporting |  |
| R3-234023 | (TP to TRS_URLLC BLCR for TS 38.413, TS 38.423 and TS 38.473) Support of 5G Timing Resiliency enhancements (Huawei, China Unicom) |  | Other Rev in R3-234599 (TP for TS 38.413 BL CR) TSS reporting and RAN feedback |  |
| R3-234428 | Discussion and TPs for timing synchronization status and reporting (ZTE) |  | Other Rev in R3-234600 (TP for TS 38.473 BL CR) TSS reporting and RAN feedback |  |
| R3-234321 | Discussion on NR Timing Resiliency and URLLC enhancements (Ericsson) |  | discussion |  |
| R3-233904 | Interworking with TSN network and RAN feedback (Nokia, Nokia Shanghai Bell) |  | discussion |  |
| R3-234429 | Discussion on TSN integration and RAN feedback (ZTE) |  | other |  |
| R3-234025 | (TP to TRS_URLLC BLCR for TS 38.413) Support of TSN enabled transport network (Huawei, China Unicom, China Telecommunication) |  | other |  |
| R3-233986 | Discussion on Open Issues in Timing Resiliency and uRLLC (Qualcomm Incorporated) |  | discussion |  |
| R3-234024 | (TP to TRS_URLLC BLCR for TS 38.413, TS 38.423 and TS 38.473) Support of RAN feedback enhancements (Huawei, China Unicom) |  | other |  |
| R3-234322 | Text Proposals on Support NR Timing Resiliency and URLLC enhancements (Ericsson) |  | Other Rev in R3-234601 (TP for TS 38.423 BL CR) TSS reporting and RAN feedback |  |
| R3-234396 | Discussion on Network timing synchronization status and reporting (CATT) |  | discussion |  |
| R3-234397 | TP for BLCR to TS38.413 Adapting downstream and upstream scheduling (CATT) |  | other |  |
| In NGAP, introduce a new Timing Synchronisation Status procedure (class 1) to enable the AMF to initiate RAN TSS reporting by the gNB? HW: Support class2 procedure only. ZTE: Current legacy procedure is enough, e.g., AMF Configuration Procedure. QC: Support new class1 procedure In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF. Proposal 3: The (NGAP) TIMING SYNCHRONISATION STATUS REQUEST message includes the RAN TSS Request Type IE (start, stop). The (NGAP) TIMING SYNCHRONISATION STATUS REPORT message includes the RAN Timing Synchronisation Status IE and the RAN TSS Scope IE (indicating whether the scope of the RAN TSS is “all cells within a single gNB” or “list of Cell IDs within a single gNB”). The detail IE design needs to be further discussed. ZTE: It makes sense. Proposal 5: In F1AP, introduce two new procedures corresponding to the new NGAP procedures. Proposal 6: The (F1AP) TIMING SYNCHRONISATION STATUS REQUEST message includes the RAN TSS Request Type IE (start, stop). Proposal 7: The (F1AP) TIMING SYNCHRONISATION STATUS REPORT message includes the RAN Timing Synchronisation Status Information IE (containing all RAN TSS attributes supported by the gNB-DU). CB: # R18URLLC_Solution - Discuss the open issue above on RAN TSS report over NG and F1 - Discuss open issue left for other topics - Capture agreements to TP - Capture agreements and open issue - LS to RAN2/SA2? (moderator - Nok) Summary of offline disc R3-234552 |  |  |  |  |
| ## 26.3. RAN Slicing WI WID [eNS_Ph3-NR-Core]: RP-231396 (target: RAN #102) Evaluate the impact, if any, on RAN to support Network Slice Service continuity scenario. [RAN3] Evaluate the impact, and specify the solution, if needed, to support Partially Allowed NSSAI in RRC_Connected Mode. [RAN3] QUOTA: 2 RAN3#120: To include the partially allowed S-NSSAI over NGAP in messages where the Allowed NSSAI is sent and to encode the partially allowed S-NSSAI separately from an Allowed S-NSSAI. Potential signaling impact on Alternative S-NSSAI and issue in mobility scenario? It is also agreed the Partially allowed NSSAI IE is not needed in the Xn Handover Request because for the propagation between gNBs, the information can be obtained by gNB via Path switch acknowledge message? UE moves from non-supporting TA to supporting TA, PDU session modify and Path switch Check SA2 conclusion |  |  |  |  |
| R3-233747 | Introduction of Enhancement of RAN Slicing for NR (Nokia, Nokia Shanghai Bell) |  | CR1006r1, TS 38.413 v17.5.0, Rel-18, Cat. B Endorsed as BL CR |  |
| R3-233860 | (TP for TS 38.413 and TS 38.423) Support of Network Slice Service Continuity (Nokia, Nokia Shanghai Bell, Orange) |  | other |  |
| R3-234419 | (TPs for BLCRs for 38.413 38.423 38.473 38.300)RAN impact on supporting Network Slice Service continuity scenario (ZTE) |  | other |  |
| R3-233962 | (TP to eNS_Ph3 BLCR for TS 38.300) Enhanced network slicing phase 3 (Huawei) |  | other |  |
| R3-234034 | Further discussion on the enhancement of RAN slicing (Samsung) |  | discussion |  |
| R3-234304 | Clarifications and Way Forward on Network Slice Service Continuity (Ericsson) |  | other |  |
| R3-234448 | Discussion on network slice service continuity (CMCC) |  | discussion |  |
| R3-234399 | TP for 38.423 for supporting alternative S-NSSAI (CATT) |  | other |  |
| R3-234005 | Discussion on remaining issues for RAN Slicing enhancement (China Telecommunication) |  | Discussion Late contribution |  |
| R3-233861 | (TP for TS 38.413 and TS 38.423) Support of partially allowed S-NSSAI in RRC connected mode (Nokia, Nokia Shanghai Bell, Orange) |  | other |  |
| R3-234420 | (TPs for BLCRs for TS 38.413)RAN impact on support Partially Allowed NSSAI (ZTE) |  | other |  |
| R3-233963 | (TP to eNS_Ph3 BLCR for TS 38.413) Partially allowed NSSAI in a registration area (Huawei) |  | other |  |
| R3-234303 | Clarifications on Partially Allowed NSSAI (Ericsson, Deutsche Telekom) |  | other |  |
| R3-234398 | Discussion on supporting Partially Allowed NSSAI (CATT) |  | discussion |  |
| R3-234449 | Discussion on Partially Allowed NSSAI (CMCC) |  | discussion |  |
| R3-234006 | (TP to TS 38.300/38.413) Support of Partially Allowed S-NSSAI (China Telecommunication) |  | Other Late contribution |  |
| Partially Allowed NSSAI: The total number of allowed/partially allowed S-NSSAI should not exceed 8. Add the Slice Availability IE in the PDU Session Notify message? Exchange the Slice Availability IE in Xn Setup/RAN Configuration Update to report neighbour cells configured outside or inside AoS in a TA? Enhancements on target S-NSSAI? E///: Comments on the signaling flow in Figure1. HW: The solution proposed by Nok needs the support from SA2. Service continuity: Enhancement on performing Network Slice Replacement during handover? CB: # R18Slice_Solution - Provide TP to capture the agreement above - Identify the left issue to be solved in R18 (moderator - ZTE) Summary of offline disc R3-234553 |  |  |  |  |
| # 31. Corrections and Enhancements to Rel-18 [TU: 0.5 (0.5, 0.5, 0.5, 0.5)] (shared with AI 9) QUOTA: 3 (was 2) All Rel-18 technically endorsed CRs need to be resubmitted when the first R18 specs are created. |  |  |  |  |
| ## 31.1. Corrections Including whether RAN1 agreements need to be captured in stage2 for NCR, no consensus in RAN3#119. |  |  |  |  |
| R3-233946 | Discussion on HARQ Related Assistance Information in NR User Plane (Samsung, China Telecom, CMCC, Lenovo) |  | Discussion HW, ZTE: CU can derive the rate data locally. SS: CU cannot know how many HARQ attemp tries QC: There is no detail on how to calculate the rate in paper, how to use the rate infor for PDCP duplication optimization, it can be used by UP to choose the path noted |  |
| R3-233947 | Correction for HARQ Related Assistance Information in NR User Plane (Samsung, China Telecom, CMCC, Lenovo) |  | CR0147r, TS 38.425 v17.3.0, Rel-18, Cat. F noted |  |
| R3-233964 | Introduction of measurements without gap with interruption (Huawei, Deutsche Telekom, BT) |  | CR1194r, TS 38.473 v17.5.0, Rel-18, Cat. B E///: CR looks ok, the reference to RAN2 CR is endorsed rather than agreed, wait till RAN2 formally agreed the CR Nok: Fine for us from technical point of view, share view as E/// ZTE: Fine with this CR To be resubmitted after the RAN4 led WI includes RAN3 spec in the WID in RAN plenary |  |
| R3-234485 | Discussion on corrections of PDU session split (ZTE, CMCC, China Telecom, China Unicom) |  | Discussion HW: The title shows the signaling procedure during PDU session resource setup/modify procedure CATT: Understand the concern, the AMF can set suitable timer takes latency of split PDU session procedure into account Nok: See the point here, good to have note CB: # 11_PDUSessionSplit - Check the scenario and work on the note if agreeable (moderator - ZTE) Summary of offline disc R3-234536 |  |
| R3-234486 | Corrections of PDU session split (ZTE, CMCC, China Telecom, China Unicom) |  | draftCR |  |
| R3-234213 | TEI18 RedCap UE MBS broadcast reception (ZTE) |  | Discussion HW: Tends to agree the motivation, IE can be defined as bitstring Nok: Different understanding, it has been discussed in RAN2/RAN, the solution is not completed, e.g., MBS SMF behavior, MBS SMF does not know the service is fit for Redcap UE ZTE: The requirement came from SA2 one year ago, there will be 5GC impact QC: There are two ways to transfer the assistance information to RAN, one is the explicit infor like CR proposed, another one is via application layer, LS to SA2 E///: Configured by 5QI HW: Operator configured 5QI has not been support in 5G CB: # 12_MBSBroadcat - Check the status in other related WGs - Check the feasibility of solution proposed and find the way to proceed (moderator - ZTE) Summary of offline disc R3-234537 |  |
| R3-234337 | Correction of MBS Multicast HFN/SN Initialization (Nokia, Nokia Shanghai Bell, CATT, Orange, Qualcomm Incorporated ) |  | Discussion E///: was discussed in last meeting HW: The issue has been stopped noted |  |
| ## 31.2. Enhancements |  |  |  |  |
| R3-233823 | Discussion on SDT termination for split gNB (ZTE) |  | Discussion CATT, CT：DU controls the radio resource, the indication from DU to CU is fine, but it’s not necessary to configure the threshold from CU to DU ZTE, Nok, QC: Only DU knows BSR information which can be used to determine whether to stop the SDT or not. Try to align with DL data treatment. CB: # 26_ULdataSDT - Agree to introduce the termination indication from DU to CU and the threshold configuration from CU to DU - Check the details of 3CRs below (moderator - ZTE) Summary of offline disc R3-234555 |  |
| R3-233863 | Support of oversize UL SDT Data Arrival [Large SDT Uplink Data] (ZTE, Nokia, Nokia Shanghai Bell, Huawei) |  | draftCR Rev in R3-234556 |  |
| R3-234091 | Support of oversize UL SDT Data Arrival [Large SDT Uplink Data] (Huawei, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, ZTE) |  | CR0302r, TS 38.401 v17.5.0, Rel-18, Cat. B Rev in R3-234557 |  |
| R3-234338 | Switching from SDT to RRC Connected State [Large SDT Uplink Data] (Nokia, Nokia Shanghai Bell, Huawei, Qualcomm Incorporated, ZTE) |  | CR1213r, TS 38.473 v17.5.0, Rel-18, Cat. B Move to 31.2 Rev in R3-234558 |  |
| R3-234220 | Discussion on positioning in inactive mode for SDT without anchor relocation (Huawei, CATT) |  | Discussion LG: Comments on reusing the PARTIAL UE CONTEXT TRANSFER procedures SS: Some information in SRS Configuration are not applied to the target node QC: Align R18 handling with R17 To be continued... |  |
| R3-234221 | Positioning inactive mode for SDT without anchor relocation (Huawei, CATT) |  | CR1085r, TS 38.423 v17.5.0, Rel-18, Cat. B |  |
| R3-234366 | Discussion on Inactive Positioning in SDT without anchor relocation (CATT, Huawei) |  | discussion |  |
| R3-234367 | Support of Inactive Positioning in SDT without anchor relocation case (CATT, Huawei) |  | CR0109r, TS 38.455 v17.5.0, Rel-18, Cat. B |  |
| R3-234237 | On MRB PDCP COUNT wrap around including [draft CRs for several TSs] (Ericsson) |  | Other HW, QC: Long discussion on this issue. According to RAN2 agreement, MRB PDCP COUNT wrap is up to implementation E///: No need to have two transmissions over Uu Nok: It’s too complicated, multiple solutions proposed previously noted |  |
| R3-234339 | Relocation of MBS Contexts to another CU UP (Nokia, Nokia Shanghai Bell) |  | Discussion HW: It’s CP decides whether to change UP, it will trigger bearer setup procedure towards other UP, and using the data forwarding mechanism Nok: Without indication from UP, CP does not aware to relocation of gNB-CU-UP MBS context for Multicast SS: It’s useful, to be further discussed in R18 MBS WI E///: Just setup the new bearer towards the new UP. noted |  |
| R3-234430 | Correction on Location reporting control (ZTE, Huawei, CATT, China Telecom, China Unicom, Qualcomm) |  | CR0877r2, TS 38.413 v17.5.0, Rel-18, Cat. F - Update the coversheet: the useless information will propagate to other RAN node and would jeopardize UE’s behavior. The standard is to avoid the useless information transmission for the case when the Area Of Interest List IE and Event Type IE in the Location Reporting Request Type IE are configured. - Try to find the proper wording in the procedure text Rev R3-234562 |  |
| R3-233841 | Location Reporting Correction (Ericsson, China Telecom, Huawei, CATT, China Unicom) |  | CR0911r1, TS 38.413 v17.5.0, Rel-18, Cat. F Nok: Not align with SA2, the second change is fine HW: Need more time to check the second change Rev in R3-234561 |  |
| R3-234488 | Information on posSIBs relaying to remote UE [PosL2RemoteUE] (Ericsson, AT&T, Deutsche Telekom) |  | CR0112r, TS 38.455 v17.5.0, Rel-18, Cat. B |  |
| R3-234239 | Timing Advance Exceeded Cause [TA-Cause] (Nokia, Nokia Shanghai Bell) |  | CR1208r, TS 38.473 v17.5.0, Rel-18, Cat. B |  |
| R3-234229 | TEI18 RRC Inactive enhancements (Ericsson) |  | discussion |  |
| R3-234104 | LMF Outage Reporting [LMF_outage] (Ericsson) |  | CR0941r3, TS 38.413 v17.5.0, Rel-18, Cat. B |  |
| R3-233831 | Timing Advance Exceeded Cause (Nokia, Nokia Shanghai Bell) |  | CR1190r, TS 38.473 v17.5.0, Rel-18, Cat. B withdrawn |  |
| R3-234277 | Information on posSIBs relaying to remote UE [PosL2RemoteUE] (Ericsson, AT&T, Deutsche Telekom) |  | CR0108r, TS 38.455 v17.5.0, Rel-18, Cat. B withdrawn |  |
| R3-234238 | Timing Advance Exceeded Cause [TA-Cause] (Nokia, Nokia Shanghai Bell) |  | CR0074r, TS 37.483 v17.5.0, Rel-18, Cat. B withdrawn |  |
| # 32. Any other business |  |  |  |  |
| # 33. Closing of the meeting |  |  |  |  |

Conference Calls Schedule (tentative)

Only delegates registered to the meeting will receive invitations to conference calls
All times are local time (CET)
For sessions longer than 2 h, there will be a 5-10 min. break in the middle of the session

|  | Monday | Tuesday | Wednesday | Thursday | Friday |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0800 0830 | 0900 START OF MEETING | Offline* | Offline* | Offline* |  |  |
|  |  | Corrections: AI 9/31 (cont.) | Corrections: AI 9/31 (cont.) | Corrections: AI 31/9 (cont.)* | CBs |  |
| 0900 | Org: AI 1-8 | SON/MDT WI: AI 10 | Mobility Enh WI: AI 14 G | QoE WI: AI 11 | CBs |  |
| 1030~1100 | Coffee Break |  |  |  |  |  |
|  | AI 8 (cont.) Corrections: AI 9 | SON/MDT WI: AI 10 | Sidelink Relay WI: AI 16 G | QoE WI: AI 11(cont.) MBS WI: AI 15 | CBs |  |
| 1300~1400 | Lunch Time |  |  |  |  |  |
|  | Corrections: AI 9 | AI RAN WI: AI 12 | Mobile IAB WI: AI 13 A | MBS WI: AI 15 | CBs |  |
| 1600~1630 | Coffee Break |  |  |  |  |  |
|  | Corrections: AI 319 | AI RAN WI: AI 12 | Mobile IAB WI: AI 13 (cont.) Positioning WI: AI 23 A | NR NTN WI: AI 17 IoT NTN WI: AI 18 | CBs |  |
|  |  |  |  |  | 1700 END OF MEETING |  |
| 1800 1900 2000 2030 | SDT WI: AI20 Redcap WI: AI 21 | Basket WI: AI 26.1, 26.2, 26.3 | XR WI: AI25 Network ES WI: AI 24 | Early CBs* |  |  |
|  |  | 3GPP Charity Musical Run in Toulouse |  |  |  |  |

blue		Study Items
A, G		chaired by Vice-Chairs
red		changed
red strikethrough	not treated
*		if needed

Future meeting dates

| Title | Dates | Venue | Host |
| --- | --- | --- | --- |
| RAN3#121 | 21 -25 Aug 2023 | F2F Meeting | Toulouse, France |
| RAN #101 | 11 -15 Sep 2023 | F2F Meeting | Bangalore , India |
| RAN3#121bis | 9 -13 Oct 2023 | F2F Meeting | Xiamen, China |

Agenda color coding

10. Agenda Item

10.x. Sub Agenda Item QUOTA: 5

10.x.1. Sub-sub Agenda Item

##### 10.x.1.1. Sub-sub-sub Agenda Item

TOPIC GROUPING (used to group and highlight a topic, but it is not an Agenda Item)

##### 10.x.1.2. Sub-sub-sub Agenda Item

QUOTA: n Each company may submit up to n contributions to the Agenda Item where this number appears. This number applies to the sum of all Tdocs submitted to all the sub-Agenda Items. In the example above, a company may submit up to 5 contributions to AI 10.x in any combination: e.g. up to 4 to 10.x.1.1 and up to 1 to 10.x.1.2, or up to 3 to 10.x.1.1 and up to 2 to 10.x.1.2, and so on.

Chair’s notes color coding

| R3-xxxxxx | Available but not yet treated document | Chair notes |
| --- | --- | --- |
| R3-xxxxxx | This document has low priority |  |
| R3-xxxxxx | This document was not available at submission deadline or withdrawn | Chair notes |
| R3-xxxxxx | The quota for at least one of the sourcing companies was exceeded in this AI. This document is to be considered withdrawn and will not be treated. | Chair notes |
| R3-xxxxxx | This document was treated as noted at this meeting. Noted: Tdoc has been presented and/or taken into account. | Chair notes |
| R3-xxxxxx | Agreed (or approved) proposal: e.g. Working Assumption, tdoc proposal, etc... | Chair notes Approved – used for Report and Agenda Agreed – used for CR to be sent to RAN or LS out p-Approved (partially approved) – used for BL CR or TR subject to tdoc allocation by MCC for next meeting Merged – used for a tdoc agreed to be merge in a BL CR or TR Endorsed – used for CR to be agreed by other WG e.g. TS 36.300 or Draft Status Report (DSR) produced in meeting |
| R3-xxxxxx | Request for ComeBack (CB) during the meeting | Chair notes a Come Back is required during the meeting CB # n_FolderName - comments (Company - moderator) |
| R3-xxxxxx | Open issue which might require further clarification in next meeting | Chair notes Comments (no agreement) |
| R3-xxxxxx | E-mail discussion (typically after the meeting) | Chair notes Email#01 Deadline (Company) |
| R3-xxxxxx | “To be continued” discussion: there was no agreement at this meeting and the discussion may continue at the next meeting | Chair notes To be continued |
| R3-xxxxxx | Important warning for consideration | Chair notes Important warning for consideration |