# 3GPP TSG RAN3 Meeting#122	R3-237346

# Chicago, USA, 13 - 17 November 2023

# Agenda Item:	8.1

# Source:	ZTE

# Title:	Discussion on the LS from CT4 on finer periodicity

# Document for:	Discussions

- Introduction

- It is stated in the LS[1] that CT4 has agreed CRs from Rel-17 at CT4#115e, which specified support of periodic deferred location reporting with interval less than 1 second. The agreed CRs may have LPP and/or NRPPA impacts.

- Discussion

CT4 has agreed following CRs from Rel-17 at CT4#115e, which specified support of periodic deferred location reporting with interval less than 1 second:

-GMLC SBI interface (CR0102 of TS 29.515)

-AMF SBI interface (CR0892 of TS 29.518)

-LMF SBI interface (CR0165 of TS 29.572)

-MAP interface (CR1267 of TS 29.002).

The CRs were approved at CT#100 and implemented in 3GPP June 2023 public release. The agreed CRs may have LPP and/or NRPPA impacts.

Following is the content of the LS from CT4 [1]:

Upon checking the agreed CR C4-231275, the yellow-highlighted correction may have RAN3 impacts as follows:

# 6.1.6.2.24	Type: PeriodicEventInfo

Table 6.1.6.2.24-1: Definition of type PeriodicEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reportingAmount | ReportingAmount | M | 1 | Number of event reports |
| reportingInterval | ReportingInterval | M | 1 | Interval of event reports |
| reportingInfiniteInd | boolean | O | 0..1 | When present, this IE shall be set to the value true, indicating that as many reports as possible to be generated during allowed duration (8639999 seconds). When this IE is present in a request, the reportingAmount should be set to the largest possible value (see NOTE) for backward compatibility consideration. An LMF supporting this IE shall ignore the reportingAmount IE and an LMF not supporting this IE will use reportingAmount IE to generate indicated number of reports as legacy. When this IE is present in a response, it indicates that this IE is supported and accepted by the LMF. |
| reportingIntervalMs | ReportingIntervalMs | O | 0..1 | When present, this IE shall indicate the Interval of event reports in milliseconds. When this IE is present in a request, the reportingInterval is set ot 1 for backward compatible consideration. An LMF supporting this IE shall ignore the reportingInterval IE; an LMF not supporting this IE will use reportingInterval IE to generate report with minimal interval as legacy, i.e. 1 second. When this IE is present in a response, it indicates that this IE is supported by the LMF and it shall indicate the value accepted by the LMF. |
| NOTE: reportingAmount x reportingInterval shall not exceed 8639999 (99 days, 23 hours, 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP. |  |  |  |  |

Reviewing UE Reporting Information IE in the current NRPPa/F1AP specification, the indication of infinite reporting is supported. Value 0 in the Reporting Amount IE represents an infinite number of periodic reporting. However, the reporting interval per millisecond is not supported. Therefore, it is proposed to introduce the reporting interval per millisecond IE in the UE Reporting Information IE to align with the agreement in CT4.

- To NRPPa & F1AP spec, introduce the ReportingIntervalIMs IE in the UE Reporting Information IE, as INTEGER(1...999).

The corresponding CR to NRPPa and F1AP are provided in R3-237347[3] and R3-237348[4].

- Reference

- R3-237135, LS Out Sub One Second Report Period for Deferred Location over SBI, CT4

- C4-231275, Missing finer periodicities than 1s and an infinite reporting amount, Ericsson

- R3-237347, CR to NRPPa on missing finer periodicity, ZTE

- R3-237348, CR to F1AP on missing finer periodicity, ZTE