3GPP TSG-RAN WG3 Meeting #122	R3-237509

# Chicago, USA – Nov 13th – Nov 17th 2023

Agenda Item:	10.2.5

Source:	Ericsson

Title:	(TP for SON to BLCR for TS 38.473, TS 38.423) NR-U enhancements for MLB and MRO

Document for:	Discussion, Approval

# Introduction

In this paper we provide further discussion on NR-U related enhancements for MRO and MLB based on the agreement and FFS captured at last RAN3 meeting:

WA: RAN3 agrees to enable reporting of number of DL LBT failures from the target node to the source node in case of failed HO attempt (assuming the UE can be identified).

FFS whether in case of DL LBT problems at the last serving node that led to the TEH / HWC, the last serving node skips informing the source node or informs it with additional information.

FFS if reporting of the number of UL LBT failures in case of non-HOF RLF can be addressed in Rel.18.

FFS on whether to add a load metrics on the number of UL consistent LBT failures in the Resource Status Update.

# Discussion

## Reporting DL LBT failures

When handover preparation is completed, the source and the target gNB have exchanged the UE identifier over Xn, i.e., the Source NG-RAN node UE XnAP ID reference and the Target NG-RAN node UE XnAP ID are known. On the other hand, over the air interface, if contention based random access is used, the target gNB cannot distinguish if the UE identified by the XnAP IDs is the same attempting the random access, until the Msg3 arrives. This means, for example, that if contention based random access is used, a gNB cannot know whether a DL LBT issue causing delay or failure in transmission of Msg2 is affecting the same UE identified by the XnAP IDs above. In case of normal handover, the target gNB could “somehow” deduce that if there was a preparation done for the UE, and there was no execution following that, the failures in transmitting Msg2 impacted the handover execution for the UE. However, the same cannot be said for CHO. In this case, a candidate target gNB does not know whether the UE will try to perform RA at all, so there is no way for a candidate target gNB to pinpoint that DL LBT failures impacted the handover execution for a specific UE before the actual UE identity is known over the air interface, i.e., until Msg3 is received.

After Msg3 is received, both for normal handover and for CHO, if the target gNB suffers DL LBT failures, and cannot transmit Msg4 due to that, it can associate the DL LBT failures to the handover failure for a certain UE.

In summary, in case of failed HO execution due to DL LBT failures at the target gNB, the UE to which the HO refers to can be identified only if the DL LBT failures have impacted the transmission of Msg4.

Observation 1: In case of contention-based RA, a target gNB can identify a UE for which an HO attempt failed only after Msg3 is received.

We analyze now the case of DL LBT failures at the last serving node that led to Too Early Handover (TEH) or Handover To Wrong Cell (HWC). First of all, we note that we should not really call this failure as “TEH” or “HWC”, since they do not reflect the legacy case. Instead, we will refer to them as “Too Early Handover detected due to DL LBT failures” and “Handover to Wrong Cell detected due to DL LBT failures”.

- “Too Early Handover detected due to DL LBT failures”. This can occur in two cases, as detailed below:

  - an handover failure occurs during the handover procedure, and the UE attempts to re-establish towards the source gNB. In legacy (i.e., in licensed spectrum), this scenario would be classified as a Too Early Handover (TEH), however the failure was in this case due to DL LBT failures. Since the UE re-establishes towards the source gNB, the source gNB can fetch the RLF report with the indicaiton of the HO failure. According to legacy, the source would use this UE report for MRO. To avoid that, the target gNB would need to inform that DL LBT failures were present. However, as discussed above, the target can only identify the UE from Msg3 onwards. Therefore, reporting the presence of DL LBT failures for the purpose of excluding the UE report for MRO is only worth doing it in case of failures in transmission of Msg4. At the same time, the target knows that DL LBT failures are present, and can try to resolve the situation independently. So, we see little benefit for the target node to send to the source node the number of DL LBT failures in this scenario. We also note that a similar problem (the source should discard UE reports for MRO) already exists in legacy, in case of failures in RA due to sub-optimal RA configuration, and in such case there is no information sent from target to source.

  - shortly after a successful HO towards a target gNB (i.e., within Tstore_UE_cntxt) the UE experiences a connection failure, and the UE attempts to re-establish towards the same cell that served the UE at last handover initialization. In legacy (i.e., in licensed spectrum), the target identifies the Too Early Handover, and sends the HANDOVER REPORT message to the source gNB. In the NR-U case, since the target gNB knows that it was not a real TEH, it can avoid sending the HANDOVER REPORT alltogether. The alternative would be that the target gNB sends the HANDOVER REPORT, in which case it has to let the source gNB understand that it is not a TEH, but a different ”Handover Report Type”. We don’t think this second option is not needed, because the source gNB does not perform any optimization in this scenario.

- “Handover to Wrong Cell detected due to DL LBT failures”

  - an handover failure occurs during the handover procedure, and the UE attempts to re-establish in a cell other than the source cell and the target cell. In legacy (i.e., in licensed spectrum), this scenario would be classified as a Handover To Wrong Cell (HWC). However, as for scenario a) and b), the real issue is the presence of DL LBT failures at the target. We can distinguish three cases:

    - the reconnecting cell belongs to the same target gNB: in this case the target gNB performs the analysis. It knows that the failure was due to DL LBT issues, and no RAN3 impact is needed.

    - the reconnecting cell belongs to the source gNB: in this case the source gNB receives the UE report. In this case it is the source node that performs the analyisis, so we are in the same position as for case a)

    - the reconnecting cell belongs to a third gNB: in this case, since the problem is ”handover failure”, the node due to perform the analysis is the source gNB. After a first step, where the third gNB sends a FAILURE INDICATION to the source gNB, we end up in the same position as for case a)

  - shortly after a successful HO towards a target gNB (i.e., within Tstore_UE_cntxt) the UE experiences a connection failure and the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell. In legacy (i.e., in licensed spectrum), this scenario would be classified as Handover to Wrong Cell, and also in this case, we can distinguish three cases:

    - the reconnecting cell belongs to the same target gNB: in this case the target gNB performs the analysis. Since it knows that the failure was due to DL LBT issues, no RAN3 impact is needed.

    - the reconnecting cell belongs to the source gNB: in this case the source gNB receives the UE report, detects that it is a HWC scenario. This is similar to case a)

    - the reconnecting cell belongs to a third gNB: in this case, since the problem is a connection failure after successful handover, it is the target gNB that should perform the analysis. After a first step, where the third gNB sends a FAILURE INDICATION to the target gNB, we end up in the same position as for case b).

Overall, we can see that for the case of DL LBT problems at the last serving node that led to TEH / HWC, two main options are significant:

- the source node is the node performing the analysis. In this case, the benefit of informing the source node is limited to the case where the UE can be identified, i.e., for DL LBT failures after Msg3.

- the target node is the node performing the analyisis. In this case, there is no benefit of informing the source node, since the optimization is done at the target.

Considering the above w.r.t. the current WA and the FFS related to DL LBT problems at last serving node that can lead to TEH and HWC:

WA: RAN3 agrees to enable reporting of number of DL LBT failures from the target node to the source node in case of failed HO attempt (assuming the UE can be identified).

FFS whether in case of DL LBT problems at the last serving node that led to the TEH / HWC, the last serving node skips informing the source node or informs it with additional information.

we think that within MRO there is little benefit to report the number of DL LBT failures from the target node to the source node in case of failed HO attempt, given that for the scenarios where the source node could make use the information, the UE can only be identified by the target gNB after Msg3 is received (assuming contention based RA).

Therefore, we propose to not turn the WA into an agreement, and not pursue the reporting of DL LBT failures from target node to source node for MRO purpose.

Proposal 1: For MRO purpose, the number of DL LBT failures are not reported from the target node to the source node in case of failed HO attempt.

We want to bring to the attention of the group that, since a number of meetings, we proposed to evaluate another option, where UE reports can be enhanced with information from which the source node analysing the report can deduce whether the target node suffered from DL LBT issues. According to the text reported below from TS 38.133 (section 6.1B), this is possible, and we encourage to check this with RAN4.

Observation 2: According to RAN4 specification (TS 38.133), the UE can detect if SMTC occasion are not available, which can impact the HO interruption time and contribute to the HOF.

In the requirements of clause 6.1B.1, the term SMTC occasion not available at the UE refers to when the SMTC contains SSBs configured by gNB in a cell on a carrier frequency subject to CCA, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the corresponding detection or time tracking period; otherwise the SMTC occasion is considered as available at the UE.

Proposal 2: RAN3 to send the LS to RAN4 as in Appendix A, to request whether the UE is capable to detect unavailability of the SMTC occasions during HO to a target NR-U cell.

In relation to the other remaining FFS, i.e.:

FFS if reporting of the number of UL LBT failures in case of non-HOF RLF can be addressed in Rel.18.

it is our understanding that RAN2 has agreed that UE can collect information related to LBT failures in case of RLF, HOF, SHR generation, and RA. This is captured in various RAN2 agreements and implemented in the running CRs for 38.331 (R2-2310750, R2-2310563). Therefore, we think no further action is needed from RAN3 point of view.

Proposal 3: No further action is needed in RAN3 to address UL LBT failures in case of non-HOF RLF.

## Consistent Uplink LBT failures

In TS 38.300, chapter 5.6.1 describes the actions performed by the UE when consistent uplink LBT failures is detected by the UE.

When the UE detects consistent uplink LBT failures, it takes actions as specified in TS 38.321 [6]. The detection is per Bandwidth Part (BWP) and based on all uplink transmissions within this BWP. When consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell than the SCell(s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE. When consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE. When multiple UL BWPs are available for switching, it is up to the UE implementation which one to select. For PSCell, if consistent uplink LBT failures are detected on all the UL BWPs with configured RACH resources, the UE declares SCG RLF and reports the failure to the MN via SCGFailureInformation. For PCell, if the uplink LBT failures are detected on all the UL BWP(s) with configured RACH resources, the UE declares RLF.

We can see that, the network is informed in various ways about the presence of UL LBT failures detected by the UE, both for the case when carrier aggregation is used and when carrier aggregation is not used. For an SpCell, the UE switches to another UL BWP in the same cell and report the failure via MAC CE. For an SCell, the UE selects another Serving Cell for which consistent LBT failure has not been triggered, and there it reports the failure via MAC CE.

In the MAC specification, this is visible in the text below, the part marked in yellow refers to SpCell, the part marked in green refers to SCell:

The MAC entity shall:

1>	if consistent LBT failure has been triggered, and not cancelled, in the SpCell; and

1>	if UL-SCH resources are available for a new transmission in the SpCell and these UL-SCH resources can accommodate the LBT failure MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the LBT failure MAC CE.

1>	else if consistent LBT failure has been triggered, and not cancelled, in at least one SCell:

2>	if UL-SCH resources are available for a new transmission in a Serving Cell for which consistent LBT failure has not been triggered and these UL-SCH resources can accommodate the LBT failure MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the LBT failure MAC CE.

2>	else:

3>	trigger a Scheduling Request for LBT failure MAC CE.

The current load balancing metrics signaled between DU and CU, or between gNBs allows to report the COT UL per NR-U channel and the Total PRB utilization in UL per NR-U channel. The COT UL refers to the percentage of time for which the channel resources have been utilized. The Total PRB utilization per NR-U Channel indicates the resources used in the NR-U channel during the percentage of time represented by the COT UL. Therefore, the existing load metrics do not take into account the fact that the UE may have encountered problems in data transmissions in the NR-U channel, and the amount of detected consistent UL LBT failures reported by the UEs can be used to complement the existing information. We think this information is beneficial for load balancing decisions.

The DU receives the MAC CE information potentially from multiple UEs, so it can report to the CU how many times the UEs have reported to the “consistent UL LBT failures” for one NR-U channel in a certain cell. The reporting can be done at the same periodicity of the other load metrics. The CU can signal to the neighboring gNBs the same information. For example:

- DU1 sends to the CU that “Number of Consistent UL LBT Failures” = X for NR-U Channel 1 in cell 1

- DU2 sends to the CU that “Number of Consistent UL LBT Failures”  = Y for NR-U Channel 1 in cell 2

- CU sends to the neighbor gNB:

- “Number of Consistent UL LBT Failures” = X for NR-U Channel 1 in cell 1

- “Number of Consistent UL LBT Failures” = Y for NR-U Channel 1 in cell 2

Proposal 4: Add a new load metric per NR-U Channel (over F1 and Xn) to indicate the number of consistent uplink LBT failures detected for a certain NR-U channel in a reporting period.

Proposal 5: Agree on the proposed TPs for F1AP and XnAP respectively in Appendix B and C.

# Conclusions

In this paper we provided further discussion on NR-U related enhancements for MRO and MLB. The following observations and proposals have been derived:

Observation 1: In case of contention-based RA, a target gNB can identify a UE for which an HO attempt failed only after Msg3 is received.

Proposal 1: For MRO purpose, the number of DL LBT failures are not reported from the target node to the source node in case of failed HO attempt.

Observation 2: According to RAN4 specification (TS 38.133), the UE can detect if SMTC occasion are not available, which can impact the HO interruption time and contribute to the HOF.

Proposal 2: RAN3 to send the LS to RAN4 as in Appendix A, to request whether the UE is capable to detect unavailability of the SMTC occasions during HO to a target NR-U cell.

Proposal 3: No further action is needed in RAN3 to address UL LBT failures in case of non-HOF RLF.

Proposal 4: Add a new load metric per NR-U Channel (over F1 and Xn) to indicate the number of consistent uplink LBT failures detected for a certain NR-U channel in a reporting period.

Proposal 5: Agree on the proposed TPs for F1AP and XnAP respectively in Appendix B and C.

# Annex A – LS to RAN4

3GPP TSG-RAN WG3 Meeting #122		TDoc R3-23xxxx

Chicago, USA – November 13th – Nov 17th 2023

Title:	Unavailability of SMTC occasions in NR-U

Release:	Rel.18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	[to be RAN3]

To:	RAN4

Cc:	RAN2

Contact person:	Luca Lunardi

luca.lunardi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

As part of the Rel.18 WI on “Further enhancement of data collection for SON MDT in NR and EN-DC”, RAN3 is discussing the possibility for the UE to include in the RLF-Report, generated upon an RLF due to HO failure, information on the unavailability of the SMTC occasions experienced during HO to a target cell operating in shared spectrum (NR-U).

Q1: RAN3 would like to ask RAN4 whether the UE is capable of detecting the unavailability of the SMTC occasions during an HO to a target NR-U cell?

# 2	Actions

To RAN4

ACTION: 	RAN3 respectfully asks RAN4 to take the above information into account and reply to Q1.

# 3	Dates of next TSG-RAN WG3 meetings

RAN3#123	26th Feb – 1st March 2024		Athens, GR

# Annex B – TP for F1AP

------------------------------------------  Start of changes --------------------------------------------------

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.3.1.x1 | Indicates the radio resource status per NR-U channel. | YES | Ignore |
| >>>> Number of UL Consistent LBT Failures | O |  | INTEGER (0,.. 65535, ...) | Number of UL consistent LBT failures reported by UEs attempting to transmit in the NR-U Channel during the reporting period. | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

------------------------------------------  Next change --------------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-ChannelOccupancyTimePercentageUL,

id-RadioResourceStatusNR-U,

id-NumberOfULConsistentLBTFailures,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

[snip]

------------------------------------------  Next change --------------------------------------------------

#### -- N

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						INTEGER(1..maxnoofNR-UChannelIDs),

channelOccupancyTimePercentageDL		ChannelOccupancyTimePercentage,

energyDetectionThreshold				EnergyDetectionThreshold,

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Item-ExtIEs} } OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore EXTENSION ChannelOccupancyTimePercentage PRESENCE optional}|

{ ID id-RadioResourceStatusNR-U                 CRITICALITY ignore EXTENSION RadioResourceStatusNR-U PRESENCE optional}|

{ ID id-NumberOfULConsistentLBTFailures             CRITICALITY ignore EXTENSION NumberOfULConsistentLBTFailures PRESENCE optional},

...

}

NumberOfULConsistentLBTFailures ::= INTEGER (0..65535,...)

------------------------------------------  Next change --------------------------------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

[snip]

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList                                 ProtocolIE-ID ::= 707

id-RAReportIndicationList							ProtocolIE-ID ::= 900 -- assigned by MCC

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 901 -- assigned by MCC

id-SuccessfulPSCellChangeReportInformationList		ProtocolIE-ID ::= 902 -- assigned by MCC

id-RadioResourceStatusNR-U							ProtocolIE-ID ::= 903 -- assigned by MCC

id-NumberOfULConsistentLBTFailures					ProtocolIE-ID ::= xxx

END

------------------------------------------  End of changes --------------------------------------------------

# Annex C – TP for XnAP

------------------------------------------  Start of changes --------------------------------------------------

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | – |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Indicates the average of the maximum ED Threshold configured by the gNB for UL channel sensing. Value is in dBm. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.2.2.xx | Indicates the radio resource status per NR-U channel. | YES | Ignore |
| >>>> Number of UL Consistent LBT Failures | O |  | INTEGER (0,.. 65535, ...) | Number of UL consistent LBT failures reported by UEs attempting to transmit in the NR-U Channel during the reporting period. | YES | Ignore |

------------------------------------------  Next change --------------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-Full-and-Short-I-RNTI-Profile-List,

id-QosFlowMappingIndication,

id-ChannelOccupancyTimePercentageUL,

id-EnergyDetectionThresholdUL,

id-PSCellListContainer,

id-RadioResourceStatusNR-U,

id-NumberOfULConsistentLBTFailures,

maxEARFCN,

maxnoofAllowedAreas,

------------------------------------------  Next change --------------------------------------------------

#### -- N

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						NR-U-ChannelID,

channelOccupancyTimePercentageDL	ChannelOccupancyTimePercentage,

energyDetectionThreshold			EnergyDetectionThreshold,

iE-Extension		ProtocolExtensionContainer { {NR-U-Channel-Item-ExtIEs} } 	OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore	EXTENSION ChannelOccupancyTimePercentage	PRESENCE optional}|

{ ID id-EnergyDetectionThresholdUL			CRITICALITY ignore	EXTENSION EnergyDetectionThreshold			PRESENCE optional}|

{ ID id-RadioResourceStatusNR-U				CRITICALITY ignore	EXTENSION RadioResourceStatusNR-U 			PRESENCE optional}|

{ ID id-NumberOfULConsistentLBTFailures		CRITICALITY ignore	EXTENSION NumberOfULConsistentLBTFailures 	PRESENCE optional }|,

...

}

NumberOfULConsistentLBTFailures ::= INTEGER (0..65535,...)

------------------------------------------  Next change --------------------------------------------------

### 9.3.7	Constant definitions

[snip]

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-QosFlowMappingIndication																			ProtocolIE-ID ::= 373

id-Full-and-Short-I-RNTI-Profile-List																ProtocolIE-ID ::= 374

id-ChannelOccupancyTimePercentageUL																	ProtocolIE-ID ::= xx1

id-EnergyDetectionThresholdUL																		ProtocolIE-ID ::= xx2

id-SuccessfulPSCellChangeReportInformation															ProtocolIE-ID ::= xx3

id-PSCellListContainer																				ProtocolIE-ID ::= xx4

id-RadioResourceStatusNR-U																			ProtocolIE-ID ::= xx5

id-CPACConfiguration																				ProtocolIE-ID ::= xx6

id-RaReportIndicationList																			ProtocolIE-ID ::= xx7

id-NumberOfULConsistentLBTFailures																				ProtocolIE-ID ::= yyy

END

------------------------------------------  End of changes --------------------------------------------------