3GPP TSG-RAN WG3 #122	R3-237591

Chicago, USA, 13th – 17th November 2023

Source:	CATT

Title:	(TP for XR 38.415 and 38.425) Discussion on PDU set handling for XR

Agenda Item:	25.2.1

Document for:	other

- Introduction

This discussion paper focuses on the remaining issues related to PDU set handling.

- Discussion

  - User Plane Container

This issue is marked as an FFS last meeting:

Enhance existing container or introduce new container to support the PDU Set Parameters on user plane? Identify whether there is any issue to reusing the existing container, if not, as majority companies would prefer to go for the existing container?

And there are two remaining solutions:

Option1: define PSI and EoDB into the existing Frames (e.g frame with PDU Type =0) of NG-U/F1-U Option3: Define a new GTP-U extension PDU Set container for PSI and EoDB

As captured in the text for FFS, Option1 is preferred by a majority of companies. However, there are two technical concerns raised during the offline discussion on Option1, regarding the handover case that the source node, supporting the PDU set handling, forwards directly what it received through NG-U toward the legacy target node. Here we check them one by one.

One concern was that the target node, without the capability to decode the extended fields of PSI and EoDB, cannot identify where the GTP-U extension container ends and the GTP-U payload starts. But in fact, the target node can. Let’s see the very beginning of Clause 5.2 of TS 29.281:

## 5.2 GTP-U Extension Header ### 5.2.1 General format of the GTP-U Extension Header The format of GTP-U Extension Headers is depicted in figure 5.2.1-1. The Extension Header Length field specifies the length of the particular Extension header in 4 octets units. The Next Extension Header Type field specifies the type of any Extension Header that may follow a particular Extension Header. If no such Header follows, then the value of the Next Extension Header Type shall be 0. Octets 1; Extension Header Length. 2 – m; Extension Header Content. m+1; Next Extension Header Type. Figure 5.2.1-1: Outline of the Extension Header Format

Each “Extension Header Content” IE is accompanied by an “Extension Header Length”, making the structure de facto similar to the IE structure with IE ID used in RAN3 Control Plane AP messages (although the design on criticality is different). The target node can use this length to identify the place where the GTP-U extension container ends and the GTP-U payload starts.

Observation 1: For Option1, the target node can use the “Extension Header Length” field to identify the place where the GTP-U extension container ends and the GTP-U payload starts.

The other concern was actually that, yes the target node shall regard the presence bit(s) as spare bit(s) according to the legacy version of RAN3 UP specs, should not interpret the values of the spare bit(s), and shall regard every octets after what it consider meaningful fields as “padding”. However, the length of padding is restricted as of current specs, e.g.:

#### 5.5.3.5 Padding Description: The padding is included at the end of the frame to ensure that the PDU Session user plane protocol PDU length (including padding and the future extension) is (n*4– 2) octets, where n is a positive integer. If there is any future extension, the padding should be added after the future extensions. Field Length: 0–3 octets.

But here in the data forwarding case, the target node may receive a packet with “padding” (in its understanding) longer than 3 octets.

Observation 2: The real problem of Option1 is that, the target node may receive a packet with “padding” (in its understanding) longer than 3 octets.

There is no clear description on how to handle too-long padding. However there is one ambiguous sentence in Clause 5.5.1 of RAN3 UP specs:

The receiver should be able to remove an additional Future Extension field that may be present. See description of Future Extension field in A.1.

So we may understand that, according to the motivation of this sentence, the target node does not check the length of “padding”. If there is any “future extension” encoded by the source node within the “padding”, it is ignored.

Observation 3: It seems that according to the sentence “The receiver should be able to remove an additional Future Extension field that may be present” in Clause 5.5.1 of RAN3 UP specs, receiving “padding” longer than 3 octets should not be treated as an abnormal case.

Therefore Option1 likely works.

Proposal 1: To adopt Option1 on user plane container, i.e. to extend existing frames.

Nevertheless, we acknowledge the possibility that RAN3 delegates cannot converge on Option1 and Option3 is used instead. We would like to remind that the criticality (encoded in the “Next Extension Header Type” field) of the new container should be either “00” or “01”, so that the target node can ignore it.

Proposal 2: If Option1 is not adopted and Option3 is adopted, the criticality bits of the newly-introduced GTP-U container should be either “00” or “01”, so that the target node can ignore it.

  - N6 Jitter Information

There are two FFSs on this topic:

The encoding of the N6 Jitter Information needs further checking from RAN2 progress

FFS how gNB-CU-CP handles the received XR UE assistance information (e.g., UL jitter information, BAT). Whether signalling to gNB-DU is needed.

Here we want to simplify the discussion. Jitter information is a parameter related to data burst arrival pattern, and the pattern of DL can be different form the one of UL, and their uses are roughly separate;

- The UL jitter information, as agreed in RAN2, can only be provided through the UE assistance information. Its main use is for the gNB to determine the UL grant configuration. Moreover, RAN2 has not decided its value range yet.

- The DL jitter information, provided by the core network, is mainly used for the gNB to determine the connected-state DRX.

Considering the different orgins, different uses, and different standardising progress of the jitter information from two directions, we prefer separating them as two topics.

According to the current specification, both the UL grant configuration and the connected-state DRX are determined by the gNB-DU. Therefore both the information from the UE and the information from the core network should be transferred toward the gNB-DU as two separate IE groups.

Proposal 3: Both the XR assistance information provided by the core network and the one provided by the UE should be delivered toward the gNB-DU. They are delivered as two separate IE groups.

However, the gNB-CU can already forward the entire RRC UE assistance information toward the gNB-DU. So there is no need to change the specification to deliver the information from the UE.

Proposal 4: Reuse the existing mechanism for the gNB-CU to forward the UE Assistance information toward the gNB-DU for delivering the UL jitter information and other related information.

- Conclusion

Observation 1: For Option1, the target node can use the “Extension Header Length” field to identify the place where the GTP-U extension container ends and the GTP-U payload starts.

Observation 2: The real problem of Option1 is that, the target node may receive a packet with “padding” (in its understanding) longer than 3 octets.

Observation 3: It seems that according to the sentence “The receiver should be able to remove an additional Future Extension field that may be present” in Clause 5.5.1 of RAN3 UP specs, receiving “padding” longer than 3 octets should not be treated as an abnormal case.

Proposal 1: To adopt Option1 on user plane container, i.e. to extend existing frames.

Proposal 2: If Option1 is not adopted and Option3 is adopted, the criticality bits of the newly-introduced GTP-U container should be either “00” or “01”, so that the target node can ignore it.

Proposal 3: Both the XR assistance information provided by the core network and the one provided by the UE should be delivered toward the gNB-DU. They are delivered as two separate IE groups.

Proposal 4: Reuse the existing mechanism for the gNB-CU to forward the UE Assistance information toward the gNB-DU for delivering the UL jitter information and other related information.

Based on the proposal, we draft 2 TPs as shown in the Annexes. The TPs are in accordance with TS 26.522. The TP for TS 38.425 includes additionally the QFI, although it is marked as FFS since how to support N-flows-to-one-RB mapping is not concluded yet.

- Annex A: TP for TS 38.415 on R3-236006

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.4.1	Transfer of DL PDU Session Information

#### 5.4.1.1	Successful operation

The purpose of the Transfer of DL PDU Session Information procedure is to send control information elements related to the PDU Session from UPF to NG-RAN.

In the case of uplink and downlink data forwarding the DL PDU Session Information procedure shall also be used to send control information elements related to the PDU Session from NG-RAN node to UPF, or from UPF to NG-RAN node, or between NG-RAN nodes.

A PDU Session user plane instance making use of the Transfer of DL PDU Session Information procedure is associated to a single PDU Session. The Transfer of DL PDU Session Information procedure may be invoked whenever packets for that particular PDU Session need to be transferred across the related interface instance.

The DL PDU SESSION INFORMATION frame includes a QoS Flow Identifier (QFI) field associated with the transferred packet. The NG-RAN shall use the received QFI to determine the QoS flow and QoS profile which are associated with the received packet.

The DL PDU SESSION INFORMATION frame shall include the Reflective QoS Indicator (RQI) field to indicate whether user plane Reflective QoS shall be activated or not. The NG-RAN shall, if RQA has been configured for the involved QoS flow as specified in TS 38.413 [7], take the RQI into account as specified in TS 37.324 [4].

The DL PDU SESSION INFORMATION frame may also include a Paging Policy Indicator (PPI) field associated with the transferred packet. The NG-RAN shall use the received PPI to determine the paging policy differentiation which is associated with the received packet as described in TS 23.501 [5].

The DL PDU SESSION INFORMATION frame may also include a QoS Monitoring Packet (QMP) field and a DL sending time stamp field. The NG-RAN shall, if QoS monitoring has been configured for the included QFI field, perform delay measurement and QoS monitoring, as specified in TS 23.501 [5].

The DL PDU SESSION INFORMATION frame may also include a DL QFI Sequence Number field associated with the transferred packet. The NG-RAN shall, if the QoS flow has been configured eligible for redundant transport bearer in TS 38.413 [6], use the received DL QFI Sequence Number field to determine and eliminate duplicated packets for a given QoS flow as specified in TS 23.501 [5].

The DL PDU SESSION INFORMATION frame may also include a DL MBS QFI Sequence Number field associated with the transferred packet. The NG-RAN shall use the received DL MBS QFI Sequence Number field to determine the PDCP count that should be used when transferring the packet over the radio as specified in TS 38.300 [2].

The DL PDU SESSION INFORMATION frame may also include an End PDU of the PDU Set (EPDU), an End of Data Burst (EDB) field, a PDU Set Importance (PSI) field, a PDU Set Sequence Number (PSSN) field and a PDU Sequence Number within a PDU Set (PSN) field. Additionally, the DL PDU SESSION INFORMATION frame may also include a PDU Set Size (PSSize) field. The NG-RAN shall use them for PDU Set handling as specified in TS 38.300 [2].

When needed, the NG-RAN shall propagate the DL PDU Session Information to a peer NG-RAN.

Figure 5.4.1.1-1: Successful Transfer of DL PDU Session Information

#### 5.4.1.2	Unsuccessful operation

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| PDU Type (=0) |  |  |  | QMP | SNP | MSNP |  | EPBP | 1 |
| PPP | RQI | QoS Flow Identifier |  |  |  |  |  |  | 1 |
| PPI |  |  | Spare |  |  |  |  |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS QFI Sequence Number |  |  |  |  |  |  |  |  | 0 or 4 |
| PSPP | PSSP | Spare |  |  |  |  |  |  | 0 or 1 |
| EPDU | End of Data Burst |  |  | PDU Set Importance |  |  |  |  | 0 or 3 |
| PDU Set Sequence Number (PSSN) |  |  |  |  |  |  |  |  |  |
| PSSN continue |  | PDU Sequence Number within a PDU Set |  |  |  |  |  |  |  |
| PDU Set Size |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.y1	Extension Presence Byte Presence (EPBP)

Description: For DL PDU Session Information frame, this parameter indicates the presence of the byte including the PDU Set Parameters Presence (PSPP) and the PDU Set Size Presence (PSSP).

Value range: {0= not present, 1= present}.

Field length: 1 bit.

#### 5.5.3.y2	PDU Set Parameters Presence (PSPP)

Description: For DL PDU Session Information frame, this parameter indicates the presence the End PDU of the PDU Set (EPDU), the End of Data Burst (EDB), the PDU Set Importance (PSI), the PDU Set Sequence Number (PSSN) and the PDU Sequence Number within a PDU Set (PSN).

Value range: {0= all absent, 1= all present}.

Field length: 1 bit.

#### 5.5.3.y3	PDU Set Size Presence (PSSP)

Description: For DL PDU Session Information frame, this parameter indicates the presence the PDU Set Size (PSSize).

Value range: {0= PDU Set Size not present, 1= PDU Set Size present}.

Field length: 1 bit.

#### 5.5.3.y4	End PDU of the PDU Set (EPDU)

Description: This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set.

Value range: {0= not the last PDU per PDU Set, 1= last PDU per PDU Set}.

Field length: 1 bit.

#### 5.5.3.y5	End of Data Burst (EDB)

Description: FFS.

Value range: {0..23-1}.

Field length: 3 bits.

#### 5.5.3.y6	PDU Set Importance (PSI)

Description: This field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

Value range: {0..24-1}.

Field length: 4 bits.

#### 5.5.3.y7	PDU Set Sequence Number (PSSN)

Description: The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set.

Value range: {0..210-1}.

Field length: 10 bits.

NOTE:	This value wraps around at 1023, how a receiver may uniquely distinguish PDU Sets is FFS.

#### 5.5.3.y8	PDU Sequence Number within a PDU Set (PSN)

Description: The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..26-1}.

Field length: 6 bits.

NOTE:	This value wraps around at 63, how a receiver may uniquely packets within a PDU Set is FFS.

#### 5.5.3.y9	PDU Set Size (PSSize)

Description: The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. If the Application Server is not able to determine the PDU Set Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs. The PSSize is expressed in bytes.

Value range: {0..224-1}.

Field length: 3 octets.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- Annex B: TP for TS 38.425 on R3-236007

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

If the Request OutOfSeq Report is set to 1, the corresponding node shall, if supported, include the NR PDCP PDU sequence number successfully delivered out of sequence in the DL DATA DELIVERY STATUS to the node hosting the NR PDCP entity.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

The DL DATA DELIVERY STATUS frame may also include a QoS flow Identifier, an End PDU of the PDU Set (EPDU), an End of Data Burst (EDB) field, a PDU Set Importance (PSI) field, a PDU Set Sequence Number (PSSN) field and a PDU Sequence Number within a PDU Set (PSN) field. Additionally, the DL DATA DELIVERY STATUS frame may also include a PDU Set Size (PSSize) field. The corresponding node shall use them for PDU Set handling as specified in TS 38.300 [2].

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

#### 5.4.1.2	Unsuccessful operation

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  | QFIP | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare |  |  | Request OutofSeq Report |  | Report Delivered | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| PSPP | PSSP | QoS Flow Identifier [FFS how to support N:N:1] |  |  |  |  |  |  | 0 or 1 |
| EPDU | End of Data Burst |  |  |  | PDU Set Importance |  |  |  | 0 or 3 |
| PDU Set Sequence Number (PSSN) |  |  |  |  |  |  |  |  |  |
| PSSN continue |  | PDU Sequence Number within a PDU Set |  |  |  |  |  |  |  |
| PDU Set Size |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

NOTE 1:	In this section, information elements are also applicable to E-UTRA PDCP unless specified otherwise. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP unless specified otherwise.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.y0	QoS Flow Identifier Presence (QFIP)

Description: This parameter indicates the presence of the QoS Flow Identifier (QFIP), the PDU Set Parameters Presence (PSPP) and the PDU Set Size Presence (PSSP).

Value range: {0= not present, 1= present}.

Field length: 1 bit.

#### 5.5.3.y1	QoS Flow Identifier (QFI)

Description: This field indicates the QoS Flow Identifier of the QoS flow to which the transferred packet belongs.

Value range: {0..26-1}.

Field length: 6 bits.

#### 5.5.3.y2	PDU Set Parameters Presence (PSPP)

Description: For DL PDU Session Information frame, this parameter indicates the presence the End PDU of the PDU Set (EPDU), the End of Data Burst (EDB), the PDU Set Importance (PSI), the PDU Set Sequence Number (PSSN) and the PDU Sequence Number within a PDU Set (PSN).

Value range: {0= all absent, 1= all present}.

Field length: 1 bit.

#### 5.5.3.y3	PDU Set Size Presence (PSSP)

Description: For DL PDU Session Information frame, this parameter indicates the presence the PDU Set Size (PSSize).

Value range: {0= PDU Set Size not present, 1= PDU Set Size present}.

Field length: 1 bit.

#### 5.5.3.y4	End PDU of the PDU Set (EPDU)

Description: This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set.

Value range: {0= not the last PDU per PDU Set, 1= last PDU per PDU Set}.

Field length: 1 bit.

#### 5.5.3.y5	End of Data Burst (EDB)

Description: FFS.

Value range: {0..23-1}.

Field length: 3 bits.

#### 5.5.3.y6	PDU Set Importance (PSI)

Description: This field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

Value range: {0..24-1}.

Field length: 4 bits.

#### 5.5.3.y7	PDU Set Sequence Number (PSSN)

Description: The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set.

Value range: {0..210-1}.

Field length: 10 bits.

NOTE:	This value wraps around at 1023, how a receiver may uniquely distinguish PDU Sets is FFS.

#### 5.5.3.y8	PDU Sequence Number within a PDU Set (PSN)

Description: The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..26-1}.

Field length: 6 bits.

NOTE:	This value wraps around at 63, how a receiver may uniquely packets within a PDU Set is FFS.

#### 5.5.3.y9	PDU Set Size (PSSize)

Description: The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. If the Application Server is not able to determine the PDU Set Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs. The PSSize is expressed in bytes.

Value range: {0..224-1}.

Field length: 3 octets.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>