3GPP TSG RAN Meeting #102	RP-232789

Edinburgh, GB, December 11 – 15, 2023	(revision of RP-230077)

- Source:	OPPO

- Title:	WID revision: NR sidelink evolution

- Document for:	Approval

Agenda Item:	9.3.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	NR sidelink evolution

## Acronym: NR_SL_enh2

## Unique identifier: 	 940097

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720003 | Study on Enhancement of 3GPP support for V2X services (FS_eV2X) | Rel-15 Stage 1 study on eV2X service requirements |
| 750002 | Enhancement of 3GPP support for V2X scenarios (eV2X) | Rel-15 Stage 1 work on eV2X service requirements |
| 790002 | Study on Improvement of V2X Service Handling (FS_V2XIMP) | Rel-16 SA1 study item on eV2X service requirements |
| 820024 | Improvement of V2X service Handling (V2XIMP) | Rel-16 SA1 work item on eV2X service requirements |
| 760043 | Study on architecture enhancements for 3GPP support of advanced V2X services (FS_eV2XARC) | Rel-15/16 SA2 study item on eV2X architecture |
| 820018 | Architecture enhancements for 3GPP support of advanced V2X services | Rel-16 SA2 work item on eV2X architecture |
| 800096 | Study on NR Vehicle-to-Everything (FS_NR_V2X) | Rel-16 RAN study item on NR V2X feasibility |
| 830078 | 5G V2X with NR sidelink (5G_V2X_NRSL) | Rel-16 RAN work item on 5G V2X |
| 840042 | New WID on Complete Gap Analysis for Railways (MONASTERYEND) | Rel-17 Stage-1 PC5 communication requirement from Railway perspective |
| 840044 | Network Controlled Interactive Service (NCIS) | Rel-17 PC5 interface stage-1 requirement for interactive services |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage (REFEC) | Rel-17 Stage-1 requirement for multi hop relay |
| 840045 | Audio-Visual Service Production (AVPROD) | Rel-17 Stage-1 requirement including AV production for PC5 |
| 850013 | Study on architecture enhancements for 3GPP support of advanced V2X services – Phase 2 (FS_eV2XARC_Ph2) | Rel-17 SA2 study item on eV2X architecture – Phase 2 |
| 830033 | Study on System enhancement for Proximity based Services in 5GS (FS_5G_ProSe) | Rel-17 SA2 study item on ProSe system |
| 860042 | NR Sidelink enhancement | Rel-17 RAN work item on sidelink enhancement |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

## 3	Justification

In Rel-16, sidelink communication was developed in RAN mainly to support advanced V2X applications. In Rel-17, SA2 studied and standardized Proximity based service including public safety and commercial related service. As part of Rel-17, power saving solutions (e.g., partial sensing, DRX) and inter-UE coordination have been developed in RAN1 and RAN2 to improve power consumption for battery limited terminals and reliability of sidelink transmissions.

Although NR sidelink was initially developed for V2X applications, there is growing interest in the industry to expand the applicability of NR sidelink to commercial use cases. For commercial sidelink applications, two key requirements have been identified:

- Increased sidelink data rate

- Support of new carrier frequencies for sidelink

Increased sidelink data rate is motivated by applications such as sensor information (video) sharing between vehicles with high degree of driving automation. Commercial use cases could require data rates in excess of what is possible in Rel-17. Increased data rate can be achieved with the support of sidelink carrier aggregation and sidelink over unlicensed spectrum. Furthermore, by enhancing the FR2 sidelink operation, increased data rate can be more efficiently supported on FR2. While the support of new carrier frequencies and larger bandwidths would also allow to improve its data rate, the main benefit would come from making sidelink more applicable for a wider range of applications. More specifically, with the support of unlicensed spectrum and the enhancement in FR2, sidelink will be in a better position to be implemented in commercial devices since utilization of the ITS band is limited to ITS safety related applications.

Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop.

## 4	Objective

### Objective of SI or Core part WI or Testing part WI

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN3, RAN4]

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to intra-band CA for the ITS band in FR1 (Band n47).

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Only Mode 2 operation

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Time resources for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

- No consideration for limited transmission and reception capability

- No primary/secondary carrier differentiation

- Reuse the LTE sidelink CA design for the following aspects:

  - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication

- The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18.

- Note: The SL CA work in Rel-18 mainly targets some V2X use cases

- Study (it was concluded that the principle of NR-U channel access mechanisms was feasible for SL) and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- Focus on FR1 unlicensed bands (n46 and n96/n102).

- Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] (A summary of study outcomes from RAN1 and RAN2 is available in RP-232787)

- Update the evaluation methodology for commercial deployment scenario. [RAN1]

- Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2]

  - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

- Study (it was concluded that semi-static resource pool partitioning is feasible and dynamic resource pool sharing is needed) and specify, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A

- RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz.

  - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer

    - No change to the R16/17 resource allocation procedure in PHY due to this restriction

    - The existing SL slot structure from Rel-16 is unchanged

    - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe

  - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain

    - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered.

  - Mode 2 operation only

- UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4]

- The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are determined based on company input.

- The frequency band combination is limited to the ITS-dedicated spectrum only (Band n47) for contiguous carrier aggregation.

- Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements.

- Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

- UE RRM core requirement for the new features introduced in this WI [RAN4]

Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the following requirements [RAN4]

- UE demodulation performance requirements

- UE RRM performance requirements

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal | TR38.786 | User Equipment (UE) radio transmission and reception for NR sidelink evolution | RAN#101 | RAN#102 | Core Part Led by RAN4 WG Editor: Jinqiang Xing, OPPO xingjinqiang@oppo.com |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.201 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 38.211 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 38.212 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 38.213 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 38.214 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 38.215 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 37.213 | Introduction of NR sidelink evolution | RAN#101 | Core Part |
| 38.300 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.304 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.306 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.321 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.322 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.323 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.331 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.101-1 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.101-2 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.101-3 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.101-4 | Introduction of NR sidelink evolution | RAN#104 | Performance Part |
| 38.133 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.133 | Introduction of NR sidelink evolution | RAN#104 | Performance Part |
| 37.985 | Introduction of NR sidelink evolution | RAN#102 | Core Part |
| 38.473 | Introduction of NR sidelink evolution | RAN#102 | Core Part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Primary rapporteur

Kevin Lin

Company: OPPO

Email: Kevin.Lin@oppo.com

Secondary rapporteur

Seungmin Lee

Company: LG Electronics

Email: edison.lee@lge.com

## 7	Work item leadership

Primary: RAN1

Secondary: RAN2, RAN4

## 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

OPPO

Apple

CAICT

CATT

CEWiT

Continental Automotive Technologies GmbH

ETRI

Fraunhofer HHI

Fraunhofer IIS

Fujitsu

FUTUREWEI

GOHIGH

HiSilicon

HONOR

Huawei

III

Intel

InterDigital

Lenovo

LG Electronics

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

Panasonic

Philips

Qualcomm Inc.

Samsung

Sanechips

SHARP

Sony

Spreadtrum Communications

Transsion Holdings

Verizon

Vivo

Xiaomi

ZTE Corporation