3GPP TSG RAN3 Meeting#122	R3-237149

Chicago, USA, 13 - 17 November 2023

3GPP TSG-RAN WG4 Meeting #108bis		R4-2317390

Xiamen, China, 09 – 13 October 2023

Title:	LS on report mapping for positioning measurements with PRS_SRS bandwidth aggregation

Response to:	N/A

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	RAN4

To:	RAN2, RAN3

Cc:	RAN1

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments:	N/A

1. Overall Description:

RAN4#108bis agreed on the following report mapping tables for DL-RSTD, UE Rx-Tx, gNB Rx-Tx, and UL-RTOA measurements for k = {-1, -2}.

Table 1. Absolute RSTD measurement report mapping for k=-1.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023.5 | Tc |
| RSTD_0000002 | -985023.5 RSTD < -985023 | Tc |
|  |  | … |
| RSTD_1970048 | -0.5 RSTD < 0 | Tc |
| RSTD_1970049 | 0 RSTD < 0.5 | Tc |
| … | … | … |
| RSTD_3940095 | 985023 RSTD < 985023.5 | Tc |
| RSTD_3940096 | 985023.5 RSTD < 985024 | Tc |
| RSTD_3940097 | 985024 RSTD | Tc |

Table 2. Absolute RSTD measurement report mapping for k=-2.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023.75 | Tc |
| RSTD_0000002 | -985023.75 RSTD < -985023.5 | Tc |
|  |  | … |
| RSTD_3940096 | -0.25 RSTD < 0 | Tc |
| RSTD_3940097 | 0 RSTD < 0.25 | Tc |
| … | … | … |
| RSTD_7880191 | 985023.5 RSTD < 985023.75 | Tc |
| RSTD_7880192 | 985023.75 RSTD < 985024 | Tc |
| RSTD_7880193 | 985024 RSTD | Tc |

Table 3. Differential RSTD measurement report mapping for k=-1.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 0.5 | Tc |
| DIFFRSTD_0001 | 0.5 RSTD < 1 | Tc |
| DIFFRSTD_0002 | 1 RSTD < 1.5 | Tc |
|  |  | … |
| DIFFRSTD_16380 | 8190 RSTD < 8190.5 | Tc |
| DIFFRSTD_16381 | 8190.5 RSTD < 8191 | Tc |
| DIFFRSTD_16382 | 8191 RSTD | Tc |

Table 4. Differential RSTD measurement report mapping for k=-2.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 0.25 | Tc |
| DIFFRSTD_0001 | 0.25 RSTD < 0.5 | Tc |
| DIFFRSTD_0002 | 0.5 RSTD < 0. 75 | Tc |
|  |  | … |
| DIFFRSTD_32762 | 8190.5 RSTD < 8190.75 | Tc |
| DIFFRSTD_32763 | 8190.75 RSTD < 8191 | Tc |
| DIFFRSTD_32764 | 8191 RSTD | Tc |

Table 5. Additional path report mapping for DL RSTD for k=-1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 32699 | 8174 path < 8174.5 | Tc |
| path_ 32700 | 8174.5 path < 8175 | Tc |
| path_ 32701 | 8175 path | Tc |

Table 6. Additional path report mapping for DL RSTD for k=-2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_32700 | -0.5 path < 0 | Tc |
| path_32701 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 65399 | 8174 path < 8174.5 | Tc |
| path_ 65400 | 8174.5 path < 8175 | Tc |
| path_ 65401 | 8175 path | Tc |

Table 7. Absolute UE Rx-Tx time difference measurement report mapping for k=-1.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985023.5 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985023.5  TUE Rx-Tx < -985023 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_1970048 | -0.5  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_1970049 | 0  TUE Rx-Tx < 0.5 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_3940095 | 985023  TUE Rx-Tx < 985023.5 | Tc |
| RX-TX_TIME_DIFFERENCE_3940096 | 985023.5  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_3940097 | 985024  TUE Rx-Tx | Tc |

Table 8. Absolute UE Rx-Tx time difference measurement report mapping for k=-2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985023.75 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985023.75  TUE Rx-Tx < -985023.5 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_3940096 | -0.25  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_3940097 | 0  TUE Rx-Tx < 0.25 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_7880191 | 985023.5  TUE Rx-Tx < 985023.75 | Tc |
| RX-TX_TIME_DIFFERENCE_7880192 | 985023.75  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_7880193 | 985024  TUE Rx-Tx | Tc |

Table 9. Differential UE Rx-Tx time difference measurement report mapping for k=-1.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 0.5 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 0.5  TUE Rx-Tx < 1 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 1  TUE Rx-Tx < 1.5 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_16380 | 8190  TUE Rx-Tx < 8190.5 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_16381 | 8190.5  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_16382 | 8191  TUE Rx-Tx | Tc |

Table 10. Differential UE Rx-Tx time difference measurement report mapping for k=-2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 0.25 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 0.25  TUE Rx-Tx < 0.5 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 0.5  TUE Rx-Tx < 0.75 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_32762 | 8190.5  TUE Rx-Tx < 8190.75 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_32763 | 8190.75  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_32764 | 8191  TUE Rx-Tx | Tc |

Table 11. Additional path report mapping for UE Rx-Tx time difference measurement for k=-1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 32699 | 8174 path < 8174.5 | Tc |
| path_ 32700 | 8174.5 path < 8175 | Tc |
| path_ 32701 | 8175 path | Tc |

Table 12. Additional path report mapping for UE Rx-Tx time difference measurement for k=-2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_32700 | -0.5 path < 0 | Tc |
| path_32701 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 65399 | 8174 path < 8174.5 | Tc |
| path_ 65400 | 8174.5 path < 8175 | Tc |
| path_ 65401 | 8175 path | Tc |

Table 13.UL-RTOA measurement report mapping for k=-1.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA _0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.5 | Tc |
| UL_RTOA_0000002 | -985023.5 UL_RTOA < -985023 | Tc |
|  |  | … |
| UL_RTOA_1970048 | -0.5 UL_RTOA < 0 | Tc |
| UL_RTOA_1970049 | 0 UL_RTOA < 0.5 | Tc |
| … | … | … |
| UL_RTOA_3940095 | 985023 UL_RTOA < 985023.5 | Tc |
| UL_RTOA_3940096 | 985023.5 UL_RTOA < 985024 | Tc |
| UL_RTOA_3940097 | 985024 UL_RTOA | Tc |

Table 14. UL-RTOA measurement report mapping for k=-2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.75 | Tc |
| UL_RTOA_0000002 | -985023.75 UL_RTOA < -985023.5 | Tc |
|  |  | … |
| UL_RTOA_3940096 | -0.25 UL_RTOA < 0 | Tc |
| UL_RTOA_3940097 | 0 UL_RTOA < 0.25 | Tc |
| … | … | … |
| UL_RTOA_7880191 | 985023.5 UL_RTOA < 985023.75 | Tc |
| UL_RTOA_7880192 | 985023.75 UL_RTOA < 985024 | Tc |
| UL_RTOA_7880193 | 985024 UL_RTOA | Tc |

Table 15. Additional path report mapping for UL-RTOA measurement for k=-1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 32699 | 8174 path < 8174.5 | Tc |
| path_ 32700 | 8174.5 path < 8175 | Tc |
| path_ 32701 | 8175 path | Tc |

Table 16. Additional path report mapping for UL-RTOA measurement for k=-2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_32700 | -0.5 path < 0 | Tc |
| path_32701 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 65399 | 8174 path < 8174.5 | Tc |
| path_ 65400 | 8174.5 path < 8175 | Tc |
| path_ 65401 | 8175 path | Tc |

Table 17. gNB Rx-Tx time difference measurement report mapping for k=-1.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_ 0000 | -985024 > RX-TX | Tc |
| RX-TX_ 0001 | -985024  RX-TX < -985023.5 | Tc |
| RX-TX_0002 | -985023.5  RX-TX < -985023 | Tc |
|  |  | … |
| RX-TX_ 1970048 | -0.5  RX-TX < 0 | Tc |
| RX-TX_ 1970049 | 0  RX-TX < 0.5 | Tc |
| … | … | … |
| RX-TX_ 3940095 | 985023  RX-TX < 985023.5 | Tc |
| RX-TX_ 3940096 | 985023.5  RX-TX < 985024 | Tc |
| RX-TX_ 3940097 | 985024  RX-TX | Tc |

Table 18. gNB Rx-Tx measurement report mapping for k=-2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX _0000 | -985024 > RX-TX | Tc |
| RX-TX_ 0001 | -985024  RX-TX < -985023.75 | Tc |
| RX-TX_ 0002 | -985023.75  RX-TX -985023.5 | Tc |
|  |  | … |
| RX-TX_ 3940096 | -0.25  RX-TX < 0 | Tc |
| RX-TX_ 3940097 | 0  RX-TX < 0.25 | Tc |
| … | … | … |
| RX-TX_ 7880191 | 985023.5  RX-TX < 985023.75 | Tc |
| RX-TX_ 7880192 | 985023.75  RX-TX < 985024 | Tc |
| RX-TX_ 7880193 | 985024  RX-TX | Tc |

Table 19. Additional path report mapping for gNB Rx-Tx measurement for k=-1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 32699 | 8174 path < 8174.5 | Tc |
| path_ 32700 | 8174.5 path < 8175 | Tc |
| path_ 32701 | 8175 path | Tc |

Table 20. Additional path report mapping for gNB Rx-Tx measurement for k=-2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_32700 | -0.5 path < 0 | Tc |
| path_32701 | 0 path < 0.5 | Tc |
| … | … | … |
| path_ 65399 | 8174 path < 8174.5 | Tc |
| path_ 65400 | 8174.5 path < 8175 | Tc |
| path_ 65401 | 8175 path | Tc |

2. Actions:

To RAN2 and RAN3.

ACTION: 	RAN4 kindly requests RAN2 and RAN3 to take the above agreement on report mapping tables for DL-RSTD, UE Rx-Tx, gNB Rx-Tx, and UL-RTOA measurements into account and design the necessary signaling support for k = {-1, -2}.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #109			13  – 17 November 2023					Chicago, US

TSG RAN WG4 Meeting #110			26 February – 01 March 2023			Athens, GR