3GPP TSG RAN3 Meeting#122	R3-237151

Chicago, USA, 13 - 17 November 2023

3GPP SA WG2 Meeting #159	S2-2311359

Xiamen, China, 09 - 13 October, 2023

Title:	Reply LS on INACTIVE eDRX above 10.24sec and SDT

Response to:	LS on INACTIVE eDRX above 10.24sec and SDT (S2-2311527/ R3-235765)

Release:	Rel-18

# Work Item:	     NR_REDCAP_Ph2, NR_redcap_enh-Core, NR_MT_SDT-Core

Source:	SA2

To:	RAN3, CT4

Cc:	RAN2

Contact Person:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN3 for their reply LS (R3-235765/ S2-2311527) on “INACTIVE eDRX above 10.24sec and SDT”.

Based on the following RAN3 feedback:

- RAN3 discussed the solution recommended by SA2 and the input from CT4 and prefers the possibility of AMF providing the most recent buffered data size per QFI at the end of the long eDRX cycle in the RAN PAGING REQUEST message, which can provide accurate information to NG-RAN for applying MT-SDT during paging decision. In case the provision of such information is deemed too complex from CT4 and SA2 point of views, RAN3 will consider the possibility of less optimized alternatives at next meeting.

SA2 answer:

SA2 discussed potential solutions and would like to provide the following feedback:

- SA2 has concluded that providing accurate information to NG-RAN regarding the most recent cumulative data size of data buffered per QFI in the UPF has some impact on the N4 interface. Namely, the N4 procedures today (as SA2 understands them) allow the UPF to send the Downlink Data Notification only upon arrival of the first packet, which is insufficient for providing accurate information on the cumulative data size of data buffered per QFI at the UPF at the end of the long eDRX cycle.

SA2 would like to kindly ask CT4 to indicate whether SA2’s understanding of the N4 procedures today is correct and to provide feedback on the feasibility of N4 procedure enhancements that would allow for more accurate provision of the cumulative data size of the data buffered per QFI at the UPF, while taking into account the cost of such enhancements in term of specification impact and the incurred additional N4 signalling load.

2. Actions:

To CT4

ACTION: 	SA2 kindly asks to indicate whether SA2’s understanding of the N4 procedures today is correct and to provide feedback on the feasibility of N4 procedure enhancement that would allow for more accurate provision of the cumulative data size of the data buffered per QFI at the UPF, while taking into account the cost of such enhancements in term of specification impact and the incurred additional N4 signalling load.

To RAN3

ACTION: 	SA2 kindly asks RAN3 the take the information provided above into account.

3. Date of Next SA2 Meeting:

SA2#160		November 13 - 17, 2023 				Chicago, US