3GPP TSG-RAN WG3 Meeting #123	R3-240740

Athens, Greece, 26 February – 01 March, 2024

Agenda item:	9.1.3.1

Source:	Nokia, Nokia Shanghai Bell, AT&T

Title:	Discussion of incoming LS on the progress update of AI/ML Management specifications in SA5

Document for:	Discussion and Decision

# 1	Introduction

SA5 sent an LS [1] to RAN1, RAN2, RAN3 and SA2 groups to inform them about the progress of SA5 related to AI/ML Management, addressing OAM support for the AI/ML features and capabilities in 5GS including the RAN domain. In Rel-18 SA5 conducted a study, captured in TR 28.908, while the Rel-18 work is captured in TS 28.105. TR 28.908 captures management related use cases related to AI/ML training, AI/ML inference and ML model deployment which are further categorized to the four operational phases for the Life Cycle Management of ML Model including:

- ML training phase (which also includes validation & testing),

- AI/ML emulation phase,

- ML deployment phase, and

- AI/ML inference phase.

In particular, the SA5 work on AI/ML management focuses on the enablement and support of AI/ML capabilities in the network side, including:

- Supporting ML model provisioning for gNB inference, including:

- Management of AI/ML-based distributed Network Energy Saving,

- Management of AI/ML-based distributed Mobility Optimization,

- Management of AI/ML-based distributed Load Balancing.

- Monitoring the performance of AI/ML capabilities in the network, including:

- Performance management for ML training function,

- Performance management for AI/ML inference function.

SA5 is defining at the moment the scope of their Rel-19 study for enhancements of AI/ML management. In their LS to RAN3, SA5 asks if there are any requirements from RAN3 related to management and orchestration of AI/ML functions and models to support AI/ML capabilities in the network side.

# 2	Discussion

In the Rel-18 NG-RAN AI/ML work, RAN3 considers two possible scenarios for the deployment of AI/ML in the NG-RAN:

- AI/ML Model Training in the OAM and AI/ML Model Inference in the NG-RAN node

- AI/ML Model Training and AI/ML Model Inference in the NG-RAN node

For the case when an AI/ML Model is trained in the OAM and subsequently deployed in the NG-RAN, RAN3 sent an LS to SA5 [2] to provide more details to SA5 regarding the RAN3 AI/ML framework. Specifically, RAN3 indicated to SA5 that AI/ML algorithms and models are out of RAN3 scope and that Model Deployment/Update concerning those AI/ML Models to the NG-RAN is up to OAM when needed. Based on these requirements from RAN3, SA5 in Rel-18 provided enablement for the support of AI/ML Model provisioning for gNB inference. In addition, SA5 enabled the monitoring of UE Performance for AI/ML Model Training and Inference function.

Rel-18 work in RAN3 focused on standardization impacts over the Xn interface with respect to the exchange of information for the support of AI/ML in the NG-RAN. In this work, RAN3 did not identify any requirements towards SA5 related to management and orchestration of AI/ML functions and models to support AI/ML capabilities in the network side.

Proposal: RAN3 would like to inform SA5 that RAN3 has not identified any requirements towards SA5 related to management and orchestration of AI/ML functions and models to support AI/ML capabilities in the network side.

We have submitted a draft reply LS to SA5 in the Annex of this paper.

# 3	Conclusion

In this contribution we make the following proposal:

Proposal: RAN3 would like to inform SA5 that RAN3 has not identified any requirements towards SA5 related to management and orchestration of AI/ML functions and models to support AI/ML capabilities in the network side.

# References

- R3-240032 - S5-238107, LS on the progress update of AI/ML Management specifications in SA5, SA5

- R3-214481, LS to SA5 on model deployment and update from OAM to NG-RAN, RAN3

# Annex

3GPP TSG-RAN WG3 Meeting #123	R3-24xxxx

Athens, Greece, 26 February – 01 March, 2024

Title:	[Draft] Reply LS to SA5 on the progress update of AI/ML Management specifications in SA5

Response to:	S5-238107 – R3-240032

Release:	Release 18

Work Item:	NR_AIML_NGRAN-Core

Source:	Nokia [to be RAN3]

To:	SA5

Cc:	RAN1, RAN2, SA2

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA5 for their LS.

Rel-18 work in RAN3 focused on exchange of information between NG-RAN nodes for the support of AI/ML in the NG-RAN. In the Rel-18 work, RAN3 has not identified any requirements towards SA5 related to management and orchestration of AI/ML functions and models to support AI/ML capabilities in the network side.

2. Actions:

To SA5 : 	RAN3 would like to inform SA5 that RAN3 has not identified any requirements in Rel-18 work towards SA5 related to management and orchestration of AI/ML functions and models to support AI/ML capabilities in the network side.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3#123-bis		15 April – 19 April, 2024			     Changsha, China

3GPP TSG RAN WG3#124		20 May – 24 May, 2024				Fukuoka, Japan