3GPP TSG-RAN WG3 Meeting #125bis	R3-245050

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	13.2

Source:	ZTE Corporation

Title:	Discussion on inter-CU LTM

Document for:	Discussions & Approval

- Introduction

In the last RAN3 #125meeting, we have the following progress.

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure

Meanwhile, we also agree a TP R3-244836, with some open issues.

In this paper, we would like to further discuss on the open issues for inter-CU LTM.

- Discussion

  - Open issues on LTM BLCR 38.300

    - One or more candidate cells within each LTM configuration

Agreed TP R3-244836 to TS38.300 3. The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). Editor’s Note: FFS on whether one candidate cell or a list of candidate cells is contained in the message.

We have agreed to enhance current XnAP: Handover preparation procedure for inter-CU LTM. However, we have not achieved consensus on either one candidate cell or multiple candidate cell within the handover request message.

For the L3 (Conditional) Handover preparation procedure, there is only one candidate cell within the HO request message. The configuration procedure is triggered per cell.

So, we propose to reuse the same way, to avoid unnecessary complexity and enhancement, i.e., one handover request message includes one candidate cell for inter-CU LTM, which is aligned with the cell specific information provided in the handover messages.

Proposal 1: Parallel procedures are used to prepare LTM candidate configurations, i.e., each handover preparation procedure is for one LTM candidate cell.

    - LTM Modification

Agreed TP R3-244836 to TS38.300 6. The source gNB requests LTM configuration update to the candidate gNB(s). Editor’s Note: Details are FFS on step 6. 7. The candidate gNB(s) sends the response (LTM CONFIGURATION UPDATE ACKNOWLEDGE) to the source gNB. Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional and FFS on the message name.

In R16 CHO, we decided to reuse the HANDOVER REQUEST message to replace a prepared CHO candidate configuration by including the CHO trigger IE set to "CHO-replace". For LTM, the source node may decide to modify an already prepared LTM candidate cell in the candidate gNB, e.g., the source gNB may update the LTM reference configuration and provide the updates to the candidate gNB.

More, we shall also use LTM modification procedure to signal Common CSI resource configuration.

RAN2 agreement

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Based on the RAN2 agreement as above, regarding how to share the candidate cells information across multiple gNB-CUs/gNB-DUs, the similar coordination procedure for intra-CU LTM can be reused for inter-CU LTM by extending the interaction between source gNB-CU and candidate gNB-CU via Xn messages.

In current TS38.423, the code point “CHO-replace” in HANDOVER REQUEST message, is used to modify the prepared Handover produce.  However, in order for easy extension, we prefer to introduce a new Class1 procedure.

For the future proof, the name of LTM configuration update is suggested to be changed into Configuration update.

Proposal 2: Introduce a new UE associated Class 1 message, name as e.g., Configuration Update procedure.

This procedure is used to perform e.g., LTM modification including notification of common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs, so that, it shall be mandatory for support of subsequent LTM.

Proposal 3: The new Class 1 Configuration Update procedure is mandatory for LTM.

    - XnAP TA INFORMATION TRANSFER message

Agreed TP R3-244836 to TS38.300 10/11. Early TA acquisition to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) send the TA INFORMATION TRANSFER message to the source gNB. Editor’s Note: Details are FFS on step 10/11.

The UE may perform UL synchronization (e.g. UE-based TA measurement, PDCCH order triggered early RACH) with the candidate cell(s) before receiving the cell switch command, if indicated by the NW. Taking the PDCCH order triggered early RACH as an example, if the UL synchronization is triggered by a PDCCH order from the source cell, the UE sends preamble towards the indicated candidate cell. The candidate gNB calculates the TA value of the candidate cell. Then the candidate gNB sends the TA value of the candidate cell and the associated CFRA resource information, to the source gNB. This procedure needs to extend the existing TA information transfer procedure via F1 interface to Xn interface, e.g. via the TA information transfer message.

In the last RAN3 meeting, it is FFS whether TA INFORMATION TRANSFER message is UE associated or non-UE associated.

In R18 intra-CU LTM, this message at F1 interface is non-UE associated, in order to simply specify and decrease normative work, we prefer to use non-UE associated procedure at Xn interface to align with that at F1 interface.

Proposal 4: XnAP TA INFORMATION TRANSFER procedure is non-UE associated procedure.

    - Late data forwarding

Agreed TP R3-244836 to TS38.300 17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1. NOTE: Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. Editor’s Note: Details are FFS on above note.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

Late Data Forwarding is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

Proposal 5: Late data forwarding is initiated	after source gNB receives Handover Success message from target gNB.

    - XnAP UE CONTEXT RELEASE message

Agreed TP R3-244836 to TS38.300 20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue. Editor’s Note: Details are FFS on step 20.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

After receiving the ACCESS SUCCESS message from the target gNB-DU, the target gNB-CU initiates the path switch procedure to CN via the existing path switch message and CN can start to forward the DL data to the target gNB-CU accordingly.

Meanwhile, the target CU sends existing XnAP: Handover Success message to the source CU to indicate that UE has accessed to the target Cell.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

After completion of the LTM, the L3 handover similar procedure at the NW side after completion of the handover can be reused for inter-CU LTM. Then, if the source gNB is not configured as a candidate gNB for the subsequent LTM, the target gNB shall send the UE CONTEXT RELEASE to inform the source gNB about the success of the handover and trigger the release of radio and C-plane related resources associated to the UE context in the source gNB.

Proposal 6: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

  - Other issues on LTM

    - CHO Cancel

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

On the other hand, similar to conditional HO cancel procedure, candidate gNB is also allowed to cancel the existing LTM.

Proposal 7: The Conditional Handover Cancel message is reused by the target gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

    - Maximum Number of LTM Preparations

In current CHO preparation, the maximum numbers can be included in the HO ACK message. Detail is as below

If the Maximum Number of CHO Preparations IE is included in the Conditional Handover Information Acknowledge IE contained in the HANDOVER REQUEST ACKNOWLEDGE message, then the source NG-RAN node should not prepare more candidate target cells for a CHO for the same UE towards the target NG-RAN node than the number indicated in the IE.

For the similar reason, we also propose to introduce maximum number of LTM preparation.

Proposal 8: Introduce a new IE (i.e., Maximum Number of LTM preparations) in Handover request ACK message.

    - One or more reference configurations

In Rel-18, a single reference configuration is used for intra-CU LTM. For inter-CU LTM, each candidate CU can have different capability and different configuration, so RAN2 is now discussing whether one or more reference configurations.

From RAN3 perspective, the reference configuration design shall also impact the Xn interaction signalling design. If only one reference configuration is introduced, the source CU needs to decide which CU (e.g. the source CU, or any one of candidate CUs) to generate the reference configuration. If a candidate CU is selected to generate the reference configuration, the source CU sends a reference configuration request indication to the candidate CU, e.g. via HO request message. The candidate CU responds with the generated reference configuration to the source CU, e.g. via HO request ACK message. And then the source CU needs to inform the received reference configuration to other candidate CUs. However, if multiple reference configurations are used, e.g. each reference per CU, the candidate CU can decide whether to use the reference configuration and generate the reference configuration by itself. Then the candidate CU can send the generated reference configuration to the source CU. The source CU does not need to inform the reference to other candidate CUs. Therefore, the inter-node interaction for multiple reference solution is simpler than the single reference solution.

Observation 1: From the perspective of inter-node interaction design, multiple reference solution is simpler than the single reference solution.

In the last RAN2 #127meeting, RAN2 has made the following agreement.

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

Based on RAN2 agreement, it is the source CU to generate the union Reference Configuration, and it shall be forwarded to all of candidate gNBs. However, in this RAN2 #127bis meeting, some companies wish to re-open this issue.

Proposal 9: RAM3 shall postpone to decide either one or more reference configurations until RANs’s agreement is stable.

    - Data forwarding for subsequent inter-CU LTM

In [R3-243395], it provides several options to support subsequent inter-CU LTM.

Take the UE moves from the source gNB to the candidate gNB1 and then to the candidate gNB2 for example. There are three options for the new source gNB to know the data forwarding information associated with other candidate gNBs. - Option 1: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to all the candidate gNBs during the LTM preparation phase. For example, the source gNB sends the data forwarding information to the gNB1 and gNB2 during the LTM preparation phase. This option may introduce new XnAP signalling between the source gNB and the candidate gNBs. - Option 2: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to the selected target gNB (i.e., new source gNB), e.g., via Cell Switch Notification message. For the subsequent inter-CU LTM, the new source gNB sends the data forwarding information associated with all the candidate gNBs to the new selected target gNB (i.e., next new soure gNB). For example, the source gNB sends the data forwarding information to the gNB1 during the first-time cell switch, and then gNB1 sends the data forwarding information to the gNB2 during the second-time cell switch. - Option 3: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to the selected target gNB, no matter which time the inter-CU LTM is performed. For example, the source gNB sends the data forwarding information to the gNB1 for the first-time cell switch, and the source gNB sends the data forwarding information to the gNB2 for the second-time cell switch.

From our view, all of above options are workable, but we think option 2 is straightforward. The reason is that, data forwarding is an optional feature, in legacy, it is the source node to request data forwarding if needed, and then the target node reply the data forwarding address to the source node if agreeable.

In legacy handover procedure, The source NG-RAN node may suggest downlink data forwarding per QoS flow established for a PDU session and may provide information how it maps QoS flows to DRBs. The target NG-RAN node decides data forwarding per QoS flow established for a PDU Session. In other words, which QoS flow will be suggested depends on the new source gNB, and which QoS flow will be accepted is determined by new candidate gNB.

Further, the DRB mapping in source gNB and target gNB also has impact on the need of data forwarding. In LTM, the QoS to DRB mapping in the new source gNB may be different to that in the old source gNB. Thus, per DRB data forwarding configuration need to be negotiated between the new source gNB and candidate gNB(s).

So, it is reasonable to negotiate new data forwarding TNL information after a LTM procedure is completed and before the subsequent LTM.

On the other hand, if the data forwarding configuration collected by the old source is still applicable for the new source gNB, i.e. per QoS data forwarding proposal and per DRB data forwarding proposal and DRB configuration are the same as old source gNB, the negotiation between new source gNB and all other candidate gNB is not needed.

Proposal 10: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to perform data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Another issue related to data forwarding for subsequent LTM is how to handle the data that is received in the early data forwarding during the precedent LTM procedure.

In legacy handover, the data forwarded to candidate gNB in early data forwarding will be released after handover procedure is completed, e.g. by reception of HANDCOVER CANCEL message.

But for subsequent LTM, the data forwarded to the candidate gNB except target gNB in early data forwarding will be useless after the precedent LTM procedure is completed. And in the subsequent LTM procedure, the new source gNB may perform early data forwarding to the candidate too. For the candidate gNB, it is unclear whether and when it should flush the data received in the precedent LTM procedure. If the data received in precedent LTM procedure is not flushed, they will bring additional buffer requirement to the candidate gNB.

Proposal 11: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

    - Establish new Xn connection between new serving gNB and candidate gNBs and release old Xn connection between old serving gNB and candidate gNBs

After completion of the LTM, new UE associated signalling connection should be established between the new serving gNB and candidate gNB(s) in order to perform subsequent LTM. To establish the new signaling connections, the new serving gNB needs to know the information of other candidate gNB(s), e.g. the gNB ID and corresponding Xn AP ID allocated for the UE. With these information, the new serving gNB can initiate procedure to setup signaling connection with other candidate gNB(s).

The old serving gNB has such information of all candidate gNB(s).The old serving gNB can transfer such information to the new serving gNB in one of following procedures:

- Handover modification procedure after path switch.

- Cell switch notification

Proposal 12: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

During path switch procedure, the AMF may update configuration for the UE, e.g. new per PDU session UL TNL address, released PDU session(s), and changed QoS parameters for QoS flows. The updated configuration needs to be updated to all candidate gNB(s) before next subsequent LTM. So after receiving PATH SWITCH REQUEST ACKNOWLEDGE message from AMF, the new serving gNB needs to initiate Xn procedure to update the configuration with all candidate gNBs.

In addition, the Xn procedure should also accomplish the setup of UE associated signaling connection between new serving gNB and candidate gNB(s), and update of data forwarding TNL information based on updated configuration.

If UE side configuration is involved, candidate gNB needs to update the prepared LTM configuration and include the update LTM configuration in the message sent to new serving gNB.

So the handover modification procedure can be used to accomplish at least following functions:

- setup Xn signaling for the UE between new serving gNB and candidate gNB(s)

- update LTM configuration

- update data forwarding TNL information

Proposal 13: After path switch procedure, handover modification procedure can be used to setup signaling connection between other candidate gNB(s), update LTM configuration, and update data forwarding TNL information.

Another issue is whether we support to release the UE associated Xn signaling connections between the old serving gNB and other candidate gNB(s). In our understanding, it is unnecessary always maintain the old Xn signaling connection. Release the old signaling connections should be allowed by specification. At least release it is needed in the case the old serving gNB is not a candidate gNB after LTM completion.

There are following options to release the old UE associated Xn signaling connections:

Option 1: During handover modification procedure after path switch, the new serving gNB indicates candidate gNB(s) explicitly or implicitly to release the UE associated signalling connection between the old serving gNB.

Option 2: The new serving gNB indicates to old serving gNB that signaling connection between old serving gNB and candidate gNB can be released explicitly or implicitly after handover success or path switch completed. The old serving gNB initiates a class 1 Xn procedure to candidate gNBs to release UE associated Xn signaling connection between candidate gNB(s).

Note that to release UE associated signaling connection only release the resources related to the UE associated signalling connection between old serving gNB and candidate gNB(s). The prepared LTM configuration in the candidate gNB is not released in this procedure.

Proposal 14: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

- Conclusion

After the above analysis, we provide the following proposals.

Proposal 1: Parallel procedures are used to prepare LTM candidate configurations, i.e., each handover preparation procedure is for one LTM candidate cell.

Proposal 2: Introduce a new UE associated Class 1 message, name as e.g., Configuration Update procedure.

Proposal 3: The new Class 1 Configuration Update procedure is mandatory for LTM.

Proposal 4: XnAP TA INFORMATION TRANSFER procedure is non-UE associated procedure.

Proposal 5: Late data forwarding is initiated	after source gNB receives Handover Success message from target gNB.

Proposal 6: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Proposal 7: The Conditional Handover Cancel message is reused by the target gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Proposal 8: Introduce a new IE (i.e., Maximum Number of LTM preparations) in Handover request ACK message.

Proposal 9: RAM3 shall postpone to decide either one or more reference configurations until RANs’s agreement is stable.

Proposal 10: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to perform data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Proposal 11: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

Proposal 12: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

Proposal 13: After path switch procedure, handover modification procedure can be used to setup signaling connection between other candidate gNB(s), update LTM configuration, and update data forwarding TNL information.

Proposal 14: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

We also provide corresponding TP to 38.300 and 38.423.