3GPP TSG-RAN WG3 Meeting #125bis	R3-245051

Hefei, China, 14th – 18th Oct 2024

Title: 	Discussion on Unified logical system architecture for topology 1 and 2

Source: 	ZTE Corporation

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #125 meeting, we have achieved the following progress and have agreed the pCR in pCR R3-244850.

The logical system architecture for topology 1 are different from it for topology 2, as below.

Figure 6.3.1-1 depicts a logical system architecture for topology 1, where the Common reader function and AIoT RAN node function are deployed within an AIoT RAN. Figure 6.3.1-1 Logical system architecture for topology 1 In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface. <Skip unrelated part> Figure 6.3.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an AIoT-enabled UE, and the AIoT RAN node function is located at an AIoT-enabled gNB. The following definitions apply: AIoT-enabled gNB: a gNB supporting AIoT RAN node function, which is able to communicate with the AIoT enabled UE via NR Uu interface. AIoT-enabled UE: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface. Figure 6.3.2-1 Logical system architecture for topology 2 Editor’s Note 1: Figure 6.3.2-1 doesn’t illustrate the protocol between AIoT enabled UE and AIoT CN, if needed, the figure needs to be revised in case such is defined by SA2. Editor’s Note 2: In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS. Editor’s Note 3: The AIoT CN could include AMF and AIoT related functions. This is up to SA2 decision. Editor’s Note 4: The AIoT enabled gNB performs radio resource management for AIoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

In this paper, we assume that RAN can select suitable either UE reader and/or gNB reader, then we provides a unified logical system architecture for both topology 1 and topology 2.

- Discussion

  - Coexistence of Topology 1 and Topology 2

    - Pre-condition

AIoT CN already knows that a set of AIoT devices are within an Inventory area, e.g., within a warehouse, meanwhile, it also knows that the warehouse is controlled by a certain RAN node.

Proposal 1: AIoT CN stores the association between a set of AIoT devices and inventory area.

Figure 1: AIoT CN stores the association between AIoT devices and Inventory area

    - AIoT CN initiates Inventory procedure

When AIoT CN initiates Inventory procedure to inventory a set of AIoT devices, it send XXAP: Inventory request message to the RAN node, including the device ID list (e.g., 1-400).

Proposal 2: AIoT CN can send XXAP: Inventory request message to RAN, to inventory a set of AIoT devices within an Inventory area.

    - AIoT RAN selects suitable readers for inventorying

This AIoT RAN node can be BBU/RRU architecture including multiple antennas. For instance, this AIoT RAN is deployed at a warehouse, and this warehouse is further split into four sub-Inventory areas, each of which is deployed with a AIoT-enabled gNB or a AIoT-enabled UE.

Figure 2: AIoT RAN selects suitable readers

The AIoT RAN knows the radio condition and workload for each of readers, so that, after receiving Inventory request message from AIoT CN, it can select suitable reader(s) to inventory.

More, if the AIoT RAN does not deploy AIoT-enabled gNB, it means that it only support AIoT topology 2, and if the AIoT RAN does not control AIoT-enabled UE, it means that it only support AIoT topology 1.

And, if the AIoT RAN deploys both AIoT-enabled gNB and AIoT-enabled UE, it means that it support both AIoT topology 1 and AIoT topology 2. For example, after receiving Inventory request message from AIoT CN, including device ID list as 1-400, the AIoT RAN can send four duplicated Inventory request message to the 4 readers in subsequence or in parallel.

Proposal 2: After receiving Inventory request message from AIoT CN, the AIoT RAN can select AIoT-enabled gNB(s) and/or AIoT-enabled UE(s) to inventory.

    - AIoT RAN sends duplicated Inventory request messages to readers

After receiving XX: Inventory request message from AIoT CN, the AIoT RAN sends the same Inventory request message to each of selected readers. For topology 1, the AIoT RAN sends Inventory request message to AIoT-enabled gNB by internal interface, and for topology 2, the AIoT RAN sends Inventory request message to AIoT-enabled UE by Uu interface.

Then each of readers initiates the same Inventory procedure including the same device list as 1-400, at AIoT radio interface.

For instance, the four readers send Inventory reports to AIoT RAN, as below:

- TPR 1 (AIoT-enabled gNB): Inventory result= device ID list (1-100)

- UE 1 (AIoT-enabled UE): Inventory result= device ID list (101-200)

- TPR 2 (AIoT-enabled gNB): Inventory result= device ID list (201-300)

- UE 2 (AIoT-enabled UE): Inventory result= device ID list (301-400)

Figure 3: Readers report Inventory result to AIoT RAN

Proposal 3: After receiving Inventory request message from AIoT CN, AIoT RAN sends duplicated Inventory request messages to its controlled readers, and each of readers sends Inventory report to the AIoT RAN.

  - Unified logical system architecture

Based on the above analysis, we provide the following figure as an example for unified Inventory procedure.

Figure 4: Unified Inventory procedure for both AIoT topology 1 and topology 2

Furthermore, according to the above Unified Inventory procedure for both AIoT topology 1 and topology 2, we propose the following figure as the unified logical system architecture.

Figure 5: Unified logical system architecture for both AIoT topology 1 and topology 2

Proposal 4: Capture Figure 5 as Unified logical system architecture for both AIoT topology 1 and topology 2 into the TR38.769.

  - Data Transport

In the last RAN3 #125 meeting, three candidate solutions are agreed to support Topology 2, as below.

6.3.2.1 Solutions for Topology 2 To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer information: - RRC based solution. With this solution, AIoT CN applies AIoT upper layer information explicitly over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted over AIoT-enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as AIoT-enabled UE's user plane data. Editor’s note 1: how to enable radio resource control, i.e. trigger AIoT RAN node functions for above solutions is FFS. Editor’s note 2: depiction and further details of the options above are FFS.

  - For UP based solution, it needs to establish PDU Session/QoS flow and the related UE/reader shall be moved into RRC connected mode, it is suitable for the data packets are large and transmission duration is long. However, it is more complex than NAS based solution.

  - For NAS based solution, it does NOT need to

- XX/NG interface: establish dedicated connection between RAN and CN

- Uu interface: establish RRC connection (UE can be in RRC inactive mode)

- Dedicate bearer: establish PDU Session/QoS flow/DRB

  - Compared to UP based solution, NAS based solution is simple. For Ambient data transport, its data packets are small and its transmission duration is short, which is very similar to the legacy NB-IoT, so NAS based solution is preferred.

- Proposal 5: From RAN3 point of view, UP based solution shall be excluded to support AIoT topology 2.

- Conclusion

After the above analysis, we provide the following proposals.

Proposal 1: AIoT CN stores the association between a set of AIoT devices and inventory area.

Proposal 2: After receiving Inventory request message from AIoT CN, the AIoT RAN can select AIoT-enabled gNB(s) and/or AIoT-enabled UE(s) to inventory.

Proposal 3: After receiving Inventory request message from AIoT CN, AIoT RAN sends duplicated Inventory request messages to its controlled readers, and each of readers sends Inventory report to the AIoT RAN.

Proposal 4: Capture Figure 5 as Unified logical system architecture for both AIoT topology 1 and topology 2 into the TR38.769.

- Proposal 5: From RAN3 point of view, UP based solution shall be excluded to support AIoT topology 2.