3GPP TSG-RAN WG3 Meeting #125bis	R3-245054

Hefei, China, 14th – 18th Oct 2024

Title: 	(TP for TR38.769) content for Ambient-IoT Inventory procedure

Source: 	ZTE Corporation

Agenda item:	16.3

Document for:	Discussion and Decision

- Introduction

In the previous meeting, we have briefly discussed the content of Inventory, and have achieved some agreement within the latest TR 38.769, as below.

TR 38.769 6.4.1.1 Inventory Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAN, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note 1: It is FFS whether AIoT RAN needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report. Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

In the last RAN3 #125 meeting, we have agreed that the whole inventory procedure includes Inventory request procedure and Inventory report procedure.

Class 1: Inventory request procedure

Inventory request message

Inventory response message

Inventory failure message

Class 2: Inventory report procedure

Inventory report message

In the last RAN3 #125 meeting, we also discussed the content of Inventory procedure by email discussion, seen in SoD R3-244703. This paper addresses to further discuss the content of Inventory procedure.

- Discussion

  - The content of Inventory request message

- session id

In case that multiple parallel inventory request procedures are initiated by AIoT-CN to the same reader, the reader needs to use different session id to differentiate them.

However, if the parallel inventory request procedures include different devices (e.g., different filter criteria/set of devices) to be inventoried, the reader can differentiate them by different devices without session id.

It means that only in case that the parallel inventory request procedures are the duplicate inventory request procedures, the session id is needed.

Proposal 1: Session id is FFS. This is up to SA2 decision, e.g., Session ID is needed if AIoT-CN initiates the same inventory request procedures in parallel.

- reader id

For topology 1, it is obvious that CN selects the suitable gNB/reader. For topology 2, since the UE/reader can be in IDLE mode, the gNB will not store the IDLE-UE context, it is reasonable that CN selects the UE/reader.

Proposal 2: It is the AIoT CN to select suitable reader for both topology 1 and topology 2.

Before initiating Inventory procedure, the AIoT CN shall know which gNB(s) and UE(s) have AIoT capability by e.g., OAM, UE authorisation.

For the initial inventory procedure, the AIoT CN has no idea of the association between reader and device(s), it shall send some duplicated Inventory request messages including the same device(s) to different readers.

After initial inventory procedure, AIoT CN can acquire that each device to be inventory/command, is controlled by which reader, i.e., the CN can establish/store association between each device and reader.

This association is included in device context, and stored in the CN, e.g., device context = {device id list, reader id}. The reader id is UE id or gNB/BS/cell/TRP id.

Proposal 3: AIoT CN shall establish, update and store AIoT Device context, which includes association between device id list and reader id.

Proposal 4: Read id (e.g., UE id or gNB/BS/cell/TRP id) is needed.

- device id

We have already agreed that device id(s) are optional present in the Inventory request message. If it is absent, it means that the selected reader shall inventory all of the AIoT devices.

Proposal 5: Device id is needed.

Inventory procedure will consume more or less Ambient-Uu radio resource. In case UE/reader shares the same frequency/band with NR-gNB and in case gNB/reader is co-site with NR-gNB, the NR-gNB shall precludes the radio resource when the reader handles inventory/command.  So, the reader shall know the quantity of the device to be inventoried.

AIoT device has ultra-low power consumption. When the device is in charging phase, it even cannot respond inventory request. In order to perform efficient inventory, the reader shall perform delta inventory. This means that only devices have not responded yet shall respond inventory, to decrease the RACH collision at Ambient-Uu interface. So that, the reader shall know the device ids to perform delta inventory.

Observation 1: The reader shall occupy suitable radio resources to perform inventory/command procedure.

Proposal 6: The reader shall be able to decode AIoT device id.

- Periodicity

After initial Inventory procedure, the AIoT has acquired the reader-device association. After a while, the AIoT can also trigger another Inventory procedure to acquire the possibly updated reader-device association.

For topology 2, the UE reader can move from one area to another area, the reader-device association may be changed, so that, AIoT can initiate periodic Inventory procedure to acquire the latest reader-device association.

Proposal 7: Periodicity is FFS. This is up to SA2 decision, e.g., periodicity is needed if AIoT shall acquire the latest reader-device association.

- Estimated Number of devices

For the device id included in the Inventory request message, some companies think the device id is transparent to reader. In this case, estimated Number of devices is needed, e.g., to enable CFRA, to assists resource allocation.

Proposal 7: Estimated Number of devices is FFS, depending on whether the device id is transparent to reader.

  - The content of Inventory response/failure message

After receiving Inventory request message, if the reader agrees to handle it, it shall send Inventory response message to the AIoT CN. Later, after acquiring Inventory result, it sends Inventory report message to the AIoT CN.

Otherwise, the reader shall send Inventory failure message to the AIoT CN.

- session id

Proposal 8: Session id is FFS, if it is included in the Inventory request message.

- reader id

Proposal 9: Read id (e.g., UE id or gNB/BS/cell/TRP id) is needed. It shall be the same as that in the Inventory request message.

  - The content of Inventory report message

- session id

Proposal 10: Session id is FFS, if it is included in the Inventory request message.

- reader id

Proposal 11: Read id (e.g., UE id or gNB/BS/cell/TRP id) is needed. It shall be the same as that in the Inventory request message.

- device id

The device id is the result of inventory request procedure, it shall be included in the Inventory report message and sent to the AIoT CN. Then the AIoT CN can acquire and store the reader-device association.

Proposal 12: Device id is needed.

- Conclusion

Based on the above proposals, RAN3 agrees to capture the following TP into the TR38.769.

- Text Proposal

============================Start of Change==========================

6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

<Skip unchanged part>

6.4.X The content of Inventory signalling and procedure

6.4.X.1 The content of Inventory request message

1) Session id (FFS)

Editor’s Note 1: This is up to SA2 decision, e.g., Session ID is needed if AIoT-CN initiates the same inventory request procedures in parallel.

2) Reader id

Editor’s Note 2: The detail of reader id, e.g., UE id or gNB/BS/cell/TRP id), is FFS.

3) Device id (optional)

Editor’s Note 3: The Device ID is optional. If it is absent, it means that the selected reader shall inventory all of the AIoT devices.

Editor’s Note 4: It is FFS on whether the device id is transparent to reader.

4) Periodicity (FFS)

Editor’s Note 5: This is up to SA2 decision, e.g., periodicity is needed if AIoT shall acquire the latest reader-device association.

5) Estimated Number of devices (FFS)

Editor’s Note 6: It is FFS on whether the device id is transparent to reader.

6.4.X.2 The content of Inventory response/failure message

1) Session id (FFS)

Editor’s Note 1: It is FFS if it is included in the Inventory request message.

2) Reader id

Editor’s Note 2: The format of reader id shall be the same as that in the Inventory request message.

6.4.X.3 The content of Inventory report message

1) Session id (FFS)

Editor’s Note 1: It is FFS if it is included in the Inventory request message.

2) Reader id

Editor’s Note 2: The format of reader id shall be the same as that in the Inventory request message.

3) Device id

Editor’s Note 3: The AIoT CN shall establish, update and store the reader-device association based on the inventory result included in the received Inventory report message.

============================End of Change==========================