3GPP TSG-RAN WG3 Meeting #125bis                             R3-245066

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	Discussion on AI/ML assisted positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

Introduction

In the WID for Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], an objective to support AI/ML assisted positioning for positioning accuracy enhancements, the potential RAN3 impact sub-bullets are highlighted below:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning: - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, we will discuss the potential RAN3 impact to support AI/ML assisted positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals.

Discussion

Support of AI-based positioning in NRPPa

According to the WID [1], on support of AI/ML assisted positioning, RAN3 should focus on the AI/ML assisted positioning Case 3a for positioning accuracy enhancements. RAN1 has some discussion from RAN1#116, the discussion is from the following 3 aspects:

- Data Collection

- Model input/report for inference

- Performance Monitoring

Proposal 1: For AI/ML assisted positioning, RAN3 should focus on the Case 3a, gNB-side model.

In the following sub-sections, we will discuss the AI/ML assisted positioning from Data Collection, Model input/report for inference and Performance Monitoring.

Data collection

For the Data collection, corresponding RAN1 agreements or working assumptions could be found below:

RAN#116 Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

RAN#116 bis Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

According to RAN1 agreements, for case 3a/3b data collection, Part A and Part B may be generated by different entities. We understand the Part A is generated by gNB, while the Part B maybe generated by the LMF or UE. RAN3 can further discuss the interaction between different entities to obtain a paired Part A and Part B.

In Case 3a, channel measurement is done at gNB, while RAN1 has agreed that the label and its related data (e.g., time stamp) can be generated by at least LMF. Whether or not gNB can derive labels by itself is not decided in RAN1.Obviously, if the label is decided by RAN itself, RAN could use it itself, no extra impact to NRPPa. If the label is generated in the LMF, we should consider how to deliver the label to gNB.

Proposal 2: In Case 3a, if the gNB could derive labels by implementation, no extra specification impact to NRPPa.

Proposal 3: In Case 3a, the label and its related data (e.g., time stamp) if generated in the LMF, should be provided to the gNB for channel measurement.

Model Input/ report for inference

For Model Input/Report for inference, the corresponding agreements in RAN1 could be found below:

RAN#116 Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

RAN#117 Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

RAN1#118 Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

In RAN1#116 meeting, it’s agreed that in Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

Which alternative will be taken is pending to RAN1, RAN3 can wait for the conclusion of RAN1 for our interface design, e.g. including phase information in the channel measurement report.

Proposal 4: In Case 3a, Sample-based or Path-based measurements should be taken for the time domain channel measurements are pending to RAN1, RAN3 could wait a little bit.

According to the RAN1 agreements/conclusion, for Case 3a, the measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. We assume NRPPa Measurement procedure may need to be enhanced to support these cases.

Proposal 5: In Case 3a, NRPPa measurement procedure may need to be enhanced, e.g. training data collection, monitoring, or inference procedures.

Performance Monitoring

For performance monitoring, corresponding RAN1 agreements are shown below:

RAN#116 Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

RAN#116bis Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation

RAN1#118 Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

According to the RAN1 agreements above, both of the options (Option A and Option B) as above are feasible, RAN3 should discuss and decide which node to perform monitoring metric calculation, NG-RAN node (option A) or LMF (Option B).

If label-based monitoring is done at gNB side (Option A), the labels and related information should be provided from the LMF to the gNB. If label-based monitoring is done at LMF (Option B), we should consider what kind of assistance information should be provided to LMF.

Proposal 6: In Case 3a, RAN3 should discuss and decide which node to perform monitoring metric calculation, NG-RAN node (Option A) and/or LMF (Option B).

Support of AI-based positioning in F1

For RAN split architecture (CU/DU split), to support AI/ML Assisted Positioning Case 3a (gNB-based model), RAN3 should discuss and decide where the AI/ML model is located, in CU, in DU or in both entities. We should consider if any interaction/efficiency issue in case of different CU-DU venders as the gNB-CU and gNB-DU may have different understanding about the AI model(s).

Proposal 7: For RAN split architecture, to support Case 3a, we should first discuss where the AI model is located, the functionality split between gNB-CU and gNB-DU.

Conclusion

In this contribution, we initially discussed the RAN3 impact to support AI/ML assisted positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following observations and proposals are provided:

Proposal 1: For AI/ML assisted positioning, RAN3 should focus on the Case 3a, gNB-side model.

Proposal 2: In Case 3a, if the gNB could derive labels by implementation, no extra specification impact to NRPPa.

Proposal 3: In Case 3a, the label and its related data (e.g., time stamp) if generated in the LMF, should be provided to the gNB for channel measurement.

Proposal 4: In Case 3a, Sample-based or Path-based measurements should be taken for the time domain channel measurements are pending to RAN1, RAN3 could wait a little bit.

Proposal 5: In Case 3a, NRPPa measurement procedure may need to be enhanced, e.g. training data collection, monitoring, or inference procedures.

Proposal 6: In Case 3a, RAN3 should discuss and decide which node to perform monitoring metric calculation, NG-RAN node (Option A) and/or LMF (Option B).

Proposal 7: For RAN split architecture, to support Case 3a, we should first discuss where the AI model is located, the functionality split between gNB-CU and gNB-DU.

Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RAN1#116/#116bis/#117/#11 Chair’s Notes