3GPP TSG-RAN WG3 Meeting #125-bis	R3-245073

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) A-IoT Signalling and Procedures for Topology 2

Agenda Item:	16.3

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#125 meeting, the candidate procedures for A-IoT Inventory for Topology 2 were captured in [1], and three solutions were captured to support Topology 2.

Yet the following FFSs were list as to be continued:

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

In this paper, we futher discuss the leftover aspects about A-IoT signalling and procedure for Topology 2.

# 2	Discussion

## 2.1	A-IoT-enabled UE selection

In last meeting, the following FFS were listed:

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

In topology 1, it is reasonable to select suitable A-IoT RAN by the A-IoT CN.

In topology 2 NAS/UP based solutions, considering the A-IoT procedures are terminated between the A-IoT-enabled UE and the A-IoT CN, the A-IoT Inventory/Command procedures are transparent to the gNB, the A-IoT-enabled UE reader selection has to be performed by the A-IoT CN.

- Proposal 1: In Topology 2 NAS/UP based solution, the A-IoT-enabled UE selection is performed by A-IoT CN.

In topology 2 RRC based solution, as the gNB is aware of the ongoing A-IoT services, the A-IoT-enabled UE selection could be done by either the gNB or the CN or both. If the A-IoT-enabled UE selection is performed by the A-IoT CN, the A-IoT CN needs to provide the selected A-IoT-enabled UE list to the A-IoT-enabled gNB.

- Proposal 2: In Topology 2 RRC based solution, the A-IoT-enabled UE selection could be performed by A-IoT CN or the gNB or both. If the A-IoT-enabled UE selection is performed by the A-IoT CN, the A-IoT CN needs to provide the selected A-IoT-enabled UE list to the A-IoT-enabled gNB.

## 2.2	Information carried in Inventory Request in Topology 2

In Topology 2 NAS/UP based solutions, the interactions between the A-IoT-enabled UE and A-IoT CN are transparently forwarded by the gNB, and the information will be the same or quite similar to the information (to be) listed in TR 38.769 section 6.5.1.1. In Topology 2 RRC based solution, upon receiving the information (to be) listed in TR 38.769 section 6.5.1.1, the A-IoT-enabled gNB will forward the received information to the A-IoT-enabled UE.

Proposal 3: The information (to be) listed in TR 38.769 section 6.5.1.1 will be transmitted to the A-IoT-enabled UE from the A-IoT-enabled gNB (RRC based solution) or from the A-IoT CN (NAS/UP based solutions).

## 2.3	Command Operation in Topology 2

Based on the discussion in [2], we would like to support Command Transmission after Inventory procedure for command operation.

Based on the Figure 6.5.3.1-2 the message flow for A-IoT Inventory procedure in Topology 2 (if NAS/UP based solution is used), the command after inventory call flow could be illustrated as below:

- Figure 1. Message flow for A-IoT Command in Topology 2 (if NAS/UP based solution is used)

Considering that the Topology 2 RRC based solution can be considered as Topolgoy 1 solution with an extra NR Uu RRC step, take the proposed call flow for Topology 1 in [2] into account, the command after inventory call flow could be illustrated as below:

Figure 2. Message flow for A-IoT Command in Topology 2 (if RRC based solution is used)

- Proposal 4: Support Command Transmission after Inventory procedure for command operation also in Topology 2.

# 3	Conclusion

- In this contribution, we further discussed the signaling and procedures for Topology 2, and got the following proposals:

- Proposal 1: In Topology 2 NAS/UP based solution, the A-IoT-enabled UE selection is performed by A-IoT CN.

- Proposal 2: In Topology 2 RRC based solution, the A-IoT-enabled UE selection could be performed by A-IoT CN or the gNB or both. If the A-IoT-enabled UE selection is performed by the A-IoT CN, the A-IoT CN needs to provide the selected A-IoT-enabled UE list to the A-IoT-enabled gNB.

- Proposal 3: The information (to be) listed in TR 38.769 section 6.5.1.1 will be transmitted to the A-IoT-enabled UE from the A-IoT-enabled gNB (RRC based solution) or from the A-IoT CN (NAS/UP based solutions).

- Proposal 4:  Command Transmission after Inventory procedure for command operation also in Topology 2.

The corresponding TP for TR 38.769 is provided in section 5.

# 4. Reference

[1] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

[2] R3-245072 (TP to TR 38.769) A-IoT Signalling and Procedures for Topology 1, Huawei

# 5. TP for TR 38.769

----------------Start of the First Change----------------

## 6.5	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.5.1	Information exchanged with A-IoT CN

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

#### 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory)

----------------Start of the Next Change----------------

### 6.5.3	Signaling and Procedures for Topology 2

#### 6.5.3.1	Candidate procedures for A-IoT Inventory for Topology 2

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.1-1: Message flow for A-IoT Inventory in Topology 2 (if RRC-based solution is used)

Figure 6.5.3.1-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

Editor’s note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

#### 6.5.3.x	Candidate procedures for A-IoT Command for Topology 2

Figure 6.5.3.x-1 illustrates the message flow to support A-IoT Command Operation in Topology 2 in case of RRC based solution:

Figure 6.5.3.x-1. Message flow for A-IoT Command in Topology 2 (if RRC based solution is used)

1.	A-IoT CN triggers Inventory Request with A-IoT Device ID towards the relavent gNB, and indicates that there will be command transmission for the device.

2.	The A-IoT-enabled gNB sends Inventory Response to the A-IoT CN.

3.  The A-IoT-enabled gNB sents Inventory information received in step1 to A-IoT-enabled UE over NR Uu.

NOTE: step 2 and step 3 may happen in parallel.

4.	The A-IoT-enabled UE triggers inventory procedure over A-IoT radio interface.

5. The A-IoT-enabled UE sends the UL Data (Device ID) received from A-IoT device to the A-IoT-enabled gNB.

6.	The A-IoT-enabled gNB sends the Inventory Report towards the CN with the UL Data (Device ID) received in step5. Based on the command indication received in step1, the gNB allocates RAN Device XXAP ID for the device, and include it in the Inventory Report.

7.	After Security checking successfully, the A-IoT CN triggers DL Command Transport towards the gNB. The A-IoT CN allocates CN Device XXAP ID for the device, and provide the RAN/CN Device XXAP ID pair and the DL Data towards the gNB.

8a. The gNB sends the DL Data towards the A-IoT-enabled UE.

8b.	The A-IoT-enabled UE transmits the DL Data towards the A-IoT Device.

9a.	The A-IoT Device transmits the UL Data towards the A-IoT-enabled UE.

9b.	The A-IoT-enabled UE sends UL Data to the gNB.

10.  The gNB triggers UL Command Transport towards the A-IoT CN node, including the RAN/CN Device XXAP ID pair and the UL Data.

Figure 6.5.3.x-x illustrates the message flowto support A-IoT Command Operation in Topology 2 in case of NAS/UP based solutions:

- Figure 6.5.3.x-2. Message flow for A-IoT Command in Topology 2 (if NAS/UP based solution is used)

- 1.	A-IoT CN triggers Inventory Request with A-IoT Device ID towards the A-IoT-enabled UE, via A-IoT-enabled UE's NAS, or via A-IoT-enabled UE's user plane data.

- 2.	The A-IoT-enabled UE sends Inventory Response to the A-IoT CN, via A-IoT-enabled UE's NAS, or via A-IoT-enabled UE's user plane data.

- 3.	The A-IoT-enabled UE triggers inventory procedure over radio.

- 4.	The A-IoT-enabled UE sends the Inventory Report towards the A-IoT CN with the UL Data (Device ID) received in step3, via A-IoT-enabled UE's NAS, or via A-IoT-enabled UE's user plane data.

- 5.	After Security checking successfully, the A-IoT CN transmits DL Command towards the A-IoT-enabled UE, via A-IoT-enabled UE's NAS, or via A-IoT-enabled UE's user plane data.

- 6	The A-IoT-enabled UE transmits the DL Data towards the A-IoT Device.

- 7.	The A-IoT Device transmits the UL Data towards the A-IoT-enabled UE.

- 8.  The A-IoT-enabled UE transmits Command Response towards the A-IoT CN node, via A-IoT-enabled UE's NAS, or via A-IoT-enabled UE's user plane data.

#### 6.5.3.y	A-IoT-enabled UE selection

In NAS/UP based solution, the A-IoT-enabled UE selection is performed by A-IoT CN.

In RRC based solution, the A-IoT-enabled UE selection could be performed by A-IoT CN or the gNB or both. If the A-IoT-enabled UE selection is performed by the A-IoT CN, the A-IoT CN needs to provide the selected A-IoT-enabled UE list to the A-IoT-enabled gNB.

----------------End of the Changes----------------