3GPP TSG-RAN WG3 Meeting #125-bis	R3-245078

Hefei , CN, 14 - 18 Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1190 | rev | - | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CIoT optimization with 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Qualcomm Incorporated, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core, NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | #Issue 1 missing ULI in UP CIoT optimization procedures SA2 has discussed ULI reporting enhancements since Rel-12 and requires the RAN node to report the most up to date User Location Information (refer to S2-133865/R3-131887) to CN. Therefore, the RAN node reports the ULI during the initial access, handover, context release procedures, etc. In Rel-16, UP CIoT optimization with 5GC was specified, and the UE Context Resume and UE Context Suspend procedures were introduced. In such scenario, the UE changes from RRC_IDLE to RRC_CONNECTED state during UE Context Resume and moves to RRC_IDLE again after UE Context Suspend, but User Location Information was missing in these two procedures in the messages to CN (i.e., UE CONTEXT SUSPEND REQUEST and UE CONTEXT RESUME REQUEST messages). And there is a common understanding that only UE Context Resume and UE Context Release procedures are allowed to take place on a suspended UE-associated signalling connection. Therefore, it is preferable to send the ULI using those procedures, rather than providing it via separate Location Reporting procedures, which will lead to increased signalling load, and may not possible after UE Context Suspend. #Issue 2 wrong additional IEs in CP CIoT optimization procedure In NB-IoT CP solution, when the UE moves to a cell belongs to another ng-eNB, RLF + RRCResetablishment will happen, upon receiving INITIAL UE MESSAGE message from new ng-eNB, the AMF may inform the old ng-eNB that the UE is to be relocated via AMF CP Relocation Indication procedure, to trigger the old ng-eNB to return the non-delivered NAS PDUs to the AMF, and UE context release will happen between the AMF and the old ng-eNB afterwards. Therefore, the AMF CP Relocation Indication procedure is not a procedure to establish something, the slice related information was wrongly added here based on R3-193410 CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Issue #1, adding User Location Information in the following messages: - UE CONTEXT SUSPEND REQUEST - UE CONTEXT RESUME REQUEST For Issue #2, removing the following slicing related IEs in AMF CP RELOCATION INDICATION: - S-NSSAI - Allowed NSSAI Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only adding the missing ULI in the UP CIoT optimization related resume and suspend procedures, and removing the IEs which should not be included in the AMF CP relocation Indication procedure. This CR has functional and protocol impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CN is not able to get the up-to-date user location information in case of UE context resume and suspend. Wrong handling of slice information at old ng-eNB side upon receiving AMF CP Relocation Indication, e.g., in case these IEs are wrong, the old ng-eNB will not be able to return the non-delivered NAS PDUs to the AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.11.2, 8.3.12.2, 9.2.2.16, 9.2.2.19, 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Rel-16 WI codes used in R3-204514 which introduced these three procedures in NGAP. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------Start of the First Change--------------------

### 8.3.7	AMF CP Relocation Indication

#### 8.3.7.1	General

The purpose of the AMF CP Relocation Indication procedure is to inform the NG-RAN node that the UE’s connection is to be relocated to another NG-RAN node as described in TS 38.300 [8], for a UE using Control Plane CIoT 5GS Optimisation. This procedure applies only if the NG-RAN node is an ng-eNB.

The procedure uses UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: AMF CP Relocation Indication. Successful operation.

The AMF initiates the procedure by sending an AMF CP RELOCATION INDICATION message to the NG-RAN node.

Upon reception of the AMF CP RELOCATION INDICATION message, the NG-RAN node shall terminate the delivery of NAS messages that have been received from the AMF.

Interactions with NAS Non Delivery Indication procedure:

On reception of the AMF CP RELOCATION INDICATION message, the NG-RAN node may initiate NAS Non Delivery Indication procedure(s) to report the non-delivery of any NAS PDUs previously received from the AMF.

#### 8.3.7.3	Abnormal Conditions

Void.

-------------------Start of the Next Change--------------------

### 8.3.11	UE Context Suspend

#### 8.3.11.1	General

The purpose of the UE Context Suspend procedure is to suspend the UE-associated logical NG-connection and the NG-U transport bearer with the 5GC while keeping the UE context in the NG-RAN node. The procedure uses UE-associated signalling.

In this version of the specification, this procedure applies only if the NG-RAN node is an ng-eNB.

#### 8.3.11.2	Successful Operation

Figure 8.3.11.2-1: UE Context Suspend: Successful operation.

The NG-RAN node initiates the procedure by sending the UE CONTEXT SUSPEND REQUEST message to the AMF.

Upon receipt of the UE CONTEXT SUSPEND REQUEST message the AMF shall act as defined in TS 23.502 [10].

Upon receipt of the UE CONTEXT SUSPEND RESPONSE message the NG-RAN node shall suspend the UE context, the UE-associated logical NG-connection and the related PDU session contexts and send the UE to RRC_IDLE.

If the Information on Recommended Cells and RAN Nodes for Paging IE is included in the UE CONTEXT SUSPEND REQUEST message, the AMF shall, if supported, store it and may use it for subsequent paging.

If the Paging Assistance Data for CE Capable UE IE is included in the UE CONTEXT SUSPEND REQUEST message, the AMF shall, if supported, store it and use it for subsequent paging, as specified in TS 23.502 [10].

If the Security Context IE is included in the UE CONTEXT SUSPEND RESPONSE message, the NG-RAN node shall store the received Security Context IE in the UE context and remove any existing unused stored {NH, NCC} as specified in TS 33.501 [13].

If the Suspend Indicator IE is included in the UE CONTEXT SUSPEND REQUEST message, the SMF shall, if supported, consider the associated PDU session as suspended.

If the User Location Information IE is included in the UE CONTEXT SUSPEND REQUEST message, the AMF shall handle this information as specified in TS 23.501 [9].

#### 8.3.11.3	Unsuccessful Operation

Figure 8.3.11.3-1: UE Context Suspend: unsuccessful operation.

If the AMF decides to not suspend the connection e.g. due to pending downlink data to be sent, it shall send the UE CONTEXT SUSPEND FAILURE message to the NG-RAN node.

#### 8.3.11.4	Abnormal Conditions

Void.

### 8.3.12	UE Context Resume

#### 8.3.12.1	General

The purpose of the UE Context Resume procedure is to resume the UE context, the suspended UE-associated logical NG-connection and the related NG-U transport bearer in the 5GC for this UE. The procedure uses UE-associated signalling.

In this version of the specification, this procedure applies only if the NG-RAN node is an ng-eNB.

#### 8.3.12.2	Successful Operation

Figure 8.3.12.2-1: UE Context Resume procedure. Successful operation.

The NG-RAN node initiates the procedure by sending the UE CONTEXT RESUME REQUEST message to the AMF. If the NG-RAN node is not able to admit any suspended PDU sessions, the NG-RAN node shall indicate this in the PDU Session Resource Failed to Resume List IE. If the NG-RAN node is not able to admit certain QoS flows for a PDU session, the NG-RAN node shall indicate this in the QoS Flow Failed to Resume List IE included in the UE Context Resume Request Transfer IE for that PDU session.

Upon receipt of the UE CONTEXT RESUME REQUEST message the AMF shall act as defined in TS 23.502 [10] and respond with the UE CONTEXT RESUME RESPONSE message. If the AMF is not able to admit any suspended PDU sessions, the AMF shall indicate this in the PDU Session Resource Failed to Resume List IE. If the SMF is not able to admit certain QoS flows for a PDU session, the SMF shall indicate this in the QoS Flow Failed to Resume List IE included in the UE Context Resume Response Transfer IE for that PDU session.

The NG-RAN node shall release resources for each PDU session or QoS flow failed to resume and shall assume that the 5GC has released respective resources as well.

If the Security Context IE is included in the UE CONTEXT RESUME RESPONSE message, the NG-RAN node shall store the received Security Context IE in the UE context and the NG-RAN node shall use it for the next suspend/resume or Xn handover or Intra NG-RAN node handovers as specified in TS 33.501 [13].

If the Suspend Request Indication IE is included in the UE CONTEXT RESUME REQUEST message, the AMF shall, if supported, consider that the NG-RAN node is requesting immediate transition to RRC IDLE with Suspend as specified in TS 23.502 [10]. If the Suspend Response Indication IE is included in the UE CONTEXT RESUME RESPONSE message, the NG-RAN node shall suspend the UE context, the UE-associated logical NG-connection and the related PDU session contexts and send the UE to RRC_IDLE.

If the Information on Recommended Cells and RAN Nodes for Paging IE is included in the UE CONTEXT RESUME REQUEST message, the AMF shall, if supported, store it and may use it for subsequent paging.

If the Paging Assistance Data for CE Capable UE IE is included in the UE CONTEXT RESUME REQUEST message, the AMF shall, if supported, store it and use it for subsequent paging, as specified in TS 23.502 [10].

If the Extended Connected Time IE is included in the UE CONTEXT RESUME RESPONSE message, the NG-RAN node shall, if supported, use it as described in TS 23.501 [9].

If the User Location Information IE is included in the UE CONTEXT RESUME REQUEST message, the AMF shall handle this information as specified in TS 23.501 [9].

#### 8.3.12.3	Unsuccessful Operation

Figure 8.3.12.3-1: UE Context resume: unsuccessful operation.

If the AMF is not able to resume a single PDU session, it releases the UE-associated logical NG-connection by sending the UE CONTEXT RESUME FAILURE message to the NG-RAN node. Upon reception of the UE CONTEXT RESUME FAILURE message the NG-RAN node shall release the RRC connection as specified in TS 36.331 [21] and release all related signalling and user data transport resources.

-------------------Start of the Next Change--------------------

#### 9.2.2.12	AMF CP RELOCATION INDICATION

This message is sent by the AMF to inform the NG-RAN node that the UE is to be relocated as described in TS. 38.300 [8].

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| S-NSSAI | O |  | 9.3.1.24 | In this version of the specification, this IE is ignored and not used. | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. In this version of the specification, this IE is ignored and not used. | YES | ignore |

-------------------Start of the Next Change--------------------

#### 9.2.2.16	UE CONTEXT SUSPEND REQUEST

This message is sent by the NG-RAN node to request the AMF to suspend the UE context and the related PDU session contexts.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| PDU Session Resource Suspend List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Suspend Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>UE Context Suspend Request Transfer | M |  | Containing the UE Context Suspend Request Transfer IE specified in subclause 9.3.4.26. |  | - |  |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

-------------------Start of the Next Change--------------------

#### 9.2.2.19	UE CONTEXT RESUME REQUEST

This message is sent by the NG-RAN node to request the AMF to resume the UE-associated logical NG-connection and UE context.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| RRC Resume Cause | M |  | RRC Establishment Cause 9.3.1.111 |  | YES | ignore |
| PDU Session Resource Resume List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Resume Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>UE Context Resume Request Transfer | M |  | OCTET STRING | Containing the UE Context Resume Request Transfer IE specified in subclause 9.3.4.24 | - |  |
| PDU Session Resource Failed to Resume List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Failed to Resume Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Suspend Request Indication | O |  | 9.3.1.158 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

-------------------Start of the Next Change--------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

//skip unchanged part

-- **************************************************************

--

-- UE CONTEXT RESUME REQUEST

--

-- **************************************************************

UEContextResumeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextResumeRequestIEs} },

...

}

UEContextResumeRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RRC-Resume-Cause						CRITICALITY ignore	TYPE RRCEstablishmentCause							PRESENCE mandatory	}|

{ ID id-PDUSessionResourceResumeListRESReq		CRITICALITY reject	TYPE PDUSessionResourceResumeListRESReq			PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToResumeListRESReq		CRITICALITY reject	TYPE PDUSessionResourceFailedToResumeListRESReq			PRESENCE optional		}|

{ ID id-Suspend-Request-Indication				CRITICALITY ignore	TYPE Suspend-Request-Indication						PRESENCE optional		}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging		PRESENCE optional		}|

{ ID id-PagingAssisDataforCEcapabUE					CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE							PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation					PRESENCE optional		},

...

}

//skip unchanged part

-- **************************************************************

--

-- UE CONTEXT SUSPEND REQUEST

--

-- **************************************************************

UEContextSuspendRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextSuspendRequestIEs} },

...

}

UEContextSuspendRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging		PRESENCE optional		}|

{ ID id-PagingAssisDataforCEcapabUE					CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE							PRESENCE optional		}|

{ ID id-PDUSessionResourceSuspendListSUSReq			CRITICALITY reject	TYPE PDUSessionResourceSuspendListSUSReq				PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation								PRESENCE optional		},	...

}

//skip unchanged part

-- **************************************************************

--

-- AMF CP Relocation Indication

--

-- **************************************************************

AMFCPRelocationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container { { AMFCPRelocationIndicationIEs} },

...

}

AMFCPRelocationIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-S-NSSAI						CRITICALITY ignore	TYPE S-NSSAI					PRESENCE optional		}—-this IE is not used and ignored|

{ ID id-AllowedNSSAI				CRITICALITY ignore	TYPE AllowedNSSAI				PRESENCE optional		}—-this IE is not used and ignored,

...

}

-------------------End of the Changes--------------------