3GPP TSG-RAN WG3 Meeting #125bis	R3-245084

Hefei, China, 14th – 18th October 2024

Agenda item:	14.2

Source:	Xiaomi

Title:	(TP to BL CR for TS 38.413) Support of broadcast service

Document for:	Other

1	Introduction

In this contribution, we’d like to further discuss the intended service area of a broadcast service according to the progresses in RAN3 and RAN2.

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

RAN3 agreements in RAN3 125 meeting

RAN2 progress in RAN2 127 meeting

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

According to RAN2 progress, the intended broadcast service area can be represented by a geographical area. In this paper, two approaches are discussed to support RAN2 agreement, and corresponding actions are provided accordingly.

2	Discussion

According to TS 23.247[1] and TS 29.522[2], initially the AF may provide the internal MBS service area or external MBS service area, if the external MBS service area is provided, the NEF shall translate the received geographical area(s) or civic address(es) to a list of cell ID(s) and/or list of TAI(s) before relaying it to the MB-SMF;

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

Reference from TS 23.247[1]

Reference from TS 29.522[2]

##### 5.29.5.4.1 Type: MbsServArea Table 5.29.5.4.1-1: Definition of type MbsServArea as a list of mutually exclusive alternatives Data type; Cardinality; Description; Applicability. MbsServiceArea; 0..1; Represents the internal MBS Service Area (i.e., in the form of a list of cell ID(s) and/or list of TAI(s)).. ExternalMbsServiceArea; 0..1; Represents the external MBS Service Area (i.e., in the form of geographical area(s) or civic address(es))..

Observation 1, either internal service area (i.e., list of cells or TAIs) or external service area (i.e., geographical area(s) or civic address(es)) may be provided from AF to establish the MBS broadcast session.

Observation 2, if the external MBS service area is provided, the NEF shall translate the received geographical area(s) or civic address(es) to a list of cell ID(s) and/or list of TAI(s) before relaying it to the MB-SMF and then to the NG-RAN node.

The codec of geographical area, as specified in TS 29.571[3] and TS 29.572[4], includes both circles and polygons, thereby supporting the RAN2 agreement.

##### 6.1.6.2.5 Type: GeographicArea Table 6.1.6.2.5-1: Definition of type GeographicArea as a list of mutually exclusive alternatives Data type; Cardinality; Discriminator property name; Discriminator mapping; Description. Point; 1; shape; POINT; Geographical area consisting of a single point, represented by its longitude and latitude.. PointUncertaintyCircle; 1; shape; POINT_UNCERTAINTY_CIRCLE; Geographical area consisting of a point and an uncertainty value.. PointUncertaintyEllipse; 1; shape; POINT_UNCERTAINTY_ELLIPSE; Geographical area consisting of a point, plus an uncertainty ellipse and a confidence value.. Polygon; 1; shape; POLYGON; Geographical area consisting of a list of points (between 3 to 15 points).. PointAltitude; 1; shape; POINT_ALTITUDE; Geographical area consisting of a point and an altitude value.. PointAltitudeUncertainty; 1; shape; POINT_ALTITUDE_UNCERTAINTY; Geographical area consisting of a point, an altitude value and an uncertainty value.. EllipsoidArc; 1; shape; ELLIPSOID_ARC; Geographical are consisting of an ellipsoid arc.. NOTE: The "anyOf" keyword (instead of "oneOf" keyword which is normally used for mutually exclusive alternatives) is used for GeographicArea type in yaml file to avoid validation failure of OpenAPI. According to current definition, a PointUncertaintyCircle object will always pass the validation with both PointUncertaintyCircle and Point, which fails the qualification of "oneOf" keyword..

Observation 3, the existing codes for geographical area defined in core network can be used to represent the circle(s) and polygon(s) that agreed to be broadcast to UE as agreed by RAN2.

Based on existing procedures and signalling, we think there’re two approaches to support gNB to broadcast the geographical area represented by a (set of) referenceLocation and radius (i.e., circle) or by a (set of) polygon(s).

Approach 1 RAN perform translation

If the RAN node is responsible for the translation, i.e., RAN node translates the Cell ID list or TAI list in the MBS service area information to a geographical area information. The overall call flow can be shown in figure 1, it can be seen that it will have no SA/CT impacts and no stage 3 impacts on RAN3.

If approach 1 is adopted, we think new procedure text is needed for this case, the following is an example and the corresponding TP is provided in Annex A.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44]. In case of NTN, the NG-RAN node shall translate the Cell ID list or TAI list in MBS Service Area IE to the area information broadcast to UE as specified in TS 38.331 [18].

There may be one issue for gNB to translate the cell ID list or TAI list to geographical area, the area after gNB’s translation may be different from the original one, which implies that the original purpose will not be achieved.

Observation 4, if NG-RAN node performs the translation, it will have no stage 3 impacts, but the geographical area broadcast to the UE may not be the same as the original geographical area from AF.

Approach 2 CN perform the translation

In case of NTN, AF may or may not transfer the geographical area information (i.e., the external MBS service area) to NEF/MBSF. If the AF provides an external service area, the CN needs to make sure the geographical area information can be transferred to the NG-RAN node. If the AF provides an internal service area, at least one of the involved CN functions needs to perform the translation to make sure the geographical area information can be transferred to the NG-RAN node. But the details of how to achieve this needs to be discussed in SA2 and CT4, RAN3 needs to send LS to SA2 and CT4 to ask them update their specifications accordingly.

If approach 2 is adopt, RAN3 needs to introduce a new IE (i.e. geographical area information) in the MBS service area information, the following is an example for RAN3 impacts, and corresponding TP and draft LS area provide in Annex B and Annex C.

#### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. Geographical Area Information; O; OCTET STRING(SIZE(FFS)).

Observation 5, if CN performs the translation, SA2, CT4 and RAN3 are all involved.

Proposal 1, RAN3 to discuss and down-select the following approaches to support broadcast geographical area to UE.

- Approach 1 RAN perform translation, update RAN3 procedure text, but may not work well after two rounds of translations.

- Approach 2 CN perform the translation, introduce new IE and send LS to SA2 and CT4.

In the Annex, a TP to TS 38.413 is to reflect the proposals in the contribution.

Proposal 2, RAN3 agree the TP to TS 38.413 or draft LS in the Annex based on the discussion.

3	Conclusion

In this contribution, we discussed how to support the broadcast service in NTN, and have the following observations and proposals:

Observation 1, either internal service area (i.e., list of cells or TAIs) or external service area (i.e., geographical area(s) or civic address(es)) may be provided from AF to establish the MBS broadcast session.

Observation 2, if the external MBS service area is provided, the NEF shall translate the received geographical area(s) or civic address(es) to a list of cell ID(s) and/or list of TAI(s) before relaying it to the MB-SMF and then to the NG-RAN node.

Observation 3, the existing codes for geographical area defined in core network can be used to represent the circle(s) and polygon(s) that agreed to be broadcast to UE as agreed by RAN2.

Observation 4, if NG-RAN node performs the translation, it will have no stage 3 impacts, but the geographical area broadcast to the UE may not be the same as the original geographical area from AF.

Observation 5, if CN performs the translation, SA2, CT4 and RAN3 are all involved.

Proposal 1, RAN3 to discuss and down-select the following approaches to support broadcast geographical area to UE.

- Approach 1 RAN perform translation, update RAN3 procedure text, but may not work well after two rounds of translations.

- Approach 2 CN perform the translation, introduce new IE and send LS to SA2 and CT4.

Proposal 2, RAN3 agree the TP to TS 38.413 or draft LS in the Annex based on the discussion.

4	Reference

[1]TS 23.247, Architectural enhancements for 5G multicast-broadcast services

[2] TS 29.522, 5G System; Network Exposure Function Northbound APIs; Stage 3

[3] TS 29.571, 5G System; Common Data Types for Service Based Interfaces; Stage 3

[4] TS 29.572, 5G System; Location Management Services; Stage 3

5	Annex A (TP to TS 38.413 for approach 1)

<<<<<<<<<<<<<<<<<<<< Change Start>>>>>>>>>>>>>>>>>>>>

## 8.17	Broadcast Session Management Procedures

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44]. In case of NTN, the NG-RAN node shall translate the Cell ID list or TAI list in MBS Service Area IE to the area information broadcast to UE as specified in TS 38.331 [18].

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8]. If the NG-RAN node decides to not establish shared NG-U resources towards the 5GC it shall include the Shared NG-U Not Established IE set to "not established" in the MBS Session Setup or Modification Response Transfer IE.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

8.17.2	Broadcast Session Modification

8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. In case of NTN, the NG-RAN node shall translate the Cell ID list or TAI list in MBS Service Area IE to the area information broadcast to UE as specified in TS 38.331 [18].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>

6	Annex B (TP to TS 38.413 for approach 2)

<<<<<<<<<<<<<<<<<<<< Change Start>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| Geographical Area Information | O |  | OCTET STRING(SIZE(FFS)) |  |

Editor note, the Size of the IE can be wait for RAN2 progress.

<<<<<<<<<<<<<<<<<<<< Change end>>>>>>>>>>>>>>>>>>>>

6	Annex C (Draft LS to SA2/CT4 for approach 2)

3GPP TSG-RAN WG3 #125bis		R3-24xxxx

Hefei, China, 14th – 18th October 2024

Title:	LS on broadcast intended area in NTN

Response to:

Release:	Release 19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN3

To:	SA2, CT4

Cc:	RAN2

Contact Person:	Lisi Li (lilisi@xiaomi.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 would like to inform SA2/CT4 about recent decisions made in broadcast intended area in NTN in RAN3:

RAN3 agreed to introduce geographical area information (i.e., Circle(s) or Polygon(s)) in MBS service area over NGAP, so that gNB onboard can broadcast this information to the UE, this geographical area information can be initially from AF to the SMF/AMF and then relayed to NG-RAN node in Broadcast Session Setup procedure and Broadcast Session Modification over NGAP.

2. Actions:

To SA2 and CT4

RAN3 kindly asks SA2 and CT4 to update their specifications to support gNB onboard obtains the geographical area information.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #126	18-22 Nov 2024			Orlando, USA

TSG-RAN3 Meeting #127	17-21 Feb 2025			Athens, GR