3GPP TSG-RAN WG3 Meeting #125bis	R3-245086

# Hefei, China, 14 – 18 October 2024

Agenda item:	20.2

Source:	Xiaomi

Title:	Discussion on AI-based positioning

Document for:	agreement

1	Introduction

According to the WID in [1], case 3b and case 3a will be discussed in this contribution.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Architecture scope:

The split architecture may not need to be supported in the positioning use case or can be considered as a low priority, given that other AI Air use cases (CSI and BF) do not consider it due to the absence of RAN3 related objectives for these two use cases. RAN3 should primarily focus on the non-split architecture and its impacts on NRPPa, while considering F1 impacts only if necessary.

Proposal 1, RAN3 discuss non-split architecture and focus on the stage2 procedure and NRPPa impacts first.

2	Case 3b

According to the protocol aspects outlined in 7.2.4 of TR 38.843[2], LMF can serve as the entity for model training, model inference, and management in case 3b. While SA2 is currently investigating the interaction between LMF and NWDAF regarding model training aspects, it falls outside the scope of RAN.

- Model Training: o For LMF-side model, the LMF is the termination point for training data. - Inference: o For LMF-side model inference, the UE or gNB can generate the necessary input data while the termination point for this input data lies within the LMF where the inference process is performed. - Management: o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE. o Monitoring:  For monitoring at the LMF side, the gNB or UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination points for these metrics is the LMF.

For case 3b, the deployment scenario can be as follows:

| Deployment | Inference entity | Training entity | Management entity |
| --- | --- | --- | --- |
| Scenario | LMF | LMF | LMF |

Figure 1 illustrates a potential deployment scenario for case 3b, based on the functional framework detailed in section 4.4 of TR 38.843[2].

Figure 1 deployment scenario for case 3b

From RAN3 perspective, we think RAN3 can focus on data transmission including training data, inference data and monitoring data from gNB to LMF.

AI model training

According to the RAN1 agreements, there are two sets of parameters for model training. Part A consists of measurements from gNBs, while Part B includes label information that does not involve gNB. RAN3 can focus on the Part A parameters.

Agreement (RAN1#116 bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Figure 2 illustrates the possible call flows for data collection and model training in case 3b. It can also be seen that the existing NRPPa measurement procedures can be used for training data collection from gNB to LMF.

Figure 2 call flows of data collection and training for case 3b

AI model inference

Figure 3 illustrates the possible call flows for data collection and model inference in case 3b. It can be seen that the existing NRPPa measurement procedures can be used for inference data collection.

Figure 3 call flows of data collection and inference for case 3b

The content of training data and inference data

For the content of the measurement data (either training or inference), two types of measurements are studied in RAN1: sample-based and path-based, as per the RAN1 agreement below.

If path-based measurements are used, the existing measurement signaling can be reused.

For sample-based measurements, new measurement types such as DP, PDP, and CIR need to be introduced, the detailed definitions are under RAN1 discussion. RAN3 can wait for RAN1’s progress to design stage 3 signaling to support sample-based measurements.

Agreement (RAN1#116) In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

Observation 1, the training or inference data from gNB in case 3b can be sample-based or path-based, which are still under RAN1 discussion.

Observation 2, existing measurement signaling can be reused for path-based measurements.

Observation 3, new measurement types and parameters need to be introduced for sample-based measurements.

Proposal 1, to support case 3b, RAN3 agrees that existing signaling can be reused for training and inference data collection in case of path-based measurements.

Proposal 2, RAN3 wait for RAN1’s progress on the details of sample-based measurements for case 3b.

Model monitoring

For monitoring data, it is unclear whether any assistance information is needed from the gNB side or if the LMF can handle it independently. We can wait for RAN1’s clarification

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Observation 4, According to RAN1 agreements, it is unclear whether and what assistance information and/or measurement reports may be sent from gNB to LMF to assist with performance monitoring.

Proposal 3, RAN3 wait for RAN1’s progress on the monitoring data from gNB for case 3b.

3	Case 3a

According to the protocol aspects in 7.2.4 of TR 38.843[2], in case 3a (i.e. gNB-side model), gNB can be the entity for model training, model inference and management.

- Model Training: o For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. - Inference: o For gNB-side model inference, input data is internally available at gNB. For this case, the UE can also generate the necessary input data while the termination point for this input data lies within the gNB where the inference process is performed. - Management: o For gNB-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB. o Monitoring:  For monitoring at the gNB side, and if needed, calculated performance metrics or data required for performance metric calculation, can at least be generated by the gNB.

For case 3b, the deployment scenario can be as follows:

| Deployment Scenario | Inference entity | Training entity | Management entity |
| --- | --- | --- | --- |
|  | gNB | gNB (OAM) | gNB (OAM) |

According to RAN1 agreement, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB, while the label and its related data (e.g., time stamp) can be generated by at least LMF, which means LMF needs to provide the label information to gNB for training purpose.

Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

Figure 4 illustrates the potential deployment scenario for case 3a, the gNB performs AI training, inference and management.

Figure 4 deployment scenario 1 for case 3b (gNB is responsible for management)

AI model training

According to SA2’s progress, the LMF can select UE with consent for positioning data collection. If the gNB needs the training data, it sends the data collection request to LMF, so that the LMF can select the UE with consent and initiates the exciting positioning procedure.

Same as case 3b, there’re two parts of parameters for AI training data. i.e. measurements info and label info. The measurement info can be collected by gNB itself and the measurement is triggered by LMF via measurement request. The label info can be also provided to gNB in measurement request message, so that the gNB can be aware of the relation between the measurements and the corresponding label for AI model training and management. In our view, to protect the user location privacy, the label info can be the AoA or TDOA that are calculated by the LMF considering the TRP location and the ground truth location of the UE (or PRU).

Figure 5 illustrates the possible call flows for data collection and model training in case 3a.

Figure 5 call flows of data collection and training for case 3a

Observation 5, according to SA2 progress, LMF is responsible for UE selection by knowing the UE consent and data requirement in positioning.

Proposal 4, LMF can select UE with consent and trigger existing positioning procedures as per gNB’s indication of data collection so that the gNB and perform the measurement for AI model training in case 3a.

Proposal 5, LMF provide the label info in the measurement request, so that the gNB can use it for AI model training in case 3a.

AI model inference

For gNB-side model inference (Case 3a), input data is internally available at gNB, the output data can be the existing measurement results. Thus, the existing signaling can be reused for the model inference in case 3a. Figure 6 illustrates the possible call flows for data collection and model inference in case 3a. It can be seen that the existing NRPPa measurement procedures can be used for inference data collection.

The additional conditions had been discussed in TR 38.843[2], which can be divided into two categories: NW-side additional conditions and UE-side additional conditions. For gNB-side model, the additional conditions for model management (e.g. whether to use AI feature or not, when to fall back) should also be considered, but what’s the additional conditions may need further discussion in RAN1.

Figure 6 call flows of data collection and inference for case 3a

Proposal 6, RAN3 agree the existing procedure and signalling can be re-used for AI model inference of case 3a.

Proposal 7, gNB needs to manage the gNB-side AI model based on the additional conditions, the details are pending to RAN1 discussion.

4	Conclusion

In this contribution, we discussed the AI-based positioning from RAN3 perspective and have the following proposals.

Architecture scope

Proposal 1, RAN3 discuss non-split architecture and focus on the stage2 procedure and NRPPa impacts first.

Case 3b

Observation 1, the training or inference data from gNB in case 3b can be sample-based or path-based, which are still under RAN1 discussion.

Observation 2, existing measurement signaling can be reused for path-based measurements.

Observation 3, new measurement types and parameters need to be introduced for sample-based measurements.

Observation 4, According to RAN1 agreements, it is unclear whether and what assistance information and/or measurement reports may be sent from gNB to LMF to assist with performance monitoring.

Proposal 1, to support case 3b, RAN3 agrees that existing signaling can be reused for training and inference data collection in case of path-based measurements.

Proposal 2, RAN3 wait for RAN1’s progress on the details of sample-based measurements for case 3b.

Proposal 3, RAN3 wait for RAN1’s progress on the monitoring data from gNB for case 3b.

Case 3a

Observation 5, according to SA2 progress, LMF is responsible for UE selection by knowing the UE consent and data requirement in positioning.

Proposal 4, LMF can select UE with consent and trigger existing positioning procedures as per gNB’s indication of data collection so that the gNB and perform the measurement for AI model training in case 3a.

Proposal 5, LMF provide the label info in the measurement request, so that the gNB can use it for AI model training in case 3a.

Proposal 6, RAN3 agree the existing procedure and signalling can be re-used for AI model inference of case 3a.

Proposal 7, gNB needs to manage the gNB-side AI model based on the additional conditions, the details are pending to RAN1 discussion.

5	Reference

[1]	 RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] 3GPP TR 38.843 V18.0.0 (2023-12), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface