3GPP TSG-RAN WG3 Meeting #125bis	R3-245098

Hefei, China, from 14th to 18th Oct 2024

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	Discussion on Direct Positioning (Case 3b)

Document for:	Discussions & Approval

1	Introduction

The objectives of AI/ML for air interface WI [1] are listed as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the current discussions in RAN1 and RAN2, the topic that requires normative work in RAN3 is AI/ML-based positioning. Further analysis indicates that Case 3a and Case 3b may have impacts on RAN3. In this contribution, we provide our understanding on the case 3b about direct AI/ML positioning.

2	Discussion

As described in TR 38.843 [2], the AI/ML assisted positioning (case 3a) is illustrated below:

Figure 1. Direct AI/ML Positioning

For Direct AI/ML positioning, the principle is to generate the UE location directly using an AI/ML model based on the input information. In Case 3b, the model is located on the LMF side, meaning the LMF uses input information from the UE/gNB to produce the UE location. Therefore, the case 3a during measurement report is further illustrated in Figure 2 below:

Figure 2. General procedures of NG-RAN node assisted positioning with LMF-side model, AI/ML direct positioning

- In Case 3b, the UE and NG-RAN node provide input information to the LMF, which uses an AI/ML model to calculate the UE location.

RAN3 needs to specify new measurements, which are not supported in the current specification. For the new measurement, following is the corresponding agreement from RAN1:

Agreement

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

Agreement

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

Agreement

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time.

- FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time

Agreement

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

It can be seen that time domain channel measurements are considered as input information reported from the NG-RAN node to the LMF. The existing Measurement Report message over NRPPa and F1AP can be reused.

However, RAN1 still needs more time to finalize whether sample-based or path-based measurements will be used. Currently, only a more general reporting method has been agreed upon: for timing, the UL discussion is to reuse the existing RTOA time, and for power, RSRPP is used as the starting point.

- In Case 3b, time domain channel measurements are considered as input information reported from the NG-RAN node to the LMF. The details of time domain channel measurements are still pending and subject to further discussion in RAN1.

- The existing Measurement Report message over NRPPa and F1AP can be reused for input information transferring.

3	Conclusion

Following is our observations and proposals:

- In Case 3b, the UE and NG-RAN node provide input information to the LMF, which uses an AI/ML model to calculate the UE location.

- In Case 3b, time domain channel measurements are considered as input information reported from the NG-RAN node to the LMF. The details of time domain channel measurements are still pending and subject to further discussion in RAN1.

- The existing Measurement Report message over NRPPa and F1AP can be reused for input information transferring.

4	Reference

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] TR 38.843