3GPP TSG-RAN WG3 Meeting #125 bis	             R3-245106

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	17.2

Source:	CMCC

Title:	(TP for TS 38.473) Discussion on On-Demand SSB SCell Operation

Document for:	Discussion

Introduction

In the RAN#102 meeting, a new Work Item for network energy saving enhancement has been approved in [1] including the following objective on on-demand SSB SCell operation:

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss on the support of on-demand SSB SCell operation relevant to above objective.

Discussions

RAN 1 Progress and Agreement

In the RAN1 #116 meeting, the following agreements were made for on-demand SSB SCell operation [2]:

Agreement (RAN1#116) Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission.

In the RAN1 #116 Bis meeting, the following agreements were made for on-demand SSB SCell operation [3]:

Agreement (RAN1#116 Bis) Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

In the RAN1 #117 meeting, the following agreements were made for on-demand SSB SCell operation [4]:

Agreement (RAN1#117) Agreement (Signalling) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement (Contents) For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB.

In the RAN1 #118 meeting, the following agreements were made for on-demand SSB SCell operation [5]:

Agreement (RAN1#118) Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, and aperiodic L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB. - FFS: Any other relevant parameters.

RAN1 has outlined two cases for on-demand SSB: one where no always-on SSB is present on the cell, and another where always-on SSB is periodically transmitted on the cell with configurable periodicity. Additionally, four scenarios have been identified, primarily focusing on the timing related to the UE receiving the SCell activation.

Discussions on On-Demand SSB SCell Operation

In the case of the split architecture, RAN3 discusses whether the gNB-CU or gNB-DU is responsible for deciding on activation/deactivation of on-demand SSB on cells.

In the current specification [7], the gNB-CU could indicate the list of cells allowed to be deactivated through the Cells Allowed to be Deactivated List IE contained in the GNB-CU CONFIGURATION UPDATE message, after receiving this, the gNB-DU considers that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

The gNB-CU has comprehensive knowledge of the status of all cells connected to the gNB-DUs it controls, as well as information about neighbouring cells similar to its indication of the list of cells allowed to be deactivated. As a result, the gNB-CU could indicate the list of cells allowed to be on-demand SSB activated.  After a gNB-DU receives this through the GNB-CU CONFIGURATION UPDATE message, it considers that is allowed to enter on-demand SSB operation mode for network energy saving purpose. Furthermore, the gNB-CU is responsible for configuring the mode (on-demand, periodic) and parameters associated with SSB transmission.

The gNB--DU is responsible for both controlling and performing the SSB transmission. Consequently, it is the gNB--DU that consumes energy for these transmissions. Because the gNB-DU handles the transmission, it can directly monitor and be aware of the energy used for SSB transmissions. Most of the energy required for baseband processing in the network is also consumed by the gNB-DU, which means that the gNB-DU is well-positioned to evaluate the energy efficiency of a cell. The gNB-DU can monitor the energy efficiency of a cell. If the energy efficiency is low—perhaps due to low user data rates or because only a few UEs are connected to the cell—the gNB-DU can take action to improve efficiency. How energy efficiency is defined and when to trigger the gNB-DU to take action are out of scope of this contribution. However, we view that the gNB-DU has the knowledge of cells’ status in terms of energy efficiency. The gNB-DU could send the energy efficiency to the gNB-CU in the ﻿RESOURCE STATUS UPDATE.

Based on the updated resource status of the gNB-DU, one approach is that the gNB-CU might determine to update the list of cells allowed to be on-demand SSB activated. This could involve deactivating some cells that are currently in the on-demand SSB activated state and adding new cells to the list of those allowed for on-demand SSB activation. For cells that are currently in the on-demand SSB activated state but need to be deactivated, the GNB-CU CONFIGURATION UPDATE and Cells to be Activated List Item IE can be reused to allow the gNB-CU to request, via the F1 interface, that the gNB-DU switch on certain cells.

The gNB-CU is responsible for configuring the mode (on-demand, periodic) and parameters associated with SSB transmission.

The gNB-CU could indicate the list of cells allowed to be on-demand SSB activated to gNB-DU.

For F1 interface, a new IE needs to be defined to carry the list of cells allowed to be on-demand SSB activated, and associated SSB transmission mode and parameter(s).

The gNB-DU could update the gNB-CU its cells status for the gNB-CU to update the list of cells allowed to be on-demand SSB activated.

After SCells are established for a UE through the F1 UE context setup or modification procedure, the gNB-DU is responsible for deciding whether to activate or deactivate these SCells for the UE. It is also logical for the gNB-DU to decide when to start and stop on-demand SSB transmissions for these cells. As agreed in the previous RAN1 meetings, the RRC and MAC CE based signaling to indicate on-demand SSB transmission are supported.

- RRC-Based Signaling:

  - The gNB-DU decides on the SSB transmission for each SCell.

  - This decision is communicated to the gNB-CU and then to the UE.

  - RAN3 needs to discuss whether this signaling can be explicitly indicated via the F1AP or within the RRC container.

- MAC CE-Based Signaling:

  - No impact on RAN3 is anticipated with MAC CE based signaling.

For RRC-based signalling for indication of on-demand SSB transmission to UE, the gNB-DU decides on the SSB transmission for each SCell.

RAN3 needs to discuss whether this RRC-based signaling can be explicitly indicated via the F1AP or within the RRC container.

No impact on RAN3 is anticipated with MAC CE based signaling.

Conclusions

In this submission, we discuss the support of on-demand SSB SCell operation. The following observations and proposals are made:

- RAN1 has outlined two cases for on-demand SSB: one where no always-on SSB is present on the cell, and another where always-on SSB is periodically transmitted on the cell with configurable periodicity. Additionally, four scenarios have been identified, primarily focusing on the timing related to the UE receiving the SCell activation.

- The gNB-CU is responsible for configuring the mode (on-demand, periodic) and parameters associated with SSB transmission.

- The gNB-CU could indicate the list of cells allowed to be on-demand SSB activated to gNB-DU.

- For F1 interface, a new IE needs to be defined to carry the list of cells allowed to be on-demand SSB activated, and associated SSB transmission mode and parameter(s).

- The gNB-DU could update the gNB-CU its cells status for the gNB-CU to update the list of cells allowed to be on-demand SSB activated.

- For RRC-based signalling for indication of on-demand SSB transmission to UE, the gNB-DU decides on the SSB transmission for each SCell.

- RAN3 needs to discuss whether this RRC-based signaling can be explicitly indicated via the F1AP or within the RRC container.

- No impact on RAN3 is anticipated with MAC CE based signaling.

References

- RP-234065 New WID: Enhancements of network energy savings for NR

- Final Report of 3GPP TSG RAN WG1 #116 v1.0.0 	(Athens, Greece, February 26th - March 1st, 2024)

- Final Report of 3GPP TSG RAN WG1 #116b v1.0.0 (Changsha, China, April 15th – 19th, 2024)

- Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0 (Fukuoka, Japan, May 20th – 24th, 2024)

- Draft Report of 3GPP TSG RAN WG1 #118 v0.1.0 	(Maastricht, The Netherlands, August 19th – 23rd, 2024)

- TS 38.473 V18.2.0

TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

- 8.2.5	gNB-CU Configuration Update

- 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procegNB-DUre is to update application level configuration data needed for the gNB-GNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procegNB-DUre does not affect existing UE-related contexts, if any. The procegNB-DUre uses non-UE associated signalling.

- 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procegNB-DUre: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-GNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-GNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-GNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-GNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-GNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-GNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-GNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to Be On-Demand SSB Activated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-GNB-DU shall, if supported, consider that it is allowed to activate on-demand SSB operation status in the indicated cells for network energy saving purpose.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

- 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-GNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBGNB-DU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBGNB-DU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-GNB-DU | O |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-GNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBGNB-DU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. | - |  |
| >>Mobile IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the mobileIAB-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the mobileIAB-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the mobileIAB-Support is sent in SIB1. | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >>E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>>E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBGNB-DU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-GNB-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-GNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-GNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBGNB-DU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to Be On-Demand SSB Activated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be On-Demand SSB Activated List Item |  | 1 .. <maxCellingNBGNB-DU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBGNB-DU | Maximum numbers of cells that can be served by a gNB-GNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-GNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |