3GPP TSG-RAN WG3 Meeting #125 bis	             R3-245110

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on UL PSI based PDU discarding in NR-DC

Document for:	Discussion

Introduction

A reply to RAN3 LS on UL PSI based PDU discarding in NR-DC is returned from RAN2 [1]. The response is as following:

RAN2 thank RAN3 for the questions on PSI-based SDU discard on UL split bearer in NR-DC. The following is the current UE behavior: - UE activates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE activating the discard, regardless of whether the MAC CE is received from MN or SN; - UE deactivates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE deactivating the discard, regardless of whether the MAC CE is received from MN or SN. It is up to RAN3 to discuss whether any network side solution to improve PSI-based SDU discard is needed.

Discussions

It has been verified by RAN 2 that the PSI-based SDU discard on a UL split bearer can be activated independently at UE after it receives a PSI-Based SDU Discard activation MAC CE either from MN or SN.  In the meantime, the PSI-based SDU discard on a UL split bearer can be deactivated independently at UE after it receives a PSI-Based SDU Discard deactivation MAC CE either from MN or SN.

The congestion at MN and SN is local to each respective node, with each node independently sending a MAC CE to the UE to activate or deactivate the UL PSI-based SDU discard, depending on the congestion level of the specific node.

Howeverr, the congestion status of MN and SN is unknown to each other, and each node activate and deactivate the UL PSI-based SDU discard at the UE independently, without consideration of the other node's congestion state. This can result in sub-optimal handling of UL split bearer traffic.

RAN3 needs to resume the discussion on whether/how to support UL PSI-based SDU discard coordination between MN and SN for the split bearer scenario in DC.

Conclusions

In this submission, we discussed the PSI-based SDU discard on UL split bearer in NR-DC. The following proposal is given:

- RAN3 needs to resume the discussion on whether/how to support UL PSI-based SDU discard coordination between MN and SN for the split bearer scenario in DC.

References

R3-245011, Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC