3GPP TSG-RAN WG3 Meeting #125bis	R3-245127

# Hefei, China, October 14th – 18th, 2024

Agenda item:	16.2

Source: 	Samsung

Title: 	Further discussion on RAN architecture for Ambient IoT

Document for:	Discussion and decision

Introduction

Last meeting RAN3 continued to discuss the RAN architecture for Ambient IoT, and the following open issue is captured,

System architecture figures agreed for Topology1 and Topology2.

For Topology2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting.

Continue to discuss the architectures and protocol stack.

In this contribution, we further discuss the RAN architecture aspects for Ambient IoT.

Discussion

The RAN architecture aspect for Ambient IoT was discussed last several meetings. During last meeting, separate architecture figures have been agreed and captured in the latest version of TR 38.769 [1] for Topology 1 and Topology 2.

In addition, the system architecture and protocol stack of Topology 1 have been captured clearly in TR 38.769, and the further details are still up to SA2 progress. According to TR 23.700-13, SA2 has not achieved conclusion yet on the architecture, so it would be better for RAN3 to wait for SA2 progress for Topology 1.

While for the Topology 2, only the system architecture has been captured in TR 38.769, while the protocol stack for Topology 2 has not been discussed last meeting due to the lack of time. So the most urgent thing for now to is to provide the protocol stack for Topology 2.

Note that there are basically three agreed solutions for Topology 2 which directly impact the design of the the protocol stack. The solutions are listed as follows,

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: Depiction and further details of the options above are FFS

And also note that the captured protocol stack for Topology 1 still assumes a SCTP-based transport between AIoT RAN and AIoT CN. As a result, we see no showstopper to also adopt the SCTP-based transport between AIoT CN and AIoT-enabled gNB for Topology 2, especially when the current 5G CN could be enhanced to serve as the AIoT CN for Topology 2.

Consequently, similar to the protocol stack captured for Topology 1, the protocol stack for three solutions for Topology 2 can be shown as follows,

- RRC based solution:

- NAS based solution:

- UP based solution:

Recall that the main AIoT-enabled gNB functionality is to allocate the radio resources used over AIoT radio. And we can directly reuse the radio resource allocation mechanisms for the RRC based solution and NAS based solution without too much extra effort, i.e. both dynamic radio resource allocation as well as the semi-persistent radio resource allocation (AIoT-enabled gNB configured resource pool for AIoT-enabled UE, and AIoT-enabled UE can select the resources within the resource pool).

While for the UP based solution, it is still unclear how the radio resources are allocated by AIoT-enabled gNB. It seems that the only way the AIoT-enabled gNB allocating the AIoT radio resources is to configure the AIoT radio resource pool blindly, which is not a proper way of resource allocation.

As a result, we prefer RRC based solution or NAS based solution as a way forward.

Proposal 1: Capture the above figures as the protocol stack for Topology 2.

Proposal 2: Either RRC based solution or NAS based solution is further pursued as the solution for Topology 2.

Conclusion

In this paper, we provide further discussion on RAN architecture for Ambient IoT. The following proposals are provided,

Proposal 1: Capture the above figures as the protocol stack for Topology 2.

Proposal 2: Either RRC based solution or NAS based solution is further pursued as the solution for Topology 2.

Reference

[1] TR 38.769 v1.0.0