3GPP TSG-RAN WG3 Meeting #125bis	R3-245128

# Hefei, China, October 14th – 18th, 2024

Agenda item:	16.3

Source: 	Samsung

Title: 	Further discussion on RAN-CN interface impact for Ambient IoT

Document for:	Discussion and decision

Introduction

Last meeting continued to discuss the RAN-CN interface impact for Ambient IoT, and the open issues are provided as follows,

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

In this contribution, we further discuss these open issues.

Discussion

The RAN-CN interface impact for Ambient IoT continued to be discussed last meeting. And several open issues are captured.

For the content in Inventory/Command, there was little discussion on this open issue during last meeting due to the lack of time. According to the outcome of the May meeting, there are several open issues captured as follows,

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

Note that the May meeting has agreed to introduce AIoT Device Identification as well as Scope (related to area) in the Inventory Request message, while no agreement has been achieved on the content in Command message.

For the remaining potential parameters that can be included in the Inventory Request message, we think the Inventory Session ID is also needed. Recall that RFID supports multiple sessions for inventory. In order to provide a comparable capability for Ambient IoT to RFID. We see no obstacle to support multiple inventory sessions. In our understanding, the inventory session is determined by AF/CN, and a session related identifier should be provided in the Inventory Request message.

While for other potential parameters, the Estimated Number of expected responses from devices can be further discussed depending on the use cases and solutions. In our understanding, the purpose of the Inventory is to let the server understand how many and which AIoT devices are in the scope. So whether the Server or AIoT CN could estimate the number of expected responses is actually depending on SA2 progress. On the other hand, the usage of the Estimated number of expected responses may be used for determining the Q value used for Random Access which is depending on RAN2 progress. So RAN3 may not be able to achieve consensus on whether to introduce such parameter in Inventory Request message until more progress can be achieved in other WGs.

In addition, we see no strong need to introduce Periodicity in Inventory Request message since the periodic inventory can be easily achieved by sending another Inventory Request message.

Proposal 1: Consider the Inventory Session ID as the content of Inventory Request message.

From the other aspect, the content of Command sent from CN to RAN has not been discussed last meeting. In our understanding, at least the following information can be included in the Command message,

- The Command container which is provided at NAS level.

- AIoT Device Identification. If the Command message is sent by setting up device context after inventory and before the command, such device related information can be AP UE IDs.

- The information related to the session. If the information related to the targeted Ambient IoT device, e.g. an device identifier used over RAN-CN interface is not globally unique, then the session related information may be needed if the device identifier used over RAN-CN interface can be allocated per session.

- The information on whether a reply is needed in response to the Command. The Command may contain Read command, Write command, etc. at the NAS or application layer level to perform corresponding operations with the storage of AIoT devices. For some command such as Write command, a reply from the AIoT device may not be needed; while for some other command such as Read command, a reply from the AIoT device is needed. In order to leverage the AIoT RAN to perform resource allocation for AIoT radio, the AIoT RAN needs to know whether the UL resources for a transmission should be allocated. Therefore the information on whether a reply is needed in response to the Command should be signalled from AIoT CN to AIoT RAN.

Proposal 2: Consider the following information as the content of Command message,

- The NAS container containing Command sent to UE.

- AIoT device identification.

- The information on whether a reply is needed in response to the Command.

For the next open issue,

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Regarding the question on which node performs the reader selection, potentially there are two options:

- AIoT CN performs the reader selection

- AIoT RAN/AIoT-enabled gNB performs the reader selection

In our understanding, firstly we need to understand the purpose to select a reader. For now there are two kinds of services that are considered firstly which need to select a reader, namely, inventory-only service and command service. For inventory-only service, the purpose to select a reader is to let the reader performs inventory operation over a service area. While for the command service, the purpose to select a reader is to find a suitable reader that can communicate with the targeted AIoT device. As a result, the node required to perform reader selection needs to be able to understand the coverage area for each reader, and needs to be able to understand which reader can reach the targeted AIoT device.

In our opinion, for Topology 1,

- Both AIoT CN and AIoT RAN understand which reader can reach the targeted AIoT device by previous Inventory procedure.

- AIoT RAN potentially has better understanding than AIoT CN on the coverage area for each reader.

While for Topology 2,

- Both AIoT CN and AIoT-enabled gNB understand which reader can reach the targeted AIoT device if the RRC based solution is adopted

- Only AIoT CN understands which reader can reach the targeted AIoT device if the NAS based solution is adopted.

- AIoT-enabled gNB potentially has better understanding than AIoT CN on the coverage area for each reader.

As a summary, each option on reader selection has pros and cons, and for Topology 2 it also depends on which solution for Topology 2 to adopt.

Proposal 3: RAN3 is kindly asked to discuss which node to perform reader selection functionality by taking the above pros and cons into account.

Conclusion

In this paper, we provide primary discussion on RAN-CN interface impact for Ambient IoT. The following proposals are provided,

Proposal 1: Consider the Inventory Session ID as the content of Inventory Request message.

Proposal 2: Consider the following information as the content of Command message,

- The NAS container containing Command sent to UE.

- AIoT device identification.

- The information on whether a reply is needed in response to the Command.

Proposal 3: RAN3 is kindly asked to discuss which node to perform reader selection functionality by taking the above pros and cons into account.

Reference