3GPP TSG-RAN WG3 Meeting #125bis	   				         R3-245139

Hefei, China, 14th – 18th Oct 2024

Agenda Item:		14.2

Source:			China Telecom

Title:		Support of MBS broadcast service

Document for:	 	Discussion & Decision

- Introduction

The following agreement has been made at the end of previous RAN3 meeting:

Turn WA into agreement: Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

Other options can be further checked.

In this contribution, we will further discuss the potential enhancement to MBS broadcast service topic.

- Discussion

Due to the fact that the actual MBS broadcast service area in NTN scenarios may be polygonal or irregular shapes, RAN3 can discuss whether additional indicator information needs to be introduced to support finer grained area partitioning (e.g. GNSS description). However, some companies are concerned that introducing new IE require additional enhancements to NGAP, which may affect AF and CN nodes. In our view, geographical area information may help operators provide accurate E2E broadcasting services, The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information in CN. Introducing new geographical area information into RAN and CN interface protocols is not complicated.

On the other hand, considering RAN2 has already made such agreement, it is proposed for RAN3 to introduce the geographical area information for the MBS broadcast service area in NGAP. The signaling format can be further studied.

Support of Broadcast

Agreements:

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Proposal 1: Geographical area information may be introduced as the intended service area for MBS broadcast service in NGAP.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Geographical area information may be introduced as the intended service area for MBS broadcast service in NGAP.

- References

[1]	RP-240775, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.