3GPP TSG-RAN WG3 #125-bis                                                                 R3-245164

Hefei, China, 14 - 18 October, 2024

Agenda item:	13.2

Source:	Samsung

Title:	(TP to BLCR for TS 38.401) Procedure on Inter-gNB-CU LTM

Document for:	Approval

Introduction

This contribution contains a TP for LTM BLCR for TS38.401 to introduce the procedure for inter-gNB-CU LTM.

Text Proposal to 38.401

============ Start of the change ==============

### 8.9.x	Inter-gNB-CU LTM

Figure 8.9.x-1 shows the overall procedures used for inter-gNB-CU LTM involving F1.

Figure 8.9.x-1: Inter-gNB-CU LTM procedure

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	Source gNB-CU gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s).

Editor’s Note: FFS on whether one candidate cell or a list of candidate cells is contained in the message.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the LTM lower layer RRC configurations.

6.  If the candidate gNB-CU accepts the request of LTM configuration, it responds with a HANDOVER REQUEST ACKNOWLEDGE including LTM configuration information of LTM candidate cell to the source gNB-CU.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB-CU may send a new UE associated Class-1 XnAP procedure to the candidate gNB(s) containing the information for subsequent LTM.

Editor’s Note: Details are FFS on step 9.

10.	The candidate gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information of candidate cells for subsequent LTM.

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

12.	The candidate gNB-CU sends the response to the source gNB-CU.

Editor’s Note: FFS on whether step 9 and 12 are mandatory or optional and FFS on the message name.

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 1: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via the TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 20.

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB-DU.

23.	The source gNB-DU determines to initiate inter-gNB-CU LTM.

24.	The source gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM.

26.	The source gNB-CU sends CELL SWITCH NOTIFICATION message to target gNB-CU.

Editor’s Note: Details are FFS on step 26.

27.	The target gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32/33. The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell. In return, the source gNB-CU sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Editor’s Note: Details are FFS on above note.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	The target gNB-CU may send UE CONTEXT RELEASE to inform the source gNB-CU about the success of the handover if no LTM candidate cell(s) exist in the source gNB. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

37.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

38.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

NOTE 3: The steps 17-38 can be performed multiple times for subsequent LTM cell switch executions.

============ End of the change ==============