3GPP TSG-RAN WG3 Meeting #125bis														R3-245178

Hefei, China, 14th - 18th October 2024

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.300) Discussion on the support of Regenerative payload

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 meeting discussed the options to manage the NG interface:

-	Option 1: NG removal/Setup

-	Option 2: NG suspend/resume

This contribution analyses the options and proposes a way forward. This contribution also discusses the FLSO, and the support for RRC INACTIVE UE.

# 2	Management of NG interface

Last RAN3 meeting agreed Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Further discussions on benefits to introduce Option 3 NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companies to have 2 methods for NG interface management.

The Stage-2 and Stag-3 TP have been proposed in last meeting, and resubmitted for this meeting. We proposed to agree the Stage-2/3 TPs.

Proposal 1-1: agree the Stage-2/3 TPs for NG Removal.

We think there is no need to introduce Option 3 NG Suspend/Resume for following reasons:

- The saved NG context is very limited.

Unlike UE suspend that a lot of UE context is stored and resumed for further resume, the NG interface does not have much context. The main saved context for NG interface is the supported TAIs.

A NTN system based on NGSO may have thousands of satellites or tens of thousands of satellites. For example, Starlink has launched ~5400 satellites, and plan to expand the number of satellites to 12000. Due to earth rotation, every satellite in a NGSO system can visit a specific geographical area but at different time. Similar to terrestrial network, an AMF in NTN serving UEs in specific areas. For example, an AMF deployed in USA may only serve the UEs that are located in the USA. For every satellite to serve USA, it needs to have setup NG with the AMF in the USA. Currently, there is no method to remove an established NG interface. This means an AMF need to maintain a very large number of NG interfaces (including IPSec, TNL connection), e.g. NG interface with every satellite in NGSO system.

On the other hand, at any specific timing duration (e.g. 1-hour), only a limited number of satellites pass USA and need to setup NG (or have “active” NG) with the AMF in USA.

It is very inefficient for AMF to manage a very large number of NG interfaces, while only a very small number of NG interfaces are “active” at any timing duration.

Observation 1-1: considering the potential very large number of satellites in NGSO, it is inefficient for an AMF to manage the NG with all satellites in a NGSO system.

It is similar situation to the gNB. There can be many (tens of AMFs or hundreds of AMFs) around the world, a NGSO satellite need to setup NG with AMFs around the world. At any specific timing duration (e.g. 1-hour), the gNB only need to have one or a few “active” NG interfaces with the AMFs in that specific geographical area. It is also not efficient for a NGSO satellite to maintain NG with an AMF when it does not serve the AMF’s serving area for a long duration (e.g. 12-hour).

Observation 1-2: considering the large number of AMFs around the world, it is inefficient for a gNB to manage the NG with all AMFs around the world.

To reduce the complexity in both gNB and AMF, there is no need to keep the NG when the NG is “not active” for a long period of time. Up to the implementation/configuration, the gNB may decide to remove the NG when it moves out of the serving area of the AMF. By removing the NG, both gNB and AMF does not need to maintain the NGAP application level data, and no need to maintain the NG TNL connection (i.e. IPSec tunnel, SCTP association)

Observation 1-3: It can reduce the complexity to both gNB and AMF if NG interface can be removed.

One may argue that the NG connection with the old AMF may be suspended when the satellite/gNB leaves, and be resumed when the satellite/gNB re-visit the area. However, there is no benefit for the NG suspend/resume. First, the argument for introducing the NG suspend/resume is to keep the NG application level configuration data when the gNB left. But it does not save the NG signaling (i.e. initiate a NG Setup procedure vs. initiate a NG Resume procedure). Second, the saved configuration data may be useless, especially in NSGO. This is due to two reasons: 1) The application level data (e.g. TACs) saved during suspend procedure is useless when the gNB resumes NG. For example, the last geo-area that satellites serves before it leaves USA is New York, but the first geo-area that the satellite serves for its next visit may be start from San Francisco. So the saved TACs in AMF during suspend is useless when the satellite start to serve USA from San Franciso in its next pass.  2) Due to the earth rotation, the geo-area that a NGSO satellite visit in each pass is different. This also causes the saved application level data useless.

Observation 1-4: there is no benefit to introduce NG suspend/resume procedure.

Based on above analysis, we propose to introduce a NG Removal procedure to reduce the complexity in both gNB and AMF.

Proposal 1-2: No need to introduce NG Suspend/Resume.

A draft NGAP TP can be found in Contribution ([3])

# 3	Feeder Link Switch Over without changing UE’s serving CN

Last RAN3 meeting agreed RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. So the following analysis only consider no change of UE’s serving AMF.

In Regenerative architecture, the TNL address of a satellite/gNB is anchored in the NTN GW, which ensures the DL NG-C/U IP packet with destination set to the gNB’s TNL address is routed to the correct NTN GW. The FLSO causes the changes of the gNB’s TNL address, i.e. from the old TNL address anchored in old NTN GW to the new TNL address anchored in the new NTN GW.

Soft FLSO

RAN3#124 has agreed the reply LS ([4]) to SA2 with following:

With respect to IP address changes due to soft feeder link switch:

RAN3 confirms that the gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures.

An example call flow is shown below:

Figure 1: Soft FLSO without changing 5GC

Stage-2 TP is needed to clarify the procedure for soft feeder link switch.

Proposal 2-1: Agree Stage-2 TP for soft Feeder Link Switch Over.

Stage-2 TP for FLSO can be found in Annex A – TP for TS 38.300

Hard FLSO

In case Hard FLSO is performed, the gNB only connect with the new NTN GW after disconnect from the old NTN GW. Since 5GC does not know the hard FLSO until gNB informs 5GC, the 5GC may continue sending the DL NG-C/U packet to the old NTN GW. The old NTN GW has no way to deliver it to the gNB or forward it to the new NTN GW. This causes packet loss.

Please note the issue does not exist in Transparent architecture. In transparent architecture, the gNB is on the ground, and is aware of the FLSO. The DL data is always sent to the gNB irrespective of the FLSO. For a UE within the gNB’s coverage, the FLSO only affect which NTN GW to be used by the gNB to send the DL data to the satellite. When a hard FLSO is to be performed, the gNB just stop the DL transmission and buffer the DL data. After the hard FLSO is completed, the gNB forward the buffered DL data to the satellite via the new NTN GW. So there may be no packet loss during the hard FLSO.

Observation 2-1: without knowing the hard FLSO, the 5GC (AMF/UPF) continue sending the DL NG-C/U to the gNB via old TNL address anchored in old NTN GW, and will be discarded at the old NTN GW.

It is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U. This can be supported by the RAN Configuration Update procedure or other NGAP procedure, which can be further discussed.

Observation 2-2: it is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U.

After the hard FLSO, gNB need to inform 5GC (AMF/UPF) for the change of its TNL address, so the AMF/UPF can use the new TNL address to send the DL NG-C/U. To change the DL TNL address for NG-U, the gNB can initiate the PDU Session Resource Modify Indication procedure per UE. Accordingly, the AMF need to request SMF then request UPF to modify the DL TNL address of DL F-TEID for every affected UE.

Considering the large coverage area of the NTN cell, it may be many UEs that will be affected by the FLSO.  This takes time, and can cause signaling storm to gNB/AMF/SMF/UPF.

Observation 2-3: existing Per-UE PDU Session Resource Modify Indication procedure may cause signaling storm and interruption, e.g. when many UEs are served by the gNB/satellite.

We propose RAN3 to study possible solutions to address this issue. Since it is same gNB before and after the FLSO, the TEID part of the DL F-TEID can remain unchanged. The UPF only need to switch the TNL address of the gNB (i.e. from the old TNL address anchored in old NTN GW to the new TNL address anchored in the new NTN GW). One potential solution is to use a non-UE-associated NGAP procedure to request CN to switch the TNL address of the gNB (i.e. from the old TNL address anchored in old NTN GW to the new TNL address anchored in the new NTN GW). By using this non-UE-associated procedure, the update the DL F-TEID for all UEs can be done in one procedure. This saves time and avoid signaling storm. It is FFS whether this procedure can be based on existing procedure or a new procedure.

It is worthy to note that there is similar issue discussed and concluded for mobile IAB in Rel-18. After the mobile IAB performed partial migration and received a new TNL address, the mobile IAB need to inform the gNB-CU-UP to use the new TNL address for DL F1-U. RAN3 agreed a new non-UE associated F1AP IAB UP Configuration Update procedure to update the DL TNL address for F1-U tunnels of all connected UEs, instead of using UE-associated procedure to update the DL TNL address for each connected UE.

An example call flow is shown as below.

Figure 2: Hard FLSO without changing 5GC

Step 1: before the hard FLSO, the gNB informs the 5GC about the hard FLSO, so the 5GC can suspend DL NG-C/U transmission. It is possible to enhance the RAN Configuration Update procedure.

Step 2: The gNB disconnect from old NTN Gateway, and connect with new NTN Gateway. The gNB receives a new TNL address anchored in the new NTN Gateway.

Step 3: The gNB setup the new TNL by using the new TNL address.

Step 4 and 5: The gNB send a RAN CONFIGURATION UPDATE message over the new TNL. The AMF can know the new TNL association is available in the gNB.

Step 6: The gNB request the 5GC (e.g. AMF, then AMF request SMF, then UPF) to use the new TNL address in DL-F-TEID for all connected UEs.  It is possible to reuse the RAN Configuration Update procedure performed in Step 4 for this purpose.

Step 7 and 8: After the TNL address of the DL F-TEID is updated, the gNB can initiate a RAN Configuration Update procedure to remove the old TNL address.

Proposal 2-2: RAN3 study the solution to reduce interruption and avoid signaling storm for hard FLSO,  and consider using a non-UE associated NGAP procedure to update the DL TNL address for NG-U tunnels of all connected UEs

# 4	INACTIVE UE

In Regenerative architecture, a UE may transition to RRC INACTIVE via Satellite1/gNB1, then resume in another Satellite4/gNB4. The Xn may only exist between the neighboring gNBs/Satellites. There may be no Xn between gNB1 and gNB4. This can cause the failure of Retrieve UE context.

Following are some options that RAN3 can consider:

- Option 1: before Xn is about to be disconnected, relocate UE context to next satellite.

This is described in Contribution ([4]):

The UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

Here is an example based on above solution:

- UE transition to RRC INACTIVE via gNB1. gNB1 only have ISL/Xn with next satellite/gNB (i.e. gNB2) of same orbit plane.

- gNB2 arrives. Before gNB2 move out, gNB1 initiate XnAP RAN paging. UE resume in gNB2. UE context is relocated to gNB2.  gNB2 have ISl/Xn with next satellite/gNB (i.e. gNB3)

- gNB3 arrives. Before gNB3 move out, gNB2 initiate XnAP RAN paging. UE resume in gNB3. UE context is relocated to gNB3.  …

This solution requires the UE need to be waken up in every next satellite, in order to relocate the UE context to the next satellite, in case ISL/Xn only exists between the neighboring satellites/gNBs. This may be performed very frequent, considering a LEO satellite may only serve a specific geo-area for a few minutes. Current TS 38.331 defines

PeriodicRNAU-TimerValue ::= ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

The minimal periodic RNA Update timer is 5-min. This solution consumes more battery.

- Option 2: Pro-actively move UE context to next satellite: Since the satellite’s trajectory is predictable. It can be predicted on which satellite will serve the UE’s geographical area in the future. One possible solution is to proactively push the UE context with updated AS security context to the next satellite via Xn interface at the time of satellite switch.

- Option 3: Relocate the UE context to the AMF on the ground. When the UE resumes, the retrieve UE context procedure is initiated towards the AMF on the ground, without relying on the Xn interface. Also, for a mobile terminated data/signaling, the AMF can initiate RNA-based paging procedure, then the serving gNB retrieve UE context from AMF.

- Option 4: introduces NG-based UE context retrieve procedure. The UE context remains in last serving gNB. When UE resumes, the current serving gNB initiate NG-based UE context retrieve procedure to retrieve the UE context from last serving gNB. For MT data/signaling, the AMF can initiate RNA-based paging procedure.

There are just some possible options. Other solutions are not precluded. It is suggested that RAN3 perform a further study.

Proposal 3: RAN3 study how to support RRC INACTIVE UE in Regenerative architecture.

# 5	Conclusion

In this contribution, we discussed the issues to support regenerative architecture. Our proposals are:

Proposal 1-1: agree the Stage-2/3 TPs for NG Removal.

Proposal 1-2: No need to introduce NG Suspend/Resume.

A draft NGAP TP can be found in Contribution ([3])

For soft FLSO:

Proposal 2-1: Agree Stage-2 TP for soft Feeder Link Switch Over.

Stage-2 TP for FLSO can be found in Annex A – TP for TS 38.300

For hard FLSO:

Observation 2-1: without knowing the hard FLSO, the 5GC (AMF/UPF) continue sending the DL NG-C/U to the gNB via old TNL address anchored in old NTN GW, and will be discarded at the old NTN GW.

Observation 2-2: it is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U.

Observation 2-3: existing Per-UE PDU Session Resource Modify Indication procedure may cause signaling storm and interruption, e.g. when many UEs are served by the gNB/satellite.

Proposal 2-2: RAN3 study the solution to reduce interruption and avoid signaling storm for hard FLSO,  and consider using a non-UE associated NGAP procedure to update the DL TNL address for NG-U tunnels of all connected UEs

Proposal 3: RAN3 study how to support RRC INACTIVE UE in Regenerative architecture.

# References

- TS 38.300

- TS 38.413

- R3-245179, (TP for TS 38.413) Introduce NG Removal procedure

- R3-243954, Reply LS on Support of Regenerative-based Satellite Access

- R3-244548, (TP for BLCR for TS 38.300) Support of regenerative payload (Huawei)

# Annex A – TP for TS 38.300

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. In Transparent NTN payload, the feeder link switchover is a Transport Network Layer procedure. In Regenerative NTN payload, the feeder link switchover causes the change of the TNL address of the gNB, i.e. from the old TNL address anchored in the old NTN Gateway to the new TNL address anchored in the new NTN Gateway.

Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

In Transparent NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

In Regenerative NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed.

-	In case of soft feeder link switch without relocation of the AMF, the gNB can setup the additional new TNL association with the AMF by using the new TNL address. The gNB initiates the NGAP procedure over the new TNL association to inform the 5GC about the update of the gNB’s TNL address. After the 5GC has completed the update, the gNB initiates the RAN node configuration Update procedure to remove the TNL association using the old TNL address and disconnect from the old NTN Gateway.

-	In case of hard feeder link switch over without relocation of the AMF, the gNB may inform the AMF about the hard FLSO is to be performed. After the gNB receive the new TNL address anchored in the new NTN Gateway, the gNB setup the new TNL association with the AMF by using the new TNL address. The gNB initiates a NGAP procedure over the new TNL association to inform the 5GC about the update of the gNB’s TNL address. After the 5GC has completed the update, the gNB initiates the RAN node configuration Update procedure to remove the TNL association using the old TNL address.