3GPP TSG-RAN WG3 Meeting #125bis	R3-245191

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	11.4

Source:	CMCC

Title:	Discussion on AI/ML based Mobility Optimization for NR-DC

Document for:	Discussion and Decision

# 1 Introduction

During the discussion of AI/ML for NG-RAN SI, the following agreements were achieved for AI/ML based Mobility Optimization for NR-DC.

RAN3#123bis

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope).

RAN3#124

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

RAN3#125bis is the first meeting for AI/ML for NG-RAN WI. In this contribution, we would continue the discussion on the scenarios and solutions for NR-DC mobility optimization and provide some proposals.

# 2 Discussion

2.1 Scenarios of AI/ML based Mobility optimization for NR-DC

For NR-DC, there are many sub-scenarios supported in 3GPP, for example, SN Addition, SN Change (MN/SN initiated), inter-Master Node handover with/without SN change, etc. Considering the potential benefits and complexity, we think SN Addition and MN-initiated SN change should be considered as the baseline in Rel-19 Mobility Optimization for NR-DC.

Proposal 1: RAN3 considers the following scenarios in Rel-19 Mobility Optimization for NR-DC.

- SN Addition

- MN-initiated SN change

In Rel-18, the UE trajectory prediction and measured UE trajectory collection for first-hop target NG-RAN node was supported. The complete mechanism was defined, including the predicted UE Trajectory, the measured UE Trajectory collection as the feedback, and the related signaling procedures.

Observation 1: The complete mechanism of UE Trajectory prediction/collection for one hop target NG-RAN node was specified in Rel-18, including

- The predicted UE Trajectory

- The measured UE Trajectory collection, including trigger conditions of UE Trajectory Collection

- The related signaling procedures

From our point of view, we could consider the normative work of Mobility Optimization for NR-DC from the above three aspects. And we prefer to reuse the existing mechanisms in Rel-18 as much as possible and only introduce the necessary enhancements for application to NR-DC scenario.

Proposal 2: For the normative work of AI/ML based Mobility Optimization for NR-DC, it is proposed to reuse the existing mechanisms in Rel-18 as much as possible and only introduce the necessary enhancements for application to NR-DC scenario.

2.2 Prediction Information of Mobility Optimization for NR-DC

In the case that AI/ML model deployed in NG-RAN node has the capability to support the mobility prediction in NR-DC and the enough data collection for AI/ML model training is available, we think the mobility prediction in NR-DC can be used as the assistance information by MN for the target SN decision and by target SN for resource reservation. The following prediction information could be considered for NR-DC Mobility Optimization:

- The predicted target PSCell(s) within the target SN could be helpful for MN to select the target PSCell(s). In Rel-18, a list of cells is provided in UE Trajectory prediction which is only limited to the next one hop target NGRAN node. Therefore, the predicted target PSCell(s) could be limited to the target SN for NR-DC.

- The predicted time duration of connectivity with the predicted target PSCell(s) could help MN to decide whether to set up the dual-connectivity for UE or help the target SN to decide the proper resource allocation for UE. In Rel-18, the presence of predicted time UE stays in cell is optional in UE Trajectory prediction. So the predicted time duration of connectivity with the target PSCell(s) could also be optional for NR-DC.

Observation 2: The predicted target PSCell(s) within the target SN could be helpful for MN to select the target PSCell(s).

Observation 3: The predicted time duration of connectivity with the predicted target PSCell(s) could help MN to decide whether to set up the dual-connectivity for UE or help the target SN to decide the proper resource allocation for UE.

Therefore, we have the proposal as below.

Proposal 3: RAN3 to discuss the prediction information for NR-DC Mobility Optimization, e.g.,

- The predicted target PSCell(s) within the target SN could be considered as the baseline of NR-DC Mobility Optimization;

- The predicted time duration of connectivity with the predicted target PSCell(s) could be considered as the optional prediction information.

Regarding to AI/ML model input, we think UHI/MHI, the current or historical UE Trajectory, and the resource status information of MN and the candidate target SN(s) could be considered as the inputs of AI/ML model for model training or inference.

Proposal 4: RAN3 to consider the AI/ML model input used for model training or inference, e.g., UHI/MHI, the current or historical UE Trajectory and the resource status information of MN and the candidate target SN(s).

2.3 Measured information collection of Mobility Optimization for NR-DC

As per the agreement achieved in RAN3#124 meeting, the collection of measured UE performance shall be supported as the feedback in Rel-19. Besides that, we think the PSCell ID(s) and the measured time duration of connectivity with target PSCell(s) could be introduced into the measured information as the feedback for NR-DC Mobility Optimization. The PSCell ID(s) is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured information reported by target SN. And the measured time duration of connectivity with target PSCell(s) could be used for AI/ML model performance testing and model retraining.

Observation 4: The PSCell ID(s) is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured information reported by target SN.

Proposal 5: Besides measured UE performance, RAN3 to further consider the measured information collection as the feedback for NR-DC Mobility Optimization, e.g.,

- PSCell ID(s)

- The measured time duration of connectivity with target PSCell(s)

In Rel-18, the measured information collection starts from successful Handover Execution, and the following four conditions introduced to terminate the measured information collection:

- The time since UE was successfully handed over to the target node is equal to the value of the Collection Time Duration;

- The configured number of hand-overs within the same target node is reached;

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- UE is handed over to a cell belonging to an NG-RAN node different from the target node.

From our point of view, the trigger conditions to start or terminate the measured information collection for NR-DC could reuse the Rel-18 mechanisms, but the necessary rewording for the trigger conditions are needed.

Observation 5: The trigger conditions to start or terminate the measured information collection for NR-DC could reuse the Rel-18 mechanisms, but the necessary rewording for the trigger conditions is needed.

For example, there is no handover for SN Addition and MN-initiated SN change, so the collection of measured information for NR-DC could start from the successful establishment of NR-DC with SN. So the trigger condition to start the measured information collection for NR-DC could be rewording as successful PSCell Addition or PSCell change with the target SN, that means the successful establishment of NR-DC with the target SN.

In order to terminate the measured information collection of NR-DC in time and avoid the unnecessary measurements in target SN, the trigger conditions used to terminate the measured information collection are also needed for NR-DC. For example, the target SN could terminate the measured information collection when the time since the successful establishment of NR-DC with the target SN is equal to the value of the collection time duration of NR-DC, which is similar with the mechanism that the time since UE was successfully handed over to the target node is equal to the value of the Collection Time Duration in Rel-18. And the target SN could terminate the measured information collection when a cell belonging to another node different from the target SN becomes the PSCell or the target SN receives SN Release Request message, that means UE will move to single connectivity with MN. It is similar with the mechanism that UE is handed over to a cell belonging to an NG-RAN node different from the target node.

Proposal 6: RAN3 to discuss whether the existing trigger conditions in Rel-18 could be applicable for the collection of measured information for NR-DC.

Proposal 7: RAN3 to consider to reword the trigger condition to start the measured information collection for NR-DC, e.g., successful PSCell Addition or PSCell change with the target SN, that means the successful establishment of NR-DC with the target SN.

Proposal 8: RAN3 to consider to reword the trigger condition to terminate the collection of measurement for NR-DC, e.g.,

- The time since the successful establishment of NR-DC with the target SN is equal to the value of the collection time duration of NR-DC;

- A cell belonging to another node different from the target SN becomes the PSCell;

- The execution of Secondary Node Release procedure or UE moves to single connectivity with MN.

2.4 Signaling procedures

In light of the agreement achieved in RAN3#123bis meeting [1], we have “inference at the MN” as the study assumption for NR-DC.

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN.

In the case that AI/ML model is deployed in MN, the MN could generate the prediction information of NR-DC in advance. So, we think on one hand, the prediction information generated by the MN could be used by MN itself as the assistance information for target SN selection for SN Addition and MN-initiated SN Change. On the other hand, the MN could send the prediction information to the target SN for resource reservation.

Proposal 9: After MN generate the prediction information (e.g., the predicted target PSCell(s) within the target SN and the predicted time duration of connectivity with the predicted target PSCell(s)) of NR-DC,

- The MN could use the prediction information as the assistance information for target SN selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation.

For the collection of measured information of Mobility Optimization for NR-DC, we think the target SN could report the measured information to MN for SN Addition and MN-initiated SN Change.

Proposal 10: For the collection of measured information for NR-DC, the target SN could report the measured information to MN for SN Addition and MN-initiated SN Change.

# 3 Conclusion

In this paper, we discuss on the scenarios and solutions for NR-DC mobility optimization, we have the following observations and proposals:

Scenarios of AI/ML based Mobility optimization for NR-DC

Proposal 1: RAN3 considers the following scenarios in Rel-19 Mobility Optimization for NR-DC.

- SN Addition

- MN-initiated SN change

Observation 1: The complete mechanism of UE Trajectory prediction/collection for one hop target NG-RAN node was specified in Rel-18, including

- The predicted UE Trajectory

- The measured UE Trajectory collection, including trigger conditions of UE Trajectory Collection

- The related signaling procedures

Proposal 2: For the normative work of AI/ML based Mobility Optimization for NR-DC, it is proposed to reuse the existing mechanisms in Rel-18 as much as possible and only introduce the necessary enhancements for application to NR-DC scenario.

Prediction Information of Mobility Optimization for NR-DC

Observation 2: The predicted target PSCell(s) within the target SN could be helpful for MN to select the target PSCell(s).

Observation 3: The predicted time duration of connectivity with the predicted target PSCell(s) could help MN to decide whether to set up the dual-connectivity for UE or help the target SN to decide the proper resource allocation for UE.

Proposal 3: RAN3 to discuss the prediction information for NR-DC Mobility Optimization, e.g.,

- The predicted target PSCell(s) within the target SN could be considered as the baseline of NR-DC Mobility Optimization;

- The predicted time duration of connectivity with the predicted target PSCell(s) could be considered as the optional prediction information.

Proposal 4: RAN3 to consider the AI/ML model input used for model training or inference, e.g., UHI/MHI, the current or historical UE Trajectory and the resource status information of MN and the candidate target SN(s).

Measured information collection of Mobility Optimization for NR-DC

Observation 4: The PSCell ID(s) is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured information reported by target SN.

Proposal 5: Besides measured UE performance, RAN3 to further consider the measured information collection as the feedback for NR-DC Mobility Optimization, e.g.,

- PSCell ID(s)

- The measured time duration of connectivity with target PSCell(s)

Observation 5: The trigger conditions to start or terminate the measured information collection for NR-DC could reuse the Rel-18 mechanisms, but the necessary rewording for the trigger conditions is needed.

Proposal 6: RAN3 to discuss whether the existing trigger conditions in Rel-18 could be applicable for the collection of measured information for NR-DC.

Proposal 7: RAN3 to consider to reword the trigger condition to start the measured information collection for NR-DC, e.g., successful PSCell Addition or PSCell change with the target SN, that means the successful establishment of NR-DC with the target SN.

Proposal 8: RAN3 to consider to reword the trigger condition to terminate the collection of measurement for NR-DC, e.g.,

- The time since the successful establishment of NR-DC with the target SN is equal to the value of the collection time duration of NR-DC;

- A cell belonging to another node different from the target SN becomes the PSCell;

- The execution of Secondary Node Release procedure or UE moves to single connectivity with MN.

Signaling procedures

Proposal 9: After MN generate the prediction information (e.g., the predicted target PSCell(s) within the target SN and the predicted time duration of connectivity with the predicted target PSCell(s)) of NR-DC,

- The MN could use the prediction information as the assistance information for target SN selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation.

Proposal 10: For the collection of measured information for NR-DC, the target SN could report the measured information to MN for SN Addition and MN-initiated SN Change.

# Reference

- RAN3_123bis_agenda_20240419_EOM, April 2024.

- RAN3_124_agenda_20240524_eom, May 2024.

- RAN3_125_agenda_20240823EOM, Aug 2024.