3GPP TSG-RAN WG3 Meeting #125bis	R3-245192

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	10.3.1

Source:	CMCC

Title:	(TP to 38.300) Discussion on SON/MDT enhancement for NTN

Document for:	Discussion and Decision

# 1 Introduction

In RAN3#125 meeting, there was discussion on SON/MDT enhancement for Intra-NTN mobility. Some agreements were achieved and the following issues were captured and need to be continued in next meeting:

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

In this contribution, we would discuss on the issues of MRO mechanisms for intra-NTN and provide some proposals.

# 2 Discussion

Based on the agreements achieved in RAN3#124 meeting, we can see that the following single trigger conditions and multiple trigger conditions should be considered in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

2.1 MRO for single trigger condition in intra-NTN mobility

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

Based on the above agreements achieved in last meeting, we can see that there are two cases (Too Late CHO Execution, Too Early CHO Execution/CHO Execution to Wrong Cell) in MRO for time-based CHO. The case of Too Late CHO Execution means RLF happens in the source cell before UE executes CHO, and the UE re-connects the cell other than the source cell. And this case can be further divided into RLF happens before and after T1 threshold starts. Network could consider to move T1 threshold to early, so that UEs can start the CHO execution earlier for RLF happens before T1 threshold starts in Case 1. For RLF happens after T1 threshold starts in Case 1, it may lead to RLF happens before CHO execution that the value of Duration is set too large. Network could consider to decrease the value of the Duration in this sub-case. Based on the above analysis for Case 1, we can observe that the optimization objectives of MRO for RLF happens before and after T1 threshold starts are different.

Observation 1: There are different optimization objectives of MRO for RLF happens before and after T1 threshold starts in Case 1 of MRO for time-based CHO. Therefore, it is needed to distinguish if the RLF happens before or after T1 threshold in Case 1.

For the case of Too Early CHO Execution in Case 2, CHO execution before HOF happens or RLF happens. It seems the value of the T1 threshold should be considered to optimize no matter RLF happens within the duration or after the duration period. Network could consider to move T1 threshold to right to avoid UEs start the CHO execution early. For the case of CHO Execution to Wrong Cell, UE successfully executes CHO before RLF happens and the UE re-connects in a third cell. So it seems the candidate cell of CHO should be optimized rather than the configuration of T1 threshold and Duration. We can observe that there are different optimization objectives of MRO for Too Early CHO Execution and CHO Execution to Wrong Cell. And we think the existing IE reconnectCellId in RLF report could be used by network to distinguish these two sub cases.

Observation 2: For the case of Too Early CHO Execution in Case 2, the value of the T1 threshold should be considered to optimize no matter RLF happens within the duration or after the duration period. For the case of CHO Execution to Wrong Cell in Case 2, the candidate cell of CHO should be optimized rather than the configuration of T1 threshold and Duration.

Therefore, we have the following proposals:

Proposal 1: There is no need to distinguish if RLF happens within the duration or after the duration period in Case 2 of MRO for time-based CHO.

Proposal 2: The objective of MRO for time-based CHO is to optimize configuration issues of T1 threshold, Duration or CHO candidate cell.

For the scenario where a location-based trigger condition configured, we think the information about the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs should be considered in enhancements to RLF report. Since it is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 3: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

Proposal 4: The objective of MRO for measurement-based CHO is to optimize configuration issues of the Thresholds in CHO events A3/A4/A5.

Moreover, we think the configured CHO execution condition (e.g., time-based trigger/location-based trigger/measurement-based trigger) could help NW identify the potential issues of CHO trigger condition.

Observation 3: The configured CHO execution condition (e.g., time-based trigger/location-based trigger/ measurement-based trigger) could help NW identify the potential issues of CHO trigger condition.

Proposal 5: For single trigger condition in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

  - The configured CHO execution condition, e.g., an indicator or bitmap for time-based trigger, location-based trigger, measurement-based trigger;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs, for the scenario where a location-based trigger condition configured;

Proposal 6: The existing IE reconnectCellId in RLF report could be used by network to distinguish these two sub cases in MRO for time-based CHO Case 2 and is helpful for the optimization of candidate cell of CHO.

2.2 MRO for multiple trigger conditions in intra-NTN mobility

Based on the agreements achieved in RAN3#124 meeting, we can see that there are two cases for multiple trigger conditions configured:

-	A time-based and Measurement-based combined trigger condition;

-	A location-based and Measurement-based combined trigger condition;

Let’s take the time-based and Measurement-based combined trigger condition as an example, there are several possible scenarios that cause the connection failure:

- Both the time-based trigger condition and the Measurement-based trigger condition are fulfilled;

- The time-based trigger condition is fulfilled but the Measurement-based trigger condition is not fulfilled;

- The Measurement-based trigger condition is fulfilled but the time-based trigger condition is not fulfilled;

- Neither the time-based trigger condition and the Measurement-based trigger condition is not fulfilled.

We think with the information about the trigger condition(s) fulfilled before the failure occurs, NW can identify whether there are trigger condition(s) that cannot be fulfilled or fulfilled too late, or whether there are trigger condition(s) that are fulfilled too early. And this information could be applicable for both a time-based and Measurement-based combined trigger condition and a location-based and Measurement-based combined trigger condition. This information could be the sequence of the fulfilled trigger condition, for example, timeBetweenEvents and firstTriggeredEvent IEs have been introduced for MRO for CHO in Rel-17.

Observation 4: With the information about the trigger condition(s) fulfilled before the failure occurs, NW can identify whether there are trigger conditions that cannot be satisfied or satisfied too late, or whether there are CHO trigger conditions that are satisfied too early.

Besides that, from our point of view, the configured CHO execution conditions are also needed for multiple trigger conditions. Especially, in the case where there is one or even two trigger conditions not fulfilled before RLF occurs, the information could help network to identify the potential problem trigger condition(s). Therefore, the configured CHO execution conditions could be applicable for all scenarios including MRO for single trigger condition and MRO for multiple trigger conditions.

Observation 5: The configured CHO execution conditions could be applicable for all scenarios including MRO for single trigger condition and MRO for multiple trigger conditions.

That means for the scenario where a time-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The configured CHO execution conditions;

  - The trigger condition(s) fulfilled before the failure occurs.

And for the scenario where a location-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The configured CHO execution conditions;

  - The trigger condition(s) fulfilled before the failure occurs;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs.

Proposal 7: The following information should be included in RLF report for the scenario where a time-based and Measurement-based combined trigger condition configured:

  - The configured CHO execution conditions;

  - The trigger condition(s) fulfilled before the failure occurs, e.g., the sequence of the fulfilled trigger condition;

Proposal 8: The following information should be included in RLF report for the scenario where a location-based and Measurement-based combined trigger condition configured:

  - The configured CHO execution conditions;

  - The trigger condition(s) fulfilled before the failure occurs, e.g., the sequence of the fulfilled trigger condition;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs.

Proposal 9: For multiple trigger conditions in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

  - The configured CHO execution conditions, which is applicable for both MRO for the single trigger condition and multiple trigger conditions;

  - The trigger condition(s) fulfilled before the failure occurs, e.g., the sequence of the fulfilled trigger condition;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs, for the scenario where a location-based and Measurement-based combined trigger condition configured.

Proposal 10: For the trigger condition(s) fulfilled before the failure occurs, the existing IEs timeBetweenEvents and firstTriggeredEvent for MRO for CHO could be reused or new IEs could be introduced to indicate the sequence of the fulfilled trigger condition.

Based on the agreements of MRO for CHO with a time-based trigger condition in RAN3#125 meeting, the corresponding TP for TS38.300 is attached in the Annex.

# 3 Conclusion

In this paper, we discuss on MRO mechanisms for intra-NTN mobility, we have the following observations and proposals:

MRO for single trigger condition in intra-NTN mobility

Observation 1: There are different optimization objectives of MRO for RLF happens before and after T1 threshold starts in Case 1 of MRO for time-based CHO. Therefore, it is needed to distinguish if the RLF happens before or after T1 threshold in Case 1.

Observation 2: For the case of Too Early CHO Execution in Case 2, the value of the T1 threshold should be considered to optimize no matter RLF happens within the duration or after the duration period. For the case of CHO Execution to Wrong Cell in Case 2, the candidate cell of CHO should be optimized rather than the configuration of T1 threshold and Duration.

Proposal 1: There is no need to distinguish if RLF happens within the duration or after the duration period in Case 2 of MRO for time-based CHO.

Proposal 2: The objective of MRO for time-based CHO is to optimize configuration issues of T1 threshold, Duration or CHO candidate cell.

Proposal 3: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

Proposal 4: The objective of MRO for measurement-based CHO is to optimize configuration issues of the Thresholds in CHO events A3/A4/A5.

Observation 3: The configured CHO execution condition (e.g., time-based trigger/location-based trigger/ measurement-based trigger) could help NW identify the potential issues of CHO trigger condition.

Proposal 5: For single trigger condition in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

  - The configured CHO execution condition, e.g., an indicator or bitmap for time-based trigger, location-based trigger, measurement-based trigger;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs, for the scenario where a location-based trigger condition configured;

Proposal 6: The existing IE reconnectCellId in RLF report could be used by network to distinguish these two sub cases in MRO for time-based CHO Case 2 and is helpful for the optimization of candidate cell of CHO.

MRO for multiple trigger conditions in intra-NTN mobility

Observation 4: With the information about the trigger condition(s) fulfilled before the failure occurs, NW can identify whether there are trigger conditions that cannot be satisfied or satisfied too late, or whether there are CHO trigger conditions that are satisfied too early.

Observation 5: The configured CHO execution conditions could be applicable for all scenarios including MRO for single trigger condition and MRO for multiple trigger conditions.

Proposal 7: The following information should be included in RLF report for the scenario where a time-based and Measurement-based combined trigger condition configured:

  - The configured CHO execution conditions;

  - The trigger condition(s) fulfilled before the failure occurs, e.g., the sequence of the fulfilled trigger condition;

Proposal 8: The following information should be included in RLF report for the scenario where a location-based and Measurement-based combined trigger condition configured:

  - The configured CHO execution conditions;

  - The trigger condition(s) fulfilled before the failure occurs, e.g., the sequence of the fulfilled trigger condition;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs.

Proposal 9: For multiple trigger conditions in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

  - The configured CHO execution conditions, which is applicable for both MRO for the single trigger condition and multiple trigger conditions;

  - The trigger condition(s) fulfilled before the failure occurs, e.g., the sequence of the fulfilled trigger condition;

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs, for the scenario where a location-based and Measurement-based combined trigger condition configured.

Proposal 10: For the trigger condition(s) fulfilled before the failure occurs, the existing IEs timeBetweenEvents and firstTriggeredEvent for MRO for CHO could be reused or new IEs could be introduced to indicate the sequence of the fulfilled trigger condition.

# Reference

- RAN3_125_agenda_20240823EOM, Aug 2024.

# Annex: TP for TS38.300

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

<<<<<< Start of CHANGES >>>>>>

In case of CHO with a time-based trigger condition in Intra-NTN mobility, the Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell are defined as follows:

-	Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

-	Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

<<<<<< End of CHANGES >>>>>>