3GPP TSG-RAN WG3 Meeting #125bis														R3-245195

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda item:		11.3

Source:				CMCC

Title:	Discussion on AI/ML-based CCO

Document for:		Discussion

Introduction

During RAN#105, new WI “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved [1].

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states

In this paper, we provide our consideration of AI/ML based Coverage and Capacity Optimization.

Discussion

During the R19 SI phase, RAN3 has agreed the AI/ML-based CCO solutions for non-split and split architecture, mainly follow the legacy CCO mechanism:

For non-split architecture:

Step 0: gNB predicts the CCO issue

Step 1: gNB generates the future coverage state based on the predicted CCO issue and other information

Step 2: gNB sends the future coverage status to neighbour gNBs

For split architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

Also, RAN3 has agreed that the locations of AI/ML model training and inference are the same as R18, and the future CCO state can be exchanged between nodes. The detailed input/output/feedback/Standard Impact have been captured in TR 38.743.

Based on the conclusions of the R19 SI, for the start point of R19 WI, we propose RAN3 first focus on the standard impacts of the input/output/feedback information and enhance the Xn/F1 interface to support the predicted/feedback information transmission.

Proposal 1: RAN3 focuses on the standard impacts and interface enhancement to support the predicted/feedback information transmission first.

Input/Output/Feedback

Input/Feedback:

In R18, new procedures, including Data Collection Reporting Initiation and Data Collection Reporting procedures, were introduced for AI/ML related data transmission. For the input and feedback information, we propose to follow the R18 mechanism that the Data Collection Request message is used to configure the requested input/feedback and the Data Collection Update message is used to report the requested information.

Proposal 2: For the input/feedback information, reuse 	Data Collection Reporting Initiation and Data Collection Reporting procedures to collect the information.

Output:

Then, the output of the AI/ML-based CCO solution including predicted CCO issue and future CCO state.

For the non-split architecture, the future CCO state and coverage modification cause are agreed to send from local node to neighbour nodes.

For non-split arch, gNB sends the future coverage status to neighbour gNBs, together with an associated coverage modification cause.

In the existing CCO mechanism, the CCO issue and CCO state are exchanged via NG-RAN NODE CONFIGURATION UPDATE message. Since there is not much new information to be introduced, we propose to reuse NG-RAN NODE CONFIGURATION UPDATE message to transfer AI/ML-related future CCO state and coverage modification cause.

Proposal 3: Reuse the NG-RAN NODE CONFIGURATION UPDATE message to transfer future CCO state and coverage modification cause to neighbour nodes.

Then, considering that the NG-RAN node can inform its neighbor nodes the cell/beam coverage configuration before and after the cell coverage is modified by using the Coverage Modification List IE in the current spec, we think the future CCO state exchanged between NG-RAN node is similar with the NG-RAN node inform its neighbor nodes the cell coverage state planned to be used at the next reconfiguration, the difference is that the future CCO state can be generated with AI/ML and it’s for the future use. Thus, two options for implementation:

Opt 1: Reusing the current defined CCO state and modification cause IE for future CCO state and coverage modification cause.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and coverage modification cause.

Observation 1: Two options for delivering future CCO state and coverage modification cause to neighbor NG-RAN nodes:

Opt 1: Reusing the current defined CCO state and modification cause IE for future CCO state and coverage modification cause.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and coverage modification cause.

From our perspective, both options can be discussed. But either way needs to distinguish legacy CCO state from future CCO state to avoid neighbor node confusing about the receiving information. If opt 2 is selected, the new IE can be used to only transmit AI/ML related predicted/future information, which can be distinguished from the existing IE to avoid neighbor node confusing about the received information is legacy CCO state or future CCO state.

For Opt 1, if reusing existing CCO cause and CCO state IE, an indication needs to be introduced to distinguish whether the CCO state is legacy or for future use.

Observation 2: Regardless of which way to transmit future CCO state and coverage modification cause to neighbor node, it is essential to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

In RAN3#125, RAN3 agreed to introduce time information of future CCO state and predicted CCO issue.

Future CCO state and predicted CCO issue will be provided together with timing information.

The time information can be used to distinguish legacy CCO state from future CCO state. For example, when the CCO state is for legacy use, the time information is not included. When the CCO state is predicted or for future use, the future CCO state exchanges between nodes together with the time information. With this time information, Opt 1 can be selected, which also has the minimum standard impact as only the time information needs to be added. However, if opt 1 is agreed, the time information should be a mandatory field. If the time information is optional, there is no difference between the legacy and AI/ML-based solution.Therefore, for Opt 1, RAN3 needs to specify that the time information is mandatory.

Observation 3: Opt 1 has minimum standard impacts if the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause. The time information should be a mandatory information for this use.

Proposal 4: If opt 1 is agreed, the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Proposal 5: RAN3 agrees that the time information is a mandatory information.

For the split architecture, it is similar to the non-split architecture, as the new introduced information is not much, the GNB-CU CONFIGURATION UPDATE message can be reused to transfer the predicted CCO issue from gNB-CU to gNB-DU, and the GNB-DU CONFIGURATION UPDATE message can be resued to transfer the future CCO state from gNB-DU to gNB-CU.

Proposal 6: For split architecture, reuse GNB-CU CONFIGURATION UPDATE message to transfer predicted CCO issue.

Proposal 7: For split architecture, reuse GNB-DU CONFIGURATION UPDATE message to transfer future CCO state.

Time information

Then, for the details of time information, from our perspective, it’s no need to include two kinds of time information in one message at the same time. For non-split architecture, it’s better for the neighbour nodes aware when the coverage configuration of local node is predicted to be modified, so that the neighbour nodes can make corresponding preparations in advance and adjust its own coverage configuration accordingly. So, for the Xn enhancement, we propose to introduce the time information that indicate the time when the coverage configuration of local node is predicted to be modified.

Observation 4: For non-split architecture, the time information of future CCO state is beneficial for the neighbour nodes aware when the local node coverage configuration will be modified. The neighbour nodes can make some preparations in advance and adjust its own coverage configuration accordingly.

Proposal 8: The time information transferred via Xn interface indicates when the local node coverage configuration will be modified.

Proposal 9: No need to introduce two separate time information for Xn interface enhancement.

For split architecture, considering that gNB-CU sends the predicted CCO issue to the gNB-DU, gNB-CU can indicate when the CCO issue is predicted to be happened. The gNB-DU provides the future CCO state to gNB-CU, gNB-DU can also indicate the time information of when the future CCO state will be modified. Thus, for split architecture, the time information of predicted CCO issue is transferred from gNB-CU to gNB-DU, and the time information of future CCO state is transferred from gNB-DU to gNB-CU.

Proposal 10: For split architecture, the time information of predicted CCO issue is sent from gNB-CU to gNB-DU.

Proposal 11: For split architecture, the time information of future CCO state is sent from gNB-DU to gNB-CU.

Others

In RAN3#125, RAN3 also discussed whether the CU generates the suggested future CCO state and sends it to DU(s) as assistance information, and the following agreement was captured:

In TP, capture whether the CU1 generates the suggested future CCO state to DU1 as assistance information is left to normative discussion

In legacy CCO mechanism, gNB-CU does not provide the suggested future CCO state to the gNB-DU, the gNB-CU detects the CCO issue and sends it to the gNB-DU, and gNB-DU provides the CCO state to gNB-CU. It seems that gNB-CU does not need to provide suggested CCO state to gNB-DU as assistance information. As the gNB-DU has the ability to derive CCO state in legacy, we also think that gNB-DU should also be able to derive future CCO state in AI/ML-based CCO.

Proposal 12: RAN3 discusses more motivation about CU provides suggested future CCO state to DU.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: RAN3 focuses on the standard impacts and interface enhancement to support the predicted/feedback information transmission first.

Proposal 2: For the input/feedback information, reuse 	Data Collection Reporting Initiation and Data Collection Reporting procedures to collect the information.

Proposal 3: Reuse the NG-RAN NODE CONFIGURATION UPDATE message to transfer future CCO state and coverage modification cause to neighbour nodes.

Observation 1: Two options for delivering future CCO state and coverage modification cause to neighbor NG-RAN nodes:

Opt 1: Reusing the current defined CCO state and modification cause IE for future CCO state and coverage modification cause.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and coverage modification cause.

Observation 2: Regardless of which way to transmit future CCO state and coverage modification cause to neighbor node, it is essential to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Observation 3: Opt 1 has minimum standard impacts if the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause. The time information should be a mandatory information for this use.

Proposal 4: If opt 1 is agreed, the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Proposal 5: RAN3 agrees that the time information is a mandatory information.

Proposal 6: For split architecture, reuse GNB-CU CONFIGURATION UPDATE message to transfer predicted CCO issue.

Proposal 7: For split architecture, reuse GNB-DU CONFIGURATION UPDATE message to transfer future CCO state.

Observation 4: For non-split architecture, the time information of future CCO state is beneficial for the neighbour nodes aware when the local node coverage configuration will be modified. The neighbour nodes can make some preparations in advance and adjust its own coverage configuration accordingly.

Proposal 8: The time information transferred via Xn interface indicates when the local node coverage configuration will be modified.

Proposal 9: No need to introduce two separate time information for Xn interface enhancement.

Proposal 10: For split architecture, the time information of predicted CCO issue is sent from gNB-CU to gNB-DU.

Proposal 11: For split architecture, the time information of future CCO state is sent from gNB-DU to gNB-CU.

Proposal 12: RAN3 discusses more motivation about CU provides suggested future CCO state to DU.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.423

    - 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>Served Cells To Update NR | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| >>Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | M |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | O |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be modified. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Predicted Coverage Modification Time | M |  | FFS | Indicates the time in point of future coverage state to be applied |  |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0.. 1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |

- Annex: TP for 38.473

9.3.1.211	CCO Assistance Information

This IE provides assistance information for the Capacity and Coverage (CCO) actions for specific CCO issues detected, and for network energy saving.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the type of CCO issue detected, or network energy saving cause. |
| Affected Cells and Beams | O |  | 9.3.1.212 |  |
| CCO issue prediction time | FFS |  | FFS | Indicates the time in point of when CCO issue predicted to be happened. |

    - 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  |  |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …, network energy saving) |  | YES | ignore |
| >>Future Coverage Modification Time | M |  | FFS | Indicates the time in point of future coverage state to be applied | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |