3GPP TSG-RAN WG3 Meeting #125-bis	R3-245230

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP for BLCR for SON for 38.300, 38.401,38.473, 37.340, 38.423) MRO enhancement for R18 mobility mechanisms

Agenda Item:	10.2

Source:	Huawei

Document for:	other

# 1	Introduction

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms.

# 2	Discussion

## 2.1	MRO for LTM

The following was agreed in RAN3#125

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

### 2.1.1	Failure scenarios

Per the current BLCR R2-244853, the successful handover in case of LTM cell switch is defined as follows:

In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

However, the clarification of successful handover in case of RACH-less and RACH-based LTM per current specification TS 38.300 is as below :

The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

From our point of view, the current specification shows that the successful reception of first UL data on the target cell terminates the RACH-less LTM cell switch and declares the completion of RACH-less cell switch execution. The transmission of the RRCReconfigurationComplete message depends on the configured or dynamically signalled UL grant, which may come really late, since this message is not that delay sensitive compared to the UL data transmission. Therefore, we propose:

The description of the successful handover in stage 2 CR is : the "successful handover" refers to successful reception of first UL data on the target cell in case of RACH-less LTM.

In last meeting we agreed that the CU identifies which node to send the information to. There was also a discusison on whether the CU need to determine the failure type. From our point of view, the analysis for CU to perform should be:

- CU first analyze whether the failure is caused by the wrong candidate cell list prepared by the CU by evaluating whether the suitable cell or the cell UE re-establish the radio link to is within the candidate cell list or not. If so, CU is responsible for the failure to the most extent.

- If it is not the wrong candidate cell list, CU needs to decide whether source DU and/or target DU is the corresponding DU for the failure event, which is equivalent to whether CU dertermines the failure type.

- The corresponding DU receive the RLF report from CU and perform further analysis (root cause analysis)

- (optional) The corresponding DU may declare to the CU that the failure does not occur in this DU.

(optional) The CU may further forward the RLF report to other DU. The CU will first analyse whether the failure is caused by the wrong candidate Cell list prepared itself.

Reagrding the failure type determination, one crucial part of information needed for the node to distinguish the exact error type is the T_storeUEcontext. This is currently configured to the CU which means that the DU does not have currently have access to this the information. To resolve this, we think there are different options:

- gNB-CU performs root cause analysis and send it to the gNB-DU – which means the DU does not have to analyse the timer info

- gNB-CU is configured the timer threshold. gNB-CU sends the RLF information to the gNB-DU together with the timer threshold and the gNB-DU performs root cause analysis

- gNB-CU sends the RLF information to the gNB-DU and gNB-DU is configured with the timer values from OAM

It should however be noted that the since the LTM cell switch is designed for shorter interruption time and shorter stay time, the timer for LTM cell switch could be very different from legacy mobility. Therefore, it probably makes sense to consider the possibility that we specify a new, separate timer for LTM cell switch. If we go for solution (2) above, this results in a specific IE, and a new timer configured from OAM which translates into a new OAM requirement. For solutions (1) or (3) this translates into a new OAM requirement.

For simplicity we suggest to go for option1 and add a new timer for LTM cell switch for LTM in the OAM requirements.

In order to enable the forwarding of information, the gNB receiving the RLF report from another node needs to understand whether the failure report is due to a legacy mobility or an LTM mobility event. This is already agreed in RAN2.

For failures shortly after successful handover, the source cell need to identify the UE context based on the C-RNTI used in the source to perform the MRO analysis. It is assumed that the gNB-CU that supports this MRO feature will be able to store the association between the C-RNTI used in the target cell (last serving cell) and the corresponding C-RNTI used in the source cell. gNB-CU translate the C-RNTI in the target cell to the C-RNTI used in the source cell and convey it outside the RLF report when forwarding the information to the source gNB-DU.

Additional information regarding source cell information and C-RNTI used in the source cell is added to the existing procedure (i.e., Access and Mobility Indication) over F1 for failure cases with RLF shortly after successful HO.

### 2.3.2	Near failure scenarios

RAN2 has excluded the RACH-based access and HO interruption time as new trigger conditions for SHR. There is no doubt to resue the the timer related trigger, i.e. T310/T312/T304, similar to legacy L3 handover. Once the timer related trigger is satisfied, the network would better know whether the handover is a LTM cell switch or L3 handover, in order to adjust the parameter precisely and efficiently, e.g. whether it is the CU or DU to optimize accordingly. Therefore, we propose, an indication of  LTM cell switch is introduced to SHR report.

An indication of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

Furthermore, RAN2 has agreed to indicate the RACH-based or RACH-less access in the RLF report. Similarly, the logging of access type is needed for SHR report and RA report.

An indication of specific access type, i.e RACH-based of RACH-less, is introduced to SHR report and RA report.

In last meeting, there is a discussion on the number of RLC re-transmission as a new trigger for SHR report. However, there is no RLC re-transmission for the first UL data transmission on the target cell when the UE is attempting to access the target cell, since RLC re-transmission at the UE side is triggered by the feedback from the network, i.e RLC status report from the target cell. It is up to the UE implementation to retransmit in case the DCI received within T304. Therefore, it is not valueable to report the number of retransmissions, since this is not controlled by network.

Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Regarding the analysis of the SHR associated to an LTM mobility, there is also a role split between the source DU and target DU. Similar to the successful L3 handover analysis mechanism, the CU may forward to the DU which configures the timers causing the SHR to be generated. If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

### 2.3.3	Successful scenarios

Also, a potential risk has been raised during the Rel-18 discussion of supporting subsequent LTM feature, which is about the ping-pong issue. The L1 measurement may be unstable and leading the handover decision directing UE to access to the same cell back and forth in a limited time. Therefore, we suggest to consider the UHI enhancement for LTM to identify the intra-gNB ping-pong issue. The CU should be responsible to collect the UHI and further provide the latest UHI to the new DU timely. Specifically, the CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU.

The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

For enabling the CU to pass the UHI to DU, a new IE over the existing F1AP signalling or a new F1AP signalling should be introduced. We believe, the existing Acess and Mobility Indication message would be the best choice to deliver UHI from CU to DU, since this message is designed for the MRO purpose to pass list of RA reports, RLF reports and SHR reports.

A new IE over the Acess and Mobility Indication message is introduced to sent UHI from CU to DU.

The UHI for DU to perform ping-pong analysis caused by LTM mobility events could be a simplified version of UHI. Because there is no need for DU to know the UHI caused by the legacy L3 handover. For example, the CU could collect the UHI, filter the UHI associcated to LTM cell switch, and passed the filtered LTM UHI to the DU.

The CU could filter the LTM related UHI and pass to DU via the new IE.

At least, we further discuss when the CU pass the UHI to DU for ping-pong purpose. The ping-pong purpose could be dived into two sub-cases: ping-pong event detection and mobility parameter optimization, ping-pong avoidance. For the former sub-case, CU forwards the UHI to source DU after the source DU makes the cell switch command. For the latter sub-case, CU forwards the UHI to the target DU as assistance information for target DU’s further handover decision once the UE accesses to the target DU. RAN3 discuss which case is the ping-pong purpose.

RAN3 discuss which case for ping-pong purpose: source DU detects the ping-pong optimization, and/or, target DU does the ping-pong avoidance for subsequent LTM execution.

## 2.2	MRO for CHO with candidate SCGs

The agreements in last meeting were:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

### 2.3.1	Down prioritization of use cases

| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| --- | --- | --- |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |

For case 4/5/6, the execution condition for CHO only or CHO with candidate SCG is satisfied. The failure occurs by executing CHO only or CHO with target SCG configuration. The legacy MRO for CHO can detect this issue and make proper optimization.

No specification impact is forseen in MRO support for case 4/5/6.

Another FFS is whether to support concurrent dual failure case (MCG+SCG). The concurrent dual failure scenario occur very rarely in pratical. Specific discussion to support MRO solution for concurrent dual failure case is not needed in Rel-19.

Specific discussion on the solution to support case 1c, 2c, 3c, 7c, 8c, 9c is not needed in Rel-19.

### 2.3.2	Failure scenarios

The summary of the failure use cases so far isas below:

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

For the network to distinguish among case1, case 2 and case 3, all of which are identified as too late failure case, the key is to identify which type of the two execution conditionsare blocking a handover for MCG mobility and SCG mobility. Without this information, it is impossible for network to determine which type of execution conditionsto adjust. Therefore, we believe it would be beneficial to indicate which of which type of execution is fullfilled, the corresponding PCell/PSCell ID information, which type of the exeution is not fulfilled and the corresponding PCell/PSCell ID, and also the time sincethe first fulfilment tillthe failure in the RLF report.

For case 1, 2 and 3, RLF report indicates which of which type of execution is fullfilled, the corresponding PCell/PSCell ID information, which type of the exeution is not fulfilled and the corresponding PCell/PSCell ID, and also the time since the first fulfilment till the failure in the RLF report.

For case 7 and 8, the two types of execution condition are fulfilled. For case 7a and case 8a, the RLF report reports the MCG failure event and log the time gap between the CHO and CPAC fulfilled, which is also the explicit indication of this new HO type to distinguish the legacy HO type. NO further specification impact is needed. For case 7b and case 7c, the SCGFailureInformation is resued to report the SCG failure event. No enhancement to SCGFailureInformation is needed.

For case 7a, 7b, 8a and 8b, no further MRO enhancement is needed.

For case 9, the two types of execution conditions, are subsequently fulfilled but never simultaneously fulfilled since the RRC reconfig till the RLF or SCG failure.  The network need to identify which type first fulfilled, fulfilled Cell ID, the time since the first fulfillment till RLF/SCG failure, the time since the first fulfilment till unfulfillment, which type subsequently fulfilled, subsequently fulfilled Cell ID, and the time since the second fulfillment till RLF/SCG failure. We propose to enhance these informatons to the RLF report and SCG Failure Information.

For case 9, enhance the following informations to RLF report and SCG failure information: which type first fulfilled, first fulfilled Cell ID, the time since the first fulfillment till RLF/SCG failure, the time since the first fulfilment till unfulfillment, which type subsequently fulfilled, the subsequently fulfilled Cell ID, and the time since the subsequent fulfillment till RLF/SCG failure.

### 2.3.3	Near failure scenarios

For near failure case as below illustrated we assume the legacy mechanism can be re-used to the maximum extent. For near failure in PCell mobility and PSCell mobility, the UE generate SHR and SPR.

In order for the source node to correlate the SHR and SPR reports, the UE should also report the time between the execution of event and the reporting. With this information, the current solution would be able to combine the information from SHR and SPR associated to the same mobility event.

Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR.

In order for the receiving node to understand that this is from this new mobility event, the UE may also indicate the time between PCell triggered and associated PSCell triggered in SHR/SPR.

Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type.

To conclude, at the moment it seems that the impact on RAN3 is limited but that information is required form the UE. We propose to send an LS to RAN2 to request the information mentioned above.

For the SHR and SPR caused by CHO with candidate SCGs, the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

## 2.3	MRO for subsequent CPAC

The agreements in last meeting were:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN

For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution.

Editor’s note: FFS whether the differentiation between initial and following CPC is needed.

### 2.3.1	Failure scenarios

A subsequent Conditional PSCell Addition or Change (subsequent CPAC) is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. In last meeting, RAN3 agreed to consider the failures in the S-CPAC procedure. We’d like to first specify some typical S-CPAC failure/near-failure cases respectively and to discuss the potential solutions. Likewise, the failure cases of S-CPAC are similar to that of CPAC, including the too-early CPAC, to-late CPAC, and CPAC to wrong cell, as defined in TS 37.340:

- Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

- Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

- CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

The difference mainly lies on the cause of the S-CPAC failures, since some information about UE or candidate PSCells are not updated timely. Thus, RAN3 can follow the failure cases of CPAC as the reference for S-CPAC. The main difference here is that there is an initiating node determining the candidate list, and each candidate will determine the exit condition (when to handover to another candidate). Hence, the mechanism must enable the forwarding of correct information to all involved nodes.

MN shall decide which SN should be informed when failure occurs. In the case of SN initiated S-CPAC, the source SN may not be aware of the failures and what is happening to the UE, since SN is not responsible for collecting the related information. Therefore, MN should inform the initiating SN for optimization such as candidate cell selection, etc.

Likewise, it is beneficial to let the last serving candidate SN to know such information to analyse the failure causes and further CPAC strategy optimization, e.g., by informing the “source” SN of the failure information. According to above, we conclude the mechanism enhancements for S-CPAC failure scenarios as follows.

MN is responsible for determining where to forward the information for failed S-CPAC events.

If an SN is initiating the S-CPAC, and the problem is related to candidate cell selection, the MN forwards information to this SN in order to optimise the candidate cell selection.

Otherwise MN forwards the information to the last serving candidate SN in order to optimise the execution parameters

### 2.3.2	Near failure scenarios

For near failure cases, we assume the legacy mechanism and the above enhancements can be re-used.

### 2.3.3	Successful scenarios

Similar as the root causes of the failure, we’d like to address two main aspects related to S-CPAC. The first one is that the expired UE information. As standardized in Rel-18, the target SN(s) receive the SCG UHI from the MN via SN Addition Request message for CPC, which is at the preparation phase of the subsequent CPAC. Hence, for further CPC, the subsequent target SNs are not able to access the latest SCG UHI, which may hinder the timely and precise operations of target SNs, e.g., parameter optimization, resource allocation, etc. Notice that updated SCG UHI may be sent to MN from SN via SN MODIFICATION REQUEST ACKNOWLEDGE message when UE leaves this SN in S-CPAC, i.e., MN is already aware of the latest SCG UHI. Thus, MN is responsible to send such information to the candidate SNs if needed. Symmetrically, MN can use SN MODIFICATION REQUEST message to inform the candidate SNs about SCG UHI.

Add UHI to SN MODIFICATION REQUEST

Another similar information exchange that may be beneficial is that the initiating node may be informed of the outcome of the S-CPAC configuration. S-CPAC requires reservation of resources in advance. The initiating node (MN or SN) could evaluate the selection of candidate cells and possibly the time spent in each cell to evaluate if S-CPAC was beneficial or not.

In case of MN-initiated S-CPAC, we believe the MN already can get this information, since MN is already responsible for collecting the UHI. But in the case of SN initiated S-CPAC, the source SN may not be aware of what is happening to this UE, since SN is not responsible for collecting this information. Herefore the MN has to provide this to the SN.

The MN could be informaed also about intra SN changes since the MN:

- can get the updated UHI after the UE leaves the SN, or

- can subscribe to UHI updates from the SN.

Then, MN could inform the initiating SN of the UHI via the SN MODIFICATION REQUEST.

RAN3 agree that it is beneficial to inform the source SN about the outcome of mobility events for SN initiated S-CPAC using the SN MODIFICATION REQUEST.

# 3	Conclusion

This document proposes:

Proposal 1:	The description of the successful handover in stage 2 CR is : the "successful handover" refers to successful reception of first UL data on the target cell in case of RACH-less LTM.

Proposal 2:	(optional) The CU may further forward the RLF report to other DU. The CU will first analyse whether the failure is caused by the wrong candidate Cell list prepared itself.

Proposal 3:	For simplicity we suggest to go for option1 and add a new timer for LTM cell switch for LTM in the OAM requirements.

Proposal 4:	Additional information regarding source cell information and C-RNTI used in the source cell is added to the existing procedure (i.e., Access and Mobility Indication) over F1 for failure cases with RLF shortly after successful HO.

Proposal 5:	An indication of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

Proposal 6:	An indication of specific access type, i.e RACH-based of RACH-less, is introduced to SHR report and RA report.

Proposal 7:	Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Proposal 8:	If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

Proposal 9:	The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

Proposal 10:	A new IE over the Acess and Mobility Indication message is introduced to sent UHI from CU to DU.

Proposal 11:	The CU could filter the LTM related UHI and pass to DU via the new IE.

Proposal 12:	RAN3 discuss which case for ping-pong purpose: source DU detects the ping-pong optimization, and/or, target DU does the ping-pong avoidance for subsequent LTM execution.

Proposal 13:	No specification impact is forseen in MRO support for case 4/5/6.

Proposal 14:	Specific discussion on the solution to support case 1c, 2c, 3c, 7c, 8c, 9c is not needed in Rel-19.

Proposal 15:	For case 1, 2 and 3, RLF report indicates which of which type of execution is fullfilled, the corresponding PCell/PSCell ID information, which type of the exeution is not fulfilled and the corresponding PCell/PSCell ID, and also the time since the first fulfilment till the failure in the RLF report.

Proposal 16:	For case 7a, 7b, 8a and 8b, no further MRO enhancement is needed.

Proposal 17:	For case 9, enhance the following informations to RLF report and SCG failure information: which type first fulfilled, first fulfilled Cell ID, the time since the first fulfillment till RLF/SCG failure, the time since the first fulfilment till unfulfillment, which type subsequently fulfilled, the subsequently fulfilled Cell ID, and the time since the subsequent fulfillment till RLF/SCG failure.

Proposal 18:	Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR.

Proposal 19:	Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type.

Proposal 20:	the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

Proposal 21:	MN is responsible for determining where to forward the information for failed S-CPAC events.

Proposal 22:	If an SN is initiating the S-CPAC, and the problem is related to candidate cell selection, the MN forwards information to this SN in order to optimise the candidate cell selection.

Proposal 23:	Otherwise MN forwards the information to the last serving candidate SN in order to optimise the execution parameters

Proposal 24:	Add UHI to SN MODIFICATION REQUEST

Proposal 25:	RAN3 agree that it is beneficial to inform the source SN about the outcome of mobility events for SN initiated S-CPAC using the SN MODIFICATION REQUEST.

# Annex	TP for LTM for 38.300

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of first UL data on the target cell in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Editor’s note 1: FFS if LTM recovery needs to be captured in the definitions.

Editor’s note 2: For the too late LTM cell switch, FFS if the re-establishment cell is a candidate cell, a cell not selected by CU, or both.

#### 15.5.2.5	O&M Requirements

All automatic changes of the HO and/or reselection parameters for mobility robustness optimisation shall be within the ranges allowed by OAM and specified below.

The following control parameters shall be provided by OAM to control MRO behaviour:

-	Maximum deviation of Handover Trigger: this parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation defined by the parameter values assigned by OAM.

-	Minimum time between Handover Trigger changes: this parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm.

Furthermore, in order to support the solutions for detection of mobility optimisation, the parameter Tstore_UE_cntxt and the Tstore_UE_cntxt_LTM shall be configurable by the OAM system.

# Annex	TP for LTM for 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU performs the detection of LTM cell switch failure as defined in 38.300 [2] and may forward the RLF report to the gNB-DU responsible for the failure, together with the detected failure cause.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU in case the SHR was triggered by source cell timer values (e.g. T310/T312) or the target gNB-DU in case the SHR was triggered by target cell timer values (e.g. T304).

# Annex	TP for LTM for 38.473

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>UE Identifier | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

# Annex	TP for CHO with candidate SCGs for 37.340

## 10.X	Self-optimisation for CHO with candidate SCGs

The procedure follows the steps described for Mobility Robustness Optimization in 38.300 [3] and Self-optimisation for PSCell addition/change in clause 10.18.

Handling multiple reports from a single CHO with candidate SCGs event

In case both the execution towards the target PCell and the execution towards the associated target PSCell succeed, MRO evaluation of sub-optimal conditional PCell execution and condition PSCell execution may be triggered simultaneously for the same mobility event. In this case, the corresponding SHR and SPR should be correlated.

# Annex	TP for S-CPAC for 37.340

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For S-CPAC, if the suitable PSCell is one of the candidate PSCells provided to the MN by the SN at S-CPAC initiation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, for MN initiated the MN performs the final MRO related optimisation, or for SN initiated the MN forwards the information to the initiating SN who performs the final MRO related optimisation.

## 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in the cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN.

For S-CPAC the MN may send an updated UE History Information to the initiating SN (if applicable) and may also send it to canadidate SN(s) using the MN initiated SN Modification procedure.

# Annex	TP for S-CPAC for 38.423

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

[snip]

If the PSCell History Information Retrieve IE set to "query" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information as specified in TS 37.340 [8].

If the UE History Information from the UE IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the UE History Information IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the SCG UE History Information IE is included in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| ... |  |  |  |  |  |  |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

# Annex	LS to RAN2 on MRO for CHO with candidate SCGs

Title:	[DRAFT] LS on MRO for CHO with candidate SCGs

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN3 has discussed MRO for CHO with candidate SCGs and agreed it is beneficial if the following information can be reported by the UE to the network:

- For case 1, 2 and 3, enhance the following information to RLF report: which of which type of execution is fullfilled, the corresponding PCell/PSCell ID information, which type of the exeution is not fulfilled and the corresponding PCell/PSCell ID, and also the time since the first fulfilment till the failure in the RLF report.

- For case 7a, 7b, 8a and 8b, no further MRO enhancement is needed.

- For case 9, enhance the following informations to RLF report and SCG failure information: which type first fulfilled, first fulfilled Cell ID, the time since the first fulfillment till RLF/SCG failure, the time since the first fulfilment till unfulfillment, which type subsequently fulfilled, the subsequently fulfilled Cell ID, and the time since the subsequent fulfillment till RLF/SCG failure.

RAN3 would like to ask RAN2 if it would be feasible to instead attach the SCG failure information to the RLF report used to indicate he connection failure for the PCell.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

RAN3#127	2025-02-17  – 2024-02-21	Athens, GR