3GPP TSG-RAN3 Meeting #125bis	R3-245243

# Hefei, China, 14th – 18th Oct 2024

Agenda Item:	11.4

Source:	CATT

Title:	Scenario clarification for continuous MDT

Document for:	Decision

- Introduction

In SI phase, we concluded using Management based MDT for continuous data collection, and split it into two tasks [1]:

Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In this paper, we will analyse the typically scenarios of continuous MDT and allocated the two tasks in each scenario.

2. Discussion

In legacy management based MDT, The management system sends a Trace Session activation request to the gNB, which including the Trace Reference (TR). After that, gNB shall select the suitable UEs for MDT data collection and assign Trace Recording Session Reference (TRSR) corresponding to the selected UE. The selection is based on the area received from the management system, the area where UE is located, user consent information and UE capability.

Observation 1: In legacy MDT, TR is assigned by the management system for specific trace task, while the TRSR is assigned by gNB corresponding to the selected UE.

Observation 2: UE selection is based on the area received from the management system, the area where UE is located, user consent information and UE capability

In the following scenario, to simplify the discussion, it is assumed that both gNBs (gNB1 and gNB2) belong to the same area scope and all satisfy the user consent requirement of UE. The management system configures both gNBs (gNB 1 and gNB2) with same immediate MDT configuration(TR1) and management-based logged MDT configuration(TR2). The gNB1 choose UE to execute continuous MDT and configured the two MDT configurations to UE separately.

Scenario 1：UE across different RRC states in same gNB

Figure 1 UE across different RRC states in same gNB

In figure 1 we illustrate a scenario of maintaining continuous management based MDT when UE transits such as idle-connected-idle state.

gNB selected UE for continuous MDT, and configured both immediate MDT configuration and logged MDT configuration during UE connection state. During the connection state, UE shall report the UE mesaurement to the gNB, and the gNB shall store it in the immediate MDT record and transfer it to the TCE combining the assigned TR, TRSR. When UE transits to IDLE/INACTIVE, UE shall start logged MDT based on the previous received logged MDT configuration.

Base on the figure 1, the measurement continuity issue may appear at step 11 due to the UE context is delete at the network side. For trace correlation, it need to enable TCE correlation the MDT received at step 8, step 13. E.g., correlation between immediate MDT record and logged MDT record in the same gNB .

Observation 3: MDT measurement interrupt issues exist when UE across different RRC states in same gNB

- Measurement continuity: when UE transit from Idle/Inactive to Connected in the same gNB

- Trace correlation: correlation between immediate MDT record and logged MDT record in the same gNB.

Scenario 2：UE across different RRC states in different gNB

Figure 2 UE across different RRC states in different gNB

In figure 2 we illustrate a scenario of maintaining continuous management based MDT when UE transits to RRC_idle at gNB1, and back to connection at gNB2. As shown above, UE is selected to execute continuous MDT by gNB1, similar with scenario 1, gNB1 configured both immediate MDT configuration and logged MDT configuration to the UE. UE excuses immediate MDT after receive the configuration and gNB1 report the immediate MDT record to the TCE. UE transits to RRC_idle at step 10 and starts logged MDT according to previous configuration. UE back to RRC_connected at step13, and based on network request, UE can send the logged MDT record to the TCE via gNB2.

Base on the figure 2, the measurement continuity issue shall appeare at step 13 due to the UE context is delete at the network side. For trace correlation, it need to enable TCE correlation the MDT received at step 10 and step 15 together. E.g., correlation between immediate MDT record session and logged MDT record session report configured at same gNB but report at different gNBs.

What’s more, after UE back to connected, for realize continuous measurements, the CU shall re-configured the immediate MDT to the UE with new assigned TRSR, which should also be correlation with previous logged MDT record. The similar issue is exist in both figure 1(step 13 and step 17) and figure 2( step 13 and step 17). E.g., correlation between logged MDT record session and immediate MDT record session report configured at same gNB.

Observation 4: MDT measurement interrupt issues exist when UE across different RRC states in different gNB

- Measurement continuity: when UE transit from Idle/Inactive to Connected at different gNBs

- Trace correlation: correlation between immediate MDT record session and logged MDT record session configured at same gNB but report from different gNB, logged MDT record session and immedicate record session from the same gNB.

.

Scenario 3：UE HO in different gNB

Figure 3 UE HO in different gNB at same RRC state

In figure 3 we illustrate a scenario of maintaining continuous management based MDT when UE HO between two gNBs. As shown in above figure, the management system configures the same immediate MDT configuration for the gNB1 and gNB2. gNB1 selected UE for immediate MDT and report the immediate MDT report to the TCE. When UE HO to gNB2, the immediate MDT shall stop unless the UE be re-selected by gNB2 again.

Base on the figure 3, the measurement continuity issue shall appear at step 8 due to UE out of the previous configured cells. For trace correlation, it need to enable TCE correlation the MDT received at step 7 and step 13 together. E.g., correlation between immediate MDT record session and immediate MDT record session provide by different gNBs.

Observation 5: MDT measurement interrupt issues exist when the UE HO to different gNB at same RRC state.

- Measurement continuity: when UE HO to another gNB

- Trace correlation: correlation between two immediate MDT record session from different gNB.

Proposal 1: Consider the below 3 scenario for continuous MDT

- UE across different RRC states in the same gNB

- UE across different RRC states in different gNB

- UE HO in different gNB

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: In legacy MDT, TR is assigned by the management system for specific trace task, while the TRSR is assigned by gNB corresponding to the selected UE.

Observation 2: UE selection is based on the area received from the management system, the area where UE is located, user consent information and UE capability

Observation 3: MDT measurement interrupt issues exist when UE across different RRC states in same gNB

- Measurement continuity: when UE transit from Idle/Inactive to connected in the same gNB

- Trace correlation: correlation between immediate MDT record and logged MDT record in the same gNB.

Observation 4: MDT measurement interrupt issues exist when UE across different RRC states in different gNB

- Measurement continuity: when UE transit from Idle/Inactive to connected at different gNBs

- Trace correlation: correlation between immediate MDT record session and logged MDT record session configured at same gNB but report from different gNB, logged MDT record session and immedicate record session from the same gNB.

Observation 5: MDT measurement interrupt issues exist when the UE HO to different gNB at same RRC state.

- Measurement continuity: when UE HO to another gNB

- Trace correlation: correlation between two immediate MDT record session from different gNB.

Proposal 1: Consider the below 3 scenario for continuous MDT

- UE across different RRC states in the same gNB

- UE across different RRC states in different gNB

- UE HO in different gNB

4. Reference

[1] TR38.743