3GPP TSG-RAN WG3 Meeting #125bis                                                              R3-245247

Hefei, China, October 14 – 18, 2024

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(draft Reply LS to SA2) Discussion on SA2 questions on multi-hop WAB and UE ULI

Document for:		Other

# 1	Introduction

SA2 sent an LS to RAN3 on questions regarding FS_VMR_Ph2 in R3-244019/S2-2407345 with the following content:

1 Overall description SA2 would like to inform RAN3 that SA2 had progressed the Rel-19 FS_VMR_Ph2 study and documented the conclusions in clause 8 in TR 23.700-06. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06. 2 Actions To RAN WG3 ACTION: - SA2 kindly asks RAN WG3 to provide feedback on above issues.

This contribution discusses the replies to SA2’s questions, and it contains a draft LS in the Annex.

# 2	Discussion

## Issue 1

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

The solutions defined by SA2 and captured in clause 6.2.3.6 of TS 23.700-6 involve the CN and handle the following scenarios:

- Avoiding service request by a WAB-MT at a non-collocated WAB-gNB.

- Avoiding NG handover of a WAB-MT to a WAB-gNB.

The solutions proposed do not handle the following scenarios:

- Avoiding Xn handover of a WAB-MT to a WAB-gNB.

- Avoiding resume procedure by a WAB-MT at a WAB-gNB with context retrieval via Xn.

The CN-based solutions further require enhancements on NGAP and AMF.

Observation 1: SA2’s CN-based solutions to avoid WAB-MT from connecting to WAB-gNB do not address all scenarios of issues 1a and 1b, and they require enhancements to NGAP and AMF.

In the following, RAN-based solutions are discussed with the goal to address all relevant scenarios, i.e., including those not addressed by SA’s CN-based solutions. Further, an attempt is made to minimize enhancements to specifications and to avoid enhancements to the BH RAN.

RRC-Idle WAB-MT performs service request at WAB-gNB

The following RAN-based options can be considered to avoid this scenario:

Solution 1: WAB-gNB cells use dedicated frequencies and/or PCI range. The WAB-MT avoids (re)selection of cells using these frequencies and/or PCI range.

Solution 2: WAB-gNB cells broadcast a “WAB” indicator in SIB. The WAB-MT avoids (re)selection of cells broadcasting this indicator.

Both solutions can handle issues 1a) and 1b) raised by SA2. Solution 1 has no stage-3 impact. Solution 2 has stage-3 impact. Neither Solution 1 nor Solution 2 require enhancements to the BH RAN.

RRC-Inactive WAB-MT performs Resume procedure at WAB-gNB

The RAN-based solutions 1 and 2 above can also be used to avoid the scenario.

RRC-Connected UE is handed over via Xn to WAB-gNB

The following RAN-based solutions can be considered to avoid this scenario:

Solution 1: WAB-gNB cells use dedicated frequencies and/or PCI range. The WAB-MT avoids reporting cells using these frequencies and/or PCI range.

Solution 2: WAB-gNB cells broadcast a “WAB” indicator in SIB1. The WAB-MT avoids reporting cells using these frequencies and/or PCI range.

Solution 3: WAB-gNB rejects Xn Handover Request of a WAB-MT based on a WAB-indicator in the UE capabilities.

As discussed above, solution 1 does not have stage-3 impact. Solutions 2 and 3 have stage-3 specification impact. None of the options require enhancements to the BH RAN.

Proposal 1-1: The following RAN-based solutions can avoid multi-hop WAB and address SA2’s issues:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCI partitions, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Solution 2: WAB-gNB-cells broadcast a “WAB” indicator in SIB, and the WAB-MT avoids (re)selection or reporting of cells broadcasting this indicator.

Solution 3: WAB-MT includes a “WAB” indicator in UE capabilities, and a WAB-gNB rejects an access attempt or a handover request based on this indicator.

Proposal 1-2: The Reply LS to SA2 to capture the above RAN-based solutions to avoid multi-hop WAB.

## Issue 2

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

TS 23.700-06, clause 8.5 captures the following:

The following aspects are taken as way forward during normative work: - Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11. - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request. NOTE: If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

From RAN3’s perspective, there is no issue in adding WAB-MT location information to the UE’s ULI. Further, from RAN3’s perspective, all three options proposed by SA2 are feasible.

Observation 2-1: From RAN3’s perspective, all three options identified by SA2 for the additional location information to be included with the UE’s ULI are feasible.

All three options have stage-3 impact on RAN3 specification:

Impact of Option 1: The User Location Information IE in NGAP needs to include an additional IE to carry the information the WAB-node’s mapped location information.

Impact of Option 2: The User Location Information IE in NGAP needs to include an additional IE to carry the information of the WAB-node’s geolocation.

Impact of Option 3: The User Location Information IE in NGAP already contains an additional ULI IE referred to as Mobile IAB-MT User Location Information. Since this additional IE explicitly refers to mobile IAB-MT, it cannot be used to also support WAB. Therefore, yet another addition ULI IE needs to be included for the WAB-MT’s ULI.

The specification impact for the three options is comparable from RAN3-perspective. Since there is no obvious preference for one or the other option, RAN3 can leave the down selection to SA2.

Observation 2-2: The enhancements for all three options identified by SA2 have stage-3 impact on RAN3 specification, the impact is comparable and rather small.

Proposal 2: Reply to SA2 that from RAN3’s perspective, all three options to include the WAB-MT’s location information in the UE’s ULI are feasible, and that RAN3 prefers to leave the down-selection to SA2.

## Draft Reply LS

A draft reply LS has been provided in the Annex.

Proposal 3: RAN3 to approve the draft Reply LS provided in the Annex.

# 3	Conclusion

This contribution discussed the reply to SA2’s LS on questions regarding FS_VMR_Ph2. The following proposals have been made:

Issue 1

Observation 1: SA2’s CN-based solutions to avoid WAB-MT from connecting to WAB-gNB do not address all scenarios of issues 1a and 1b, and they require enhancements to NGAP and AMF.

Proposal 1-1: The following RAN-based solutions can avoid multi-hop WAB and address SA2’s issues:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCI partitions, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Solution 2: WAB-gNB-cells broadcast a “WAB” indicator in SIB, and the WAB-MT avoids (re)selection or reporting of cells broadcasting this indicator.

Solution 3: WAB-MT includes a “WAB” indicator in UE capabilities, and a WAB-gNB rejects an access attempt or a handover request based on this indicator.

Proposal 1-2: The Reply LS to SA2 to capture the above RAN-based solutions to avoid multi-hop WAB.

Issue 2

Observation 2-1: From RAN3’s perspective, all of the three options identified by SA2 for the additional location information to be included with the UE’s ULI are feasible.

Observation 2-2: The enhancements for all three options identified by SA2 have stage-3 impact on RAN3 specification, the impact is comparable and rather small.

Proposal 2: Reply to SA2 that from RAN3’s perspective, all three options to include the WAB-MT’s location information in the UE’s ULI are feasible, and that RAN3 prefers to leave the down-selection to SA2.

Draft Reply LS

Proposal 3: RAN3 to approve the draft Reply LS provided in the Annex.

# Annex: Draft Reply LS to SA2

3GPP TSG RAN WG3#125bis	R3-24xxxx

Hefei, China, October 14 – 18, 2024

Title:	[Draft] Reply LS on questions regarding FS_VMR_Ph2

Response to:	LS in R3-244019 (S2-2407345) on questions regarding FS_VMR_Ph2

Release:	Rel-19

Study Item:	FS_VMR_Ph2

Source:	Qualcomm [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on FS_VMR_Ph2 solution impacts to RAN (R3-244019/S2-2407345).

On issue 1:

SA2’s question:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3’s answer:

RAN3 observed that SA2’s CN-based solutions to avoid WAB-MT from connecting to WAB-gNB discussed in clause 6.2.3.6 of TS 23.700-06 did not handle all scenarios of issue 1a and 1b. RAN3 further observed that these CN-based solutions required enhancements to NGAP and AMF.

As alternative, RAN3 identified the following RAN-based solutions:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCI partitions, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Solution 2: WAB-gNB-cells broadcast a “WAB” indicator in SIB, and the WAB-MT avoids (re)selection or reporting of cells broadcasting this indicator.

Solution 3: WAB-MT includes a “WAB” indicator in UE capabilities, and a WAB-gNB rejects an access attempt or a handover request based on this indicator.

RAN3 believes that these solutions can handle all scenarios of issues 1a and 1b. None of these solutions have impact on the BH RAN. RAN3 will specify at least solution 1 to address SA2’s concerns.

On issue 2:

SA2’s question:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3’s answer:

From RAN3’s perspective, all three options to include the WAB-MT’s location information in the UE’s ULI are feasible. RAN3 prefers to leave further down-selection to SA2.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #126, November 18 to 22, 2024       Orlando, FL, USA

TSG-RAN3 Meeting	 #127, February 17 to 21, 2024       Athens, Greece