3GPP TSG-RAN WG3 Meeting #125bis                                                              R3-245248

Hefei, China, October 14 – 18, 2024

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on assumptions and architecture for WAB

Document for:		Discussion

# 1	Introduction

TSG RAN#105 agreed the New WID on additional topological enhancements for NR in RP-242395 [1]. The following discussion addresses aspects related to WAB in compliance with the objectives of this WID.

# 2	Discussion

## 2.1	Assumptions for WAB

In the first step of the normative phase, RAN3 should aim to converge on terminology and initial assumptions for WAB.  In this context, the WID in [1] states:

The normative work for WAB should be based on the functionalities, terminology and requirements captured in TR 38.799.

TR 38.799 contains the following assumptions and requirements:

The study is based on the following requirements: - The WAB-node includes a gNB component (WAB-gNB) and an MT component (WAB-MT). - The WAB-gNB is based on the gNB functionality specified in TS 38.300 [4] and TS 38.401 [3]. - The CU-DU split of the WAB-gNB is not considered in this study. - The WAB-MT supports at least a subset of UE functionalities. - The NR Uu is used for the radio link between WAB-gNB and the served UEs. - The NR Uu radio link between the WAB-gNB and the served UEs does not use NTN. - The study focuses on NR-Uu backhaul. - In-band scenario for access and backhaul is not precluded to be studied. - The study precludes the scenario where the access and the backhaul are in-band while the backhaul uses NTN. - The study focuses on the use of WAB-MT´s PDU session via NR Uu as backhaul of WAB-gNB. Other options for the backhaul (including non-3GPP radio technology) are not precluded but are not a part of the study. - A WAB-gNB cannot serve WAB-MT(s). - The study includes a scenario where the WAB-gNB and the WAB-MT connect to the same PLMN or to different PLMNs. - The WAB-MT may connect to a public PLMN or an SNPN. - The WAB-gNB may connect to a public PLMN or an SNPN. - Legacy UEs can connect to the WAB-gNB. There are no WAB-specific enhancements for UEs that connect to the WAB-gNB.

The green highlighted requirements have already been captured in the WID. The yellow highlighted requirement follows from the requirements for backhauling via PDU-session. The cyan highlighted requirement does not need discussion since it follows from the WID objective:

- The handling of: - … - Radio-resource coordination between access and backhaul links.

The remaining requirements of TR 38.799 can be captured as assumptions for the normative phase:

-	The WAB-gNB is based on the gNB functionality specified in TS 38.300 [4] and TS 38.401 [3].

-	The WAB-MT supports at least a subset of UE functionalities.

-	The NR Uu is used for the radio link between WAB-gNB and the served UEs.

-	The NR Uu radio link between the WAB-gNB and the served UEs does not use NTN.

-	WAB does not support the scenario where the access and the backhaul are in-band while the backhaul uses NTN.

-	A WAB-gNB should not serve WAB-MT(s).

-	The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

-	The WAB-MT may connect to a public PLMN or an SNPN.

-	The WAB-gNB may connect to a public PLMN or an SNPN.

Proposal 1.1: In compliance with the WID and TR 38.799, WAB to meet the following assumptions:

-	The WAB-gNB is based on the gNB functionality specified in TS 38.300 [4] and TS 38.401 [3].

-	The WAB-MT supports at least a subset of UE functionalities.

-	The NR Uu is used for the radio link between WAB-gNB and the served UEs.

-	The NR Uu radio link between the WAB-gNB and the served UEs does not use NTN.

-	WAB does not support the scenario where the access and the backhaul are in-band while the backhaul uses NTN.

-	A WAB-gNB should not serve WAB-MT(s).

-	The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

-	The WAB-MT may connect to a public PLMN or an SNPN.

-	The WAB-gNB may connect to a public PLMN or an SNPN.

Proposal 1.2: The above assumptions for WAB to be captured on stage-2 in TS 38.401.

## 2.2	Architecture and protocol stacks for WAB

The architecture for WAB was discussed during the study and capture in TR 38.799 in Figure 4.2-1. The protocol stacks for WAB’s NG and Xn interfaces were also discussed during the study and captured in TR 38.799 in Figures 4.2-3 and 4.2-4, respectively. The architecture figure and these protocol stacks should be adopted for stage 2, preferably in 38.401.

Proposal 2: The architecture figure for WAB in TR 38.799 Figure 4.2-3 and the protocol stacks for NG in TR 38.799, Figure 4.2-3 and Xn in TR 38.799 Figure 4.2-4 to be added to stage-2 description of WAB in TS 38.401.

## 2.3	Authorization for WAB-node

During the study phase, RAN3 agreed that the authorization procedure of WAB-MT and WAB-gNB to be left up to SA2. While SA2 discussed WAB authorization during the SA2 study on VMR2, they have not yet captured WAB authorization on normative stage. Since SA2 is scheduled to complete the WI on VMR2 before RAN’s WI on Additional Topological Enhancements, RAN3 can wait for SA2 on the procedure for WAB-node authorization.

Proposal 3.1: RAN3 to wait for SA2 on the definition for the WAB-node authorization procedure.

However, the WID for RAN’s WI on Additional Topological Enhancements defines the following objective for RAN3 on the authorization of the WAB-node [1]:

- Specifications for the support of WAB including [RAN3]: … - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes.

This behaviour was already discussed during the study phase and captured in TR 38.799. Based on this prior discussion, RAN3 should agree on the description of TR 38.799:

Proposal 3.2: In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

Proposal 3.3: In case the WAB-gNB authorization status changes from “not authorized” to “authorized”, the WAB-gNB to reinstate NG and the operation of the WAB-gNB’s air interface.

Proposal 3.4: The above agreements to be captured on stage 2.

## 2.4	PCI configuration and collision avoidance

The WID for RAN’s WI on Additional Topological Enhancements defines the following objective for RAN3 on the authorization of the WAB-node [1]:

- Specifications for the support of WAB including [RAN3]: … - The handling of: - PCI collision avoidance. NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB.

The PCI configuration for gNB cells is defined in TS 38.401, clause 7.8. This clause further defines the following procedure for PCI collision avoidance for mobile IAB:

For mobile IAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells and stationary cells by implementation.

Based on NOTE 1 in WID [1], these procedures should also be applied to handle PCI configuration and collision avoidance of WAB-gNB cells.

Proposal 4: PCI configuration and PCI collision avoidance to follow the same procedure as defined for mobile IAB in TS 38.401, clause 7.8.

## 2.5	Avoiding Xn establishment between WAB-gNBs

The WID for RAN’s WI on Additional Topological Enhancements defines the following objective for RAN3 on the authorization of the WAB-node [1]:

- Specifications for the support of WAB including [RAN3]: … - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

The establishment of Xn between WAB-gNBs may have benefits in some scenarios, e.g., when the WAB-nodes are stationary and in close vicinity, but it may have shortcomings in others, e.g., when the WAB-nodes move with high relative speed. It may therefore be beneficial to allow for Xn establishment between WAB-gNBs as on optional feature.

The WAB-gNB should therefore be aware whether a neighbour RAN Node is a peer WAB-gNB, or not. To enable this awareness, the following options can be considered:

Option 1: A dedicated frequency and/or PCI range is assigned to WAB-gNB cells. The WAB-gNB rejects Xn Setup Requests in case they contain served cells using this dedicated frequency and/or PCI range.

Option 2: WAB-gNBs includes a “WAB” indicator in the served-cell information contained in the Xn Setup Request. The WAB-gNB rejects Xn Setup Requests in case they contain the “WAB” indicator in the served cell information.

Option 1 has no stage-3 impact, but it requires allocating a dedicated frequency and/or PCI range to WAB-gNB cells. Option 2 does have stage-3 impact, but it allows using same frequency and PCI range for WAB-gNB cells and BH RAN cells.

Proposal 5.1: For avoiding Xn establishment between WAB-gNBs, the following two options to be considered:

Option 1: The WAB-gNB rejects Xn Setup Requests based on WAB-specific frequency or PCI range in the served cell information.

Option 2: The WAB-gNB rejects Xn Setup Request based on a “WAB” indicator in the served cell information.

Proposal 4 recommends that PCI collision avoidance be handled by allocating a separate PCI range to WAB-gNB cells. In case Proposal 4 is agreed, Option 1 would automatically apply and Option 2 can be omitted.

Proposal 5.2: In case RAN3 agrees to PCI partitioning as the solution to PCI collision avoidance, the same solution to be used for avoidance of Xn establishment between WAB-gNBs.

## 2.6	Change of UE’s AMF using single logical WAB-gNB

The WID for RAN’s WI on Additional Topological Enhancements defines the following objective for RAN3 [1]:

- Specifications for the support of WAB including [RAN3]: … - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB.

During the study, RAN3 discussed procedures to support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. RAN3 agreed that such AMF change can be conducted via a two-logical-gNB solution. RAN3 further contemplated the following options that only use a single logical WAB-gNB. These options need further discussion:

Option 1: Single WAB-gNB with a single cell using mobility registration update due to TAC change

Option 2: Single WAB-gNB with two cells with different TACs, using NG-based HO

Option 3: Single WAB-gNB with single cell without TAC change

In all three options, the WAB-gNB is concurrently connected to both AMFs. In options 1 and 2, the WAB-gNB reports a different TAC to each AMF. In Option 3, the WAB-gNB reports the same TAC to both AMFs.

Problem with Option 1:

CM-IDLE UE:

When the CM-IDLE UE observes the change of TAC in SI, it sends an MRU. The WAB-gNB will direct the MRU to the new AMF. The new AMF can pull the UE’s context from the old AMF. No problem here.

CM-CONNECTED UE:

When the CM-CONNECTED UE observes the change of TAC in SI, it sends an MRU. Since the UE is already CM-CONNECTED with context in the initial AMF, the WAB-gNB directs the MRU including the initial TAC to the initial AMF. The initial AMF has no reason to perform AMF reallocation since the initial TAC is still registered for the cell the UE is connected to. Therefore, enhancements would be necessary to RAN/CN procedures and/or AMF behavior to enable Option 1 for CM-CONNECTED UEs.

Observation 1: Enhancements are needed to RAN/CN procedures and/or AMF behavior to enable Option 1 for CM-CONNECTED UEs.

Problem with Option 2:

CM-IDLE UE:

When the initial cell is removed from the air interface, the CM-IDLE UE will reselect the new cell. Since the TAC has changed, the UE will send an MRU. The WAB-gNB will direct the MRU to the new AMF. The new AMF can pull the UE’s context from the old AMF. No problem here.

CM-CONNECTED UE:

The WAB-gNB will initiate an NG handover for the CM-CONNECTED UE from the initial cell to the new cell. When the initial AMF receives the HO Required message, it will not perform an AMF reallocation since the target cell belongs to the same gNB. Therefore, enhancements would be necessary to RAN/CN procedures and/or AMF behavior to enable Option 2 for CM-CONNECTED UEs.

Observation 2: Enhancements are needed to the AMF behavior to support Option 2 for CM-CONNECTED UEs.

Problem with Option 3:

CM-IDLE UE:

Since neither the cell nor TAC change during the AMF change, the CM-IDLE UE will not send an MRU, and therefore, its context will be retained in the initial AMF. This implies that when the WAB-node moves out of the initial AMF’s coverage area, the CM-IDLE UE cannot be paged anymore. Significant enhancements to existing RAN/CN procedures would be necessary to support Option 3 for CM-IDLE UEs.

CM-CONNECTED UE:

As for the CM-IDLE UE, the CM-CONNECTED UE will not invoke an MRU at the time of the AMF change. With only single cell, the WAB-gNB cannot cannot hand it over, and therefore, its context will be retained in the initial AMF. This implies that when the WAB-node moves out of the initial AMF’s coverage area, the CM-CONNECTED UE is losing AMF connectivity. Significant enhancements to existing RAN/CN procedures would be necessary to support Option 3 for CM-CONNECTED UEs.

Observation 3: Significant enhancements to existing RAN/CN procedures are necessary to support Option 3 for CM-IDLE and CM-CONNECTED UEs.

Based on observations 1, 2 and 3, all options considered for AMF change with a single logical WAB-gNB require enhancements to AMF and/or RAN/CN procedures. Since such enhancements are not needed for AMF change with two logical WAB-gNBs, the single-logical-WAB-gNB solutions should be deprioritized.

Proposal 6: Deprioritize solutions to facilitate AMF change using single-WAB-gNB since these solutions require enhancements to AMF and/or RAN/CN procedures which are not needed when using two logical WAB-gNBs.

## 2.7	Handling of WAB-gNB’s traffic during PDU session change

The WID for RAN’s WI on Additional Topological Enhancements defines the following objective for RAN3 on the authorization of the WAB-node [1]:

- Specifications for the support of WAB including [RAN3]: … - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes.

The only critical scenario to be considered is when the WAB-MT’s PDU session changes. In this case, the IP address(es) of the WAB-MT change(s), too. NG-, Xn- and OAM traffic need to cope with the change of IP addresses.

The following solutions can be considered:

IP address change for NG-C: According to TS 38.412, the WAB-gNB can establish a new SCTP association with the AMF using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

IP address change for Xn-C: According to TS 38.422, the WAB-gNB can establish a new SCTP association with the peer gNB using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

IP address change for NG-U: According to TS 38.413, the WAB-gNB can migrate the NG-U GTP-U tunnels to the new IP address(es) via the NGAP PDU session Resource Modify Indication procedure.

IP address change for Xn-U: Xn-U is used for data forwarding during handover and for DC (split bearers, MN-terminated SCG bearer, SN-terminated MCG bearer). Data forwarding during handover usually covers only a short time frame. In case data forwarding is interrupted due to change of tunnel end point IP addresses, it may be easier to retransmit the packets on upper layers. For DC, the GTP-U tunnels can be migrated via the procedures S-NG-RAN node MODIFICATION PROCEDURES.

IP address change for OAM traffic: The impact of IP address change on OAM traffic is out-of-scope. Also, an equivalent scenario was already supported for IAB, where OAM traffic can be exchanged via the backhaul, whose IP addresses can change.

Proposal 7: In case the WAB-gNB’s IP addresses change, e.g., due to the change of the WAB-MT’s PDU session, legacy procedures can be used and are sufficient to migrate the WAB-gNB’s NG and Xn traffic to the new IP addresses.

# 3	Conclusion

This contribution discussed the aspects of WAB. The following observations and proposals have been made:

Proposal 1.1: In compliance with the WID and TR 38.799, WAB to meet the following assumptions:

-	The WAB-gNB is based on the gNB functionality specified in TS 38.300 [4] and TS 38.401 [3].

-	The WAB-MT supports at least a subset of UE functionalities.

-	The NR Uu is used for the radio link between WAB-gNB and the served UEs.

-	The NR Uu radio link between the WAB-gNB and the served UEs does not use NTN.

-	WAB does not support the scenario where the access and the backhaul are in-band while the backhaul uses NTN.

-	A WAB-gNB should not serve WAB-MT(s).

-	The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

-	The WAB-MT may connect to a public PLMN or an SNPN.

-	The WAB-gNB may connect to a public PLMN or an SNPN.

Proposal 1.2: The above assumptions for WAB to be captured on stage-2 in TS 38.401.

Proposal 2.1: The architecture figure for WAB in TR 38.799 Figure 4.2-3 and the protocol stacks for NG in TR 38.799, Figure 4.2-3 and Xn in TR 38.799 Figure 4.2-4 to be added to stage-2 description of WAB in TS 38.401.

Proposal 3.1: RAN3 to wait for SA2 on the definition for the WAB-node authorization procedure.

Proposal 3.2: In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

Proposal 3.3: In case the WAB-gNB authorization status changes from “not authorized” to “authorized”, the WAB-gNB to reinstate NG and the operation of the WAB-gNB’s air interface.

Proposal 3.4: The above agreements to be captured on stage 2.

Proposal 4: PCI configuration and PCI collision avoidance to follow the same procedure as defined for mobile IAB in TS 38.401, clause 7.8.

Proposal 5.1: For avoiding Xn establishment between WAB-gNBs, the following two options to be considered:

Option 1: The WAB-gNB rejects Xn Setup Requests based on WAB-specific frequency or PCI range in the served cell information.

Option 2: The WAB-gNB rejects Xn Setup Request based on a “WAB” indicator in the served cell information.

Proposal 5.2: In case RAN3 agrees to PCI partitioning as the solution to PCI collision avoidance, the same solution to be used for avoidance of Xn establishment between WAB-gNBs.

Observation 1: Enhancements are needed to RAN/CN procedures and/or AMF behavior to enable Option 1 for CM-CONNECTED UEs.

Observation 2: Enhancements are needed to the AMF behavior to support Option 2 for CM-CONNECTED UEs.

Observation 3: Significant enhancements to existing RAN/CN procedures are necessary to support Option 3 for CM-IDLE and CM-CONNECTED UEs.

Proposal 6: Deprioritize solutions to facilitate AMF change using single-WAB-gNB since these solutions require enhancements to AMF and/or RAN/CN procedures which are not needed when using two logical WAB-gNBs.

Proposal 7: In case the WAB-gNB’s IP addresses change, e.g., due to the change of the WAB-MT’s PDU session, legacy procedures can be used and are sufficient to migrate the WAB-gNB’s NG and Xn traffic to the new IP addresses.

# Reference

[1]  RP-242395, New WID on additional topological enhancements for NR, TSG RAN Meeting #105, Melbourne, Australia, September 2024