3GPP TSG-RAN WG3 Meeting #125bis                                                             R3-245253

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Other issues on WAB

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In the last meeting, the study of additional topological enhancements for NR was finished, and the conclusion and recommendation for Wireless Access Backhaul (WAB) is produced [1].

In this contribution, we will discuss other open issues on WAB.

2 Discussion

2.1 Xn connection management

2.1.1 Dynamic Xn setup

Based on the WID approved during RAN#105 [2], Xn interface setup for the WAB-gNB with BH-RAN-node or surrounding NG-RAN nodes should be specified. Different to normal gNB, Xn setup is dynamic for WAB-gNB, due to the neighbour relationship may change with the movement of WAB-node.

As a legacy mechanism, UEs served by WAB-node can detect the PCI of the neighbour cell of WAB-gNB, and UE’s CGI reporting procedure can be used to derive the RAN-node ID of the neighbour nodes. However, the CGI acquisition is optionally supported by UE. Other method should be studied. Basically, the cell serving the WAB-MT under the BH-RAN-node is probably the nearest neighbour cell of WAB-gNB. As the RAN-node ID of the BH-RAN-node is broadcasted in SIB1, the WAB-gNB can recognize the BH-RAN-node after the WAB-MT reading SIB1 of the serving cell. Then, the WAB-gNB can obtain IP address of the BH-RAN-node and set up Xn connection with the BH-RAN-node. IP address of the BH-RAN-node can be obtained by the legacy Xn-C TNL address discovery mechanism. This procedure only impacts stage-2.

Proposal 1-1: WAB-gNB can recognize BH-RAN-node via the SIB1 of the cell serving the WAB-MT.

Proposal 1-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism and set up Xn connection with it.

If setting up Xn with the BH-RAN-node, the WAB-gNB may discover some neighbour cells’ CGI from the serving cell information delivered by the BH-RAN-node over Xn interface.

There may be other neighbour cells of WAB-gNB, which are supposed in scope of the neighbour cells of the BH-RAN-node. The WAB-gNB can also discover neighbour cells’ CGI from the neighbor cells of the BH-RAN-node. The information of BH-RAN-node’s neighbour cells is sent from BH-RAN-node via Neighbour Information NR IE or Neighbour Information E-UTRA IE in some Xn messages, including Xn SETUP REQUEST/RESPONSE message, NG-RAN NODE CONFIGURATION UPDATE message, and NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

After discovering the neighbour cells’ CGI, the WAB-gNB can add the neighbour cells to the NCRT and look up the TNL address of the neighbour nodes to set up Xn interface. The WAB-gNB can get the IP addresses of the neighbour RAN-nodes using the legacy Xn-C TNL address discovery mechanism. This procedure impacts only stage-2.

Proposal 1-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 1-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism and set up Xn connections with surrounding RAN nodes.

On the other hand, according to the WID approved during RAN#105 [2], setting up Xn between WAB-gNBs should be avoided. Due to the mobility of WAB-node is unpredictable, the WAB-gNBs is recommended not to set up Xn connection with another WAB-gNB in most cases, in order to avoid the frequent Xn setup and removal. For avoidance of setting up Xn between WAB-gNBs, WAB-cell indication needs to be introduced in Xn interface.

When each WAB-gNB sets up Xn connections with its BH-RAN-node or other neighbour RAN nodes, it can exchange WAB-Cell indicator via Xn Setup Request/Response messages. And the BH-RAN-node or neighbour nodes can spread the information of WAB cell to their neighbour by the neighbour cell information IE in Xn messages, that the WAB-gNB can discover the WAB cells in the neighbour cell information of Xn messages. A WAB-Cell indicator should be added to the serving cell information IE and the neighbour cell information IE of Xn interface.

Take following figure as an example: WAB-node 1 exchanges WAB-Cell indicator to BH-RAN-node 1 by Xn connection and WAB-node 2 exchanges WAB-Cell indicator to BH-RAN-node 2 and BH-RAN-node 1 over Xn. BH-RAN-node 1 can know the WAB cell of WAB-node 2 directly from Xn message of WAB-node 2 or from Xn message of BH-RAN-node 2. BH-RAN-node 2 knows the WAB cell of WAB-node 1 from Xn messages of BH-RAN-node 1. As a result, WAB-node 1 can know the WAB cell of WAB-node 2 through the neighbour cell information sent by BH-RAN-node 1, WAB-node 1 can know the WAB cell of WAB-node 2 from the neighbour cell information sent by BH-RAN-node 2 or BH-RAN-node 1.

Proposal 1-5: To avoid setting up Xn between WAB-nodes, a WAB-Cell indicator can be added to the serving cell information and the neighbour cell information in Xn messages.

2.1.2 Dynamic Xn removal

Due to the mobility of WAB-node, the NCRT in the WAB-gNB may change more frequently than legacy gNB. The WAB-node would need to set up Xn with new neighbour nodes and release Xn with the original neighbour nodes during its movement. The WAB-node can discover PCI of the new neighbour cell through the UE’s measurement result.

If the WAB-MT connects to a new BH-RAN-node and WAB-gNB establishes Xn with the new BH-RAN-node, the WAB-gNB can derive CGI of the current neighbour cells through serving cell information or neighbour cell information sent by the new BH-RAN-node over Xn. The WAB-gNB may add the new neighbour cells to the neighbour relationship, and set up Xn connection with the new neighbour nodes. Accordingly, the WAB-gNB would have to remove cells from the NCRT dynamically.

The WAB-gNB will consider the cell(s), which is not included in the serving/neighbour cell information of current BH-RAN-node, no longer acting as the neighbouring cells and remove the cell(s) from the NCRT. If there is any NG-RAN node which has already set up Xn connection with the WAB-gNB no longer serves the neighbour cell of the WAB-gNB, the WAB-gNB can initiate Xn removal to the NG-RAN node.

Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-7: WAB-gNB removes Xn with the with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

2.2 Additional ULI

According to SA2’s conclusion [3], indication of an additional ULI from the WAB-gNB to the AMF for UE is required. One LS [4] was sent from SA2 for asking RAN to discuss the issue of additional ULI and feedback:

S2-2407345 Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

According to clause 8.5 of TR 23.700-06 [3], SA2 has some progress on the issue of additional ULI. It was agreed that TAC/Cell ID the cell serving WAB-MT is reported as the additional ULI of UE when the PLMN serving the WAB-MT and that serving the UE is the same, whereas, 3 potential options are listed for the content of additional ULI in case the PLMN serving the WAB-MT and the PLMN serving the UE are not the same PLMN.

clause 8.5 of TR 23.700-06 - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative in phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request. NOTE: If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

In Rel-18, due to the cell ID/TAC broadcasted by the mobile IAB cell may kept unchanged during the IAB-node’s movement over a long distance, it may not reflect the real location of UE. So, there is problem to support the services that use cell ID/TAC as location reference, e.g., regulatory services. In Rel-18, the IAB-donor-CU can provide an Additional ULI which includes TAI/NCGI of the cell serving the mIAB-MT in addition to the ULI of UE, then the AMF can determine UE’s location according to the additional ULI.

The same problem is applied to WAB, i.e., the cell ID/TAC of the WAB-node cell may not be able to reflect the real location of UE. The method of reporting the additional ULI in mobile IAB can be reused for WAB. However, the AMF of UE may not recognize the cell ID/TAC of a PLMN which is not the PLMN of UE, that the AMF would not be able to derive the UE’s location if the additional ULI pertains to cell ID/TAC of a different PLMN to the UE’s PLMN. Above 3 options are presented to address this issue.

On option 1, additional ULI indlcudes the mapped ULI information. OAM can provide the WAB-gNB the configuration of mapped TAC/Cell ID according to different geographic locations.

On option 2, additional ULI includes geolocation of WAB-node. NGAP has to introduce the new IE of WAB-node’s geolocation in UE’s ULI.

Option 3 is workable from RAN3 perspective, but it’s not clear how does AMF of a PLMN identify the cell ID of other PLMNs. The normative work is same with option 1.

From RAN3's point of view, all 3 options are feasible. It’s up to SA2 to determine which option to adopt.

Proposal 2-1: All three options for the WAB node providing additional ULI are feasible, it's up to SA2 to determine which option to use.

Proposal 2-2: RAN3 send response LS to SA2 on additional ULI as Annex B of [5].

2.3 Resource multiplexing

Based on the WID approved during RAN#105 [2], radio-resource coordination between access and backhaul links should be specified. According to the TR [1] from last meeting, the resource coordination mechanism introduced for IAB can be considered as the starting point to resource multiplexing for WAB. However, the TR does not specify what information to reuse from the resource coordination mechanism introduced for IAB. The issue needs to be refined.

Observation 1: According to the TR [1], it’s not clear what information of IAB is reused for WAB on resource multiplexing.

Resource multiplexing for IAB is specified in Rel-16/17. The transmission of each IAB-DU’s cell depends on resource configuration or implicit determination to make sure the access link will have no impact to the BH link. The same principle should be applied to WAB, that DL/UL transmission of each WAB-gNB’s cell should be constrained for potential impacts to transmission over the BH link.

Each slot is assigned to be hard, soft, or unavailable for D/U/F symbols for IAB-DU’s cell. When a symbol is configured as hard, the IAB-DU can use it for schedule. When a symbol is configured to be unavailable, the IAB-DU cannot use it except in some special cases. When a symbol is configured as soft, the IAB-DU can conditionally use it in cases where the parent node explicitly indicates availability via DCI format 2_5, or implicitly in cases where the IAB-node determines availability.

Similar with IAB, there can be three types for D/U/F symbols in a slot for WAB-gNB’s cell. The hard symbols can be used by WAB cell, the unavailable symbols cannot be used by WAB cell and the soft symbols can be conditionally used by WAB cell depending on whether it has interference to the backhaul link.

Proposal 3-1: Three types of symbol (i.e., hard, soft and unavailable) can be used for resource multiplex of WAB.

For resource multiplexing, WAB-node can send resource configurations of the WAB-node to the BH-RAN-node. The resource configuration of the WAB-gNB’s cell contains the configuration for slot format and symbol type of each slot. The slot format indicates the number and position of D/U/F symbols.

The BH-RAN-node can also transport resource configurations of its cell(s) to the WAB-node. Since BH-RAN-node is legacy NG-RAN node, the resource configuration of BH-RAN-node’s cell only needs to include the slot format to indicate the number and position of D/U/F symbols. The symbol type of each slot is not necessary.

Observation 2: The resource configuration of WAB-gNB’s cell(s) sent to BH-RAN-node contains slot format and symbol type of each slot. The resource configuration of BH-RAN-node’s cell(s) sent to WAB-gNB contains slot format only.

The multiplexing capability of WAB-node should also be sent to the BH-RAN-node. The multiplexing capability includes simultaneous operation of WAB-MT Rx / WAB-gNB Rx, WAB-MT Tx / WAB-gNB Tx, WAB-MT Rx / WAB-gNB Tx, WAB-MT Tx / WAB-gNB Rx, should also be supported by WAB. If the simultaneous operation of WAB-MT and WAB-gNB is not supported, time multiplexing between access and BH links should be applied, otherwise, frequency multiplexing can be utilized between access and BH links.

Observation 3: The multiplexing capability of WAB-node can be sent to the BH-RAN-node by WAB-node.

To achieve resource multiplexing between access link and backhaul link, resource information for each cell of BH-RAN-node and each cell of WAB-gNB is exchanged between the BH-RAN-node and the WAB-node. Besides that, the WAB node can send the proposed slot format of the BH-RAN-node’s cell to the BH-RAN-node and BH-RAN-node can modify the slot format based on the proposed configuration. The BH-RAN-node can also send the proposed slot format and/or symbol type of each slot for each WAB-gNB’s cell to the WAB node. The WAB node can also modify the slot format and/or symbol type of each slot base on the proposed configuration from the BH-RAN-node.

Observation 4: The WAB-node can send proposed slot format for each BH-RAN-node’s cell to the BH-RAN-node, and the BH-RAN-node can send proposed slot format and/or symbol type of each slot for each WAB-gNB’s cell to the WAB node.

In summary, the information from WAB-node may include the slot format and the symbol type of each slot for each cell, the duplex capability of the WAB node and the proposed slot format of the BH-RAN-node’s cell. And the information from BH-RAN-node may include the slot format for each cell, the proposed slot format and/or symbol type of each slot for each WAB-gNB’s cell.

Proposal 3-2: The resource coordination information of WAB-node includes slot format and symbol type for each WAB-gNB cell, WAB-node duplex capability and proposed slot format for BH-RAN-node’s cell(s). The resource coordination information of BH-RAN-node includes slot format for each cell, proposed slot format and/or proposed symbol type for WAB-gNB’s cell(s).

The resource coordination information can be exchanged between WAB-node and its BH-RAN-node via RRC or XnAP (If Xn interface between BH-RAN-node and WAB-node is established).

As the solution of using RRC signaling can be done without the Xn connection between WAB-node and BH-RAN-node, this solution is less constraint. However, it has large impact to the RRC specification.

Using XnAP signaling to achieve resource coordination is similar with the mechanism of IAB, but the disadvantage is that it relies on the Xn connection between the BH-RAN-node and WAB-gNB. Further, it requires co-location discovery of WAB-MT and WAB-gNB.

Observation 5: If the resource coordination is done by XnAP, the BH-RAN-node needs to discover co-location of the WAB-MT and the WAB-gNB.

However, considering resource multiplexing is optional and Xn is also optional, the Xn solution is feasible. Although co-location discovery is required, limited impact is expected for XnAP. In our view, the resource multiplexing between access and backhaul links can be achieved via XnAP.

Proposal 3-3: Exchanging resource coordination information between BH-RAN-node and WAB-node over Xn.

The identity of WAB-MT is used in RRC with the BH-RAN-node and the identity of WAB-gNB is used in XnAP with the BH-RAN-node. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB by either the identity of WAB-gNB or the identity of WAB-MT. According to the WAB-node integration procedure, the WAB-MT of a WAB-node connects to the network in the way as a UE by performing RRC connection setup procedure with the BH-RAN-node in phase 1. The WAB-gNB establishes Xn connection towards the BH-RAN-node in phase 2-3.

There are two options for co-location discovery:

- Option 1: WAB-MT reporting identity of WAB-gNB.

The WAB-MT reports the gNB ID to the BH-RAN-node via RRC message, e.g., during phase 1 for WAB-MT setup. The BH-RAN-node can discover the WAB-MT is co-located with the WAB-gNB through the gNB ID, e.g., when the WAB-gNB sets up Xn interface with it during phase 2-3.

- Option 2: WAB-gNB reporting identity of WAB-MT.

The BH-RAN-node allocates C-RNTI in the servicing cell for WAB-MT in phase 1. WAB-gNB sends the C-RNTI and the cell identity of the serving cell of WAB-MT to the BH-RAN-node via XnAP signaling, e.g., during the Xn setup procedure in phase 2-3. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB via the C-RNTI and cell identity.

Comparing the two options, option 2 has an advantage over option 1 on avoiding the impact to RRC. It is proposed to adopt option 2 for co-location discovery.

Proposal 3-4: BH-RAN-node discovers the co-location of WAB-gNB WAB-MT via the WAB-gNB reporting the identity of the WAB-MT over Xn.

3 Conclusion

In the previous sections we made the following proposals:

Xn connection management

Proposal 1-1: WAB-gNB can recognize BH-RAN-node via the SIB1 of the cell serving the WAB-MT.

Proposal 1-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism and set up Xn connection with it.

Proposal 1-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 1-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism and set up Xn connections with surrounding RAN nodes.

Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-7: WAB-gNB removes Xn with the with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

Additional ULI

Proposal 2-1: All three options for the WAB node providing additional ULI are feasible, it's up to SA2 to determine which option to use.

Proposal 2-2: RAN3 send response LS to SA2 on additional ULI as Annex B of [5].

Resource multiplexing

Observation 1: According to the TR [1], it’s not clear what information of IAB is reused for WAB on resource multiplexing.

Proposal 3-1: Three types of symbol (i.e., hard, soft and unavailable) can be used for resource multiplex of WAB.

Observation 2: The resource configuration of WAB-gNB’s cell(s) sent to BH-RAN-node contains slot format and symbol type of each slot. The resource configuration of BH-RAN-node’s cell(s) sent to WAB-gNB contains slot format only.

Observation 3: The multiplexing capability of WAB-node can be sent to the BH-RAN-node by WAB-node.

Observation 4: The WAB-node can send proposed slot format for each BH-RAN-node’s cell to the BH-RAN-node, and the BH-RAN-node can send proposed slot format and/or symbol type of each slot for each WAB-gNB’s cell to the WAB node.

Proposal 3-2: The resource coordination information of WAB-node includes slot format and symbol type for each WAB-gNB cell, WAB-node duplex capability and proposed slot format for BH-RAN-node’s cell(s). The resource coordination information of BH-RAN-node includes slot format for each cell, proposed slot format and/or proposed symbol type for WAB-gNB’s cell(s).

Observation 5: If the resource coordination is done by XnAP, the BH-RAN-node needs to discover co-location of the WAB-MT and the WAB-gNB.

Proposal 3-3: Exchanging resource coordination information between BH-RAN-node and WAB-node over Xn.

Proposal 3-4: BH-RAN-node discovers the co-location of WAB-gNB WAB-MT via the WAB-gNB reporting the identity of the WAB-MT over Xn.

4 Reference

- R3-244849, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v1.2.0 (2024-08)

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- 3GPP TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v 19.0.0 (2024-06)

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2#163

- R3-245253, Discussion on stage-2 aspects for WAB, CATT, RAN3#125bis