3GPP TSG RAN WG3 #125bis		R3-245263

Hefei, China, October 14th – 18th, 2024

Title:	Discussion on LTM MRO

Agenda item:	10.2

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN3#125 meeting, there were some agreements and FFSs for LTM MRO.

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

RAN3#123bis meeting, it was agreed to study the near failure LTM:

Work on scenarios of near failure LTM

In this paper, we further discuss the remaining issues of LTM MRO and give our considerations.

# 2 Discussion

## LTM RLF Report

During LTM cell switch procedure, the CU prepares the LTM candidate cells, and the source DU selects the target LTM cell for the UE. If a connection failure occurs during LTM procedure, the UE selects a cell for reestablishment attempt. To assist the NW for robustness of the LTM procedure, the UE provides the failure information in the RLF report to the NW.

To address the failures, the NW needs to perform the following analysis:

- the failure scenario analysis: identifying whether the connection failure is due to inappropriate LTM cell switch triggering;

- failure type detection analysis: determining the type of LTM failure, e.g., too late LTM, too early LTM or LTM to wrong cell

- root cause analysis: analysing the underlying reason or sub-optimal parameters for the LTM failure.

As agreed in RAN#125 meeting, it is an intuitive idea that the failure scenario analysis should be performed at CU side.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

For the connection failure during L3 mobility procedure, it is the failure node to perform the failure type detection analysis. For LTM procedure, it involves DUs as the source and target nodes. It may bring too much complexity, e.g., signalling among CU, source DU and target DU, to following the previous principle for the failure type detection. One alternative solution is that the CU performs the failure type detection analysis and identifies the LTM failure type for the connection failure initiating the RLF report. In this way, the handover report kind of procedure between the failure DU and the source DU can be definitely avoided.

Proposal 1: CU performs LTM failure type detection analysis for failures due to inappropriate cell switch triggering by DU.

Currently, there are three failure types for LTM, including too late LTM, too early LTM and LTM to wrong cell. As described in TS38.300, the node determines the failure type by comparing the UE reported time with the configured threshold (e.g., Tstore_UE_cntxt). To accurately detect the failure type at CU side, it is necessary for CU to get the configured threshold parameter. Therefore, the OAM should provide the parameter Tstore_UE_cntxt to CU to facilitate accurate LTM failure analysis.

Proposal 2: OAM provides the parameter Tstore_UE_cntxt to CU for LTM failure type detection.

After determining the LTM failure type, the CU forwards the RLF report to the appropriate DU for root cause analysis. For the too late LTM, the CU forwards the RLF report to the failure DU controlling the cell where the sub-optimal LTM configuration resulted in the RLF. For too early LTM or LTM to wrong cell, the CU forwards the RLF report to the source DU, which inappropriately initiated the LTM cell switch procedure.

Proposal 3: CU forwards RLF report to the correct DU based on LTM failure type:

- The failure DU in case of too late LTM

- The source DU in case of too early LTM, or LTM to wrong cell

To aid the root cause analysis during L3 mobility procedure, the node performing initial analysis will include additional information alongside the RLF report to the right node responsible for the connection failure. The additional information can include handover report type and other information.

The similar principle can be considered for CU to provide the additional information to DU. Correspondingly, the CU needs to provide the LTM report type to assist the root cause analysis at DU. The LTM report type can indicate the LTM failure type determined at CU, e.g., too late LTM, too early LTM or LTM to wrong cell.

For other additional information, it can be derived from the RLF report. There is no need to provide redundant information already present in the RLF report container.

Proposal 4: CU provides the LTM failure type to DU as the additional information.

The receiving DU can then perform a root cause analysis based on the RLF report and the LTM failure type. This analysis can help optimize LTM trigger parameters and prevent future failures.

Proposal 5: DU performs root cause analysis to optimize the LTM trigger related parameters

## LTM SHR

In RAN3#123bis meeting, it was agreed on the study of near failure LTM:

Work on scenarios of near failure LTM

For previous SHR of L3 mobility, timer-based triggering conditions are introduced, including the timers T304, T310 and T312. All of the timers are still used as before during LTM procedure. It is straightforward to reuse all the above timer-based triggering conditions for LTE SHR determination.

Proposal 6: Reuse timers T304, T310 and T312 related triggering conditions for LTM SHR.

When CU receives the LTM SHR, it performs initial analysis and identifies the triggering condition(s) which triggered the SHR. Correspondingly, the CU can decide the DU responsible for the near failure LTM procedure. For example, if the LTM SHR was triggered based on the T304 related triggering condition, the CU forwards the LTM SHR to the target DU; while for T310 or T312 related triggering conditions, the CU forwards the LTM SHR to the source DU.

Furthermore, the CU forwards the LTM SHR to the DU based on the triggering condition(s).

Proposal 7: CU forwards the LTM SHR to DU based on triggering conditions for LTM SHR:

- Source DU in case of T310 or T312 related triggering conditions

- Target DU in case of T304 related triggering condition

# 3 Conclusion

In this contribution, we discussed the potential solutions for LTM MRO. The related proposals are summarized below:

LTM RLF report:

Proposal 1: CU performs LTM failure type detection analysis for failures due to inappropriate cell switch triggering by DU.

Proposal 2: OAM provides the parameter Tstore_UE_cntxt to CU for LTM failure type detection.

Proposal 3: CU forwards RLF report to the correct DU based on LTM failure type:

- The failure DU in case of too late LTM

- The source DU in case of too early LTM, or LTM to wrong cell

Proposal 4: CU provides the LTM failure type to DU as the additional information.

Proposal 5: DU performs root cause analysis to optimize the LTM trigger related parameters

LTM SHR:

Proposal 6: Reuse timers T304, T310 and T312 related triggering conditions for LTM SHR.

Proposal 7: CU forwards the LTM SHR to DU based on triggering conditions for LTM SHR:

- Source DU in case of T310 or T312 related triggering conditions

- Target DU in case of T304 related triggering condition