3GPP RAN WG3 Meeting #125-bis	R3-245275

# Hefei, China, 14th – 18th October 2024

Source:	CATT

Title:	Potential RAN3 impacts of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and decision

- Introduction

This document analyses some potential RAN3 impacts as quite many topics depends on the output of other WGs.

- Discussion

  - UE paging subgroup ID

For CN-assigned subgroup ID, RAN3 discussed last meeting and ever proposed that the “PEI information elements over RAN3 protocols should preferably not be reused”, but finally the agreement was written as:

[Agreement] New IEs should be introduced to support LP-WUS on NG, Xn and F1.

We interpret this agreement intends to express that the IEs introduced for PEI should not be reused. Nevertheless, the details still depends on the input from other WGs.

For the used range of CN-assigned subgroup IDs, we prefer aligning with PEI, i.e. configured by the OAM.

Proposal 1: The used range of CN-assigned subgroup IDs should be configured by the OAM, without any impact on RAN3 specifications.

For UE-ID-based subgroup ID, the only thing which can be determined in RAN3 now is the UE capability: this is part of UE paging capability which is delivered within containers in NGAP and XnAP, but delivered as separate RAN3-flavour IEs in F1AP. Therefore a new IE indicating the UE’s capability on LP-WUS subgrouping should be introduced in F1AP. We consider this to be a per-UE capability so adding it into the “9.3.1.270 UE Paging Capability” sufficient enough, rather than adding it as per-UE per-cell like the one for PEI.

Proposal 2: An IE indicating the UE’s support for LP-WUS subgrouping should be added into the “9.3.1.270 UE Paging Capability” of TS 38.473.

  - System Information

Configuration of LP-WUS (except for CN-based subgroup ID) is provided to the UE by the system information. It will not cost many bits and is likely included in SIB1. The content of it is assumed by RAN2 that:

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

-	LP-SS configuration

-	LP-WUS configuration

-	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

The first two bullets indicate the configuration of the signal itself and should of course be decided by the gNB-DU. The last bullet, on the other side, may be more suitable to be configured by the gNB-DU (as many conditions for measurements or cell reselection).

Therefore we may assume now that the entry/exit condition for LP-WUS should be delivered from the gNB-CU (where it is determined) to the gNB-DU (where SIB1 is encoded).

Proposal 3: To assume that the entry/exit condition for LP-WUS monitoring should be delivered as separate F1AP IEs from the gNB-CU to the gNB-DU. This may be revisited upon further RAN2 discussion.

  - Emergency service

A UE using LP-WUS will experience significant extra paging delay compared with UEs not monitoring only conventional (Rel-15) POs. This may be suboptimal for emergency service. If there is an (already established) emergency session between the UE and the network, and the UE is currently not in RRC_CONNECTED state, there may be some benefit to prevent the UE from using LP-WUS.

The case is simple at the UE side: A UE knows whether there is an established emergency session (by checking e.g. its own NAS context), and if there is, the UE may simply act as if it does not support LP-WUS for shorter paging delay.

The case in CN is simple as well: The only impact on CN is assigning the CN-assigned subgroup ID. As long as the UE does not indicate its support of LP-WUS, the CN will not assign—the same method as in PEI.

The case in RAN, however, is not so easy, since RAN is not provided with necessary information whether there is an emergency session for a UE. There are a few possible attitudes toward this issue:

- Simply ignores the drawback. The delay caused by LP-WUS is probably within 1000ms (waiting for RAN1 to decide), comparable with the delay caused by PO periodicity (and occasionally missing a couple of PO when monitoring). This may be acceptable.

- gNB begins to send Uu paging from the first available PO after it knows it shall page, rather than the first available PO after LP-WUS wake up period. Not only the UE with established emergency session but also UEs not monitoring LP-WUS due to other reason (e.g. out of enter condition) will benefit from such implementation. (Note that RAN2 has agreed that the gNB is assumed to be unaware of whether a UE monitors LP-WUS.)

- Assume that emergency session always come with the highest paging priority (an IE within NGAP paging message and/or NG-U DL packet). The gNB begins to send Uu paging from the first available PO for the highest paging priority.

- The UE updates its Uu capability (by UE Assistance Information maybe) as “not supporting LP-WUS” once an emergency session is established, fully aligned with the method in NAS. The gNB will of course act as legacy since it considers the UE not supporting LP-WUS.

- The core network indicates that a session is an emergency one, or an NG paging is triggered for an emergency session, in the corresponding NGAP messages.

The fourth attitude requires adequate design in RAN2, whereas the last attitude requires enhancement on RAN3 stage-3 specification—other attitudes are fully implementation-based and independent with each other (i.e. there would not be any extra issue if one gNB uses one implementation and another gNB uses another). We propose RAN3 to discuss this issue.

Proposal 4: To discuss the issue of emergency service handling based on the five attitudes listed above.

- Conclusion

Proposal 1: The used range of CN-assigned subgroup IDs should be configured by the OAM, without any impact on RAN3 specifications.

Proposal 2: An IE indicating the UE’s support for LP-WUS subgrouping should be added into the “9.3.1.270 UE Paging Capability” of TS 38.473.

Proposal 3: To assume that the entry/exit condition for LP-WUS monitoring should be delivered as separate F1AP IEs from the gNB-CU to the gNB-DU. This may be revisited upon further RAN2 discussion.

Proposal 4: To discuss the issue of emergency service handling based on the five attitudes listed above.

One TP is provided in [1] accordingly.

- Reference

[1] R3-245276; TP on TS 38.300 for LP-WUS; CATT.